3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230348

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3066 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcases 8.2.4.2.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 7 – 8, the parameter to change the current PSCell (NR Cell 2) to make it a SCell is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the 4th parameter of function f_NR_RRC_PSCellChangeCA_InterCell to p_PSCell2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2.1.1.ENDC, 8.2.4.2.1.2.ENDC, 8.2.4.2.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_TC_8_2_4_2_1_x_ENDC_NR_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_8_2_4_2_1_x_ENDC_NR_TestBody

| Function name | fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | At Step 7 – 8, the parameter to change the current PSCell (NR Cell 2) to make it a SCell is missing |
| Summary of change | Set the 4th parameter of function f_NR_RRC_PSCellChangeCA_InterCell to p_PSCell2 |
| TTCN module | RRC_CarrierAggregation_ENDC_NR.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. The Source SCell is reconfigured to a normal cell within f_NR_ReconfigSCellToNormalCell, so there is no need to reconfigure it as a PSCell inside f_NR_RRC_PSCellChangeCA_InterCell. |

Before Change:

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? //@sic R5s220809 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_ReconfigSCellToNormalCell(p_SCell2, v_RadioBearerList); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, omit, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //@sic R5s221199 sic@ }

After Change:

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? //@sic R5s220809 sic@ v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_ReconfigSCellToNormalCell(p_SCell2, v_RadioBearerList); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, p_PSCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //WA# //@sic R5s221199 sic@ }

MCC160 proposed implementation

function f_NR_RRC_PSCellChangeCA_InterCell(NR_CellId_Type                       p_SourcePSCellId,

NR_CellId_Type                       p_SourceSCellId,

NR_CellId_Type                       p_TargetPSCellId,

template (omit) NR_CellId_Type       p_TargetSCellId,

boolean                              p_RBConfig_KeyChange := true,

template (omit) RadioBearerConfig    p_RadioBearerConfig,

template (value) CellGroupConfig     p_SCGConfig,

template (value) NR_RadioBearerList_Type  p_RadioBearerToReleaseList

) runs on NR_BASE_PTC

{ //@sic R5s230169 p_TargetSCellId updated to template(omit) sic@

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := f_NR_GetRACHProcedureConfigPSCell( p_TargetPSCellId, -);

var NR_CellIdList_Type v_SCellList := {};   //@sic R5s230169 sic@

// Steps 1 to 6

f_NR_RRC_PSCellChange_InterCell_Steps1To6(p_SourcePSCellId,

p_TargetPSCellId,

-, //tsc_C_RNTI_Value3

p_RBConfig_KeyChange,

false, //p_IsSrb3Configured,

v_RachProcedureConfig );

//Step 7. NR Target PSCell: Configure Target PSCell as PSCell.

// NOTE 5:  The FollowOnFlag is set to true in the ASP reconfiguring PSCell.

if (isvalue (p_TargetSCellId)) {      //@sic R5s230169 sic@

v_SCellList := {valueof(p_TargetSCellId)};

f_NR_ConfigPCell_AddRelSCells(p_TargetPSCellId, v_SCellList, tsc_PCellIndex_1, cs_TimingInfo_Now, tsc_C_RNTI_Value3, -, tsc_NoCnfReq, tsc_FollowOnFlag );    //@sic R5-206369 sic@

}

f_NR_ConfigPCell_AddRelSCells(p_TargetPSCellId, v_SCellList, tsc_PCellIndex_1, cs_TimingInfo_Now, tsc_C_RNTI_Value3, -, tsc_NoCnfReq, tsc_FollowOnFlag );    //@sic R5-206369 sic@

//Step 8. NR Target SCell: Configure Target SCell as SCell with new PSCell association.

if (  ispresent(p_SCGConfig.sCellToAddModList) and isvalue(p_TargetSCellId)) {  //@sic R5s220809 change 2 sic@

// In step 7 there is no SCell

f_NR_ConfigSCell(valueof(p_TargetSCellId), p_SCGConfig, cs_TimingInfo_Now);

}

// Step 9. E-UTRA Cell: Send RRCConnectionReconfiguration.

// Step 10. E-UTRA Cell: Receive RRCConnectionReconfigurationComplete.

f_NR_SendRRCReconfigurationContentsToEUTRA(p_RadioBearerConfig, p_SCGConfig, -, -, -, reconfigurationWithSync_CheckRACH, p_TargetPSCellId); //@sic R5s221199 R5s230131 sic@

// Step 11. NR Target PSCell: Inform the SS about completion of the PSCell change (e.g. to trigger PDCP STATUS REPORT PDU).

f_NR_SS_PdcpHandoverCtrl(p_TargetPSCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetPSCellId));

// Step 12. NR Source PSCell: If source PSCell is not the same as target SCell, Release SRB3 (if necessary) and DRBs, reconfigure the cell as a normal cell.

if (isvalue (p_TargetSCellId)) {

if (p_SourcePSCellId != valueof(p_TargetSCellId)) { //@sic R5-204475 sic@

f_NR_SS_CommonRadioBearerConfig(p_SourcePSCellId, p_RadioBearerToReleaseList);

}

}

//Step 13. NR Source SCell: If source SCell is not the same as target SCell or target PSCell, reconfigure the cell as a normal cell

//NOTE 6:   The FollowOnFlag is set to true in the ASP reconfiguring PSCell

if (p_SourceSCellId != p_TargetPSCellId) {

f_NR_Rel_SCell_Common(p_SourcePSCellId, p_SourceSCellId, tsc_PCellIndex_1, -, -, tsc_NoCnfReq, tsc_FollowOnFlag );   //@sic R5-206369 sic@

}

}