3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230350

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3067 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 11.4.14 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 11.4.14 to the IWD_23wk12 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.4.14 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.14.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 11.4.14	4

2.1	f_TC_11_4_14_NR5GC_IMS2	4

3	Branches executed	6

4	Execution Log Files	6

4.1	Qualcomm	6

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.4.14 which is part of the NR5GC test suite in iWD_TTCN3-B2022-09_D23wk12.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.14.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	R&S® 5G Protocol Conformance Test platform, Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm Snapdragon® X75 5G Modem-RF System , Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution 8450,SDX65 UE

Verification Status:	PASS

# Corrections required to test case 11.4.14

## f_TC_11_4_14_NR5GC_IMS2

| Function name | f_TC_11_4_14_NR5GC_IMS2 |
| --- | --- |
| Reason for change | In the current TTCN implementation, emergency speech call setup is done as per context condition A_2_1_A6 in which INVITE for creating an emergency session in case of no registration but according to test specification context condition shall be A_2_1_A7 INVITE for creating an emergency session within an emergency. This needs to be corrected |
| Summary of change | In the function f_IMS_EmergencyCallSetup_NR5GC_A_6 passed the condition A_2_1_A7. |
| TTCN module | Emergency_NR5GC_IMS. ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_11_4_14_NR5GC_IMS2()runs on IMS_PTC { //5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call /Deregistration upon emergency registration expiration //@sic R5-227446 sic@ var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyRegistration(-, "30000", IPCAN_E); f_IMS_EmergencyCallSetup_NR5GC_A_6 (A_2_1_A6); // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Emergency Call Established")); f_IMS_IPCAN_WaitForTrigger(IPCAN, "Trigger MT Call Release"); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); //Tell NR that call is released f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); }

After Change:

function f_TC_11_4_14_NR5GC_IMS2()runs on IMS_PTC { //5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call /Deregistration upon emergency registration expiration //@sic R5-227446 sic@ var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyRegistration(-, "30000", IPCAN_E); f_IMS_EmergencyCallSetup_NR5GC_A_6 (A_2_1_A7); // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Emergency Call Established")); f_IMS_IPCAN_WaitForTrigger(IPCAN, "Trigger MT Call Release"); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); //Tell NR that call is released f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA2 Integrity and NEA2 ciphering algorithms on R&S 5G Conformance Test platform, and on NR primary band n41 using NIA1 integrity and NEA1 ciphering algorithms on Keysight S8704A Protocol Conformance Toolset.

# Execution Log Files

## Qualcomm

The Qualcomm Snapdragon® X75 5G Modem-RF System , Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution 8450,SDX65 UE  passed this test case on R&S® 5G Protocol Conformance Test platform and Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

R&S :
tc_11_4_14_QC.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

Keysight :

TC_11_4_14_LOG.html.xml

TC_11_4_14_PICS_PIXIT.xml

# References

| [1] | R5s230351: Supporting information for addition of NR5GC testcase 11.4.14 in FR1 |
| --- | --- |