3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230354

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3070 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Step 5A-5B the Harq Ack indication is not disabled before exiting function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback. Keeping Harq Ack indication enabled for p_SourceCellId after its use would interfere later flow of test steps, because the TTCN would keep expecting the HARQ indications on SYS.IND, and will fail if something else is received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that Harq ACK indication should be disabled before leaving the function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.1.NR5GC, 8.1.4.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

2.1	Function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback

| Function name | f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback |
| --- | --- |
| Reason for change | In Step 5A-5B the Harq Ack indication is not disabled before exiting function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback. Keeping Harq Ack indication enabled for p_SourceCellId after its use would interfere later flow of test steps, because the TTCN would keep expecting the HARQ indications on SYS.IND, and will fail if something else is received. |
| Summary of change | It is proposed that Harq ACK indication should be disabled before leaving the function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback. |
| TTCN module | NR5GC_Handover.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(NR_CellId_Type p_SourceCellId, charstring p_StepNo := "", NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC //@sic R5s221151 sic@ { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var IntegerList_Type v_DrbList; var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 5.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_DrbList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionInfoList[0].SessionId)); // Get list of DRBs for the first PDU session v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DrbList[0]); //Get the QosFlow ID information from the DRB ID // Configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_SourceCellId, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Step 14A. Source Cell: Send one IP packet on the DAPS DRB (first DRB of the first PDU session) DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(p_SourceCellId, v_QosFlowId, {v_IPData})); interleave { // Chg 2 - R5s230320 // Step 14B. Source Cell: Check that the UE transmit a positive HARQ feedback [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_SourceCellId, ack)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_StepNo); } // Step 14C. Target Cell: Receive loopbacked IP packet [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(p_TargetCellId, v_QosFlowId, {v_IPData})) {} //@sic R5s221151 sic@ } t_Watchdog.stop; }

After Change:

function f_NR5GC_DAPS_HO_CheckDataPath_HARQFeedback(NR_CellId_Type p_SourceCellId, charstring p_StepNo := "", NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC //@sic R5s221151 sic@ { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var IntegerList_Type v_DrbList; var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 5.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_DrbList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionInfoList[0].SessionId)); // Get list of DRBs for the first PDU session v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DrbList[0]); //Get the QosFlow ID information from the DRB ID // Configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_SourceCellId, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Step 14A. Source Cell: Send one IP packet on the DAPS DRB (first DRB of the first PDU session) DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(p_SourceCellId, v_QosFlowId, {v_IPData})); interleave { // Chg 2 - R5s230320 // Step 14B. Source Cell: Check that the UE transmit a positive HARQ feedback [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_SourceCellId, ack)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_StepNo); } // Step 14C. Target Cell: Receive loopbacked IP packet [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(p_TargetCellId, v_QosFlowId, {v_IPData})) {} //@sic R5s221151 sic@ } f_NR_SS_SystemIndCtrlConfig(p_SourceCellId, cds_NR_SystemIndCtrl_UL_HARQ(disable)); //WA#WA=1083527 t_Watchdog.stop; }