3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230376

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3082 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation at Step 25 the SIB1 is updated to include accessbarring information for NR Cell 1, and then immediately after this the NR Cell 1 is set to serving cell power (Step 26). This may result in the UE detecting the NR Cell 1 and reading the old SIB1, which does not contain access barring. This will cause testcase failure at Step 34 because the UE will trigger the IMS video Call (Step 33). This needs to be addressed. In the current TTCN implementation, at Step 33, the user is prompted to make a video call, and at Step 34 it is checked for 6s that the UE does not send a rrcSetupREQ for initiating the video call. In case the testcse is run manually (This is a special USIM testcase), then the user would 1) Press OK for the prompt to initiate a video call and, 2) Use AT command and proprietary mechanism to initiate video call on UE side. These actions take togather can easily take 10s, and by this time the check at Step 34 will have expired. Thus, there is scope for a non-conformant UE to slip through this negative check. This needs to be addressed In the current TTCN imlpementation, at Step 10B and 10Eb3, tsc_DNNValue is used as DNN in the call f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); However, this DNN is not passed on while expecting the PDU Session Establishment REQ in Steps 10Fa1-10Fa8 when function f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB2); is called. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After the call f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1111111'B)));, called functions f_NR_ModifySysinfo(nr_Cell1, false); and f_NR_WaitModificationPeriods(nr_Cell1);. A Prose CR will be raised for this. At Step 34, called f_NR_CheckNoRRCSetupReq(nr_Cell1, 30.0); instead of f_NR_CheckNoRRCSetupReq(nr_Cell1, 6.0); A Prose CR will be raised for this. Initialized a new variable v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); and then used this in call to f_NR5GC_PDUSessionEstablishment for Steps 10Fa1-10Fa8 Please see below for detailed changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required to test case 11.3.5	3

2.1	fl_TC_11_3_5_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 11.3.5

## fl_TC_11_3_5_TestBody

| Template name | fl_TC_11_3_5_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation at Step 25 the SIB1 is updated to include accessbarring information for NR Cell 1, and then immediately after this the NR Cell 1 is set to serving cell power (Step 26). This may result in the UE detecting the NR Cell 1 and reading the old SIB1, which does not contain access barring. This will cause testcase failure at Step 34 because the UE will trigger the IMS video Call (Step 33). This needs to be addressed. In the current TTCN implementation, at Step 33, the user is prompted to make a video call, and at Step 34 it is checked for 6s that the UE does not send a rrcSetupREQ for initiating the video call. In case the testcse is run manually (This is a special USIM testcase), then the user would 1) Press OK for the prompt to initiate a video call and, 2) Use AT command and proprietary mechanism to initiate video call on UE side. These actions take togather can easily take 10s, and by this time the check at Step 34 will have expired. Thus, there is scope for a non-conformant UE to slip through this negative check. This needs to be addressed In the current TTCN imlpementation, at Step 10B and 10Eb3, tsc_DNNValue is used as DNN in the call f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); However, this DNN is not passed on while expecting the PDU Session Establishment REQ in Steps 10Fa1-10Fa8 when function f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB2); is called. This needs to be addressed. |
| Summary of change | After the call f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1111111'B)));, called functions f_NR_ModifySysinfo(nr_Cell1, false); and f_NR_WaitModificationPeriods(nr_Cell1);. A Prose CR will be raised for this. At Step 34, called f_NR_CheckNoRRCSetupReq(nr_Cell1, 30.0); instead of f_NR_CheckNoRRCSetupReq(nr_Cell1, 6.0); A Prose CR will be raised for this. Initialized a new variable v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); and then used this in call to f_NR5GC_PDUSessionEstablishment for Steps 10Fa1-10Fa8 Please see below for detailed changes. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | 1. Not needed. Function f_NR_SS_ConfigureSysinfo can be used to update SysInfo immediately, not needed to wait for 2.1 MPs. Draft Prose CR must be updated. 2. Accepted subject to Prose CR agreement 3. Accepted |

Before Change:

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var TypeOfRegistration_Type v_TypeOfRegistration; var float v_TimerValue1 := 10.0; timer t_WaitTime; timer t_Wait; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, -80, -80) // Power level for FR2 is FFS }; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell1, -80, -80), // Power level for FR2 is FFS cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 1A" siclog@ //Void @sic R5-223436 sic@ //@siclog "Step 1B" siclog@ //Void @sic R5-223436 sic@ //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message on NR cell 12 including the establishmentCause which is any value except mps-PriorityAccess. SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp; if(match(v_ReceivedAsp.Signalling.Rrc.Ccch.message_.c1.rrcSetupRequest.rrcSetupRequest.establishmentCause, mps_PriorityAccess)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Steps 2A- 2B" siclog@ //Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 12 // @sic R5-223436 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def(nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); //@siclog "Steps 2Ca1-2Ca15" siclog@ //IF 5GS registration type is set as Initial Registration in step 2B, THEN Steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed without RRC release.on NR Cell 12 // @sic R5-223436 sic@ // @sic R5s230200 sic@ select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) { case (tsc_NG_RegistrationInitial) { //@siclog "Steps 2Ca1-2Ca15" siclog@ //Perform steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, noRrcConnectionRelease, Initial_Secure); } case (tsc_NG_RegistrationMobility) { //@siclog "2Cb1-2Cb2" siclog@ //Perform steps 4 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); } case else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect"); } } //@siclog "Steps 3 - 10" siclog@ //Void @sic R5-223436 sic@ //@siclog "Steps 10A" siclog@ // @sic R5-223436 sic@ f_NR_RRCRelease(nr_Cell12); //@siclog "Steps 10B" siclog@ // @sic R5-223436 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Steps 10C" siclog@ // @sic R5-223436 sic@ t_WaitTime.start(21.0); alt { //Check: Does the UE transmit an RRCSetupRequest message within 21s on NR Cell 12? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(?, ?))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10C"); } [] t_WaitTime.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10C"); } } //@siclog "Steps 10D" siclog@ // @sic R5-223436 sic@ //SS changes SIB1 of NR Cell 12 according to Table 11.3.5.3.3-2 and sends Short Message on PDCCH using P-RNTI. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12, omit); f_NR_ModifySysinfo(nr_Cell12); //@siclog "Steps 10E" siclog@ // @sic R5-223436 sic@ t_WaitTime.start(28.0); alt { //@siclog "Steps 10Ea1" siclog@ // @sic R5-223436 sic@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mo_Data? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data, ?))) //@sic R5s221172 sic@ { //@siclog "Steps 10Ea2" siclog@ // @sic R5-223436 sic@ t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10Ea1"); } //@siclog "Steps 10Eb1" siclog@ // @sic R5-223436 sic@ [] t_WaitTime.timeout { //@siclog "Steps 10Eb2" siclog@ // @sic R5-223436 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); t_Wait.start(10.0); alt { //@siclog "Steps 10Eb3" siclog@ // @sic R5-223436 sic@ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data, ?))) { t_Wait.stop; // @sic R5s230200 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10Eb3"); } []t_Wait.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10Eb3"); } } } } //@siclog "Steps 10Fa1-10Fa8" siclog@ // @sic R5-223436 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { // @sic R5s230103 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12,-,tsc_NR_RbId_SRB2); // @sic R5s230200 sic@ //@siclog "Steps 10H" siclog@ // @sic R5-223436 sic@ //The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4] f_NR_UE_DeRegisterOnSwitchOff(nr_Cell12, STATE_CONNECTED_3A); // Tell IMS PTC to stop waiting for De-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s230200 sic@ f_NR5GC_MobileInfo_Init(); //@siclog "Steps 11A" siclog@ // @sic R5-223436 sic@ f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Step 11B" siclog@ // @sic R5-223436 sic@ //The UE is Switched ON. The generic test procedure in TS 38.508-1 [4] Table 4.5.2.2-2 indicate that the UE performs registration on NR Cell 1. f_NR5GC_SwitchOnAndResetIMS(); f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); //@siclog "Step 11C" siclog@ // @sic R5-223436 sic@ f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 11D- 11F" siclog@ // @sic R5-223436 sic@ //Steps 2-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 12. (Note 2) v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); // @sic R5s230200 sic@ select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) { case (tsc_NG_RegistrationInitial) { //Perform steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 11Fa1-11Fa16a1" siclog@ //IF 5GS registration type is set as Initial Registration in step 11F, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 12. //f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_Secure, NOT_SUPPORTED); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, cds_NG_NetworkFeatureSupport_mpsi_NoEMC); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 15 f_NR5GC_Registration_Complete(nr_Cell12); } case (tsc_NG_RegistrationMobility) { //Perform steps 4 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 11Fb1-11Fb3a1" siclog@ //IF 5GS registration type is set as Mobility Registration in step 211F, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 12. // @sic R5-223436 sic@ //f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, -); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, cds_NG_NetworkFeatureSupport_mpsi_NoEMC); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_Registration_Complete(nr_Cell12); // No SRB2 so must be on SRB1 } case else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect"); } } f_NR_RRCRelease(nr_Cell12); //@siclog "Step 12 siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell12, RRC_IDLE); //@siclog "Step 14 siclog@ t_WaitTime.start(v_TimerValue1); alt { //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } [] t_WaitTime.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } } //@siclog "Step 15 - 20 siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12 ,v_ReceivedMsg); //@siclog "Step 24 siclog@ f_NR_RRCRelease(nr_Cell12); //Set SIB1 for NR cell 1 according to Table 11.3.5.3.3-1 for Step 25 and 35 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1111111'B))); // @sic R5-223436 sic@ //@siclog "Step 26" siclog@ //The SS adjusts the NR Cells power levels according to row "T2" in table 11.3.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Step 26A - 26C" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) { case (tsc_NG_RegistrationInitial) { v_TypeOfRegistration := Initial_Secure; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_TypeOfRegistration)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 26Da1-26Da16a1" siclog@ //IF 5GS registration type is set as Initial Registration in step 26C, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } case (tsc_NG_RegistrationMobility) { v_TypeOfRegistration := Mobility; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_TypeOfRegistration)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 26Db1-26Db3a1" siclog@ //IF 5GS registration type is set as Mobility Registration in step 26C, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } case else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect"); } } //@siclog "Step 33" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); //@siclog "Steps 34" siclog@ // Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message? f_NR_CheckNoRRCSetupReq(nr_Cell1, 6.0); //@siclog "Step 35 siclog@ //SS changes SIB1 of NR cell 1 according to Table 11.3.5.3.3-1 and notifies the UE of change of System Information on NR Cell 1 by send Short Message on PDCCH using P-RNTI f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); //@siclog "Step 35A siclog@ t_WaitTime.start(26.0); alt { //@siclog "Step 35Aa1 siclog@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { //@siclog "Step 35Aa2 siclog@ t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Aa1"); } //@siclog "Step 35Ab1 siclog@ [] t_WaitTime.timeout { //@siclog "Step 35Ab2" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); t_Wait.start(10.0); alt { //@siclog "Step 35Ab3" siclog@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_Wait.stop; // @sic R5s230331 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Ab3"); } [] t_Wait.timeout { // @sic R5s230331 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35Ab3"); } } } } //@siclog "Step 36 - 38 siclog@ //Void @sic R5-223436 sic@ //@siclog "Step 39 - 49b3 siclog@ //Steps 3-13b3 of generic test procedure for IMS MO Video call establishment in 5GC as specified in 38.508-1 [4] Table 4.9.xx24.2.2-1 are performed. f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_NR_Cell1, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // Wait until need to add video QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Video Bearer"); // @sic R5-225362 sic@ f_NR5GC_ModifyPDUSession_AddVideo(nr_Cell1, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Video Bearer Added")); // @sic R5-225362 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Video Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Video Call Established"); // @sic R5-225362 sic@ //@siclog "Step 50" siclog@ //The generic test procedure for IMS MT call release in 5GC as specified in TS 38.508-1 [4] Table 4.9.18.2.2-1 takes place f_NR5GC_CallRelease(nr_Cell1, IPCAN_VideoCall, false); // @sic R5-225362 sic@ }

After Change:

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NR_CellPowerList_Type v_CellPowerList_T2; var TypeOfRegistration_Type v_TypeOfRegistration; var template (omit) DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); //WA#11_3_5 var float v_TimerValue1 := 10.0; timer t_WaitTime; timer t_Wait; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, -80, -80) // Power level for FR2 is FFS }; v_CellPowerList_T2 := { cs_NR_CellPower (nr_Cell1, -80, -80), // Power level for FR2 is FFS cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 1A" siclog@ //Void @sic R5-223436 sic@ //@siclog "Step 1B" siclog@ //Void @sic R5-223436 sic@ //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message on NR cell 12 including the establishmentCause which is any value except mps-PriorityAccess. SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp; if(match(v_ReceivedAsp.Signalling.Rrc.Ccch.message_.c1.rrcSetupRequest.rrcSetupRequest.establishmentCause, mps_PriorityAccess)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Steps 2A- 2B" siclog@ //Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 12 // @sic R5-223436 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def(nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); //@siclog "Steps 2Ca1-2Ca15" siclog@ //IF 5GS registration type is set as Initial Registration in step 2B, THEN Steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed without RRC release.on NR Cell 12 // @sic R5-223436 sic@ // @sic R5s230200 sic@ select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) { case (tsc_NG_RegistrationInitial) { //@siclog "Steps 2Ca1-2Ca15" siclog@ //Perform steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, noRrcConnectionRelease, Initial_Secure); } case (tsc_NG_RegistrationMobility) { //@siclog "2Cb1-2Cb2" siclog@ //Perform steps 4 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); } case else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect"); } } //@siclog "Steps 3 - 10" siclog@ //Void @sic R5-223436 sic@ //@siclog "Steps 10A" siclog@ // @sic R5-223436 sic@ f_NR_RRCRelease(nr_Cell12); //@siclog "Steps 10B" siclog@ // @sic R5-223436 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Steps 10C" siclog@ // @sic R5-223436 sic@ t_WaitTime.start(21.0); alt { //Check: Does the UE transmit an RRCSetupRequest message within 21s on NR Cell 12? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(?, ?))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10C"); } [] t_WaitTime.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10C"); } } //@siclog "Steps 10D" siclog@ // @sic R5-223436 sic@ //SS changes SIB1 of NR Cell 12 according to Table 11.3.5.3.3-2 and sends Short Message on PDCCH using P-RNTI. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12, omit); f_NR_ModifySysinfo(nr_Cell12); //@siclog "Steps 10E" siclog@ // @sic R5-223436 sic@ t_WaitTime.start(28.0); alt { //@siclog "Steps 10Ea1" siclog@ // @sic R5-223436 sic@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mo_Data? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data, ?))) //@sic R5s221172 sic@ { //@siclog "Steps 10Ea2" siclog@ // @sic R5-223436 sic@ t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10Ea1"); } //@siclog "Steps 10Eb1" siclog@ // @sic R5-223436 sic@ [] t_WaitTime.timeout { //@siclog "Steps 10Eb2" siclog@ // @sic R5-223436 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); t_Wait.start(10.0); alt { //@siclog "Steps 10Eb3" siclog@ // @sic R5-223436 sic@ [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data, ?))) { t_Wait.stop; // @sic R5s230200 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10Eb3"); } []t_Wait.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10Eb3"); } } } } //@siclog "Steps 10Fa1-10Fa8" siclog@ // @sic R5-223436 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { // @sic R5s230103 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@ f_NR5GC_PDUSessionEstablishment (nr_Cell12, -, tsc_NR_RbId_SRB2, -, - , v_DNN, -, v_DNN); // @sic R5s230200 sic@ //WA#11_3_5 //@siclog "Steps 10H" siclog@ // @sic R5-223436 sic@ //The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4] f_NR_UE_DeRegisterOnSwitchOff(nr_Cell12, STATE_CONNECTED_3A); // Tell IMS PTC to stop waiting for De-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s230200 sic@ f_NR5GC_MobileInfo_Init(); //@siclog "Steps 11A" siclog@ // @sic R5-223436 sic@ f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Step 11B" siclog@ // @sic R5-223436 sic@ //The UE is Switched ON. The generic test procedure in TS 38.508-1 [4] Table 4.5.2.2-2 indicate that the UE performs registration on NR Cell 1. f_NR5GC_SwitchOnAndResetIMS(); f_NR5GC_RRC_IdleState1N_Def(nr_Cell1); //@siclog "Step 11C" siclog@ // @sic R5-223436 sic@ f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 11D- 11F" siclog@ // @sic R5-223436 sic@ //Steps 2-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 12. (Note 2) v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); // @sic R5s230200 sic@ select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) { case (tsc_NG_RegistrationInitial) { //Perform steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 11Fa1-11Fa16a1" siclog@ //IF 5GS registration type is set as Initial Registration in step 11F, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 12. //f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_Secure, NOT_SUPPORTED); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, cds_NG_NetworkFeatureSupport_mpsi_TC1135); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 15 f_NR5GC_Registration_Complete(nr_Cell12); } case (tsc_NG_RegistrationMobility) { //Perform steps 4 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 11Fb1-11Fb3a1" siclog@ //IF 5GS registration type is set as Mobility Registration in step 211F, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 12. // @sic R5-223436 sic@ //f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, -); v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, cds_NG_NetworkFeatureSupport_mpsi_TC1135); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); f_NR5GC_Registration_Complete(nr_Cell12); // No SRB2 so must be on SRB1 } case else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect"); } } f_NR_RRCRelease(nr_Cell12); //@siclog "Step 12 siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(nr_Cell12, RRC_IDLE); //@siclog "Step 14 siclog@ t_WaitTime.start(v_TimerValue1); alt { //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } [] t_WaitTime.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } } //@siclog "Step 15 - 20 siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12 ,v_ReceivedMsg); //@siclog "Step 24 siclog@ f_NR_RRCRelease(nr_Cell12); //Set SIB1 for NR cell 1 according to Table 11.3.5.3.3-1 for Step 25 and 35 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1111111'B))); // @sic R5-223436 sic@ f_NR_ModifySysinfo(nr_Cell1, false); //WA#11_3_5 f_NR_WaitModificationPeriods(nr_Cell1); //WA#11_3_5 //@siclog "Step 26" siclog@ //The SS adjusts the NR Cells power levels according to row "T2" in table 11.3.5.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T2); //@siclog "Step 26A - 26C" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) { case (tsc_NG_RegistrationInitial) { v_TypeOfRegistration := Initial_Secure; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_TypeOfRegistration)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 26Da1-26Da16a1" siclog@ //IF 5GS registration type is set as Initial Registration in step 26C, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1. f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } case (tsc_NG_RegistrationMobility) { v_TypeOfRegistration := Mobility; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_TypeOfRegistration)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 26Db1-26Db3a1" siclog@ //IF 5GS registration type is set as Mobility Registration in step 26C, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } case else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect"); } } //@siclog "Step 33" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); //@siclog "Steps 34" siclog@ // Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message? f_NR_CheckNoRRCSetupReq(nr_Cell1, 30.0); //WA#11_3_5 6.0 --> 30.0 //@siclog "Step 35 siclog@ //SS changes SIB1 of NR cell 1 according to Table 11.3.5.3.3-1 and notifies the UE of change of System Information on NR Cell 1 by send Short Message on PDCCH using P-RNTI f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); //@siclog "Step 35A siclog@ t_WaitTime.start(26.0); alt { //@siclog "Step 35Aa1 siclog@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { //@siclog "Step 35Aa2 siclog@ t_WaitTime.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Aa1"); } //@siclog "Step 35Ab1 siclog@ [] t_WaitTime.timeout { //@siclog "Step 35Ab2" siclog@ f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); t_Wait.start(10.0); alt { //@siclog "Step 35Ab3" siclog@ //Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess? [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?))) { t_Wait.stop; //WA#R5s230331 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Ab3"); } [] t_Wait.timeout { //WA#R5s230331 f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35Ab3"); } } } } //@siclog "Step 36 - 38 siclog@ //Void @sic R5-223436 sic@ //@siclog "Step 39 - 49b3 siclog@ //Steps 3-13b3 of generic test procedure for IMS MO Video call establishment in 5GC as specified in 38.508-1 [4] Table 4.9.xx24.2.2-1 are performed. f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@ f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_NR_Cell1, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // Wait until need to add video QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Video Bearer"); // @sic R5-225362 sic@ f_NR5GC_ModifyPDUSession_AddVideo(nr_Cell1, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Video Bearer Added")); // @sic R5-225362 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Video Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Video Call Established"); // @sic R5-225362 sic@ //@siclog "Step 50" siclog@ //The generic test procedure for IMS MT call release in 5GC as specified in TS 38.508-1 [4] Table 4.9.18.2.2-1 takes place f_NR5GC_CallRelease(nr_Cell1, IPCAN_VideoCall, false); // @sic R5-225362 sic@ }

MCC160 Implementation

function fl_TC_11_3_5_TestBody() runs on NR5GC_PTC

{

var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell12);

var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams);

var NG_NAS_GutiParameters_Type v_GutiParams_NRCell1 := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1);

var template (present) NG_MobileIdentity v_S_TMSI_NR_Cell1 := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams_NRCell1);

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12);

var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (nr_Cell12);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams);

var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)});

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (value) NR_CellPowerList_Type v_CellPowerList_T1;

var template (value) NR_CellPowerList_Type v_CellPowerList_T2;

var TypeOfRegistration_Type v_TypeOfRegistration;

var template (omit) DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); //WA#11_3_5

var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig; //@sic R5s230376 sic@

var float v_TimerValue1 := 10.0;

timer t_WaitTime;

timer t_Wait;

v_CellPowerList_T1 := {

cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower (nr_Cell12, -80, -80)  // Power level for FR2 is FFS

};

v_CellPowerList_T2 := {

cs_NR_CellPower (nr_Cell1, -80, -80),  // Power level for FR2 is FFS

cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)

};

//@siclog "Step 1" siclog@

//The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.5.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_T1);

//@siclog "Step 1A" siclog@

//Void @sic R5-223436 sic@

//@siclog "Step 1B" siclog@

//Void @sic R5-223436 sic@

//@siclog "Step 2" siclog@

//Check: Does the UE transmit an RRCSetupRequest message on NR cell 12 including the establishmentCause which is any value except mps-PriorityAccess.

SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest())) -> value v_ReceivedAsp;

if(match(v_ReceivedAsp.Signalling.Rrc.Ccch.message_.c1.rrcSetupRequest.rrcSetupRequest.establishmentCause, mps_PriorityAccess))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2");

}

else

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

}

//@siclog "Steps 2A- 2B" siclog@

//Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 12

// @sic R5-223436 sic@

f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@

f_NR_RRCSetup_Def(nr_Cell12);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def));

//@siclog "Steps 2Ca1-2Ca15" siclog@

//IF 5GS registration type is set as Initial Registration in step 2B, THEN Steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed without RRC release.on NR Cell 12

// @sic R5-223436 sic@

// @sic R5s230200 sic@

select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) {

case (tsc_NG_RegistrationInitial) {

//@siclog "Steps 2Ca1-2Ca15" siclog@

//Perform steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, noRrcConnectionRelease, Initial_Secure);

}

case (tsc_NG_RegistrationMobility) {

//@siclog "2Cb1-2Cb2" siclog@

//Perform steps 4 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease);

}

case else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect");

}

}

//@siclog "Steps 3 - 10" siclog@

//Void @sic R5-223436 sic@

//@siclog "Steps 10A" siclog@

// @sic R5-223436 sic@

f_NR_RRCRelease(nr_Cell12);

//@siclog "Steps 10B" siclog@

// @sic R5-223436 sic@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue);

//@siclog "Steps 10C" siclog@

// @sic R5-223436 sic@

t_WaitTime.start(21.0);

alt {

//Check: Does the UE transmit an RRCSetupRequest message within 21s on NR Cell 12?

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(?, ?)))

{

t_WaitTime.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10C");

}

[] t_WaitTime.timeout {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10C");

}

}

//@siclog "Steps 10D" siclog@

// @sic R5-223436 sic@

//SS changes SIB1 of NR Cell 12 according to Table 11.3.5.3.3-2 and sends Short Message on PDCCH using P-RNTI.

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell12, omit);

f_NR_ModifySysinfo(nr_Cell12);

//@siclog "Steps 10E" siclog@

// @sic R5-223436 sic@

t_WaitTime.start(28.0);

alt {

//@siclog "Steps 10Ea1" siclog@

// @sic R5-223436 sic@

//Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mo_Data?

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data, ?))) //@sic R5s221172 sic@

{

//@siclog "Steps 10Ea2" siclog@ // @sic R5-223436 sic@

t_WaitTime.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10Ea1");

}

//@siclog "Steps 10Eb1" siclog@ // @sic R5-223436 sic@

[] t_WaitTime.timeout {

//@siclog "Steps 10Eb2" siclog@ // @sic R5-223436 sic@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue);

t_Wait.start(10.0);

alt {

//@siclog "Steps 10Eb3" siclog@ // @sic R5-223436 sic@

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mo_Data, ?)))

{

t_Wait.stop; // @sic R5s230200 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10Eb3");

}

[]t_Wait.timeout {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10Eb3");

}

}

}

}

//@siclog "Steps 10Fa1-10Fa8" siclog@

// @sic R5-223436 sic@

f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@

f_NR_RRCSetup_Def (nr_Cell12);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *),

tsc_SHT_IntegrityProtected

);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { // @sic R5s230103 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); // @sic R5s221172 sic@

f_NR5GC_PDUSessionEstablishment (nr_Cell12, -, tsc_NR_RbId_SRB2, -, - , v_DNN, -, v_DNN); // @sic R5s230200 sic@ //WA#11_3_5

//@siclog "Steps 10H" siclog@ // @sic R5-223436 sic@

//The UE is switched off by executing generic procedure in Table 4.9.6.3-1 in TS 38.508-1 [4]

f_NR_UE_DeRegisterOnSwitchOff(nr_Cell12, STATE_CONNECTED_3A);

// Tell IMS PTC to stop waiting for De-register

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s230200 sic@

f_NR5GC_MobileInfo_Init();

//@siclog "Steps 11A" siclog@

// @sic R5-223436 sic@

f_NR_SetCellPowerList(v_CellPowerList_T2);

//@siclog "Step 11B" siclog@ // @sic R5-223436 sic@

//The UE is Switched ON. The generic test procedure in TS 38.508-1 [4] Table 4.5.2.2-2 indicate that the UE performs registration on NR Cell 1.

f_NR5GC_SwitchOnAndResetIMS();

f_NR5GC_RRC_IdleState1N_Def(nr_Cell1);

//@siclog "Step 11C" siclog@ // @sic R5-223436 sic@

f_NR_SetCellPowerList(v_CellPowerList_T1);

//@siclog "Step 11D- 11F" siclog@ // @sic R5-223436 sic@

//Steps 2-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 12. (Note 2)

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

// @sic R5s230200 sic@

select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) {

case (tsc_NG_RegistrationInitial) {

//Perform steps 5 to 19 of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Steps 11Fa1-11Fa16a1" siclog@

//IF 5GS registration type is set as Initial Registration in step 11F, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 12.

//f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure);

f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_Secure, NOT_SUPPORTED);

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

v_GMM_MobilityInfo,

v_GUTIToSend,

v_TAIList,

omit,

-, -, -,

cds_NG_NetworkFeatureSupport_mpsi_TC1135);

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

// Step 15

f_NR5GC_Registration_Complete(nr_Cell12);

}

case (tsc_NG_RegistrationMobility) {

//Perform steps 4 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Steps 11Fb1-11Fb3a1" siclog@

//IF 5GS registration type is set as Mobility Registration in step 211F, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 12.

// @sic R5-223436 sic@

//f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, -);

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility,

v_GMM_MobilityInfo,

v_GUTIToSend,

v_TAIList,

omit,

-, -, -,

cds_NG_NetworkFeatureSupport_mpsi_TC1135);

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

f_NR5GC_Registration_Complete(nr_Cell12); // No SRB2 so must be on SRB1

}

case else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect");

}

}

f_NR_RRCRelease(nr_Cell12);

//@siclog "Step 12 siclog@

//The SS transmits a Paging message including a matched identity (correct fullI-RNTI).

f_NR_Paging_Def(nr_Cell12, RRC_IDLE);

//@siclog "Step 14 siclog@

t_WaitTime.start(v_TimerValue1);

alt {

//Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess?

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?)))

{

t_WaitTime.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

}

[] t_WaitTime.timeout {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14");

}

}

//@siclog "Step 15 - 20 siclog@

f_NR_ULGrantConfiguration_Start(nr_Cell12, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@

f_NR_RRCSetup_Def (nr_Cell12);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *),

tsc_SHT_IntegrityProtected

);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, ?)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12 ,v_ReceivedMsg);

//@siclog "Step 24 siclog@

f_NR_RRCRelease(nr_Cell12);

//Set SIB1 for NR cell 1 according to Table 11.3.5.3.3-1 for Step 25 and 35

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1,

cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(5,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '1111111'B))); // @sic R5-223436 sic@

v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(nr_Cell1); //@sic R5s230376 sic@

f_NR_SS_ConfigureSysinfo(nr_Cell1, -, v_NR_BcchToPdschConfig); //@sic R5s230376 sic@

//@siclog "Step 26" siclog@

//The SS adjusts the NR Cells power levels according to row "T2" in table 11.3.5.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_T2);

//@siclog "Step 26A - 26C" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) {

case (tsc_NG_RegistrationInitial) {

v_TypeOfRegistration := Initial_Secure;

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_TypeOfRegistration)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Step 26Da1-26Da16a1" siclog@

//IF 5GS registration type is set as Initial Registration in step 26C, THEN Steps 5 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1.

f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure);

}

case (tsc_NG_RegistrationMobility) {

v_TypeOfRegistration := Mobility;

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_TypeOfRegistration)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Step 26Db1-26Db3a1" siclog@

//IF 5GS registration type is set as Mobility Registration in step 26C, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1.

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo);

}

case else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect");

}

}

//@siclog "Step 33" siclog@

f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri);

//@siclog "Steps 34" siclog@

// Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message?

f_NR_CheckNoRRCSetupReq(nr_Cell1, 30.0); //WA#11_3_5 6.0 --> 30.0

//@siclog "Step 35 siclog@

//SS changes SIB1 of NR cell 1 according to Table 11.3.5.3.3-1 and notifies the UE of change of System Information on NR Cell 1 by send Short Message on PDCCH using P-RNTI

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit);

f_NR_ModifySysinfo(nr_Cell1);

//@siclog "Step 35A siclog@

t_WaitTime.start(26.0);

alt {

//@siclog "Step 35Aa1 siclog@

//Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess?

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?)))

{

//@siclog "Step 35Aa2 siclog@

t_WaitTime.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Aa1");

}

//@siclog "Step 35Ab1 siclog@

[] t_WaitTime.timeout {

//@siclog "Step 35Ab2" siclog@

f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri);

t_Wait.start(10.0);

alt {

//@siclog "Step 35Ab3" siclog@

//Check: Does the UE transmit an RRCSetupREQ message with establishmentCause of mps-PriorityAccess?

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mps_PriorityAccess, ?)))

{

t_Wait.stop; //WA#R5s230331

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 35Ab3");

}

[] t_Wait.timeout { //WA#R5s230331

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 35Ab3");

}

}

}

}

//@siclog "Step 36 - 38 siclog@

//Void @sic R5-223436 sic@

//@siclog "Step 39 - 49b3 siclog@

//Steps 3-13b3 of generic test procedure for IMS MO Video call establishment in 5GC as specified in 38.508-1 [4] Table 4.9.xx24.2.2-1 are performed.

f_NR_ULGrantConfiguration_Start(nr_Cell1, -, -, -, cs_NR_UplinkTimeAlignment_Stop, -, -, tsc_NoCnfReq ); // @sic R5s221172 sic@

f_NR_RRCSetup_Def (nr_Cell1);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *, *),

tsc_SHT_IntegrityProtected

);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_NR_Cell1, ?)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg);

v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN);

// Wait until need to add video QoS

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Video Bearer"); // @sic R5-225362 sic@

f_NR5GC_ModifyPDUSession_AddVideo(nr_Cell1, valueof(v_GSM_MobilityInfo));

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Video Bearer Added"));  // @sic R5-225362 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Video Call Established");

// Call now established

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Video Call Established"); // @sic R5-225362 sic@

//@siclog "Step 50" siclog@

//The generic test procedure for IMS MT call release in 5GC as specified in TS 38.508-1 [4] Table 4.9.18.2.2-1 takes place

f_NR5GC_CallRelease(nr_Cell1, IPCAN_VideoCall, false); // @sic R5-225362 sic@

}