3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230382

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3086 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NRDC test case 8.2.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With change from R5s230339, at step 5, the SK Counter value is incremented from 0 to 1 in the reconfiguration message. However, Keys are not regenerated with new SK Counter Value. This will lead to mismatch in Keys used by NW and Device.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step 5 , Sk counter is update and security init function is called to generate the Key with the new SK counter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | f_TC_8_2_2_1_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | With change from R5s230339, at step 5, the SK Counter value is incremented from 0 to 1 in the reconfiguration message. However, Keys are not regenerated with new SK Counter Value. This will lead to mismatch in Keys used by NW and Device.. |
| Summary of change | At step 5 , Sk counter is update and security init function is called to generate the Key with the new SK counter. |
| TTCN module | \8_2_2\RRC_Reconfiguration_NRDC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_8_2_2_1_2_NR5GC_TestBody() runs on NR5GC_PTC { ......... ......... //@siclog "Step 5" siclog@ //The SS transmits an RRCReconfiguration message via SRB3 to reconfigure PDCP. //@sic R5s230098 R5-231411 sic@ v_Timing_NextSendOccasion := f_NR_GetNextSendOccasion_DL(nr_Cell10); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB3)}, {cs_NR_DRB_ToAddMod_ReestPDCP(v_DrbId, cs_38508_PDCP_Config)}); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(omit,cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount),omit,omit,omit,cs_RRCReconfiguration_v1540_IEs(-, cs_RRCReconfiguration_v1560_IEs((v_SecurityParams.SK_Counter + 1)))); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell10, cs_TimingInfo_NR(v_Timing_NextSendOccasion), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_RRCReconfiguration_v1530))); //Reset PDCP of SRB3 in SS //@sic R5s221315 R5s230098 R5-231411 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing_NextSendOccasion, 5); v_PdcpCountInfoListSCG := f_NR_SS_PdcpCount_Get(nr_Cell10); for( i := 0 ; i < lengthof(v_PdcpCountInfoListSCG) ; i := 1+1 ) { if( ( ischosen(v_PdcpCountInfoListSCG[i].RadioBearerId.Srb) and (v_PdcpCountInfoListSCG[i].RadioBearerId.Srb == tsc_SRB3) ) or ( ischosen(v_PdcpCountInfoListSCG[i].RadioBearerId.Drb) and (v_PdcpCountInfoListSCG[i].RadioBearerId.Drb == v_DrbId))) { // Convert PdcpCount to integer value (UL an DL) v_PdcpCountInfoListSCG[i].UL.Value := int2bit(0, 32); v_PdcpCountInfoListSCG[i].DL.Value := int2bit(0, 32); } }

After Change

function f_TC_8_2_2_1_2_NR5GC_TestBody() runs on NR5GC_PTC { ......... ......... //@siclog "Step 5" siclog@ //The SS transmits an RRCReconfiguration message via SRB3 to reconfigure PDCP. //@sic R5s230098 R5-231411 sic@ v_Timing_NextSendOccasion := f_NR_GetNextSendOccasion_DL(nr_Cell10); v_SecurityParams.SK_Counter := v_SecurityParams.SK_Counter + 1; v_SecurityParams := f_NR_Security_InitSCG(v_SecurityParams); f_NR_Security_Set(v_SecurityParams); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB3)}, {cs_NR_DRB_ToAddMod_ReestPDCP(v_DrbId, cs_38508_PDCP_Config)}); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(omit,cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount),omit,omit,omit,cs_RRCReconfiguration_v1540_IEs(-, cs_RRCReconfiguration_v1560_IEs(v_SecurityParams.SK_Counter))); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell10, cs_TimingInfo_NR(v_Timing_NextSendOccasion), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_RRCReconfiguration_v1530))); //Reset PDCP of SRB3 in SS //@sic R5s221315 R5s230098 R5-231411 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing_NextSendOccasion, 5); v_PdcpCountInfoListSCG := f_NR_SS_PdcpCount_Get(nr_Cell10); for( i := 0 ; i < lengthof(v_PdcpCountInfoListSCG) ; i := 1+1 ) { if( ( ischosen(v_PdcpCountInfoListSCG[i].RadioBearerId.Srb) and (v_PdcpCountInfoListSCG[i].RadioBearerId.Srb == tsc_SRB3) ) or ( ischosen(v_PdcpCountInfoListSCG[i].RadioBearerId.Drb) and (v_PdcpCountInfoListSCG[i].RadioBearerId.Drb == v_DrbId))) { // Convert PdcpCount to integer value (UL an DL) v_PdcpCountInfoListSCG[i].UL.Value := int2bit(0, 32); v_PdcpCountInfoListSCG[i].DL.Value := int2bit(0, 32); } }