3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230442

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3098 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.3.2b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the case when pc_configuredUL_GrantType1 as "TRUE", the following changes are needed: - The UE should be allowed to act upon the transmitted CG-PUSCH Type 1 for the configured DRBs by setting the IE configuredGrantType1Allowed to TRUE in the ul_SpecificParameters in the peer signalling message as per Table 7.1.1.3.2b.3.1-3: maxPUSCH-Duration and configuredGrantType1Allowed settings - The configured grant period for the various sub carrier spacing should be according to Table 7.1.1.3.2b.3.3-3: CellGroupConfig (Table 7.1.1.3.2b.3.3-2: RRCReconfiguration) - Configured the dmrs_UplinkForPUSCH_MappingTypeB as per the second entry of Table 7.1.1.3.2b.3.1-2a: PUSCH-TimeDomainResourceAllocationList as well as the startSymbolAndLength since the expected TB size to be used is 9224. - For FR1 (SCS 30 Khz), the K2 value should be n4 rather than 2 as if it is 2, then it would lead to a DL slot in the configured TDD_UL_DL_ConfigurationCommon Pattern of 7 DL Slots and 2 UL Slots. Also, the periodicityAndOffset for schedulingRequestResourceToAddModList should be changed to sl20:9 as done in Prose CR R5-225377 NOTE: A Prose CR for this shall be raised at the next RAN5 - The timeDomainOffset is set to 9. NOTE: A Prose CR for this shall be raised at the next RAN5 for correcting the timeDomainOffset to 9 in Table 7.1.1.3.2b.3.3-3: CellGroupConfig (Table 7.1.1.3.2b.3.3-2: RRCReconfiguration) as the existing value of 0 does not match with an UL Slot as per the TDD_UL_DL_ConfigurationCommon Pattern for both FR 1 and FR 2 FR 1 Slot Pattern: DDDDDDDFUU FR 2 Slot Pattern: DDDFU - The parameter p_T2 should be set to 0 so that normal DCI based cyclic UL grants are not transmitted as the UE has to use the CG-PUSCH Type 1 grants to loop back the data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - configuredGrantType1 is set to TRUE for DRB1 and DRB3 as per Table 7.1.1.3.2b.3.1-3: maxPUSCH-Duration and configuredGrantType1Allowed settings - Config Grant period corrected as per Table 7.1.1.3.2b.3.3-3: CellGroupConfig (Table 7.1.1.3.2b.3.3-2: RRCReconfiguration) - Configured Mapping type to dmrs_UplinkForPUSCH_MappingTypeB as well as the startSymbolAndLength - K2 set to 4 for FR1 - The parameter p_TimeDomainOffset in template cs_RRC_ConfiguredUplinkGrant is set to 9. - The parameter p_T2 is set to 0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case when run with pc_configuredUL_GrantType1 as "TRUE". |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2b.NR5GC, 7.1.1.3.2b.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Prose CR R5-232872 raised for RAN5#99 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_2b_NR5GC () |
| --- | --- |
| Reason for change | For the case when pc_configuredUL_GrantType1 as "TRUE", the following changes are needed: - The UE should be allowed to act upon the transmitted CG-PUSCH Type 1 for the configured DRBs by setting the IE configuredGrantType1Allowed to TRUE in the ul_SpecificParameters in the peer signalling message as per Table 7.1.1.3.2b.3.1-3: maxPUSCH-Duration and configuredGrantType1Allowed settings - It is recommended to use the same configured grant period values for the various sub carrier spacing should be modified to reflect the values as used in the CG-PUSCH Type 1 TC 7.1.1.6.2 NOTE: A Prose CR for this shall be raised at the next RAN5 - Configured the dmrs_UplinkForPUSCH_MappingTypeB as per the second entry of Table 7.1.1.3.2b.3.1-2a: PUSCH-TimeDomainResourceAllocationList as well as the startSymbolAndLength since the expected TB size to be used is 9224. - For FR1 (SCS 30 Khz), the K2 value should be n4 rather than 2 as if it is 2, then it would lead to a DL slot in the configured TDD_UL_DL_ConfigurationCommon Pattern of 7 DL Slots and 2 UL Slots. Also, the periodicityAndOffset for schedulingRequestResourceToAddModList should be changed to sl20:9 as done in Prose CR R5-225377 NOTE: A Prose CR for this shall be raised at the next RAN5 - The timeDomainOffset is set to 9. NOTE: A Prose CR for this shall be raised at the next RAN5 for correcting the timeDomainOffset to 9 in Table 7.1.1.3.2b.3.3-3: CellGroupConfig (Table 7.1.1.3.2b.3.3-2: RRCReconfiguration) as the existing value of 0 does not match with an UL Slot as per the TDD_UL_DL_ConfigurationCommon Pattern for both FR 1 and FR 2 FR 1 Slot Pattern: DDDDDDDFUU FR 2 Slot Pattern: DDDFU - The parameter p_T2 should be set to 0 so that normal DCI based cyclic UL grants are not transmitted as the UE has to use the CG-PUSCH Type 1 grants to loop back the data. - The variables v_RRC_MsgRcvd and v_DRB_Rcvd should be re-initialised to false otherwise the peer signalling message RRC Reconfiguration Complete will not be received in the TestBody |
| Summary of change | - configuredGrantType1 is set to TRUE for DRB1 and DRB3 as per Table 7.1.1.3.2b.3.1-3: maxPUSCH-Duration and configuredGrantType1Allowed settings - Config Grant period corrected for different SCS to reflect the values as used in the CG-PUSCH Type 1 TC 7.1.1.6.2 - Configured Mapping type to dmrs_UplinkForPUSCH_MappingTypeB as well as the startSymbolAndLength - K2 set to 4 for FR1 - The parameter p_TimeDomainOffset in template cs_RRC_ConfiguredUplinkGrant is set to 9. - The parameter p_T2 is set to 0. - Variables v_RRC_MsgRcvd and v_DRB_Rcvd re-initialised to false |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions <<SKIPPED CODE>> //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, f_NR_GetServinCellConfig(nr_Cell1)); //@sic R5-202561 sic@ v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_LocationAndBandwidth := v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth; v_BwpSize := fl_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); v_RIV := fl_NR_RIV_Calculate(v_BwpSize, 23, 0); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_DMRS_UL_AdditionalPosition := pos1;} else { v_DMRS_UL_AdditionalPosition := pos0;} select (v_SubcarrierSpacing) { case(kHz15) { v_ConfigGrant_Period := sym20x14} case(kHz30) { v_ConfigGrant_Period := sym40x14} case(kHz60) { v_ConfigGrant_Period := sym80x14} case(kHz120) { v_ConfigGrant_Period := sym160x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig.setup := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),16))); f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s210863 sic@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,cs_TimingInfo_NR(v_Timing),false); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_UplinkBWP_List)); t_Watchdog.start; alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 11-14 sic@ //@sic R5s220045 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,20,360,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B)); //T1 =0 to C-RNTI grant. }; f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions <<SKIPPED CODE>> //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ //WA# v_RRC_MsgRcvd := false; v_DRB_Rcvd := false; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,-,-,-,-,true_)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,-,-,-,-,-)) ; v_RLC_BearerConfigList[2] := cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,-,-,-,-, true_)) ; v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, f_NR_GetServinCellConfig(nr_Cell1)); //@sic R5-202561 sic@ v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_LocationAndBandwidth := v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth; v_BwpSize := fl_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); v_RIV := fl_NR_RIV_Calculate(v_BwpSize, 23, 0); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_DMRS_UL_AdditionalPosition := pos1;} else { v_DMRS_UL_AdditionalPosition := pos0;} select (v_SubcarrierSpacing) { //WA# case(kHz15) { v_ConfigGrant_Period := sym40x14} case(kHz30) { v_ConfigGrant_Period := sym80x14} case(kHz60) { v_ConfigGrant_Period := sym160x14} case(kHz120) { v_ConfigGrant_Period := sym320x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig.setup := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),16,9))); //WA# //WA# R5s230287 v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , v_RLC_BearerConfigList, //WA# omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s210863 sic@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,cs_TimingInfo_NR(v_Timing),false); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_UplinkBWP_List)); t_Watchdog.start; alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 11-14 sic@ //@sic R5s220045 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,20,0,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B)); //T1 =0 to C-RNTI grant. WA# }; f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };