3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230454

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4738 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RRC testcase 8.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - When segmented ueCapabilityInformation is received, it is necessary to firstly decode it as UL-DCCH message rather than decoding it as eutraCapabilityMessage. - Present definition of function f_EUTRA_DecodeULDcchMsg doesn’t return the decoded UL-DCCH message but it is necessary to return the decoded value at times. - Failing to include requestedFrequencyBands_r11 IE in the ueCapabilityEnquiry message causes UE to not perform segmentation even though the UE supports UL segmentation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Segmented ueCapabilityInformation is initially decoded as UL-DCCH message and then decoding of ueCapabilityRAT_Container is done. - Added return value for function f_EUTRA_DecodeULDcchMsg and move the function definition to the module ‘EUTRA_CapabilityFunctions’ - Populate requestedFrequencyBands_r11 IE and include it in the ueCapabilityEnquiry message when UL segmentation is supported by the UE (pc_LTE_UL_Segmentation=TRUE) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant Rel16 UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

2.3	Change 3	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_GetCaps |
| --- | --- |
| Reason for change | When segmented ueCapabilityInformation is received, it is necessary to firstly decode it as UL-DCCH message rather than decoding it as eutraCapabilityMessage. |
| Summary of change | Segmented ueCapabilityInformation is initially decoded as UL-DCCH message and then decoding of ueCapabilityRAT_Container is done. |
| TTCN module | EUTRA_CapabilityFunctions.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_EUTRA_GetCaps(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC return UE_EUTRA_Capability

{ // @sic R5-217903 sic@

var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo;

var UE_EUTRA_Capability v_RxEutraCapability;

var SRB_COMMON_IND v_Received_DedicatedMessageSegment;

var octetstring v_Rx_SegmentOct := ''O;

var boolean v_Completed_AllSegments := false;

var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15)

// Rx UE Capability Information in segments or in one container

// IF pc_LTE_UL_Segmentation and UE's encoded UECapabilityInformation message >= Max PDCP SDU size then segments will be handled else one container

alt {

// EXCEPTION: Segment handling is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment.

[pc_LTE_UL_Segmentation] SRB.receive( car_SRB1_RrcPdu_IND(p_CellId, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(v_MessageSegmentRx))) ) -> value v_Received_DedicatedMessageSegment

{

v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16;   //compact segment

if (v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentType_r16 == lastSegment){ // no more segments to receive

v_Completed_AllSegments := true;

}

else { // notLastSegment received

if (v_MessageSegmentRx == 15) { f_EUTRA_SetVerdictFailOrInconc(__FILE__,__LINE__, "Step 2a1: Received a notLastSegment as 16th segment from the UE");}

v_MessageSegmentRx := v_MessageSegmentRx + 1;

repeat;

}

}

[not pc_LTE_UL_Segmentation] SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, cr_508_ULDedicatedMessageSegment_r16(?))) // VerdictFail case

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "pc_LTE_UL_Segmentation not set for segmentation when UECapabilityInformation message >= Max PDCP SDU size");

}

// The UE transmits a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable

[] SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, cr_508_UeCapabilityInformation(p_RRC_TI, cr_EutraCapInfo))) -> value v_ReceivedAspForUeCapabilityInfo

{

if (v_MessageSegmentRx>0) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE radio access capability information reived after a segment"); // VerdictFail case: step 2b1 is mutually exclusive with 2ax

}

}

};//end alt

// Decode and store the UE-CapabilityRAT-Container

if (v_Completed_AllSegments){ //UECapabilityInformation Rx. in segments

if (lengthof(v_Rx_SegmentOct) < tsc_Max_LTE_PDCP_SDU_size) {//Set to <8188 by default - @sic R5s220330 sic@

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size");

}

v_RxEutraCapability := f_EUTRA_DecodeEutraCapMsg(v_Rx_SegmentOct);

if (match (v_RxEutraCapability.accessStratumRelease, (rel8, rel9, rel10, rel11, rel12, rel13, rel14, rel15))) { // accessStratumRelease < rel16

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "AccessStratumRelease < release16 so segmentation is not allowed");

}

}

else{ //UECapabilityInformation Rx. in one message

v_Rx_SegmentOct  := v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.c1.ueCapabilityInformation_r8.ue_CapabilityRAT_ContainerList[0].ueCapabilityRAT_Container;

if (lengthof(v_Rx_SegmentOct) > tsc_Max_LTE_PDCP_SDU_size){

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE sent one segment of size > tsc_Max_LTE_PDCP_SDU_size");

}

v_RxEutraCapability := f_EUTRA_DecodeEutraCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.c1.ueCapabilityInformation_r8.ue_CapabilityRAT_ContainerList[0].ueCapabilityRAT_Container);

}

return v_RxEutraCapability;

}

After change:

function f_EUTRA_GetCaps(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC return UE_EUTRA_Capability

{ // @sic R5-217903 sic@

var SRB_COMMON_IND v_ReceivedAspForUeCapabilityInfo;

var UE_EUTRA_Capability v_RxEutraCapability;

var UL_DCCH_Message v_RxSegEutraCapability;

var SRB_COMMON_IND v_Received_DedicatedMessageSegment;

var octetstring v_Rx_SegmentOct := ''O;

var boolean v_Completed_AllSegments := false;

var ULDedicatedMessageSegment_r16_IEs.segmentNumber_r16 v_MessageSegmentRx := 0; //INTEGER (0..15)

// Rx UE Capability Information in segments or in one container

// IF pc_LTE_UL_Segmentation and UE's encoded UECapabilityInformation message >= Max PDCP SDU size then segments will be handled else one container

alt {

// EXCEPTION: Segment handling is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment.

[pc_LTE_UL_Segmentation] SRB.receive( car_SRB1_RrcPdu_IND(p_CellId, cr_508_ULDedicatedMessageSegment_r16(cr_ULDedicatedMessageSegment_r16_enabled(v_MessageSegmentRx))) ) -> value v_Received_DedicatedMessageSegment

{

v_Rx_SegmentOct := v_Rx_SegmentOct & v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentContainer_r16;   //compact segment

if (v_Received_DedicatedMessageSegment.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ulDedicatedMessageSegment_r16.criticalExtensions.ulDedicatedMessageSegment_r16.rrc_MessageSegmentType_r16 == lastSegment){ // no more segments to receive

v_Completed_AllSegments := true;

}

else { // notLastSegment received

if (v_MessageSegmentRx == 15) { f_EUTRA_SetVerdictFailOrInconc(__FILE__,__LINE__, "Step 2a1: Received a notLastSegment as 16th segment from the UE");}

v_MessageSegmentRx := v_MessageSegmentRx + 1;

repeat;

}

}

[not pc_LTE_UL_Segmentation] SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, cr_508_ULDedicatedMessageSegment_r16(?))) // VerdictFail case

{

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "pc_LTE_UL_Segmentation not set for segmentation when UECapabilityInformation message >= Max PDCP SDU size");

}

// The UE transmits a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable

[] SRB.receive(car_SRB1_RrcPdu_IND(p_CellId, cr_508_UeCapabilityInformation(p_RRC_TI, cr_EutraCapInfo))) -> value v_ReceivedAspForUeCapabilityInfo

{

if (v_MessageSegmentRx>0) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE radio access capability information reived after a segment"); // VerdictFail case: step 2b1 is mutually exclusive with 2ax

}

}

};//end alt

// Decode and store the UE-CapabilityRAT-Container

if (v_Completed_AllSegments){ //UECapabilityInformation Rx. in segments

if (lengthof(v_Rx_SegmentOct) < tsc_Max_LTE_PDCP_SDU_size) {//Set to <8188 by default - @sic R5s220330 sic@

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE segmented the UECapabilityInformation message being smaller than Max PDCP SDU size");

}

v_RxSegEutraCapability := f_EUTRA_DecodeULDcchMsg(v_Rx_SegmentOct);

v_RxEutraCapability := f_EUTRA_DecodeEutraCapMsg(v_RxSegEutraCapability.message_.c1.ueCapabilityInformation.criticalExtensions.c1.ueCapabilityInformation_r8.ue_CapabilityRAT_ContainerList[0].ueCapabilityRAT_Container);

if (match (v_RxEutraCapability.accessStratumRelease, (rel8, rel9, rel10, rel11, rel12, rel13, rel14, rel15))) { // accessStratumRelease < rel16

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "AccessStratumRelease < release16 so segmentation is not allowed");

}

}

else{ //UECapabilityInformation Rx. in one message

v_Rx_SegmentOct  := v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.c1.ueCapabilityInformation_r8.ue_CapabilityRAT_ContainerList[0].ueCapabilityRAT_Container;

if (lengthof(v_Rx_SegmentOct) > tsc_Max_LTE_PDCP_SDU_size){

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "UE sent one segment of size > tsc_Max_LTE_PDCP_SDU_size");

}

v_RxEutraCapability := f_EUTRA_DecodeEutraCapMsg(v_ReceivedAspForUeCapabilityInfo.Signalling.Rrc.Dcch.message_.c1.ueCapabilityInformation.criticalExtensions.c1.ueCapabilityInformation_r8.ue_CapabilityRAT_ContainerList[0].ueCapabilityRAT_Container);

}

return v_RxEutraCapability;

}

## 2.2 Change 2

| Function name | f_EUTRA_DecodeULDcchMsg |
| --- | --- |
| Reason for change | Present definition of function f_EUTRA_DecodeULDcchMsg doesn’t return the decoded UL-DCCH message but it is necessary to return the decoded value at times. |
| Summary of change | Added return value for function f_EUTRA_DecodeULDcchMsg and move the function definition to the module ‘EUTRA_CapabilityFunctions’ to be accessible by other testcases in LTE suite. |
| TTCN module | RRC_Others_PRO.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_EUTRA_DecodeULDcchMsg(octetstring p_Octetstring)

{

var UL_DCCH_Message v_UL_Dcch_Msg;

var bitstring v_Bitstring := oct2bit(p_Octetstring);

var integer v_Result := decvalue(v_Bitstring, v_UL_Dcch_Msg);

if (v_Result != 0) {

FatalError(__FILE__, __LINE__, "UE EUTRA Capability cannot be decoded");

}

}

After change:

function f_EUTRA_DecodeULDcchMsg(octetstring p_Octetstring) return UL_DCCH_Message

{

var UL_DCCH_Message v_UL_Dcch_Msg;

var bitstring v_Bitstring := oct2bit(p_Octetstring);

var integer v_Result := decvalue(v_Bitstring, v_UL_Dcch_Msg);

if (v_Result != 0) {

FatalError(__FILE__, __LINE__, "UE EUTRA Capability cannot be decoded");

}

return v_UL_Dcch_Msg;

}

## 2.3 Change 3

| Function name | fl_EUTRA_FullCapabilityCheck |
| --- | --- |
| Reason for change | Failing to include requestedFrequencyBands_r11 IE in the ueCapabilityEnquiry message causes UE to not perform segmentation even though the UE supports UL segmentation. |
| Summary of change | Populate requestedFrequencyBands_r11 IE and include it in the ueCapabilityEnquiry message when UL segmentation is supported by the UE (pc_LTE_UL_Segmentation=TRUE) |
| TTCN module | LTE\8_5\RRC_Others.ttcn |
| MCC160 Comment | Rejected Required prose CR could not be provided in time |

Before change:

function fl_EUTRA_FullCapabilityCheck(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC

{

var UE_EUTRA_Capability v_ReceivedCapabilityMsg;

var template UE_EUTRA_Capability v_EutraCapability;

var template UE_EUTRA_Capability_v9a0_IEs v_Expected_UE_EUTRA_Capability_v9a0_IEs := omit;

// Transmit UE Capability Enquiry message to ask for EUTRA Capability information

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI,-,-,-,-,enabled))); // @sic R5-217903 sic@

// Receive UE Capability Information message for EUTRA capabilities and decode

// Either a container is received or segments from which the container is to be established (only Rel-16 onwards)

v_ReceivedCapabilityMsg := f_EUTRA_GetCaps(p_CellId, p_RRC_TI);  // @sic R5-217903 sic@

// Skipped code

} // fl_EUTRA_FullCapabilityCheck

After change:

import from EUTRA_AuxiliaryCapCheckFunctions all;

import from EUTRA_Parameters all;

function fl_EUTRA_FullCapabilityCheck(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC

{

var UE_EUTRA_Capability v_ReceivedCapabilityMsg;

var template UE_EUTRA_Capability v_EutraCapability;

var template UE_EUTRA_Capability_v9a0_IEs v_Expected_UE_EUTRA_Capability_v9a0_IEs := omit;

var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_RequestedFrequencyBands := {px_ePrimaryFrequencyBand};

var integer v_Index := 0;

var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics();

var UECapabilityEnquiry_v1180_IEs.requestedFrequencyBands_r11 v_BandToBeAdded;

if(pc_LTE_UL_Segmentation) {

// Get bands to be requested, from all the supported bands

do {

if(v_SuppEutraBandsAccToPICS[v_Index]== '1'B and (v_Index+1) != px_ePrimaryFrequencyBand) { // if the UE supposts that band and it is not the primary band (which is already added in the list)

v_BandToBeAdded := {v_Index+1};

v_RequestedFrequencyBands := v_RequestedFrequencyBands & v_BandToBeAdded; // add supported band to the list

}

v_Index := v_Index + 1;

} while ((lengthof(v_RequestedFrequencyBands) < 16) or ((v_Index < lengthof(v_SuppEutraBandsAccToPICS)) and (lengthof(v_SuppEutraBandsAccToPICS) < 16)));

// Transmit UE Capability Enquiry message to ask for EUTRA Capability information

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI,-,-,-,-,enabled ,v_RequestedFrequencyBands))); // @sic R5-217903 sic@

} else {

// Transmit UE Capability Enquiry message to ask for EUTRA Capability information

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI,-,-,-,-,enabled ))); // @sic R5-217903 sic@

}

// Receive UE Capability Information message for EUTRA capabilities and decode

// Either a container is received or segments from which the container is to be established (only Rel-16 onwards)

v_ReceivedCapabilityMsg := f_EUTRA_GetCaps(p_CellId, p_RRC_TI);  // @sic R5-217903 sic@

// Skipped code

} // fl_EUTRA_FullCapabilityCheck