3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230478

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4742 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-17 NB-IoT NTN test case 22.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 22.1.2 to the NBIOT ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 22.1.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

2.3	Change 3	8

2.4	Change 4	11

3	Branches executed	12

4	Execution Log Files	12

4.1	MediaTek Dimensity D930	12

5	References	12

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 22.1.2 which is part of the NBIOT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	22.1.2

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	MediaTek Dimensity D930

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_NBIOT_InitSystemInformation() |
| --- | --- |
| Reason for change | For system information combination c8 and c9, the new value of SIB2 including ntn_ConfigCommon_r1 is not updated in v_SI_List |
| Summary of change | Update v_SI_List[0] with modified SIB2 contents |
| TTCN module | NBIOT_CellInfo.ttcn |
| MCC160 Comment | Accepted |

Before change

function f_NBIOT_InitSystemInformation(NBIOT_CellId_Type                         p_CellId,

EUTRA_ASN1_CellIdentity_Type              p_CellIdentity,

NB_Combination_Type                       p_Combination,

integer                                   p_FrequencyBand,

NB_OperationMode_Type                     p_OperationMode,

template (value) PLMN_IdentityList_NB_r13 p_PLMN_Identity,

EUTRA_ASN1_TrackingAreaCode_Type          p_TrackingAreaCode,

EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN)

runs on NBIOT_PTC return template (value) NB_BcchInfo_Type

{

var template (value) NB_BcchInfo_Type v_CellBcchInfo;

var template (value) BCCH_BCH_Message_NB v_MIB;

var template (value) BCCH_DL_SCH_Message_NB v_SIB1;

var template (value) NB_SI_List_Type v_SI_List;

var template (value) SystemInformation_NB_r13_IEs v_SIB2;

var template (value) SystemInformation_NB_r13_IEs v_SIB_SI2;

var template (value) SystemInformation_NB_r13_IEs v_SIB_SI3;

var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset;

var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize;

var template (value) NB_OperationModeInfo_Type v_OperationMode;

var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList;

var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI2;

var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI3;

var NB_CarrierFreq_Type v_CarrierFreq := fl_NBIOT_Get_CarrierFreq(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN); // @sic R5-185113 sic@

select (p_OperationMode) {

case (standalone) {

v_OperationMode := cs_NB_OperationModeInfo_Standalone;

v_NB_EutraControlRegionSize := omit;

v_NRS_CRS_PowerOffset := omit;

}

case (inband_samePCI) {

v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); // @sic R5-181295 sic@

v_NB_EutraControlRegionSize := n2;

v_NRS_CRS_PowerOffset := dB0;

}

case (inband_differentPCI) {

v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@

v_NB_EutraControlRegionSize := n2;

v_NRS_CRS_PowerOffset := omit;

}

case (guardband) {

v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@

v_NB_EutraControlRegionSize := omit;

v_NRS_CRS_PowerOffset := omit;

}

}

// --- Initialise MIB ---

v_MIB :=  cs_508_MasterInformationBlock_NB_Init(v_OperationMode);

// --- Initialise SIB1 ---

// -- A default scheduling assigned ----------

v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(p_PLMN_Identity,

p_TrackingAreaCode,

p_CellIdentity,

p_FrequencyBand,

v_NB_EutraControlRegionSize,

v_NRS_CRS_PowerOffset,

cs_SchedulingInfo_NB_1SI);

// --- Initialise SIB2 ---

v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN));

v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2);

// --- Initialise all SYSINFO --

if (p_Combination == c1) {

v_SchedulingInfoList := cs_SchedulingInfo_NB_1SI;

} else if (p_Combination == c5) { // @sic R5-185113 sic@

v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB20(cs_508_SystemInformationBlockType20_NB_Def));

v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType20_NB_r14});

} else if (p_Combination == c6) { // @sic R5-185113 sic@

v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL)));

v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType22_NB_r14});

} else if (p_Combination == c7) { //@sic R5-2226052 sic@

v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL)));

v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB23(cs_508_SystemInformationBlockType23_NB_Def));

v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {sibType22_NB_r14}, -, -, -, {});

v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.schedulingInfoList_v1530 := cs_SchedulingInfo_NB_c7_r15;

} else if (p_Combination == c8 or p_Combination == c9) { //@sic R5-231904, R5-231933, R5-233367, R5-233365 sic@

v_SI_List[1] := f_NBIOT_InitSIB31(p_CellId);

v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({});

v_MIB.message_.partEARFCN_17.earfcn_LSB := fl_NBIOT_Get_EARFCN_2LSB(v_CarrierFreq);

v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred;

v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17;

if (p_Combination == c8){

v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode);

}

else{//c9 (c8+SIB3)

v_SI_List[2] := v_SI_List[1];

v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def));

v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType3_NB_r13});

v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {sibType3_NB_r13}, -, -, -, {});

v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c9_r15);

}

} else {

// --- Initialise SIB3 ---

v_SIB_MappingInfo_SI2 := {sibType3_NB_r13};

v_SIB_SI2 := cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def);

select (p_Combination) { // @sic R5-185113 sic@

case (c2) { // SIB3 & SIB4 Intra-Frequency

v_SIB_MappingInfo_SI3 := {sibType4_NB_r13};

v_SIB_SI3 := cs_SI_NB_SIB4(cs_508_SystemInformationBlockType4_NB_Def);

}

case (c3) { // SIB 3 & SIB5 Inter-Frequency

v_SIB_MappingInfo_SI3 := {sibType5_NB_r13};

v_SIB_SI3 := cs_SI_NB_SIB5(f_NBIOT_InitSystemInformation5(p_CellId, p_FrequencyBand, p_OperationMode));

}

case (c4) { // SIB 3 & 14

v_SIB_MappingInfo_SI3 := {sibType14_NB_r13};

v_SIB_SI3 := cs_SI_NB_SIB14(cs_SystemInformationBlockType14_NB_Def);

}

case else {

FatalError(__FILE__, __LINE__, "invalid sysinfo combination");

}

}

v_SI_List[1] := cs_SystemInformation_Common_NB(v_SIB_SI2);

v_SI_List[2] := cs_SystemInformation_Common_NB(v_SIB_SI3);

v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, v_SIB_MappingInfo_SI2, -, -, -, v_SIB_MappingInfo_SI3);

}

v_SIB1.message_.c1.systemInformationBlockType1_r13.schedulingInfoList_r13 := v_SchedulingInfoList;

v_CellBcchInfo := cs_BCCHInfo_NB_C1(v_MIB, v_SIB1, v_SI_List);

return v_CellBcchInfo;

}

After change

function f_NBIOT_InitSystemInformation(NBIOT_CellId_Type                         p_CellId,

EUTRA_ASN1_CellIdentity_Type              p_CellIdentity,

NB_Combination_Type                       p_Combination,

integer                                   p_FrequencyBand,

NB_OperationMode_Type                     p_OperationMode,

template (value) PLMN_IdentityList_NB_r13 p_PLMN_Identity,

EUTRA_ASN1_TrackingAreaCode_Type          p_TrackingAreaCode,

EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN)

runs on NBIOT_PTC return template (value) NB_BcchInfo_Type

{

var template (value) NB_BcchInfo_Type v_CellBcchInfo;

var template (value) BCCH_BCH_Message_NB v_MIB;

var template (value) BCCH_DL_SCH_Message_NB v_SIB1;

var template (value) NB_SI_List_Type v_SI_List;

var template (value) SystemInformation_NB_r13_IEs v_SIB2;

var template (value) SystemInformation_NB_r13_IEs v_SIB_SI2;

var template (value) SystemInformation_NB_r13_IEs v_SIB_SI3;

var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset;

var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize;

var template (value) NB_OperationModeInfo_Type v_OperationMode;

var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList;

var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI2;

var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI3;

var NB_CarrierFreq_Type v_CarrierFreq := fl_NBIOT_Get_CarrierFreq(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN); // @sic R5-185113 sic@

select (p_OperationMode) {

case (standalone) {

v_OperationMode := cs_NB_OperationModeInfo_Standalone;

v_NB_EutraControlRegionSize := omit;

v_NRS_CRS_PowerOffset := omit;

}

case (inband_samePCI) {

v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); // @sic R5-181295 sic@

v_NB_EutraControlRegionSize := n2;

v_NRS_CRS_PowerOffset := dB0;

}

case (inband_differentPCI) {

v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@

v_NB_EutraControlRegionSize := n2;

v_NRS_CRS_PowerOffset := omit;

}

case (guardband) {

v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@

v_NB_EutraControlRegionSize := omit;

v_NRS_CRS_PowerOffset := omit;

}

}

// --- Initialise MIB ---

v_MIB :=  cs_508_MasterInformationBlock_NB_Init(v_OperationMode);

// --- Initialise SIB1 ---

// -- A default scheduling assigned ----------

v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(p_PLMN_Identity,

p_TrackingAreaCode,

p_CellIdentity,

p_FrequencyBand,

v_NB_EutraControlRegionSize,

v_NRS_CRS_PowerOffset,

cs_SchedulingInfo_NB_1SI);

// --- Initialise SIB2 ---

v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN));

v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2);

// --- Initialise all SYSINFO --

if (p_Combination == c1) {

v_SchedulingInfoList := cs_SchedulingInfo_NB_1SI;

} else if (p_Combination == c5) { // @sic R5-185113 sic@

v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB20(cs_508_SystemInformationBlockType20_NB_Def));

v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType20_NB_r14});

} else if (p_Combination == c6) { // @sic R5-185113 sic@

v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL)));

v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType22_NB_r14});

} else if (p_Combination == c7) { //@sic R5-2226052 sic@

v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL)));

v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB23(cs_508_SystemInformationBlockType23_NB_Def));

v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {sibType22_NB_r14}, -, -, -, {});

v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.schedulingInfoList_v1530 := cs_SchedulingInfo_NB_c7_r15;

} else if (p_Combination == c8 or p_Combination == c9) { //@sic R5-231904, R5-231933, R5-233367, R5-233365 sic@

v_SI_List[1] := f_NBIOT_InitSIB31(p_CellId);

v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({});

v_MIB.message_.partEARFCN_17.earfcn_LSB := fl_NBIOT_Get_EARFCN_2LSB(v_CarrierFreq);

v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred;

v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17;

v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2);

if (p_Combination == c8){

v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode);

}

else{//c9 (c8+SIB3)

v_SI_List[2] := v_SI_List[1];

v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def));

v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType3_NB_r13});

v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {sibType3_NB_r13}, -, -, -, {});

v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c9_r15);

}

} else {

// --- Initialise SIB3 ---

v_SIB_MappingInfo_SI2 := {sibType3_NB_r13};

v_SIB_SI2 := cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def);

select (p_Combination) { // @sic R5-185113 sic@

case (c2) { // SIB3 & SIB4 Intra-Frequency

v_SIB_MappingInfo_SI3 := {sibType4_NB_r13};

v_SIB_SI3 := cs_SI_NB_SIB4(cs_508_SystemInformationBlockType4_NB_Def);

}

case (c3) { // SIB 3 & SIB5 Inter-Frequency

v_SIB_MappingInfo_SI3 := {sibType5_NB_r13};

v_SIB_SI3 := cs_SI_NB_SIB5(f_NBIOT_InitSystemInformation5(p_CellId, p_FrequencyBand, p_OperationMode));

}

case (c4) { // SIB 3 & 14

v_SIB_MappingInfo_SI3 := {sibType14_NB_r13};

v_SIB_SI3 := cs_SI_NB_SIB14(cs_SystemInformationBlockType14_NB_Def);

}

case else {

FatalError(__FILE__, __LINE__, "invalid sysinfo combination");

}

}

v_SI_List[1] := cs_SystemInformation_Common_NB(v_SIB_SI2);

v_SI_List[2] := cs_SystemInformation_Common_NB(v_SIB_SI3);

v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, v_SIB_MappingInfo_SI2, -, -, -, v_SIB_MappingInfo_SI3);

}

v_SIB1.message_.c1.systemInformationBlockType1_r13.schedulingInfoList_r13 := v_SchedulingInfoList;

v_CellBcchInfo := cs_BCCHInfo_NB_C1(v_MIB, v_SIB1, v_SI_List);

return v_CellBcchInfo;

}

## Change 2

| Function name | f_NBIOT_InitSIB31() |
| --- | --- |
| Reason for change | The array vc_NBIOT_Global.CellArray() is not initialized until the function f_NBIOT_CellArray_Init() returns (in function fl_NBIOT_Common_Init()) so it cannot be referenced in this function which is called multiple times during f_NBIOT_CellArray_Init() |
| Summary of change | Remove check on length of vc_NBIOT_Global.CellArray |
| TTCN module | NBIoT_CellInfo.ttcn |
| MCC160 Comment | Accepted |

Before change

function f_NBIOT_InitSIB31(NBIOT_CellId_Type p_CellId,

template (omit) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17 := omit)

runs on NBIOT_PTC return template (value) BCCH_DL_SCH_Message_NB

{

var template (value) SystemInformationBlockType31_NB_r17 v_SIB31 := cs_508_SystemInformationBlockType31_NB_GSO_Def;

var integer v_Index := f_NBIOT_CellInfo_GetIndex(p_CellId);

if(lengthof(vc_NBIOT_Global.CellArray)==lengthof(cs_SystemInformationBlockType31_NB_r17_Array))

{

if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO)

{

v_SIB31 :=cs_SystemInformationBlockType31_NB_r17_Array[v_Index];

if(isvalue(p_UL_SyncValidityDuration_r17))

{

v_SIB31.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 := p_UL_SyncValidityDuration_r17  //compiler warning may be ignored due to isvalue check

}

}

else

{

FatalError(__FILE__, __LINE__, "NGSO configuration is not yet supported");

}

}

else

{

FatalError(__FILE__, __LINE__, "SIB 31 configuration for this NBIOT cell is not yet supported");

}

return cs_SystemInformation_Common_NB(cs_SI_NB_SIB31(v_SIB31));

}

After change

function f_NBIOT_InitSIB31(NBIOT_CellId_Type p_CellId,

template (omit) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17 := omit)

runs on NBIOT_PTC return template (value) BCCH_DL_SCH_Message_NB

{

var template (value) SystemInformationBlockType31_NB_r17 v_SIB31 := cs_508_SystemInformationBlockType31_NB_GSO_Def;

var integer v_Index := f_NBIOT_CellInfo_GetIndex(p_CellId);

/*REMOVED if(lengthof(vc_NBIOT_Global.CellArray)==lengthof(cs_SystemInformationBlockType31_NB_r17_Array))

{*/

if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO)

{

v_SIB31 :=cs_SystemInformationBlockType31_NB_r17_Array[v_Index];

if(isvalue(p_UL_SyncValidityDuration_r17))

{

v_SIB31.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 := p_UL_SyncValidityDuration_r17  //compiler warning may be ignored due to isvalue check

}

}

else

{

FatalError(__FILE__, __LINE__, "NGSO configuration is not yet supported");

}

/* REMOVED }

else

{

FatalError(__FILE__, __LINE__, "SIB 31 configuration for this NBIOT cell is not yet supported");

}*/

return cs_SystemInformation_Common_NB(cs_SI_NB_SIB31(v_SIB31));

}

## Change 3

| Function name | f_TC_22_1_2_NBIOT() |
| --- | --- |
| Reason for change | UL grant transmission is not started |
| Summary of change | Start default UL grant transmission after receiving RRCConnectionRequest |
| TTCN module | NBIoT_General_NTN.ttcn |
| MCC160 Comment | Accepted in principle. It is proposed to introduce a new common wrapper function f_NBIOT_ULGrantTransmission_Def for these steps. |

Before change

function f_TC_22_1_2_NBIOT() runs on NBIOT_PTC

{ // NB-IoT / NTN

var NAS_MSG_Indication_Type v_NasInd;

//    var template (present) NAS_UL_Message_Type v_AttachReq;

var template (present) NAS_UL_Message_Type v_ESMMessage;

var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B);

var template (present) UL_DCCH_Message_NB v_RRCConnectionSetupComplete_NB := cr_508_RRCConnectionSetupComplete_NB;

//Initialise all cell, security and mobile parameters

f_NBIOT_Init(c8);

//Create and configure NCell 1

f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE);

//UE is Switched ON to configure Automatic PLMN selection mode

f_UT_SwitchOnUE(UT, false);

//Make sure the UE is in automatic PLMN selection mode

f_UT_AutomaticPLMN_Select (UT);

//Make sure that No UE location is pre-configured

f_UT_ConfigureNTNPosition(UT,"No UE location is pre-configured");

//Now Power OFF the UE

f_UT_PowerOffUE (UT);

f_NBIOT_TestBody_Set(true);

//Now bring up the cell to serving cell power for premable

f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE);

//@siclog "Step 1" siclog@

f_UT_SwitchOnUE(UT, false);

//@siclog "Step 1A" siclog@

//Check for no RRCConnectionRequest-NB message on Ncell 11

if (f_NBIOT_RRC_RRCConnectionRequest_Check(nbiot_Cell1, 360.0)) {

//Does the UE sent RRCConnectionRequest-NB message on Ncell 1 within 6 minutes

f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 1A : RRCConnectionRequest-NB message received on NCell 1 within 6 minutes");

}

//@siclog "Step 1B siclog@

f_UT_PowerOffUE (UT);

//@siclog "Step 1B siclog@

f_UT_SwitchOnUE(UT, false);

//@siclog "Step 1C siclog@

f_UT_ConfigureNTNPosition(UT);

//@siclog "Step 2 siclog@

if (f_NBIOT_RRC_RRCConnectionRequest_Check(nbiot_Cell1, 360.0)) {

//Does the UE sent RRCConnectionRequest-NB message on Ncell 1 within 6 minutes

f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 2 : RRCConnectionRequest-NB message received on NCell 1 within 6 minutes");

}

//@siclog "Steps 3 - 14 siclog@

//Steps 3 to 14 of the registration procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1.

f_NBIOT_RRC_ConnectionSetup_Def();

if (px_DoAttachWithoutPDN) {

v_ESMMessage := cr_ESM_DUMMY_MESSAGE;

} else {

v_ESMMessage := cr_CIOT_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, v_PDNType);

}

//The rrcConnectionSetupComlete should include IE gnss-ValidityDuration-r17, but contents are not checked according to Table 22.1.2.3.3-1

v_RRCConnectionSetupComplete_NB.message_.c1.rrcConnectionSetupComplete_r13.criticalExtensions.rrcConnectionSetupComplete_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.gnss_ValidityDuration_r17 := ?;

v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1,

v_RRCConnectionSetupComplete_NB,

cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected),

cr_CIOT_ATTACH_REQUEST(cr_AdditionalUpdateType_CIOT),

v_ESMMessage));

f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NasInd);

//@siclog "Step 15" siclog@

// The SS transmits an RRCConnectionRelease-NB message to release RRC connection and move to RRC_IDLE

f_NBIOT_RRC_ConnectionRelease (nbiot_Cell1);

f_NBIOT_TestBody_Set(false);

//Switch/power off UE

f_NBIOT_Postamble (nbiot_Cell1, CONTROL_PLANE, N1_IDLE);

}

After change

function f_TC_22_1_2_NBIOT() runs on NBIOT_PTC

{ // NB-IoT / NTN

var NAS_MSG_Indication_Type v_NasInd;

//    var template (present) NAS_UL_Message_Type v_AttachReq;

var template (present) NAS_UL_Message_Type v_ESMMessage;

var template (present) B3_Type v_PDNType := (f_GetPdnType(), '101'B);

var template (present) UL_DCCH_Message_NB v_RRCConnectionSetupComplete_NB := cr_508_RRCConnectionSetupComplete_NB;

var template (omit) integer v_HarqProcessNumber;

//Initialise all cell, security and mobile parameters

f_NBIOT_Init(c8);

//Create and configure NCell 1

f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE);

//UE is Switched ON to configure Automatic PLMN selection mode

f_UT_SwitchOnUE(UT, false);

//Make sure the UE is in automatic PLMN selection mode

f_UT_AutomaticPLMN_Select (UT);

//Make sure that No UE location is pre-configured

f_UT_ConfigureNTNPosition(UT,"No UE location is pre-configured");

//Now Power OFF the UE

f_UT_PowerOffUE (UT);

f_NBIOT_TestBody_Set(true);

//Now bring up the cell to serving cell power for premable

f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE);

//@siclog "Step 1" siclog@

f_UT_SwitchOnUE(UT, false);

//@siclog "Step 1A" siclog@

//Check for no RRCConnectionRequest-NB message on Ncell 11

if (f_NBIOT_RRC_RRCConnectionRequest_Check(nbiot_Cell1, 360.0)) {

//Does the UE sent RRCConnectionRequest-NB message on Ncell 1 within 6 minutes

f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 1A : RRCConnectionRequest-NB message received on NCell 1 within 6 minutes");

}

//@siclog "Step 1B siclog@

f_UT_PowerOffUE (UT);

//@siclog "Step 1B siclog@

f_UT_SwitchOnUE(UT, false);

//@siclog "Step 1C siclog@

f_UT_ConfigureNTNPosition(UT);

//@siclog "Step 2 siclog@

if (f_NBIOT_RRC_RRCConnectionRequest_Check(nbiot_Cell1, 360.0)) {

//Does the UE sent RRCConnectionRequest-NB message on Ncell 1 within 6 minutes

f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 2 : RRCConnectionRequest-NB message received on NCell 1 within 6 minutes");

}

v_HarqProcessNumber := f_NBIOT_CellInfo_GetDefaultHarqProcessNumber(nbiot_Cell1);

f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant(cs_NB_DciInfo_CcchDcchDtchUL(-, -, -, v_HarqProcessNumber))));

//@siclog "Steps 3 - 14 siclog@

//Steps 3 to 14 of the registration procedure described in TS 36.508 [18] subclause 8.1.5.2.3 are performed on Ncell 1.

f_NBIOT_RRC_ConnectionSetup_Def();

if (px_DoAttachWithoutPDN) {

v_ESMMessage := cr_ESM_DUMMY_MESSAGE;

} else {

v_ESMMessage := cr_CIOT_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, v_PDNType);

}

//The rrcConnectionSetupComlete should include IE gnss-ValidityDuration-r17, but contents are not checked according to Table 22.1.2.3.3-1

v_RRCConnectionSetupComplete_NB.message_.c1.rrcConnectionSetupComplete_r13.criticalExtensions.rrcConnectionSetupComplete_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.gnss_ValidityDuration_r17 := ?;

v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1,

v_RRCConnectionSetupComplete_NB,

cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected),

cr_CIOT_ATTACH_REQUEST(cr_AdditionalUpdateType_CIOT),

v_ESMMessage));

f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NasInd);

//@siclog "Step 15" siclog@

// The SS transmits an RRCConnectionRelease-NB message to release RRC connection and move to RRC_IDLE

f_NBIOT_RRC_ConnectionRelease (nbiot_Cell1);

f_NBIOT_TestBody_Set(false);

//Switch/power off UE

f_NBIOT_Postamble (nbiot_Cell1, CONTROL_PLANE, N1_IDLE);

}

## Change 4

| Function name | f_NBIOT_RRC_RRCConnectionRequest() |
| --- | --- |
| Reason for change | RRCConnectionRequest with Initial UE identity as “randomValue” should be allowed |
| Summary of change | Allow any value for initial UE identity |
| TTCN module | NBIOT_RRC_Templates.ttcn |
| MCC160 Comment | Accepted, conditional 36.508 CR approval, since current message contents are as per 8.1.6.1-10 in 36.508. |

Before change

function f_NBIOT_RRC_RRCConnectionRequest_Check(template (present) NBIOT_CellId_Type p_CellId,

float p_TimerValue,

template (present) EstablishmentCause_NB_r13 p_EstablishmentCause := cr_EstablishmentCause_NB_Any,

boolean p_JustCheck := false)

runs on NBIOT_PTC return boolean

{

var template NB_SRB_COMMON_IND v_SRB0_RrcPdu_IND := car_NB_SRB0_RrcPdu_IND(p_CellId, cr_RRCConnectionRequest_NB(p_EstablishmentCause));

timer t_Wait;

t_Wait.start(p_TimerValue);

alt {

// receive RRC Connection Request on SRB0 (CCCH):

[not p_JustCheck] SRB.receive(v_SRB0_RrcPdu_IND)

{

t_Wait.stop;

return true;

}

[p_JustCheck] SRB.check(receive(v_SRB0_RrcPdu_IND))

{

t_Wait.stop;

return true;

}

[] t_Wait.timeout

{

}

}

return false;

}

After change

function f_NBIOT_RRC_RRCConnectionRequest_Check(template (present) NBIOT_CellId_Type p_CellId,

float p_TimerValue,

template (present) EstablishmentCause_NB_r13 p_EstablishmentCause := cr_EstablishmentCause_NB_Any,

boolean p_JustCheck := false)

runs on NBIOT_PTC return boolean

{

var template NB_SRB_COMMON_IND v_SRB0_RrcPdu_IND := car_NB_SRB0_RrcPdu_IND(p_CellId, cr_RRCConnectionRequest_NB(p_EstablishmentCause, ?));

timer t_Wait;

t_Wait.start(p_TimerValue);

alt {

// receive RRC Connection Request on SRB0 (CCCH):

[not p_JustCheck] SRB.receive(v_SRB0_RrcPdu_IND)

{

t_Wait.stop;

return true;

}

[p_JustCheck] SRB.check(receive(v_SRB0_RrcPdu_IND))

{

t_Wait.stop;

return true;

}

[] t_Wait.timeout

{

}

}

return false;

}

## Additional change from R&S Correction to cs_NTN_TimingAdvanceConfigSemistatic

| template name | cs_NTN_TimingAdvanceConfigSemistatic |
| --- | --- |
| Reason for change | The current UE Specific value 7477248 results in 243.4msec, the value is rounded up to the first digit in msec. The UE calculated value is 243.38955, therefore it is proposed to use 7476928 The proposed changes required an update to 36.523-3 |
| Summary of change | Updated NtnUeSpecificTA 7476928 |
| TTCN module | NBIOT_CellCfg_Templates |
| MCC160 Comment | Accepted in principle, but will be implemented differently according to the CR for 36.523-3 |

Before Change:

template (value) NTN_TimingAdvanceConfig_Type cs_NTN_TimingAdvanceConfigSemistatic := { /* @status APPROVED (NBIOT) */ Semistatic := { InitialValue := 0, NtnCommonTA := 8243100, //acc 36.508 Table 8.1.4.3.3-10 NtnUeSpecificTA := 7477248 //acc 36.523-3 cl.7A.14.2.2 } }

After Change:

template (value) NTN_TimingAdvanceConfig_Type cs_NTN_TimingAdvanceConfigSemistatic := { /* @status APPROVED (NBIOT) */ Semistatic := { InitialValue := 0, NtnCommonTA := 8243100, //acc 36.508 Table 8.1.4.3.3-10 NtnUeSpecificTA := 7476928 //acc 36.523-3 cl.7A.14.2.2 . } }

## Additional change from R&S f_NBIOT_RRC_ConnectionRelease_Common

| Function name | f_NBIOT_RRC_ConnectionRelease_Common |
| --- | --- |
| Reason for change | Due to the delay being configured with Koffset of 550, the time it takes for the 2 DL PDU data to sent and takes longer as the Harq process has to wait for HARQ ACK before the next data can be sent. This causes the SS not able to honour the current activation time calculation specified in TTCN (36.523-3 sec 7A.7 requires updates to NTN scenarios. The ESM data transfer message to complete the RS/RA procedure takes longer due to the configured delay. In order to calculate the activation time for a RRC Connection release message to be sent out, the current TTCN calculates this based on 300msec in the future (i.e v_MilliSecondsDelay := 300), if there is an existing grant already sent to the UE , then the next scheduling option for SS will be greater than 550msec (due to the Koffset value), therefore the SS cannot guarantee that the RRC Connection release will be sent at the activation time specified by TTCN |
| Summary of change | Configured a delay of 1.28sec (640+640) before the RRC Connection release procedure is started. Updated v_MilliSecondsDelay := 640 (10* 64 (space cycles of 64ms)) |
| TTCN module | NBIOT_RRCSteps |
| MCC160 Comment | Postponed, has to be investigated in more details |

Before Change:

function f_NBIOT_RRC_ConnectionRelease_Common(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, template (value) DL_DCCH_Message_NB p_RRCConnectionRelease, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NB_SRB_Identity_Type p_SrbId := omit) runs on NBIOT_PTC { /* @sic R5s170250: p_IOT_State instead of p_SrbId sic@ */ var NB_SRB_Identity_Type v_SrbId; var SubFrameTiming_Type v_TimingAtT; // time T: sending of RRCConnectionRelease acc. to 36.523-3 clause 7A.7 var SubFrameTiming_Type v_TimingNow := f_NBIOT_GetCurrentTiming(p_CellId); var integer v_MilliSecondsDelay := 300; var integer v_Duration; timer t_Timer; if (isvalue(p_SrbId)) { // @sic R5s180217 sic@ v_SrbId := valueof(p_SrbId); } else { v_SrbId := f_NBIOT_GetSrb1Id(p_IOT_State); } if (f_TimingInfo_IsNow(p_TimingInfo)) { v_TimingAtT := f_NBIOT_GetNextSearchSpace(p_CellId, v_MilliSecondsDelay); } else { // dedicated timing info is given v_TimingAtT := valueof(p_TimingInfo.SubFrame); } v_Duration := f_SubFrameTiming_Duration(v_TimingNow, v_TimingAtT) + tsc_NBIOT_DelayAfterRRCConnectionRelease; /* @sic R5s170263: tsc_NBIOT_DelayAfterRRCConnectionRelease sic@ */ t_Timer.start(int2float(v_Duration) / 1000.0); SRB.send(cas_NB_SRB_RrcPdu_REQ(p_CellId, v_SrbId, cs_TimingInfo_EUTRA_NB(v_TimingAtT), p_RRCConnectionRelease)); f_NBIOT_RRC_ConnectionRelease_Local(p_CellId, p_IOT_State, v_TimingAtT); alt { [] t_Timer.timeout {} [] SRB.check(receive(car_NB_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest_NB(cr_EstablishmentCause_NB_Any, ?)))) { /* @sic R5s170366: ? as InitialUE_Identity sic@ */ // there is an RRCConnectionRequest sent by the UE which can be received by TTCN-3 in a subsequent function t_Timer.stop; } } }

After Change:

function f_NBIOT_RRC_ConnectionRelease_Common(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, template (value) DL_DCCH_Message_NB p_RRCConnectionRelease, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NB_SRB_Identity_Type p_SrbId := omit) runs on NBIOT_PTC { /* @sic R5s170250: p_IOT_State instead of p_SrbId sic@ */ var NB_SRB_Identity_Type v_SrbId; var SubFrameTiming_Type v_TimingAtT; // time T: sending of RRCConnectionRelease acc. to 36.523-3 clause 7A.7 var SubFrameTiming_Type v_TimingNow := f_NBIOT_GetCurrentTiming(p_CellId); var integer v_MilliSecondsDelay := 300; var integer v_Duration; // timer t_Timer; if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO){ f_Delay (1.28); v_MilliSecondsDelay := 640; //(640 taking into account Koffset } if (isvalue(p_SrbId)) { // @sic R5s180217 sic@ v_SrbId := valueof(p_SrbId); } else { v_SrbId := f_NBIOT_GetSrb1Id(p_IOT_State); } if (f_TimingInfo_IsNow(p_TimingInfo)) { v_TimingAtT := f_NBIOT_GetNextSearchSpace(p_CellId, v_MilliSecondsDelay); } else { // dedicated timing info is given v_TimingAtT := valueof(p_TimingInfo.SubFrame); } v_Duration := f_SubFrameTiming_Duration(v_TimingNow, v_TimingAtT) + tsc_NBIOT_DelayAfterRRCConnectionRelease; /* @sic R5s170263: tsc_NBIOT_DelayAfterRRCConnectionRelease sic@ */ t_Timer.start(int2float(v_Duration) / 1000.0); SRB.send(cas_NB_SRB_RrcPdu_REQ(p_CellId, v_SrbId, cs_TimingInfo_EUTRA_NB(v_TimingAtT), p_RRCConnectionRelease)); f_NBIOT_RRC_ConnectionRelease_Local(p_CellId, p_IOT_State, v_TimingAtT); alt { [] t_Timer.timeout {} [] SRB.check(receive(car_NB_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest_NB(cr_EstablishmentCause_NB_Any, ?)))) { /* @sic R5s170366: ? as InitialUE_Identity sic@ */ // there is an RRCConnectionRequest sent by the UE which can be received by TTCN-3 in a subsequent function t_Timer.stop; } } }

## Additional change from R&S f_NBIOT_RRC_ConnectionRelease_Local

| Function name | f_NBIOT_RRC_ConnectionRelease_Local |
| --- | --- |
| Reason for change | The current procedure defined in 36.523-3 Sec 7A.7 requires updates to local end release procedure for e.g. Step 2 (T+64) will not be possible due to an ongoing DL transmission The activation time calculation and sending of UL grant needs to take into consideration the Koffset values |
| Summary of change | Step 2 updated to T+678, Step 3 updated to T+ 694 Step 4 updated to T+ 710 The proposed changes required an update to 36.523-3 |
| TTCN module | NBIOT_RRCSteps |
| MCC160 Comment | Postponed, has to be investigated in more details |

Before Change:

function f_NBIOT_RRC_ConnectionRelease_Local(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, SubFrameTiming_Type p_TimingAtT) runs on NBIOT_PTC { /* NOTE: the function schedules all steps to release and reconfigure the SRBs (and default DRB) * => this will in fact happen in the future */ var SubFrameTiming_Type v_Timing := p_TimingAtT; var template (value) TimingInfo_Type v_TimingInfo; var DRB_IdentityList_Type v_DrbIdList := f_NBIOT_SS_ActiveDRBs(p_CellId, p_IOT_State); // @sic R5s180547 sic@ var template (omit) integer v_HarqProcessNumber := f_NBIOT_CellInfo_GetDefaultHarqProcessNumber(p_CellId); /* @sic R5w180305: support of two HARQ processes sic@ */ /* Step 1 at T: disable the UL grant */ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_ULGrantTransmission(p_CellId, cs_NB_UL_GrantScheduling_Stop, v_TimingInfo); // @sic R5s220319 - MCC160 implementation: use of timing information according to 36.523-3 clause 7A.7 sic@ /* Step 2 at T+64ms */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 64); v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); if (p_IOT_State == USER_PLANE) { /* release security */ f_NBIOT_SS_RRC_ReleaseSecurity(p_CellId, v_TimingInfo); } f_NBIOT_ULGrantTransmission(p_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(cs_NB_DciInfo_CcchDcchDtchUL(-, -, -, v_HarqProcessNumber), /* @sic R5w180305: support of two HARQ processes sic@ */ 4)), /* @sic R5-177097: give 4 sub-sequent UL grants to allow the UE to do RLC ACK sic@ */ v_TimingInfo); /* Step 3 at T+80ms: Release SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 80); v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_SS_SRBs_DRBs_Release(p_CellId, p_IOT_State, v_TimingInfo, v_DrbIdList); // @sic R5s180547: v_DrbIdList sic@ /* Step 4 at T+96ms: Configure SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 96); v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_SS_SRBs_DRBs_Config(p_CellId, p_IOT_State, v_TimingInfo, omit); }

After Change:

function f_NBIOT_RRC_ConnectionRelease_Local(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, SubFrameTiming_Type p_TimingAtT) runs on NBIOT_PTC { /* NOTE: the function schedules all steps to release and reconfigure the SRBs (and default DRB) * => this will in fact happen in the future */ var SubFrameTiming_Type v_Timing := p_TimingAtT; var template (value) TimingInfo_Type v_TimingInfo; var DRB_IdentityList_Type v_DrbIdList := f_NBIOT_SS_ActiveDRBs(p_CellId, p_IOT_State); // @sic R5s180547 sic@ var template (omit) integer v_HarqProcessNumber := f_NBIOT_CellInfo_GetDefaultHarqProcessNumber(p_CellId); /* @sic R5w180305: support of two HARQ processes sic@ */ /* Step 1 at T: disable the UL grant */ v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_ULGrantTransmission(p_CellId, cs_NB_UL_GrantScheduling_Stop, v_TimingInfo); // @sic R5s220319 - MCC160 implementation: use of timing information according to 36.523-3 clause 7A.7 sic@ /* Step 2 at T+64ms */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 64); if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO){ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 678); //(64+550+64 taking into account Koffset } v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); if (p_IOT_State == USER_PLANE) { /* release security */ f_NBIOT_SS_RRC_ReleaseSecurity(p_CellId, v_TimingInfo); } f_NBIOT_ULGrantTransmission(p_CellId, cs_NB_UL_GrantScheduling_Start(cs_NB_PeriodicGrant(cs_NB_DciInfo_CcchDcchDtchUL(-, -, -, v_HarqProcessNumber), /* @sic R5w180305: support of two HARQ processes sic@ */ 4)), /* @sic R5-177097: give 4 sub-sequent UL grants to allow the UE to do RLC ACK sic@ */ v_TimingInfo); /* Step 3 at T+80ms: Release SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 80); if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO){ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 694); //(80+550+64 taking into account Koffset } v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_SS_SRBs_DRBs_Release(p_CellId, p_IOT_State, v_TimingInfo, v_DrbIdList); // @sic R5s180547: v_DrbIdList sic@ /* Step 4 at T+96ms: Configure SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 96); if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO){ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 710); //(96+550+64 taking into account Koffset } v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_Timing); f_NBIOT_SS_SRBs_DRBs_Config(p_CellId, p_IOT_State, v_TimingInfo, omit); }

# Branches executed

The test case was executed on NB-IoT band 255 using ciphering algorithm eea1 and integrity algorithm eia1

# Execution Log Files

## MediaTek Dimensity D930

The MediaTek Dimensity D930 passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_22_1_2_LOG.html

- PICS/PIXIT parameter file:

TC_22_1_2_PIXIT.xml

# References

| [1] | R5s230479 : Supporting information for addition of Rel-17 NB-IoT NTN test case 22.1.2 |
| --- | --- |