3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230486

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0096 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 6.1.1.19 to the MCX ATS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2023-06-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add MCPTT TC 6.1.1.19 to the MCX ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX IPCAN test model. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to the MCX ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.19.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Logs in R5s230487 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Nemergent Solutions Client v7.0	3

5	References	4

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.1.1.19.MCPTT which is part of the ATS iwd-TTCN3-B2022-09_D23wk12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Endika Aldecoa
	endika.aldecoa@ehu.eus

## Verification Test Summary

Test Case: 	6.1.1.19.MCPTT

ATS Version:	ATS iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	MCSTaaSting Tester

UE used:	Nemergent Client v7.0 on COTS LTE UE

Verification Status:	PASS

# Corrections required

## Change 1

Change1 of R5s230484

## Additional changes by MCC160

In order to simplify the handling of elements in <anyExt> of mcptt-info, mcvideo-info and mcdata-info TTCN is changed to do the decoding and check of elements in <anyExt> together with the decoding and check of mcptt-info, mcvideo-info and mcdata-info in f_MCX_MIME_DecodeAndMatchXML.

MCX_MIME_Functions

type record MCX_MIME_XmlBodies_Type { MCPTT_Info_Type McpttInfo optional, McpttInfoExt_ElementList_Type McpttInfoExt optional, MCVideo_Info_Type McvideoInfo optional, McvideoInfoExt_ElementList_Type McvideoInfoExt optional, MCData_Info_Type McdataInfo optional, McdataInfoExt_ElementList_Type McdataInfoExt optional, MCPTT_LocationInfo_Type McpttLocationInfo optional, MCVideo_LocationInfo_Type McvideoLocationInfo optional, MCData_LocationInfo_Type McdataLocationInfo optional, ResourceLists_Type ResourceLists optional, MCX_SimpleFilter_Type SimpleFilter optional, MCX_PIDF_Type PIDF optional, MCX_Affiliation_CommandList_Type AffiliationCmd optional, PocSettings_Type PocSettings optional };

template (value) MCX_MIME_XmlBodies_Type cs_MCX_MIME_XmlBodies_AllOmit := {*/ McpttInfo := omit, McpttInfoExt := omit, McvideoInfo := omit, McvideoInfoExt := omit, McdataInfo := omit, McdataInfoExt := omit, McpttLocationInfo := omit, McvideoLocationInfo := omit, McdataLocationInfo := omit, ResourceLists := omit, SimpleFilter := omit, PIDF := omit, AffiliationCmd := omit, PocSettings := omit };

function f_MCX_MIME_DecodeAndMatchXML(MIME_Message p_MIME_Message, MCX_ServiceType_Type p_ServiceType, MCX_KeyIndex_Type p_KeyIndex := CSK, template MCPTT_Info_Type p_McpttInfo := omit, template McpttInfoExt_ElementList_Type p_McpttInfoExt := omit, template MCVideo_Info_Type p_McvideoInfo := omit, template McvideoInfoExt_ElementList_Type p_McvideoInfoExt := omit, template MCData_Info_Type p_McdataInfo := omit, template McdataInfoExt_ElementList_Type p_McdataInfoExt := omit, template MCPTT_LocationInfo_Type p_McpttLocationInfo := omit, template MCVideo_LocationInfo_Type p_McvideoLocationInfo := omit, template MCData_LocationInfo_Type p_McdataLocationInfo := omit, template ResourceLists_Type p_ResourceLists := omit, template PocSettings_Type p_PocSettings := omit, template MCX_PIDF_Type p_PIDF := omit, template MCX_Affiliation_CommandList_Type p_AffiliationCmd := omit, template MCX_SimpleFilter_Type p_SimpleFilter := omit) runs on MCX_IMS_PTC return MCX_MIME_XmlBodies_Type { var charstring v_MCServiceID := f_MCX_MCService_ID_User_A(p_ServiceType); var template (omit) MCX_SecurityKey_Type v_CSK := f_MCX_IMS_PTC_SecurityParameters_GetKey(p_KeyIndex); var template (value) MCX_MIME_XmlBodies_Type v_XmlBodies := cs_MCX_MIME_XmlBodies_AllOmit; v_XmlBodies.McpttInfo := fl_MIME_DecodeAndMatchMcpttInfo(p_MIME_Message, p_McpttInfo, v_MCServiceID, v_CSK); v_XmlBodies.McvideoInfo := fl_MIME_DecodeAndMatchMcvideoInfo(p_MIME_Message, p_McvideoInfo, v_MCServiceID, v_CSK); v_XmlBodies.McdataInfo := fl_MIME_DecodeAndMatchMcdataInfo(p_MIME_Message, p_McdataInfo, v_MCServiceID, v_CSK); v_XmlBodies.McpttLocationInfo := fl_MIME_DecodeAndMatchMcpttLocation(p_MIME_Message, p_McpttLocationInfo, v_MCServiceID, v_CSK); v_XmlBodies.McvideoLocationInfo := fl_MIME_DecodeAndMatchMcvideoLocation(p_MIME_Message, p_McvideoLocationInfo, v_MCServiceID, v_CSK); v_XmlBodies.McdataLocationInfo := fl_MIME_DecodeAndMatchMcdataLocation(p_MIME_Message, p_McdataLocationInfo, v_MCServiceID, v_CSK); v_XmlBodies.ResourceLists := fl_MIME_DecodeAndMatchResourceLists(p_MIME_Message, p_ResourceLists, v_MCServiceID, v_CSK); v_XmlBodies.SimpleFilter := fl_MIME_DecodeAndMatchSimpleFilter(p_MIME_Message, p_SimpleFilter); v_XmlBodies.PIDF := fl_MIME_DecodeAndMatchPIDF(p_MIME_Message, p_PIDF, p_ServiceType, v_MCServiceID, v_CSK); v_XmlBodies.AffiliationCmd := fl_MIME_DecodeAndMatchAffiliationCmd(p_ServiceType, p_MIME_Message, p_AffiliationCmd); v_XmlBodies.PocSettings := fl_MIME_DecodeAndMatchPocSettings(p_MIME_Message, p_PocSettings); if (isvalue (v_XmlBodies.McpttInfo)) { v_XmlBodies.McpttInfoExt := f_MCX_McpttInfoExt_DecodeAndMatch(valueof(v_XmlBodies.McpttInfo), p_McpttInfoExt); } if (isvalue (v_XmlBodies.McvideoInfo)) { v_XmlBodies.McvideoInfoExt := f_MCX_McvideoInfoExt_DecodeAndMatch(valueof(v_XmlBodies.McvideoInfo), p_McvideoInfoExt); } if (isvalue (v_XmlBodies.McdataInfo)) { v_XmlBodies.McdataInfoExt := f_MCX_McdataInfoExt_DecodeAndMatch(valueof(v_XmlBodies.McdataInfo), p_McdataInfoExt); } // Check Signature f_MCX_MIME_CheckSignatures(p_MIME_Message, p_ServiceType, v_CSK); return valueof(v_XmlBodies); }

MCX_XML_mcptt_info

template (value) McpttInfoExt_Element_Type crs_McpttInfoExt_AmbientListening(AmbientListeningType p_AmbientListening) := { AmbientListening := p_AmbientListening }; template (present) McpttInfoExt_Element_Type cr_McpttInfoExt_FunctionalAliasURI(XSD.AnyURI p_URI) := { FunctionalAliasURI := cr_MCPTT_Content_URI(p_URI) };

function fl_MCX_McpttInfoExt_Decode(template (omit) AnyExtType p_McpttInfoExt) runs on IMS_PTC return McpttInfoExt_ElementList_Type { // variables for decoding var Ambient_listening_type v_Ambient_listening_type; var Release_reason v_Release_reason; var Request_type v_Request_type; var Response_type v_Response_type; var Urgency_ind v_Urgency_ind; var Time_of_request v_Time_of_request; var Selected_group_change_outcome v_Selected_group_change_outcome; var Affiliation_required v_Affiliation_required; var Remotely_initiated_call_outcome v_Remotely_initiated_call_outcome; var Notify_remote_user v_Notify_remote_user; var Functional_alias_URI v_Functional_alias_URI; // other variables var AnyExtType v_McpttInfoExt; var McpttInfoExt_ElementList_Type v_McpttInfoExtElementList := {}; var McpttInfoExt_Element_Type v_Element; var bitstring v_Bitstring; var integer i; if (isvalue(p_McpttInfoExt)) { v_McpttInfoExt := valueof(p_McpttInfoExt); for (i := 0; i < lengthof(v_McpttInfoExt.elem_list); i := i + 1) { v_Bitstring := f_Unichar2Bit(v_McpttInfoExt.elem_list[i]); if (decvalue(v_Bitstring, v_Ambient_listening_type) == 0) { v_Element.AmbientListening := v_Ambient_listening_type; } else if (decvalue(v_Bitstring, v_Release_reason) == 0) { v_Element.ReleaseReason := v_Release_reason; } else if (decvalue(v_Bitstring, v_Request_type) == 0) { v_Element.RequestType := v_Request_type; } else if (decvalue(v_Bitstring, v_Response_type) == 0) { v_Element.ResponseType := v_Response_type; } else if (decvalue(v_Bitstring, v_Urgency_ind) == 0) { v_Element.UrgencyInd := v_Urgency_ind; } else if (decvalue(v_Bitstring, v_Time_of_request) == 0) { v_Element.TimeOfRequest := v_Time_of_request; } else if (decvalue(v_Bitstring, v_Selected_group_change_outcome) == 0) { v_Element.SelectedGroupChangeOutcome := v_Selected_group_change_outcome; } else if (decvalue(v_Bitstring, v_Affiliation_required) == 0) { v_Element.AffiliationRequired := v_Affiliation_required; } else if (decvalue(v_Bitstring, v_Remotely_initiated_call_outcome) == 0) { v_Element.RemotelyInitiatedCallOutcome := v_Remotely_initiated_call_outcome; } else if (decvalue(v_Bitstring, v_Notify_remote_user) == 0) { v_Element.NotifyRemoteUser := v_Notify_remote_user; } else if (decvalue(v_Bitstring, v_Functional_alias_URI) == 0) { v_Element.FunctionalAliasURI := v_Functional_alias_URI; } else { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "mcpttinfo.mcptt-Params.anyExt: element[" & int2str(i) & "] cannot be decoded"); } v_McpttInfoExtElementList := v_McpttInfoExtElementList & { v_Element }; } } return v_McpttInfoExtElementList; }

function f_MCX_McpttInfoExt_DecodeAndMatch(MCPTT_Info_Type p_McpttInfo, template McpttInfoExt_ElementList_Type p_McpttInfoExtElementList) runs on IMS_PTC return McpttInfoExt_ElementList_Type { var McpttInfoExt_ElementList_Type v_McpttInfoExtElementList; var template (present) McpttInfoExt_ElementList_Type v_McpttInfoExtElementListPermutation; if (ispresent(p_McpttInfoExtElementList)) { v_McpttInfoExtElementList := fl_MCX_McpttInfoExt_Decode(p_McpttInfo.mcptt_Params.anyExt); v_McpttInfoExtElementListPermutation := { permutation(all from p_McpttInfoExtElementList) }; // !!!! "all from" may need to be removed !!!! if (not match(v_McpttInfoExtElementList, v_McpttInfoExtElementListPermutation)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "mcpttinfo.mcptt-Params.anyExt does not match"); } } return v_McpttInfoExtElementList; }

MCX_SIP_CallControl_MCPTT_MCVideo

function f_MCX_InviteMO_ReceiveINVITE(MCX_ServiceType_Type p_ServiceType, MCX_CallType_Type p_CallType, MCX_PriorityCallType_Type p_PriorityCallType := NORMAL_CALL, MCX_CommencementMode_Type p_CommencementMode := AUTOMATIC, template (present) MCX_SdpInfoRX_Type p_SdpInfoRX, template (present) MCX_MIME_XmlBodies_Type p_XmlBodies, boolean p_IsReInvite := false) runs on MCX_IMS_PTC return MCX_InviteRequestWithSdpAndXml_Type { var charstring v_SessionId; var charstring v_PublicServiceId; var template (present) SipUrl v_RequestUri; var template (present) MessageHeader v_MessageHeader_Invite; var IMS_DATA_REQ v_IMS_DATA_REQ; var MessageBody v_MessageBody; var MIME_Message v_MIME_Message; var SDP_Message v_SDP_Message; var MCX_MIME_XmlBodies_Type v_XmlBodies; var boolean v_AlertInd; var boolean v_LocationInfoIsPresent; if (not p_IsReInvite) { /* @sic R5w220111, R5-223945: SessionId different from PSI sic@ */ v_SessionId := f_MCX_SessionId(p_ServiceType); v_PublicServiceId := f_MCX_MCServer_PublicServiceId(p_ServiceType); v_RequestUri := f_SIP_BuildSipUri_RX(v_PublicServiceId); f_MCX_IMS_PTC_ImsInfo_SetContactUrlSS(v_SessionId); /* @sic R5w220111, R5-223945: global variable ContactUrlSS sic@ */ } else { v_RequestUri := f_IMS_PTC_ImsInfo_GetContactUrlSS(); } v_MessageHeader_Invite := f_MCX_InviteRequest_MessageHeaderRX(p_ServiceType, -, p_PriorityCallType, p_CommencementMode, v_RequestUri, p_IsReInvite); v_IMS_DATA_REQ := valueof(f_MCX_InviteMO_ReceiveRequest_Optional(v_RequestUri, v_MessageHeader_Invite, cr_MessageBody_MIME, -, p_IsReInvite)); v_MessageBody := v_IMS_DATA_REQ.Request.Invite.messageBody; v_MIME_Message := v_MessageBody.mimeMessageBody; v_SDP_Message := valueof(f_MCX_DecodeMatchAndCheckSDP(p_ServiceType, v_MessageBody, p_SdpInfoRX)); f_MCX_CallSetupMO_ConfigSRTP(p_ServiceType, p_CallType); /* @sic R5s211469: SRTP config independent from p_IsReInvite sic@ */ if (not p_IsReInvite) { f_IMS_IPCAN_SendCoOrdMsg(IPCAN); /* trigger IPCAN to continue with activation of dedicated EPS bearer */ } v_XmlBodies := f_MCX_MIME_DecodeAndMatchXML(v_MIME_Message, p_ServiceType, -, p_XmlBodies.McpttInfo, p_XmlBodies.McpttInfoExt, p_XmlBodies.McvideoInfo, p_XmlBodies.McvideoInfoExt, -, -, p_XmlBodies.McpttLocationInfo, p_XmlBodies.McvideoLocationInfo, -, p_XmlBodies.ResourceLists); if (p_PriorityCallType == EMERGENCY_CALL) { // the UE may set alert-ind:=true in which case it shall include location info select (p_ServiceType) { case (MCPTT) { v_AlertInd := isvalue(v_XmlBodies.McpttInfo.mcptt_Params.alert_ind) and match(v_XmlBodies.McpttInfo.mcptt_Params.alert_ind, cr_MCPTT_Content_True); v_LocationInfoIsPresent := isvalue(v_XmlBodies.McpttLocationInfo); } case (MCVideo) { v_AlertInd := isvalue(v_XmlBodies.McvideoInfo.mcvideo_Params.alert_ind) and match(v_XmlBodies.McvideoInfo.mcvideo_Params.alert_ind, cr_MCVideo_Content_True); v_LocationInfoIsPresent := isvalue(v_XmlBodies.McvideoLocationInfo); } } if (v_AlertInd and not v_LocationInfoIsPresent) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "alert-ind==true in mcptt/mcvideo-info but mcptt/mcvideo-location-info is not present"); } else if (not v_AlertInd and v_LocationInfoIsPresent) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "alert-ind==false in mcptt/mcvideo-info but mcptt/mcvideo-location-info is present"); } } return fl_MCX_InviteRequestWithSdpAndXmlMO(v_IMS_DATA_REQ, v_SDP_Message, v_XmlBodies); }

function f_MCX_CallSetupMO_ReceiveINVITE_Send100Trying(MCX_ServiceType_Type p_ServiceType, MCX_CallType_Type p_CallType, MCX_PriorityCallType_Type p_PriorityCallType := NORMAL_CALL, MCX_CommencementMode_Type p_CommencementMode := AUTOMATIC, MCX_MediaControlCondition_Type p_MediaControlCondition := withoutImplictGrantRequest, template (omit) XSD.AnyURI p_FunctionalAliasURI := omit) runs on MCX_IMS_PTC return MCX_InviteRequestWithSdpAndXml_Type { var template (present) MCX_SdpInfoRX_Type v_SdpInfoRX := f_MCX_SdpInfo_RX(p_ServiceType, ON_DEMAND, p_CallType, SDP_OFFER, p_MediaControlCondition); var template (present) MCX_MIME_XmlBodies_Type v_XmlBodies := f_MCX_InviteMO_XmlBodies_Init(p_ServiceType, p_CallType, p_PriorityCallType); var MCX_InviteRequestWithSdpAndXml_Type v_InviteRequestWithSdpAndXml; if (isvalue(p_FunctionalAliasURI)) { select (p_ServiceType) { case (MCPTT) { v_XmlBodies.McpttInfoExt := { cr_McpttInfoExt_FunctionalAliasURI(valueof(p_FunctionalAliasURI)) }; } case (MCVideo) { FatalError(__FILE__, __LINE__, "Functional alias not supoorted for MCVideo"); } } } v_InviteRequestWithSdpAndXml := f_MCX_InviteMO_ReceiveINVITE(p_ServiceType, p_CallType, p_PriorityCallType, p_CommencementMode, v_SdpInfoRX, v_XmlBodies); f_MCX_InviteMO_Send100Trying(v_InviteRequestWithSdpAndXml.InviteRequestWithSdp); return v_InviteRequestWithSdpAndXml; }

MCX_SIP_Common

function f_MCX_ReceiveMESSAGE_Send20xResponse_MIME(MCX_ServiceType_Type p_ServiceType, charstring p_PublicServiceId, StatusLine p_StatusLineResponse, template MCPTT_Info_Type p_McpttInfo := omit, template McpttInfoExt_ElementList_Type p_McpttInfoExt := omit, template MCVideo_Info_Type p_McvideoInfo := omit, template McvideoInfoExt_ElementList_Type p_McvideoInfoExt := omit, template MCData_Info_Type p_McdataInfo := omit, template McdataInfoExt_ElementList_Type p_McdataInfoExt := omit, template MCX_Affiliation_CommandList_Type p_AffiliationCmd := omit, template ResourceLists_Type p_ResourceLists := omit, template MCPTT_LocationInfo_Type p_McpttLocationInfo := omit, template MCVideo_LocationInfo_Type p_McvideoLocationInfo := omit, template MCData_LocationInfo_Type p_McdataLocationInfo := omit) runs on MCX_IMS_PTC return MCX_MessageRequestWithXmlBodies_Type { var MESSAGE_Request v_MESSAGE_Request := f_MCX_ReceiveMESSAGE_Send20xResponse(p_ServiceType, p_PublicServiceId, p_StatusLineResponse); var MIME_Message v_MIME_Message := v_MESSAGE_Request.messageBody.mimeMessageBody; var MCX_MIME_XmlBodies_Type v_XmlBodies; v_XmlBodies := f_MCX_MIME_DecodeAndMatchXML(v_MIME_Message, p_ServiceType, -, p_McpttInfo, p_McpttInfoExt, p_McvideoInfo, p_McvideoInfoExt, p_McdataInfo, p_McdataInfoExt, p_McpttLocationInfo, p_McvideoLocationInfo, p_McdataLocationInfo, p_ResourceLists, -, -, p_AffiliationCmd); return valueof(cs_MCX_MessageRequestWithXmlBodies(v_MESSAGE_Request, v_XmlBodies)); }

function f_MCPTT_ReceiveMESSAGE_Send200OK(template MCPTT_Info_Type p_McpttInfo := omit, template McpttInfoExt_ElementList_Type p_McpttInfoExt := omit, template MCX_Affiliation_CommandList_Type p_AffiliationCmd := omit, template ResourceLists_Type p_ResourceLists := omit, template MCPTT_LocationInfo_Type p_McpttLocationInfo := omit) runs on MCX_IMS_PTC return MCX_MIME_XmlBodies_Type { var MCX_MessageRequestWithXmlBodies_Type v_MessageRequestWithXmlBodies; var charstring v_PublicServiceId := f_MCX_MCServer_PublicServiceId(MCPTT); if (ischosen(p_McpttLocationInfo.choice.report)) { // => condition LOCATION-REPORT, 24.379 clause 13.2.2 v_PublicServiceId := tsc_MCPTT_PublicServiceId_PF_A; } v_MessageRequestWithXmlBodies := f_MCX_ReceiveMESSAGE_Send20xResponse_MIME(MCPTT, v_PublicServiceId, c_statusLine200, p_McpttInfo, p_McpttInfoExt, -, -, p_AffiliationCmd, p_ResourceLists, p_McpttLocationInfo); return v_MessageRequestWithXmlBodies.XmlBodies; }

MCPTT_GroupCall_MCX_IMS

function f_TC_6_1_1_19_MCPTT_MCX_IMS() runs on MCX_IMS_PTC { // On-network / On-demand Pre-arranged Group Call / Active functional alias / Client Originated (CO) var MCX_InviteRequestWithSdpAndXml_Type v_InviteRequestWithSdpAndXml; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_MCX_Preamble_IMS(MCPTT, IPCAN_SpeechCall, REGISTRED_AUTHORIZED_FA_ACTIVATED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_MCPTT_CallSetupMO_InitiateCall("Make the UE (MCPTT client) request the establishment of a group call using the active functional alias with automatic commencement mode and implicit floor request"); // @siclog "Step 2" siclog@ v_InviteRequestWithSdpAndXml := f_MCX_CallSetupMO_ReceiveINVITE_Send100Trying(MCPTT, GROUP_CALL, -, -, withImplictGrantRequest, px_MCPTT_ID_FA_A); v_InviteRequestWithSdp := v_InviteRequestWithSdpAndXml.InviteRequestWithSdp; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); f_MCX_CallSetupMO_Send200OK_ReceiveACK(MCPTT, -, v_InviteRequestWithSdp); f_MCPTT_FloorControl_SendFloorGranted(NORMAL_CALL); // @siclog "Step 3" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (MCPTT client) provide floor granted notification to the user?", "Step 3"); // @siclog "Step 4" siclog@ f_UT_MCX_UserAction(MMI, "Make the UE (MCPTT client) release the call"); // @siclog "Step 5" siclog@ f_MCX_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 5"); f_IMS_TestBody_Set(false); f_MCX_Postamble_IMS(); }

Further changes:

- f_MCX_PreestablishedSession_CallSetupMO_ReceiveREFER
Parameters p_McpttInfoExtElementList and p_McvideoInfoExtElementList added and handed through to f_MCX_MIME_DecodeAndMatchXML

- f_MCX_PreestablishedSession_CallSetupMO_ReceiveREFER_Send200OK
Parameters p_McpttInfoExtElementList and p_McvideoInfoExtElementList added and handed through to f_MCX_PreestablishedSession_CallSetupMO_ReceiveREFER

- f_MCPTT_PreestablishedSession_CallSetupMO
Parameter p_McpttInfoExtElementList added and handed through to f_MCX_PreestablishedSession_CallSetupMO_ReceiveREFER_Send200OK

# Branches executed

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE Band 1

- EEA2/EIA2 RRC/NAS ciphering/integrity algorithms

# Execution Log Files

## Nemergent Solutions Client v7.0

The Nemergent Solutions Client v7.0 on COTS LTE UE passed this test case against the MCSTaaSting Tester. The documentation below is enclosed as evidence of the successful test case run [1]:

- SS_Log_Coversheet.txt

- Signalling-TC6.1.1.19.MCPTT.html (RRC/NAS signalling, IP flow decrypted/decoded)

- MCX-PIXIT.cfg (Titan configuration and PIXITs)

- TLI Interface log

# References

| [1] | R5s230487: Supporting information for the addition of MCPTT test case 6.1.1.19 to the MCX ATS |
| --- | --- |