3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230492

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3113 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.6.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the prose CR R5-232067 agreed on RAN5#99, E-UTRA Cell 1 power level has to be -96 for FR1 ( Please refer the table Table 8.1.6.2.4.3.2-1: Time instances of cell power level and parameter changes for FR1 From 38523-1 ) |  |  |  |  |  |  |  |  |
| Summary of change: |  | FR1 EUTRA power level has to be set as -96 instead of -85 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case and also results in Exception because MaxReferencePower of EUTRA Cell 1 is -90 and TTCN sets the power level as -85 for FR1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody	2

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

- Corrections required

  - Correction to the Function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody

| Template name | fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | As per the prose CR R5-232067 agreed on RAN5#99, E-UTRA Cell 1 power level has to be -96 for FR1 ( Please refer the table Table 8.1.6.2.4.3.2-1: Time instances of cell power level and parameter changes for FR1 From 38523-1 ) |
| Summary of change | FR1 EUTRA power level has to be set as -96 instead of -85 |
| TTCN module | NR5GC_IRAT\8_1_6\RRC_MDT_InterRAT_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Power Level"); if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA38_SetCellPower(eutra_Cell1, -85, -85); //@sic R5-230262 sic@ } else { f_EUTRA38_SetCellPower(eutra_Cell1, -96, -96); //@sic R5-232393 sic@) } }

After Change:

function fl_TC_8_1_6_2_4_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Power Level"); if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA38_SetCellPower(eutra_Cell1, -96, -96); //@sic R5-230262 sic@ } else { f_EUTRA38_SetCellPower(eutra_Cell1, -96, -96); //@sic R5-232393 sic@) } }