3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230498

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0183 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC positioning test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect GNSS signal name reported in fail verdict |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected failure message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 9.3.4.4.15s and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_POS_NR_CheckSignalsGalileo() |
| --- | --- |
| Reason for change | Incorrect GNSS signal name reported in fail verdict |
| Summary of change | Corrected failure message |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment | Accepted. |

Before change

function f_POS_NR_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GALILEO_E1_Reported := false;

var boolean v_GALILEO_E5a_Reported := false;

var boolean v_GALILEO_E5b_Reported := false;

var boolean v_GALILEO_E6_Reported := false;

var boolean v_GALILEO_E5aE5b_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GALILEO_E1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GALILEO_E5a_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GALILEO_E5b_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_GALILEO_E6_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) {

v_GALILEO_E5aE5b_Reported := true;

}

} //@sic R5s210887 sic@

if (not((v_GALILEO_E1_Reported == pc_GALILEO_E1) and (v_GALILEO_E5a_Reported == pc_GALILEO_E5a) and (v_GALILEO_E5b_Reported == pc_GALILEO_E5b) and

(v_GALILEO_E6_Reported == pc_GALILEO_E6) and (v_GALILEO_E5aE5b_Reported == pc_GALILEO_E5aE5b)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

After change

function f_POS_NR_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GALILEO_E1_Reported := false;

var boolean v_GALILEO_E5a_Reported := false;

var boolean v_GALILEO_E5b_Reported := false;

var boolean v_GALILEO_E6_Reported := false;

var boolean v_GALILEO_E5aE5b_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GALILEO_E1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GALILEO_E5a_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GALILEO_E5b_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_GALILEO_E6_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) {

v_GALILEO_E5aE5b_Reported := true;

}

} //@sic R5s210887 sic@

if (not((v_GALILEO_E1_Reported == pc_GALILEO_E1) and (v_GALILEO_E5a_Reported == pc_GALILEO_E5a) and (v_GALILEO_E5b_Reported == pc_GALILEO_E5b) and

(v_GALILEO_E6_Reported == pc_GALILEO_E6) and (v_GALILEO_E5aE5b_Reported == pc_GALILEO_E5aE5b)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GALILIEO signals not reported correctly");

}

}