3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230499

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0184 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EUTRA positioning test case 7.3.4.4_7s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2023-06-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current TTCN implementation uses function f_POS_LppMessage_DecodeAndMatch() before step 5a1/5b1. Since the LPP message is then matched after this function call for steps 5a1/5b1, it is only required to decode the received LPP message. Currently if the message does not match the template of step 5a1 the test case will fail in this function without trying the match for step 5b2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use the function f_POS_LppMessage_Decode() instead of f_POS_LppMessage_DecodeAndMatch(), before matchng for steps 5a1/5b1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4.4_7s |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | function f_TC_7_3_4_4_7s_EUTRA() |
| --- | --- |
| Reason for change | Current TTCN implementation uses function f_POS_LppMessage_DecodeAndMatch() before step 5a1/5b1. Since the LPP message is then matched after this function call for steps 5a1/5b1, it is only required to decode the received LPP message. Currently if the message does not match the template of step 5a1 the test case will fail in this function without trying the match for step 5b2. |
| Summary of change | Use the function f_POS_LppMessage_Decode() instead of f_POS_LppMessage_DecodeAndMatch(), before matchng for steps 5a1/5b1. |
| TTCN module | EUTRA_Positioning_Testcases.ttcn |
| MCC160 Comment | Accepted. |

Before change

function f_TC_7_3_4_4_7s_EUTRA() runs on EUTRA_POS_PTC
{

………

//@siclog "Step 4" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with OTDOA information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

omit,

cs_OtdoaProvideAssistanceData(v_Ecgi1, v_PrsInfo, v_Ecgi2, v_Ecgi4));

//@siclog "Step 4a" siclog@

//Receive LPP Request Assistance Data for A-GNSS

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, cr_LppReqAssistanceDataAgnss_Ack);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a");

// Get Transaction number sent by the UE

v_TransactionNumber := v_LppMessage.transactionID.transactionNumber;

v_RequestAssistData := v_LppMessage.lpp_MessageBody.c1.requestAssistanceData.criticalExtensions.c1.requestAssistanceData_r9;

// Retrieve requested data from Satellite simulator and store it in v_ProvideAssistanceData_ByRef

v_ProvideAssistData := f_POS_EUTRA_RetrieveData(v_GNSSList, //@sic R5-172963 sic@

v_RequestAssistData);

//@siclog "Step 4b" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with A-GNSS information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

v_ProvideAssistData.a_gnss_ProvideAssistanceData,

omit);

}

else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected Assistance data request");

}

//Receive LPP PROVIDE LOCATION INFORMATION

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber))); //@sic R5-152147 sic@

//@sic R5-152149 sic@

//@siclog "Step 5a1" siclog@

if (match(v_LppMessage, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber)))) { //@sic R5-152147 sic@

if ((isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

v_GNSS_MeasurementList := v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation.gnss_SignalMeasurementInformation.gnss_MeasurementList;

f_POS_CheckGnssMeasurementList(v_GNSS_MeasurementList);//@sic R5-198966, R5s210633 sic@

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation.otdoaSignalMeasurementInformation)) { //@sic R5-144626 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a1");

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "OTDOA Location Information present but SignalMeasurementInformation missing");  //@sic R5-144626 sic@

}

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GNSS or OTDOA Location Information not present");

}

//@siclog "Step 5a2" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1,

tsc_POS_RoutingId,

v_LppMessage);

}

//@siclog "Step 5b1" siclog@

else if (match (v_LppMessage, cr_LppProvideLocationInformation_NoError_EarlyFix(tsc_POS_TransactionNumber))) {

if (not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and //@sic R5s220799 sic@

not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Neither A-gnss Provide Location Information nor OTDOA Provide Location Information are present");

}

…………….

}

After change

function f_TC_7_3_4_4_7s_EUTRA() runs on EUTRA_POS_PTC
{

………

//@siclog "Step 4" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with OTDOA information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

omit,

cs_OtdoaProvideAssistanceData(v_Ecgi1, v_PrsInfo, v_Ecgi2, v_Ecgi4));

//@siclog "Step 4a" siclog@

//Receive LPP Request Assistance Data for A-GNSS

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, cr_LppReqAssistanceDataAgnss_Ack);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a");

// Get Transaction number sent by the UE

v_TransactionNumber := v_LppMessage.transactionID.transactionNumber;

v_RequestAssistData := v_LppMessage.lpp_MessageBody.c1.requestAssistanceData.criticalExtensions.c1.requestAssistanceData_r9;

// Retrieve requested data from Satellite simulator and store it in v_ProvideAssistanceData_ByRef

v_ProvideAssistData := f_POS_EUTRA_RetrieveData(v_GNSSList, //@sic R5-172963 sic@

v_RequestAssistData);

//@siclog "Step 4b" siclog@

//SS sends LPP PROVIDE ASSISTANCE DATA (with A-GNSS information)

f_POS_SendLppMessage(v_LppMessage,

v_TransactionNumber,

targetDevice,

v_ProvideAssistData.a_gnss_ProvideAssistanceData,

omit);

}

else {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected Assistance data request");

}

//Receive LPP PROVIDE LOCATION INFORMATION

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo(tsc_POS_RoutingId)))))

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_Decode(v_Octetstring);//@sic R5-152149 sic@

//@siclog "Step 5a1" siclog@

if (match(v_LppMessage, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber)))) { //@sic R5-152147 sic@

if ((isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

v_GNSS_MeasurementList := v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation.gnss_SignalMeasurementInformation.gnss_MeasurementList;

f_POS_CheckGnssMeasurementList(v_GNSS_MeasurementList);//@sic R5-198966, R5s210633 sic@

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation.otdoaSignalMeasurementInformation)) { //@sic R5-144626 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.4 Step 4a1");

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "OTDOA Location Information present but SignalMeasurementInformation missing");  //@sic R5-144626 sic@

}

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GNSS or OTDOA Location Information not present");

}

//@siclog "Step 5a2" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1,

tsc_POS_RoutingId,

v_LppMessage);

}

//@siclog "Step 5b1" siclog@

else if (match (v_LppMessage, cr_LppProvideLocationInformation_NoError_EarlyFix(tsc_POS_TransactionNumber))) {

if (not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and //@sic R5s220799 sic@

not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Neither A-gnss Provide Location Information nor OTDOA Provide Location Information are present");

}

……………

}