3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230501

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3120 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NR-DC Functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of the function f_NR_Security_InitSCG_ForDC : AS_Ciphering_S and AS_Integrity_S are re-initialized with MCG Ciphering and Integrity . This could overwrite SCG specific Algorithm values .In order to protect the SCG integrity and ciphering exisiting values , it is proposed to initialize the SCG Ciphering and integrity values with MCG Ciphering and integrity values only when the algorithm are matching between MCG and SCG. Otherwise, AS_Ciphering_S and AS_Integrity_S are expected to retain the existing values. In the current TTCN implementation of the function f_TC_7_1_3_5_2_NR5GC : as the PDCP is in TM mode , it is expected to set the MCG Ciphering as well in NEA0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_NR_Security_InitSCG_ForDC : initialize the SCG Ciphering and integrity values with MCG Ciphering and integrity values only when the algorithm are matching between MCG and SCG In the current TTCN implementation of the function f_TC_7_1_3_5_2_NR5GC : as the PDCP is in TM mode , it is expected to set the MCG Ciphering as well in NEA0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Change 1 is for all 7.1.3.5.x NR-DC cases and Change 2 is for 7.1.3.5.2.NR5GC |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function f_NR_Security_InitSCG_ForDC	3

2.2	Correction to the Function f_TC_7_1_3_5_2_NR5GC	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_NR_Security_InitSCG_ForDC

| Function name | f_NR_Security_InitSCG_ForDC |
| --- | --- |
| Reason for change | - In the current TTCN implementation of the function f_NR_Security_InitSCG_ForDC : AS_Ciphering_S and AS_Integrity_S are re-initialized with MCG Ciphering and Integrity . This could overwrite SCG specific Algorithm values .In order to protect the SCG integrity and ciphering exisiting values , it is proposed to initialize the SCG Ciphering and integrity values with MCG Ciphering and integrity values only when the algorithm are matching between MCG and SCG. Otherwise, AS_Ciphering_S and AS_Integrity_S are expected to retain the existing values. |
| Summary of change | - In the function f_NR_Security_InitSCG_ForDC : initialize the SCG Ciphering and integrity values with MCG Ciphering and integrity values only when the algorithm are matching between MCG and SCG |
| TTCN module | ttcn/develop/Common/NR/NR_SecurityFunctions.ttcn |
| MCC160 Comment | Accepted in principle. Assignments to v_SecurityParamsSCG.AS_Ciphering_S resp. v_SecurityParamsSCG.AS_Integrity_S are to be removed instead as these are done already inside f_NR_Security_Init. |

Before Change:

function f_NR_Security_InitSCG_ForDC(NR_SecurityParams_Type p_SecurityParams) return NR_SecurityParams_Type { var NR_SecurityParams_Type v_SecurityParamsSCG; // First, initialise SCG integrity/ciphering parameters (e.g. algorithms) with the existing values v_SecurityParamsSCG := p_SecurityParams; v_SecurityParamsSCG.AS_Ciphering_S := v_SecurityParamsSCG.AS_Ciphering; v_SecurityParamsSCG.AS_Integrity_S := v_SecurityParamsSCG.AS_Integrity; // Then update those specific for SCG v_SecurityParamsSCG.S_KGNB := f_NR_Authentication_A16 (v_SecurityParamsSCG.KDF, v_SecurityParamsSCG.KGNB, v_SecurityParamsSCG.SK_Counter); return v_SecurityParamsSCG; }

After Change:

function f_NR_Security_InitSCG_ForDC(NR_SecurityParams_Type p_SecurityParams) return NR_SecurityParams_Type { var NR_SecurityParams_Type v_SecurityParamsSCG; // First, initialise SCG integrity/ciphering parameters (e.g. algorithms) with the existing values v_SecurityParamsSCG := p_SecurityParams; if(v_SecurityParamsSCG.AS_Ciphering_S.Algorithm == v_SecurityParamsSCG.AS_Ciphering.Algorithm) { v_SecurityParamsSCG.AS_Ciphering_S := v_SecurityParamsSCG.AS_Ciphering; //WA#WI=1110266 } if(v_SecurityParamsSCG.AS_Integrity_S.Algorithm == v_SecurityParamsSCG.AS_Integrity.Algorithm ) { v_SecurityParamsSCG.AS_Integrity_S := v_SecurityParamsSCG.AS_Integrity; //WA#WI=1110266 } // Then update those specific for SCG v_SecurityParamsSCG.S_KGNB := f_NR_Authentication_A16 (v_SecurityParamsSCG.KDF, v_SecurityParamsSCG.KGNB, v_SecurityParamsSCG.SK_Counter); return v_SecurityParamsSCG; }

## Correction to the Function f_TC_7_1_3_5_2_NR5GC

| Function name | f_TC_7_1_3_5_2_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_TC_7_1_3_5_2_NR5GC : as the PDCP is in TM mode , it is expected to set the MCG Ciphering as well in NEA0 as per 38 523-3 V17.6.0 “On UE side Ciphering is enabled as null algorithm” |
| Summary of change | In the current TTCN implementation of the function f_TC_7_1_3_5_2_NR5GC : as the PDCP is in TM mode , it is expected to set the MCG Ciphering as well in NEA0 |
| TTCN module | ttcn/develop/NR5GC/7_1_3/PDCP_NR5GC.ttcn |
| MCC160 Comment | Rejected. not needed as per R5-232205 |

Before Change:

function f_TC_7_1_3_5_2_NR5GC() runs on NR5GC_PTC { var NR_PDCP_SS_State_Type v_NR_PDCP_State; var integer v_SCG_DRB_Id; var template (value) NR_RadioBearerList_Type v_SS_ConfigMCG; // @sic R5s230377 sic@ var template (value) NR_RadioBearerList_Type v_SS_ConfigSCG; // @sic R5s230377 sic@ //Create and configure NR cell1 f_NR5GC_Init_NRDC(NR_1,{nr_Cell1, nr_Cell10}); //@sic R5-215680 sic@ => to initialise the frequencies and cells as per NR-DC f_NR_CellConfig_Def(nr_Cell1); //@sic R5s210891 sic@ f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); //In NR-DC, we use inter-band cells => cell 1 + Cell 10. A prose CR is needed //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeA_ON); //run the preamble as normal and bring back UE to IDLE with TEST mode A on @sic R5s211471 sic@ f_NR_AS_CipheringAlgorithm_Set_SCG(nea0); //@sic R5s210888 sic@ // configure DRB with PDCP in TM at the start @sic R5s230377 sic@ v_SCG_DRB_Id := f_NR_GetNextSCGDRBId(); << SKIPPED CODE >> }

After Change:

function f_TC_7_1_3_5_2_NR5GC() runs on NR5GC_PTC { var NR_PDCP_SS_State_Type v_NR_PDCP_State; var integer v_SCG_DRB_Id; var template (value) NR_RadioBearerList_Type v_SS_ConfigMCG; // @sic R5s230377 sic@ var template (value) NR_RadioBearerList_Type v_SS_ConfigSCG; // @sic R5s230377 sic@ //Create and configure NR cell1 f_NR5GC_Init_NRDC(NR_1,{nr_Cell1, nr_Cell10}); //@sic R5-215680 sic@ => to initialise the frequencies and cells as per NR-DC f_NR_CellConfig_Def(nr_Cell1); //@sic R5s210891 sic@ f_NR_CellConfig_Def(nr_Cell10, -, tsc_NRDC_SCG_Cell); //In NR-DC, we use inter-band cells => cell 1 + Cell 10. A prose CR is needed //Preamble f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeA_ON); //run the preamble as normal and bring back UE to IDLE with TEST mode A on @sic R5s211471 sic@ f_NR_AS_CipheringAlgorithm_Set_SCG(nea0); //@sic R5s210888 sic@ f_NR_AS_CipheringAlgorithm_Set(nea0); //WA#WI=1110266 // configure DRB with PDCP in TM at the start @sic R5s230377 sic@ v_SCG_DRB_Id := f_NR_GetNextSCGDRBId(); << SKIPPED CODE >> }