3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230502

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0185 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of CNF templates in PosSystem_Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2023-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When sending the Load Scenario request and Start request to configure and start the positioning simulator, there is no feedback of when the scenario is actually configured and started. For scenarios which require a considerable number of satellites, the satellite simulator might not be ready when the LPP session is already started. This leads to fail verdicts, as UE has not enough time to get a position fix. Current version does not include confirmation messages for the Load and Start scenario and does not wait for such confirmations to continue with the test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New templates car_POS_SYSTEM_LoadScenario and car_POS_SYSTEM_Start are defined for Confirmaiton messages and included in f_PosSystem_LoadScenario and f_PosSystem_Start respectively. The Cnf messages are created in a similar way as the Retrieve Data Cnf message (car_POS_SYSTEM_RetrieveData) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.1, 7.2.2.2, 7.3.3.1, 7.3.4.1, 7.3.4.2, 7.3.4.3, 7.3.4.4, 7.3.5.1 9.3.1.2, 9.3.3.1, 9.3.4.1, 9.3.4.2, 9.3.4.3, 9.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to PosSystem_Control.ttcn	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1	Correction to PosSystem_Control.ttcn

| Function name | PosSystem_Control.ttcn |
| --- | --- |
| Reason for change | When sending the Load Scenario request and Start request to configure and start the positioning simulator, there is no feedback of when the scenario is actually configured and started. For scenarios which require a considerable number of satellites, the satellite simulator might not be ready when the LPP session is already started. This leads to fail verdicts, as UE has not enough time to get a position fix. Current version does not include confirmation messages for the Load and Start scenario and does not wait for such confirmations to continue with the test case. |
| Summary of change | New templates car_POS_SYSTEM_LoadScenario and car_POS_SYSTEM_Start are defined for Confirmaiton messages and included in f_PosSystem_LoadScenario and f_PosSystem_Start respectively. The Cnf messages are created in a similar way as the Retrieve Data Cnf message (car_POS_SYSTEM_RetrieveData) |
| TTCN module | PosSystem_Control.ttcn |
| MCC160 Comment | Accepted as an improvement. |

Before

<<SKIPPED CODE>>

//============================================================================

// Templates

//============================================================================

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_Start( PositioningSystemList_Type p_PosSystemListId ) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := true,

Request := {

Start := true

}

};

//----------------------------------------------------------------------------

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_Stop(PositioningSystemList_Type p_PosSystemListId) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := false,

Request := {

Stop := true

}

};

//----------------------------------------------------------------------------

template (value) PositioningScenario_Type cs_SatelliteScenario_AGNSS(ScenarioNumber_Type p_ScenarioNumber) :=

{ /* @status    APPROVED (POS) */

AGNSS := p_ScenarioNumber

};

//----------------------------------------------------------------------------

template (value) PositioningScenario_Type cs_SatelliteScenario_LTEPos(ScenarioNumber_Type p_ScenarioNumber) :=

{ /* @status    APPROVED (POS) */

LTE_Positioning := p_ScenarioNumber

};

//----------------------------------------------------------------------------

template (value) PositioningScenario_Type cs_SatelliteScenario_NrPos(ScenarioNumber_Type p_ScenarioNumber) :=

{ /* @status    APPROVED (POS) */

NR_Positioning := p_ScenarioNumber

};

//----------------------------------------------------------------------------

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_LoadScenario(PositioningSystemList_Type p_PosSystemListId,

template (value) PositioningScenario_Type p_PositioningScenario) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := true,

Request := {

LoadScenario := p_PositioningScenario

}

};

//----------------------------------------------------------------------------

<<SKIPPED CODE>>

//============================================================================

// Common Functions

//============================================================================

/*

* @desc      common function to start a satellite system from EUTRA/UTRAN PTC;

*            no confirmation is expected

* @param     p_Port

* @param     p_PosSystemListId

* @status    APPROVED (LTE_A_PRO, POS)

*/

function f_PosSystem_Start(POS_PTC_MTC_PORT p_Port,

PositioningSystemList_Type p_PosSystemListId)

{

p_Port.send(cas_POS_SYSTEM_Start(p_PosSystemListId));

}

/*

* @desc      common function to stop a satellite system from EUTRA/UTRAN PTC;

*            no confirmation is expected

* @param     p_Port

* @param     p_PosSystemListId

* @status    APPROVED (LTE_A_PRO, POS)

*/

function f_PosSystem_Stop(POS_PTC_MTC_PORT p_Port,

PositioningSystemList_Type p_PosSystemListId)

{

p_Port.send(cas_POS_SYSTEM_Stop(p_PosSystemListId));

}

/*

* @desc      common function to load a scenario to a satellite system from EUTRA/UTRAN PTC;

*            no confirmation is expected

* @param     p_Port

* @param     p_PosSystemListId

* @param     p_PositioningScenario

* @status    APPROVED (LTE_A_PRO, POS)

*/

function f_PosSystem_LoadScenario(POS_PTC_MTC_PORT p_Port,

PositioningSystemList_Type p_PosSystemListId,

template (value) PositioningScenario_Type p_PositioningScenario)

{

p_Port.send(cas_POS_SYSTEM_LoadScenario(p_PosSystemListId, p_PositioningScenario));

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

//============================================================================

// Templates

//============================================================================

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_Start( PositioningSystemList_Type p_PosSystemListId ) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := true,

Request := {

Start := true

}

};

//----------------------------------------------------------------------------

template (present) POS_SYSTEM_CTRL_CNF car_POS_SYSTEM_Start(PositioningSystemList_Type p_PosSystemListId) :=

{ /* @status    APPROVED (POS) */

PositioningSystemList := p_PosSystemListId,

Confirm := {

Start := ?

}

};

//----------------------------------------------------------------------------

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_Stop(PositioningSystemList_Type p_PosSystemListId) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := false,

Request := {

Stop := true

}

};

//----------------------------------------------------------------------------

template (value) PositioningScenario_Type cs_SatelliteScenario_AGNSS(ScenarioNumber_Type p_ScenarioNumber) :=

{ /* @status    APPROVED (POS) */

AGNSS := p_ScenarioNumber

};

//----------------------------------------------------------------------------

template (value) PositioningScenario_Type cs_SatelliteScenario_LTEPos(ScenarioNumber_Type p_ScenarioNumber) :=

{ /* @status    APPROVED (POS) */

LTE_Positioning := p_ScenarioNumber

};

//----------------------------------------------------------------------------

template (value) PositioningScenario_Type cs_SatelliteScenario_NrPos(ScenarioNumber_Type p_ScenarioNumber) :=

{ /* @status    APPROVED (POS) */

NR_Positioning := p_ScenarioNumber

};

//----------------------------------------------------------------------------

template (value) POS_SYSTEM_CTRL_REQ cas_POS_SYSTEM_LoadScenario(PositioningSystemList_Type p_PosSystemListId,

template (value) PositioningScenario_Type p_PositioningScenario) :=

{ /* @status    APPROVED (LTE_A_PRO, POS) */

PositioningSystemList := p_PosSystemListId,

CnfFlag := true,

Request := {

LoadScenario := p_PositioningScenario

}

};

//----------------------------------------------------------------------------

template (present) POS_SYSTEM_CTRL_CNF car_POS_SYSTEM_LoadScenario(PositioningSystemList_Type p_PosSystemListId) :=

{ /* @status    APPROVED (POS) */

PositioningSystemList := p_PosSystemListId,

Confirm := {

LoadScenario := ?

}

};

//----------------------------------------------------------------------------

<<SKIPPED CODE>>

//============================================================================

// Common Functions

//============================================================================

/*

* @desc      common function to start a satellite system from EUTRA/UTRAN PTC;

*            no confirmation is expected

* @param     p_Port

* @param     p_PosSystemListId

* @status    APPROVED (LTE_A_PRO, POS)

*/

function f_PosSystem_Start(POS_PTC_MTC_PORT p_Port,

PositioningSystemList_Type p_PosSystemListId)

{

p_Port.send(cas_POS_SYSTEM_Start(p_PosSystemListId));

p_Port.receive(car_POS_SYSTEM_Start(p_PosSystemListId));

}

/*

* @desc      common function to stop a satellite system from EUTRA/UTRAN PTC;

*            no confirmation is expected

* @param     p_Port

* @param     p_PosSystemListId

* @status    APPROVED (LTE_A_PRO, POS)

*/

function f_PosSystem_Stop(POS_PTC_MTC_PORT p_Port,

PositioningSystemList_Type p_PosSystemListId)

{

p_Port.send(cas_POS_SYSTEM_Stop(p_PosSystemListId));

}

/*

* @desc      common function to load a scenario to a satellite system from EUTRA/UTRAN PTC;

*            no confirmation is expected

* @param     p_Port

* @param     p_PosSystemListId

* @param     p_PositioningScenario

* @status    APPROVED (LTE_A_PRO, POS)

*/

function f_PosSystem_LoadScenario(POS_PTC_MTC_PORT p_Port,

PositioningSystemList_Type p_PosSystemListId,

template (value) PositioningScenario_Type p_PositioningScenario)

{

p_Port.send(cas_POS_SYSTEM_LoadScenario(p_PosSystemListId, p_PositioningScenario));

p_Port.receive(car_POS_SYSTEM_LoadScenario(p_PosSystemListId));

}

<<SKIPPED CODE>>