3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230505

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3122 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC RedCap testcase 6.1.2.26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As as per prose R5-232981 at Step 3 once NR Cell 11 is on UE is required to exclude the cell as candidate for selection for 300 sec as specified in TS 38.304 clause 5.3.1. Thus, 300s timer should start after STEP 3 and 60s timer of STEP4 needs to be within that time frame only. NOTE: TTCN CR R5s230290 change 2.1 is re-implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Moved t_WaitNoRRCSetupRequest.start for 300ms before f_NR_CheckNoRRCSetupReq for 60s function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.26.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

2.1	Function f_TC_6_1_2_26_NR5GC_TestBody	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_TC_6_1_2_26_NR5GC_TestBody

| Function name | function f_TC_6_1_2_26_NR5GC_TestBody |
| --- | --- |
| Reason for change | 1. As as per prose R5-232981 at Step 3 once NR Cell 11 is on UE is required to exclude the cell as candidate for selection for 300 sec as specified in TS 38.304 clause 5.3.1. Thus, 300s timer should start after STEP 3 and 60s timer of STEP4 needs to be within that time frame only. NOTE: TTCN CR R5s230290 change 2.1 is re-implemented. |
| Summary of change | 1. Moved t_WaitNoRRCSetupRequest.start for 300ms before f_NR_CheckNoRRCSetupReq for 60s function. |
| TTCN module | .. 6_1_2\Idle_CellSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_1_2_26_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-231531 R5-232981 sic@ var boolean v_IsFR1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var CellBarredRedCap1Rx_r17_Type v_CellBarredRedCap1Rx := notBarred; var CellBarredRedCap2Rx_r17_Type v_CellBarredRedCap2Rx := notBarred; timer t_WaitNoRRCSetupRequest; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // Table 6.1.2.26.3.3.3-1: SIB1 for NR Cell 11 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell11); if (v_IsFR1) { // FR1 v_CellBarredRedCap1Rx := barred; } else { // FR2 or pc_maxNumberMIMO_LayersPDSCH_twoLayers v_CellBarredRedCap2Rx := barred; } if (pc_maxNumberMIMO_LayersPDSCH_twoLayers) { v_CellBarredRedCap2Rx := barred; } // @siclog "Step 1" siclog@ // The SS changes the SIB1 of NR Cell 11 to set cell Barred status to "true". f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17(nr_Cell11, v_CellBarredRedCap1Rx, v_CellBarredRedCap2Rx); f_NR_SS_ConfigureSysinfo(nr_Cell11, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell11)); //@sic R5s230290 Change 1 sic@ // @siclog "Step 2 - Void" siclog@ // @siclog "Step 3" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T1" in table 6.1.2.26.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 4" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell11, 60.0); t_WaitNoRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell11, nonProtocolTimer, 300.0)); // premature retransmission would be caught in the default, after this timeout RRCSetupRequest is acceptable // @siclog "Step 5" siclog@ // The SS changes the SIB1 of NR Cell 11 to delete the configuration of cellBarredRedCap-r17 f_NR_Delete_RedCap_ConfigCommonSIB_r17(nr_Cell11); // Table 6.1.2.26.3.3.3-2 f_NR_ModifySysinfo(nr_Cell11); //Modify Sysinfo in SS // @siclog "Step 6" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 after 300s from Step 3? alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_WaitNoRRCSetupRequest.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 6.1.2.26 Step 6: UE sent an RRCSetupRequest on NR Cell 11 within 300s"); } [] t_WaitNoRRCSetupRequest.timeout { SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.26 Step 6"); } } // @siclog "Step 7" siclog@ // Steps 2-6a1 of generic test procedure in TS 38.508-1 clause 4.9.5 on NR Cell 11. // NOTE: The UE performs a registration for mobility procedure and the RRC connection is released. f_NR5GC_MobilityRegistration(nr_Cell11); // @siclog "Step 8" siclog@ // The SS changes the SIB1 of NR Cell 1 to set intraFreqReselectionRedCap to not present. f_NR_CellInfo_SetSIB1_intraFreqReselectionRedCap_r17(nr_Cell1); // Table 6.1.2.26.3.3.3-3: SIB1 for NR Cell 1 f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 9" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell1); // @siclog "Step 10" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T0" in table 6.1.2.26.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); // @siclog "Step 11" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0); // Note 2: The UE finishes in State DEREGISTRED because UE does not camp on any NR Cells after Step 11 }//end of f_TC_6_1_2_26_NR5GC_TestBody

After Change:

function f_TC_6_1_2_26_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-231531 R5-232981 sic@ var boolean v_IsFR1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var CellBarredRedCap1Rx_r17_Type v_CellBarredRedCap1Rx := notBarred; var CellBarredRedCap2Rx_r17_Type v_CellBarredRedCap2Rx := notBarred; timer t_WaitNoRRCSetupRequest; v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // Table 6.1.2.26.3.3.3-1: SIB1 for NR Cell 11 v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell11); if (v_IsFR1) { // FR1 v_CellBarredRedCap1Rx := barred; } else { // FR2 or pc_maxNumberMIMO_LayersPDSCH_twoLayers v_CellBarredRedCap2Rx := barred; } if (pc_maxNumberMIMO_LayersPDSCH_twoLayers) { v_CellBarredRedCap2Rx := barred; } // @siclog "Step 1" siclog@ // The SS changes the SIB1 of NR Cell 11 to set cell Barred status to "true". f_NR_CellInfo_SetSIB1_RedCap_ConfigCommonSIB_r17(nr_Cell11, v_CellBarredRedCap1Rx, v_CellBarredRedCap2Rx); f_NR_SS_ConfigureSysinfo(nr_Cell11, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell11)); //@sic R5s230290 Change 1 sic@ // @siclog "Step 2 - Void" siclog@ // @siclog "Step 3" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T1" in table 6.1.2.26.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 4" siclog@ // WA#WI=1110544, 60s should be within this 300s period only as per prose R5-232981. t_WaitNoRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell11, nonProtocolTimer, 300.0)); // premature retransmission would be caught in the default, after this timeout RRCSetupRequest is acceptable // Check: Does the UE send an RRCSetupRequest on NR Cell 11 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell11, 60.0); // WA#WI=1110544 - <<Moved up>> //t_WaitNoRRCSetupRequest.start(f_NR_SetTimerToleranceMin(nr_Cell11, nonProtocolTimer, 300.0)); // premature retransmission would be caught in the default, after this timeout RRCSetupRequest is acceptable // @siclog "Step 5" siclog@ // The SS changes the SIB1 of NR Cell 11 to delete the configuration of cellBarredRedCap-r17 f_NR_Delete_RedCap_ConfigCommonSIB_r17(nr_Cell11); // Table 6.1.2.26.3.3.3-2 f_NR_ModifySysinfo(nr_Cell11); //Modify Sysinfo in SS // @siclog "Step 6" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 11 after 300s from Step 3? alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_WaitNoRRCSetupRequest.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 6.1.2.26 Step 6: UE sent an RRCSetupRequest on NR Cell 11 within 300s"); } [] t_WaitNoRRCSetupRequest.timeout { SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.26 Step 6"); } } // @siclog "Step 7" siclog@ // Steps 2-6a1 of generic test procedure in TS 38.508-1 clause 4.9.5 on NR Cell 11. // NOTE: The UE performs a registration for mobility procedure and the RRC connection is released. f_NR5GC_MobilityRegistration(nr_Cell11); // @siclog "Step 8" siclog@ // The SS changes the SIB1 of NR Cell 1 to set intraFreqReselectionRedCap to not present. f_NR_CellInfo_SetSIB1_intraFreqReselectionRedCap_r17(nr_Cell1); // Table 6.1.2.26.3.3.3-3: SIB1 for NR Cell 1 f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 9" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods(nr_Cell1); // @siclog "Step 10" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 and NR Cell 11 according to row "T0" in table 6.1.2.26.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); // @siclog "Step 11" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, 60.0); // Note 2: The UE finishes in State DEREGISTRED because UE does not camp on any NR Cells after Step 11 }//end of f_TC_6_1_2_26_NR5GC_TestBody