3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230507

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3124 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.6.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Downlink SPS is not configured for this test case, so DRB_COMMON_REQ needs C-RNTI grant. Incorrect template is used in current TTCN. - For Step 27, a conformant UE will send SR after receiving the PDU due to having released the configuredGrant. The current watchdog timer of 0.2sec goes beyond the configured sr-TransMax=16, and will cause the UE to attempt RACH. It is more stable to have the watchdog timer within sr-TransMax. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change all calls of cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant into cas_NR_DRB_COMMON_REQ_MAC_PDUList - Change watchdog timer to 0.1sec |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.3.ENDC, 7.1.1.6.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:                      Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_7_1_1_6_3_NR_TestBody1 |
| --- | --- |
| Reason for change | Downlink SPS is not configured for this test case, so DRB_COMMON_REQ needs C-RNTI grant. Incorrect template is used in current TTCN. |
| Summary of change | Change all calls of cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant into cas_NR_DRB_COMMON_REQ_MAC_PDUList |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_7_1_1_6_3_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { const integer const_SPS_Period :=40; // period in milli seconds const integer const_UL_TBS_CRNTI := 144; // const integer const_UL_TBS_CSRNTI := 160; //160 bits *skipped code* DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 5" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,60); //60 ms after f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); // Prepare step 6 v_TimingUL_1 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,60); //Step 6 timing v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu1)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; //Prepare step 7 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu2)}; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_SPS_Period); //Step 7 Timing //@siclog "Step 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); //Prepare step 9 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu3)}; v_MAC_PDUListUL3 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2);//Step 9 Timing //Prepare step 10 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_MAC_PDUListUL4 := { cr_NR_UL_MAC_PDU(*, *, *) }; v_TimingUL_4 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_3,const_SPS_Period/2);//Step 10 Timing //Prepare step 11 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu4)}; v_MAC_PDUListUL5 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_5 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2);//Step 11 Timing //@siclog "Step 12" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI, true); // NDI =1 //Prepare step 13 //v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_MAC_PDUListUL6 := v_MAC_PDUListUL5; v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_5,const_SPS_Period/2);//Step 13 Timing //Prepare step 14 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu5)}; v_MAC_PDUListUL7 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period/2);//Step 14 Timing //@siclog "Step 15" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), v_NR_ResourceAllocation_CRNTI); //prepare step 16 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu6)}; v_MAC_PDUListUL8 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period);//Step 16 Timing //prepare step 17 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_MAC_PDUListUL9 := { cr_NR_UL_MAC_PDU(omit, *, *) }; v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_8,const_SPS_Period);//Step 17 Timing //@siclog "Step 18" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL1)); //prepare step 19 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu7)}; v_MAC_PDUListUL10 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,const_SPS_Period);//Step 19 Timing //@siclog "Step 20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_UL); //@siclog "Steps 18" siclog@ f_Delay(5.0); // @sic R5-227488 sic@ //@siclog "Steps 19 - 20" siclog@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); }

After Change

function f_TC_7_1_1_6_3_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { const integer const_SPS_Period :=40; // period in milli seconds const integer const_UL_TBS_CRNTI := 144; // const integer const_UL_TBS_CSRNTI := 160; //160 bits DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); //Anritsu TTCN CR for 7.1.1.6.3 *skipped code* //@siclog "Step 5" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,60); //60 ms after f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); // Prepare step 6 v_TimingUL_1 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,60); //Step 6 timing v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu1)}; v_MAC_PDUListUL1 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; //Prepare step 7 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu2)}; v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_TimingUL_2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_1,const_SPS_Period); //Step 7 Timing //@siclog "Step 8" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI); //Prepare step 9 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu3)}; v_MAC_PDUListUL3 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, *) }; v_TimingUL_3 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_2,const_SPS_Period/2);//Step 9 Timing //Prepare step 10 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_MAC_PDUListUL4 := { cr_NR_UL_MAC_PDU(*, *, *) }; v_TimingUL_4 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_3,const_SPS_Period/2);//Step 10 Timing //Prepare step 11 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu4)}; v_MAC_PDUListUL5 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_5 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2);//Step 11 Timing //@siclog "Step 12" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); f_NR_UL_CgGrantConfiguration(nr_Cell1,cs_TimingInfo_NR( v_TimingDL), v_NR_ResourceAllocation_CSRNTI, true); // NDI =1 //Prepare step 13 //v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_MAC_PDUListUL6 := v_MAC_PDUListUL5; v_TimingUL_6 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_5,const_SPS_Period/2);//Step 13 Timing //Prepare step 14 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu5)}; v_MAC_PDUListUL7 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_7 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_6,const_SPS_Period/2);//Step 14 Timing //@siclog "Step 15" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), v_NR_ResourceAllocation_CRNTI); //prepare step 16 v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu6)}; v_MAC_PDUListUL8 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_8 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_7,const_SPS_Period);//Step 16 Timing //prepare step 17 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period); v_MAC_PDUListUL9 := { cr_NR_UL_MAC_PDU(omit, *, *) }; v_TimingUL_9 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_8,const_SPS_Period);//Step 17 Timing //@siclog "Step 18" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL1)); //prepare step 19 v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period-10); v_SDU_SubPDUListUL := {cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, int2bit(16, 8)), v_EncodedRlcPdu7)}; v_MAC_PDUListUL10 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; v_TimingUL_10 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_9,const_SPS_Period);//Step 19 Timing //@siclog "Step 20" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,10); f_NR_SpsCgDeactivate(nr_Cell1, cs_TimingInfo_NR( v_TimingDL),cs_NR_SpsCgDeactivate_Pdcch_UL); *skipped code*

## Change 2

| Function name | function f_TC_7_1_1_6_3_NR_TestBody2 |
| --- | --- |
| Reason for change | - Same as Change 1 - For Step 27, a conformant UE will send SR after receiving the PDU due to having released the configuredGrant. The current watchdog timer of 0.2sec goes beyond the configured sr-TransMax=16, and will cause the UE to attempt RACH. It is more stable to have the watchdog timer within sr-TransMax. |
| Summary of change | - Same as Change 1 - Change watchdog timer to 0.1sec |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_7_1_1_6_3_NR_TestBody2(DRB_Identity p_NR_DRB_Id //SubFrameTiming_Type p_TimingUL )runs on NR_BASE_PTC { //const integer const_SPS_Period :=40; // period in milli seconds var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; //var SubFrameTiming_Type v_TimingUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(*, *, *) }; timer t_Watchdog := 0.2; //@sic R5s230067 sic@ // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_Now, cs_NR_UplinkTimeAlignment_Stop); //v_TimingUL := f_SubFrameTiming_AddMilliSeconds(p_TimingUL,const_SPS_Period *p_X); v_EncodedRlcPdu := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (9, crs_NR_PDCP_SDU_12B7, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := {cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); DRB.send(cads_NR_DRB_COMMON_REQ_MAC_PDUList_NoCRNTI_Grant(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, v_MAC_PDUListUL)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 27"); t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); } } // Get back PDU from UE f_NR_ULGrantConfiguration_Start(nr_Cell1); DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, v_MAC_PDUListUL)) };

After Change

function f_TC_7_1_1_6_3_NR_TestBody2(DRB_Identity p_NR_DRB_Id //SubFrameTiming_Type p_TimingUL )runs on NR_BASE_PTC { //const integer const_SPS_Period :=40; // period in milli seconds var NR_MAC_SDU_Type v_EncodedRlcPdu; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; //var SubFrameTiming_Type v_TimingUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(*, *, *) }; timer t_Watchdog := 0.1; //@sic R5s230067 sic@ // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_Now, cs_NR_UplinkTimeAlignment_Stop); //v_TimingUL := f_SubFrameTiming_AddMilliSeconds(p_TimingUL,const_SPS_Period *p_X); v_EncodedRlcPdu := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (9, crs_NR_PDCP_SDU_12B7, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := {cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu)}; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL), v_MAC_PDUListDL)); t_Watchdog.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, v_MAC_PDUListUL)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 27"); t_Watchdog.stop; } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); } } // Get back PDU from UE f_NR_ULGrantConfiguration_Start(nr_Cell1); DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, v_MAC_PDUListUL)) };