3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230512

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4744 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NB-IoT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-29-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the global variable vc_NBIOT_Global.MobileInfo.NTN_SatelliteType is left uninitialized if pc_NB_ntn_GSO_ScenarioSupport is set to FALSE. This can cause a run-time error in the call to function f_NBIOT_MobileInfo_GetNTNSatelliteType() during NB-IoT cell configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set vc_NBIOT_Global.MobileInfo.NTN_SatelliteType to omit if GSO satellite access is not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT test cases cannot execute for non-sattellite access devices |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NB-IoT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NBIOT_Init () |
| --- | --- |
| Reason for change | Currently the global variable vc_NBIOT_Global.MobileInfo.NTN_SatelliteType is left uninitialized if pc_NB_ntn_GSO_ScenarioSupport is set to FALSE. This can cause a run-time error in the call to function f_NBIOT_MobileInfo_GetNTNSatelliteType() during NB-IoT cell configuration. |
| Summary of change | Set vc_NBIOT_Global.MobileInfo.NTN_SatelliteType to omit if GSO satellite access is not supported. |
| TTCN module | NBIoT_CellInfo.ttcn |
| MCC160 Comment | Accepted in principle, will be implemented differently, to avoid complications caused by change2. NTN_Satellite_Type in CommonDefs module will be extended with the entry ntn_None to be used in “else“ case. |

Before Change

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq ) runs on NBIOT_PTC { var NB_InitParams_Local_Type v_Params := { CellsOnSamePLMN := p_CellsOnSamePLMN, SysinfoCombination := p_SysinfoCombination, CellSpecialFreq := p_CellSpecialFreq }; var boolean v_DoAttachWithoutPDN := false; if (p_AllowAttachWithoutPDN == CONTROL_PLANE) { // @sic R5-171992 sic@ v_DoAttachWithoutPDN := true; } if(pc_NB_ntn_GSO_ScenarioSupport) { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_GSO; //@sic R5-231915, R5-233442 sic@ } fl_NBIOT_Common_Init(v_Params, v_DoAttachWithoutPDN); }

After Change

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq ) runs on NBIOT_PTC { var NB_InitParams_Local_Type v_Params := { CellsOnSamePLMN := p_CellsOnSamePLMN, SysinfoCombination := p_SysinfoCombination, CellSpecialFreq := p_CellSpecialFreq }; var boolean v_DoAttachWithoutPDN := false; if (p_AllowAttachWithoutPDN == CONTROL_PLANE) { // @sic R5-171992 sic@ v_DoAttachWithoutPDN := true; } if(pc_NB_ntn_GSO_ScenarioSupport) { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_GSO; //@sic R5-231915, R5-233442 sic@ } else { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := omit; } fl_NBIOT_Common_Init(v_Params, v_DoAttachWithoutPDN); }

MCC160 implementation

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq ) runs on NBIOT_PTC { var NB_InitParams_Local_Type v_Params := { CellsOnSamePLMN := p_CellsOnSamePLMN, SysinfoCombination := p_SysinfoCombination, CellSpecialFreq := p_CellSpecialFreq }; var boolean v_DoAttachWithoutPDN := false; if (p_AllowAttachWithoutPDN == CONTROL_PLANE) { // @sic R5-171992 sic@ v_DoAttachWithoutPDN := true; } if(pc_NB_ntn_GSO_ScenarioSupport) { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_GSO; //@sic R5-231915, R5-233442 sic@ } else { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_None; } fl_NBIOT_Common_Init(v_Params, v_DoAttachWithoutPDN); }

## Change 2

| Function name | function f_NBIOT_MobileInfo_GetNTNSatelliteType () |
| --- | --- |
| Reason for change | Currently the global variable vc_NBIOT_Global.MobileInfo.NTN_SatelliteType is left uninitialized if pc_NB_ntn_GSO_ScenarioSupport is set to FALSE. This can cause a run-time error in the call to function f_NBIOT_MobileInfo_GetNTNSatelliteType() during NB-IoT cell configuration. |
| Summary of change | Update return type so that this function can return omit (see change 1) |
| TTCN module | NBIoT_CellInfo.ttcn |
| MCC160 Comment | Not needed due to alternative implementation of change 1 Due to return template(omit) all function calls would have to be modfified to use valueof to check, which satellite type is supported if any. |

Before Change

function f_NBIOT_MobileInfo_GetNTNSatelliteType() runs on NBIOT_PTC return NTN_Satellite_Type { return vc_NBIOT_Global.MobileInfo.NTN_SatelliteType;

After Change

function f_NBIOT_MobileInfo_GetNTNSatelliteType() runs on NBIOT_PTC return template (omit) NTN_Satellite_Type { return vc_NBIOT_Global.MobileInfo.NTN_SatelliteType; }