3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230512

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4744 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NB-IoT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-29-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the global variable vc_NBIOT_Global.MobileInfo.NTN_SatelliteType is left uninitialized if pc_NB_ntn_GSO_ScenarioSupport is set to FALSE. This can cause a run-time error in the call to function f_NBIOT_MobileInfo_GetNTNSatelliteType() during NB-IoT cell configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set vc_NBIOT_Global.MobileInfo.NTN_SatelliteType to omit if GSO satellite access is not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT test cases cannot execute for non-sattellite access devices |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NB-IoT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NBIOT_Init () |
| --- | --- |
| Reason for change | Currently the global variable vc_NBIOT_Global.MobileInfo.NTN_SatelliteType is left uninitialized if pc_NB_ntn_GSO_ScenarioSupport is set to FALSE. This can cause a run-time error in the call to function f_NBIOT_MobileInfo_GetNTNSatelliteType() during NB-IoT cell configuration. |
| Summary of change | Set vc_NBIOT_Global.MobileInfo.NTN_SatelliteType to omit if GSO satellite access is not supported. |
| TTCN module | NBIoT_CellInfo.ttcn |
| MCC160 Comment | Accepted in principle, will be implemented differently, to avoid complications caused by change2. NTN_Satellite_Type in CommonDefs module will be extended with the entry ntn_None to be used in “else“ case. |

Before Change

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq ) runs on NBIOT_PTC { var NB_InitParams_Local_Type v_Params := { CellsOnSamePLMN := p_CellsOnSamePLMN, SysinfoCombination := p_SysinfoCombination, CellSpecialFreq := p_CellSpecialFreq }; var boolean v_DoAttachWithoutPDN := false; if (p_AllowAttachWithoutPDN == CONTROL_PLANE) { // @sic R5-171992 sic@ v_DoAttachWithoutPDN := true; } if(pc_NB_ntn_GSO_ScenarioSupport) { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_GSO; //@sic R5-231915, R5-233442 sic@ } fl_NBIOT_Common_Init(v_Params, v_DoAttachWithoutPDN); }

After Change

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq ) runs on NBIOT_PTC { var NB_InitParams_Local_Type v_Params := { CellsOnSamePLMN := p_CellsOnSamePLMN, SysinfoCombination := p_SysinfoCombination, CellSpecialFreq := p_CellSpecialFreq }; var boolean v_DoAttachWithoutPDN := false; if (p_AllowAttachWithoutPDN == CONTROL_PLANE) { // @sic R5-171992 sic@ v_DoAttachWithoutPDN := true; } if(pc_NB_ntn_GSO_ScenarioSupport) { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_GSO; //@sic R5-231915, R5-233442 sic@ } else { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := omit; } fl_NBIOT_Common_Init(v_Params, v_DoAttachWithoutPDN); }

MCC160 implementation

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq ) runs on NBIOT_PTC { var NB_InitParams_Local_Type v_Params := { CellsOnSamePLMN := p_CellsOnSamePLMN, SysinfoCombination := p_SysinfoCombination, CellSpecialFreq := p_CellSpecialFreq }; var boolean v_DoAttachWithoutPDN := false; if (p_AllowAttachWithoutPDN == CONTROL_PLANE) { // @sic R5-171992 sic@ v_DoAttachWithoutPDN := true; } if(pc_NB_ntn_GSO_ScenarioSupport) { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_GSO; //@sic R5-231915, R5-233442 sic@ } else { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_None; } fl_NBIOT_Common_Init(v_Params, v_DoAttachWithoutPDN); }

## Change 2

| Function name | function f_NBIOT_MobileInfo_GetNTNSatelliteType () |
| --- | --- |
| Reason for change | Currently the global variable vc_NBIOT_Global.MobileInfo.NTN_SatelliteType is left uninitialized if pc_NB_ntn_GSO_ScenarioSupport is set to FALSE. This can cause a run-time error in the call to function f_NBIOT_MobileInfo_GetNTNSatelliteType() during NB-IoT cell configuration. |
| Summary of change | Update return type so that this function can return omit (see change 1) |
| TTCN module | NBIoT_CellInfo.ttcn |
| MCC160 Comment | Not needed due to alternative implementation of change 1 Due to return template(omit) all function calls would have to be modfified to use valueof to check, which satellite type is supported if any. |

Before Change

function f_NBIOT_MobileInfo_GetNTNSatelliteType() runs on NBIOT_PTC return NTN_Satellite_Type { return vc_NBIOT_Global.MobileInfo.NTN_SatelliteType;

After Change

function f_NBIOT_MobileInfo_GetNTNSatelliteType() runs on NBIOT_PTC return template (omit) NTN_Satellite_Type { return vc_NBIOT_Global.MobileInfo.NTN_SatelliteType; }

## Additional change

| Function name | function f_NBIOT_Init() |
| --- | --- |
| Reason for change | Based on feedback from Keysight: the NTN_SatelliteType will always be set if pc_NB_ntn_GSO_ScenarioSupport is true ; this means that NTN configuration (satellite offsets and delays) will always be configured in the ASPs regardless of the test case. These should only be configured for NTN test cases where SIB31 is present and can cause other NB-IoT test cases to fail (for a device supporting both satellite and non-satellite NB-IoT access). |
| Summary of change | Updated code to set vc_NBIOT_Global.MobileInfo.NTN_SatelliteType only when UE supports NTN and when system combination c8 or c9 is used |
| TTCN module | NBIoT_CellInfo.ttcn |
| MCC160 Comment | Accepted |

Before r1

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq ) runs on NBIOT_PTC { var NB_InitParams_Local_Type v_Params := { CellsOnSamePLMN := p_CellsOnSamePLMN, SysinfoCombination := p_SysinfoCombination, CellSpecialFreq := p_CellSpecialFreq }; var boolean v_DoAttachWithoutPDN := false; if (p_AllowAttachWithoutPDN == CONTROL_PLANE) { // @sic R5-171992 sic@ v_DoAttachWithoutPDN := true; } if(pc_NB_ntn_GSO_ScenarioSupport) { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_GSO; //@sic R5-231915, R5-233442 sic@ } else { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_None; } fl_NBIOT_Common_Init(v_Params, v_DoAttachWithoutPDN); }

After r1:

function f_NBIOT_Init(NB_Combination_Type p_SysinfoCombination, IOT_STATE_Type p_AllowAttachWithoutPDN := CONTROL_PLANE, EUTRA_NAS_NumOfPLMN_Type p_CellsOnSamePLMN := LTE_AllCellsOnSamePLMN, EUTRA_NAS_CellSpecialFreq p_CellSpecialFreq := LTE_NAS_NormalFreq ) runs on NBIOT_PTC { var NB_InitParams_Local_Type v_Params := { CellsOnSamePLMN := p_CellsOnSamePLMN, SysinfoCombination := p_SysinfoCombination, CellSpecialFreq := p_CellSpecialFreq }; var boolean v_DoAttachWithoutPDN := false; if (p_AllowAttachWithoutPDN == CONTROL_PLANE) { // @sic R5-171992 sic@ v_DoAttachWithoutPDN := true; } if(pc_NB_ntn_GSO_ScenarioSupport and (p_SysinfoCombination == c8 or p_SysinfoCombination == c9)) { vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_GSO; //@sic R5-231915, R5-233442 sic@ } else {vc_NBIOT_Global.MobileInfo.NTN_SatelliteType := ntn_None; } fl_NBIOT_Common_Init(v_Params, v_DoAttachWithoutPDN); }