3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230515

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3129 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC 5GSM test case 8.1.5.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inside the f_NR5GC_RRCReestablishment_Def there is SS configuration change for the upcoming RACH procedure, namely the C-RNTI may change. However in the 8.1.5.9.1. the cell is turned on in step 9 before TTCN changes the SS configuration. If the UE triggers the RACH procedure fast enough after detecting the cell, it may happen that SS is still configured with old C-RNTI and the RACH procedure fails. Therefore to avoid this race condition we suggest to pre-configure SS first with the new parameters and to turn on the cell after that. |  |  |  |  |  |  |  |  |
| Summary of change: |  | f_NR5GC_RRCReestablishment_Def was splitted into two functions in order to separate the SS RACH configuration and reception of the ReestablishmentRequest. The first part is now executed before step 9, the second part after. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test if it triggers RRCreestablishment procedure fast enough after step 9. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_5_9_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | Inside the f_NR5GC_RRCReestablishment_Def there is SS configuration change for the upcoming RACH procedure, namely the C-RNTI may change. However in the 8.1.5.9.1. the cell is turned on in step 9 before TTCN changes the SS configuration. If the UE triggers the RACH procedure fast enough after detecting the cell, it may happen that SS is still configured with old C-RNTI and the RACH procedure fails. Therefore to avoid this race condition we suggest to pre-configure SS first with the new parameters and to turn on the cell after that. |
| Summary of change | f_NR5GC_RRCReestablishment_Def was splitted into two functions in order to separate the SS RACH configuration and reception of the ReestablishmentRequest. The first part is now executed before step 9, the second part after. |
| TTCN module | . NR5GC\8_1_5\RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted but implemented differently. See below and changes 2 and 3. |

Before Change:

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC { … //@siclog "Step 8" siclog@ // Wait for T310 to expire f_Delay(1.0); //@sic R5s220330 sic@ //@siclog "Step 9" siclog@ // The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2. f_NR_SetCellPowerList (v_CellPowerList_AtT2); //@siclog "Step 10 - 12" siclog@ f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, ?); //@siclog "Step 13" siclog@

After Change:

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC { … //@siclog "Step 8" siclog@ // Wait for T310 to expire f_Delay(1.0); //@sic R5s220330 sic@ //@siclog "Step 9" siclog@ // The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2. f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell1, ?, f_NR_CellInfo_GetRNTI(nr_Cell1), -, -, false); f_NR_SetCellPowerList (v_CellPowerList_AtT2); //@siclog "Step 10 - 12" siclog@ f_NR5GC_RRCReestablishment_Def_fromStep2(nr_Cell1, nr_Cell1, ?); //REMOVED f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, ?); //@siclog "Step 13" siclog@

MCC160 proposed implementation

function f_TC_8_1_5_9_1_NR5GC_TestBody() runs on NR5GC_PTC

{

…

//@siclog "Step 8" siclog@

// Wait for T310 to expire

f_Delay(1.0);  //@sic R5s220330 sic@

//@siclog "Step 9" siclog@

// The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2.

//f_NR_SetCellPowerList (v_CellPowerList_AtT2);

//@siclog "Step 10 - 12" siclog@

f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, ?, -,-,-,-,-,-, v_CellPowerList_AtT2);

…

}

## Change 2

| Function name | f_NR5GC_RRCReestablishment_Steps0To2 |
| --- | --- |
| Reason for change | Needed for change 1 |
| Summary of change | Added possibility to exclude reception of RRCreestablishmentRequest |
| TTCN module | . NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted, implemented by adding the powerLevel as parameter. When provided the power level is updated before step 2. See MCC160 proposed implementation below. |

Before Change:

function f_NR5GC_RRCReestablishment_Steps0To2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit ) runs on NR_BASE_PTC { //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@ //@sic R5-221467 renamed function from f_NR5GC_RRCReestablishment_Steps1To2 to f_NR5GC_RRCReestablishment_Steps0To2 sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); //@sic R5s211169 change 1.1 sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); // 0. Source Cell: If the Source Cell is the same as the Target Cell: Release SRBs and DRBs. (Re-)configure SRBs. //@sic R5-221467 sic@ if (p_NR_SourceCellId == p_NR_TargetCellId) { f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId); //@sic R5s220275 sic@ f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211169 R5s220176 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit); //@sic R5s220176 sic@ // @sic R5-217830 sic@ } // 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, p_RNTI_Value), //@sic R5-225418 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); // 2. Target Cell: Receive RRCReestablismentRequest. SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } }

After Change:

function f_NR5GC_RRCReestablishment_Steps0To2(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, boolean p_ReceiveRRCReestablishmentRequest := true ) runs on NR_BASE_PTC { //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@ //@sic R5-221467 renamed function from f_NR5GC_RRCReestablishment_Steps1To2 to f_NR5GC_RRCReestablishment_Steps0To2 sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); //C-RNTI used by UE in RRCReestablismentRequest var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId); var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); //@sic R5s211169 change 1.1 sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); // 0. Source Cell: If the Source Cell is the same as the Target Cell: Release SRBs and DRBs. (Re-)configure SRBs. //@sic R5-221467 sic@ if (p_NR_SourceCellId == p_NR_TargetCellId) { f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId); //@sic R5s220275 sic@ f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211169 R5s220176 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit); //@sic R5s220176 sic@ // @sic R5-217830 sic@ } // 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, p_RNTI_Value), //@sic R5-225418 sic@ cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); // 2. Target Cell: Receive RRCReestablismentRequest. if (p_ReceiveRRCReestablishmentRequest) { SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA)); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } } }

MCC160 proposed implementation

function f_NR5GC_RRCReestablishment_Def(NR_CellId_Type            p_NR_SourceCellId,

NR_CellId_Type            p_NR_TargetCellId,

template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure,

NextHopChainingCount      p_NextHopChainingCount := 0,

RNTI_Value                p_RNTI_Value :=  tsc_C_RNTI_Value4,

template(omit) charstring p_VerdictMsg := omit,

template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit,

template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := *,

boolean p_ReceiveForRRCreconfigurationComplete := true,

template(omit) NR_CellPowerList_Type p_CellPowerListBeforeStep2 := omit

) runs on NR5GC_PTC

{ //@sic R5-206369 R5s220372 R5s220354: deleted parameter p_SCellConfig sic@

//@sic R5s230515: added p_CellPowerListBeforeStep2 sic@

//Steps 1-2

f_NR5GC_RRCReestablishment_Steps0To2(p_NR_SourceCellId, p_NR_TargetCellId, p_ReestablishmentCause, p_RNTI_Value, p_VerdictMsg, -, p_CellPowerListBeforeStep2);  //@sic R5-211391 sic@

//Steps 3-7

f_NR5GC_RRCReestablishment_Steps3To7(p_NR_TargetCellId, p_NextHopChainingCount, -, p_RRCReestablishmentComplete_v1610_IEs);

//Steps 8-10

f_NR5GC_RRCReestablishment_Steps8To10(p_NR_SourceCellId, p_NR_TargetCellId, p_RlfTimersAndConstants, -, p_ReceiveForRRCreconfigurationComplete);

//Steps 11-13

f_NR5GC_RRCReestablishment_Steps11To13( p_NR_SourceCellId, p_NR_TargetCellId );

}

function f_NR5GC_RRCReestablishment_Steps0To2(NR_CellId_Type            p_NR_SourceCellId,

NR_CellId_Type            p_NR_TargetCellId,

template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure,

RNTI_Value                p_RNTI_Value :=  tsc_C_RNTI_Value4,

template(omit) charstring p_VerdictMsg := omit,

template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit,

template(omit) NR_CellPowerList_Type p_CellPowerListBeforeStep2 := omit

) runs on NR_BASE_PTC

{ //@sic R5-211391 R5-213516: added p_MacBearerRoutingCA sic@

//@sic R5-221467 renamed function from f_NR5GC_RRCReestablishment_Steps1To2 to f_NR5GC_RRCReestablishment_Steps0To2 sic@

//@sic R5s230515: added p_CellPowerListBeforeStep2 sic@

var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId);  //C-RNTI used by UE in RRCReestablismentRequest

var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId);

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId);

var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId);  //@sic R5s211169 change 1.1 sic@

var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId);

// 0. Source Cell:  If the Source Cell is the same as the Target Cell:  Release SRBs and DRBs. (Re-)configure SRBs.

//@sic R5-221467 sic@

if (p_NR_SourceCellId == p_NR_TargetCellId) {

f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId);  //@sic R5s220275 sic@

f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList);  //@sic R5-211417 R5s211169 R5s220176 sic@

f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit);          //@sic R5s220176 sic@

// @sic R5-217830 sic@

}

// 1. Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI.

f_NR_CellInfo_SetRNTI(p_NR_TargetCellId, p_RNTI_Value);

f_NR_SS_C_RNTI_Config(p_NR_TargetCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag);    // NOTE 1: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, tsc_CnfReq,

cs_NR_RachProcedureConfig_RRCReestablishment(v_NR_PhysicalParameters, p_RNTI_Value),  //@sic R5-225418 sic@

cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters),

cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params)));

//@sic R5s230515 sic@

if ( isvalue(p_CellPowerListBeforeStep2)) {

f_NR_SetCellPowerList (p_CellPowerListBeforeStep2);

}

// 2. Target Cell: Receive RRCReestablismentRequest.

SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause), p_MacBearerRoutingCA));

if (isvalue(p_VerdictMsg)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg));

}

}

## Change 3

| Function name | f_NR5GC_RRCReestablishment_Def_fromStep2 |
| --- | --- |
| Reason for change | Needed for change 1 |
| Summary of change | Created new function that contains RRCReestablishment_Def from step 2 til the end. |
| TTCN module | . NR5GC_RRCSteps.ttcn |
| MCC160 Comment | See change 2 |

After Change:

function f_NR5GC_RRCReestablishment_Def_fromStep2 (NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, NextHopChainingCount p_NextHopChainingCount := 0, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := *, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 R5s220372 R5s220354: deleted parameter p_SCellConfig sic@ var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(p_NR_SourceCellId); var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(p_NR_SourceCellId); var ShortMAC_I v_ShortMAC_I:= f_NR_Calculate_ShortMAC(p_NR_SourceCellId, p_NR_TargetCellId); // 2. Target Cell: Receive RRCReestablismentRequest. SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_TargetCellId, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), p_ReestablishmentCause))); if (isvalue(p_VerdictMsg)) { f_NR_PreliminaryPass(__FILE__, __LINE__, valueof(p_VerdictMsg)); } //Steps 3-7 f_NR5GC_RRCReestablishment_Steps3To7(p_NR_TargetCellId, p_NextHopChainingCount, -, p_RRCReestablishmentComplete_v1610_IEs); //Steps 8-10 f_NR5GC_RRCReestablishment_Steps8To10(p_NR_SourceCellId, p_NR_TargetCellId, p_RlfTimersAndConstants, -, p_ReceiveForRRCreconfigurationComplete); //Steps 11-13 f_NR5GC_RRCReestablishment_Steps11To13( p_NR_SourceCellId, p_NR_TargetCellId ); }