3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230529

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3136 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-07-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN, for Steps 11-11A, following functions are called in this sequence : f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@ f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); In the highlighted function, the UE is paged about updated SIB, and then a tmer of 2*modification period is started. Once this timer expires, a coordination message is sent to IMS PTC to expect the re-REGISTER. However, some UEs could read the updated SIB1 in the first cycle of modification period itself, and then try to send a re-REGISTER immediately. This causes TC failure as IMS PTC is not yet ready for this because the coordination message that transfers control is not yet called on NR side. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the function call to f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); Before starting the call to f_NR_WaitModificationPeriods(nr_Cell1); Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case reliablity will be reduced and a conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

2.1	Function f_TC_7_1_1_1_16_NR_TestBody	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function fl_TC_11_3_10_TestBody

| Function name | fl_TC_11_3_10_TestBody |
| --- | --- |
| Reason for change | In the current TTCN, for Steps 11-11A, following functions are called in this sequence : f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@ f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); In the highlighted function, the UE is paged about updated SIB, and then a tmer of 2*modification period is started. Once this timer expires, a coordination message is sent to IMS PTC to expect the re-REGISTER. However, some UEs could read the updated SIB1 in the first cycle of modification period itself, and then try to send a re-REGISTER immediately. This causes TC failure as IMS PTC is not yet ready for this because the coordination message that transfers control is not yet called on NR side. This needs to be addressed. |
| Summary of change | Moved the function call to f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); Before starting the call to f_NR_WaitModificationPeriods(nr_Cell1); Please see below. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently. A prose CR must be submuitted to remove step 11A |

Before Change:

function fl_TC_11_3_10_TestBody() runs on NR5GC_PTC { //@siclog "Steps 9 - 9A" siclog@ //The SS changes the SIB1 according to 38.508-1[4] Table 11.3.10.3.3-2 and send Short Message on PDCCH using P-RNTI. <-- PDCCH (DCI 1_0): Short Message - - //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, //@sic R5-221459 sic@ cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(9,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@ f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 11 - 11A" siclog@ //The SS changes the SIB1 according to 38.508-1 [4] Table 4.6.1-28 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification <-- PDCCH (DCI 1_0): Short Message //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@ f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

After Change:

function fl_TC_11_3_10_TestBody() runs on NR5GC_PTC { //@siclog "Steps 9 - 9A" siclog@ //The SS changes the SIB1 according to 38.508-1[4] Table 11.3.10.3.3-2 and send Short Message on PDCCH using P-RNTI. <-- PDCCH (DCI 1_0): Short Message - - //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, //@sic R5-221459 sic@ cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(9,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@ f_NR_WaitModificationPeriods(nr_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 11 - 11A" siclog@ //The SS changes the SIB1 according to 38.508-1 [4] Table 4.6.1-28 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification <-- PDCCH (DCI 1_0): Short Message //Wait for 2.1* modification period to allow the new system information to take effect f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //WA#11_3_10 f_NR_WaitModificationPeriods(nr_Cell1); //WA#11_3_10 MOVED f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

MCC160 Implementation

function fl_TC_11_3_10_TestBody() runs on NR5GC_PTC

{

//@siclog "Steps 9 - 9A" siclog@

//The SS changes the SIB1 according to 38.508-1[4] Table 11.3.10.3.3-2 and send Short Message on PDCCH using P-RNTI.    <-- PDCCH (DCI 1_0): Short Message  -   -

//Wait for 2.1* modification period to allow the new system information to take effect

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, //@sic R5-221459 sic@

cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(9,1),

-, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B)));

f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@

f_NR_WaitModificationPeriods(nr_Cell1);

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]);

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

//@siclog "Steps 11 - 11A" siclog@

//The SS changes the SIB1 according to 38.508-1 [4] Table 4.6.1-28 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification  <-- PDCCH (DCI 1_0): Short Message

//Wait for 2.1* modification period to allow the new system information to take effect

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit);

f_NR_ModifySysinfo(nr_Cell1, -, RRC_CONNECTED); //@sic R5s230133 sic@

//f_NR_WaitModificationPeriods(nr_Cell1); REMOVED

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]);

//Postamble after receiving coordination from IMS PTC

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

}