3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230590

# Online, , 13th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3156 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-17 eCPSOR test case 6.3.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add test case 6.3.2.3 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.3.2.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR… |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	10

4	Execution Log Files	11

4.1	Mediatek MT6815	11

5	References	11

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.3.2.3 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	6.3.2.3

ATS Version:	iwd-TTCN3-B2022-09_D23wk24

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6815

Verification Status:	PASS

- Corrections required

Changes 2 and 3 of R5s230554 are also needed.

  - Change 1

| Function name | fl_TC_6_3_2_3_TestBody () |
| --- | --- |
| Reason for change | - Current TTCN is not implemented SOR-CMCI container correctly. As per 24.501 Figure: 9.11.3.51.2A it should be like below 8; 7; 6; 5; 4; 3; 2; 1. SOR transparent container IEI; octet 1. Length of SOR transparent container contents; octet 2 octet 3. SOR header; octet 4. SOR-MAC-IAUSF; octet 5 octet 20. CounterSOR; octet 21 octet 22. Length of PLMN ID and access technology list; octet 23*. PLMN ID and access technology list; octet 24* octet m*. 0 Spare; 0 Spare; 0 Spare; 0 Spare; 0 Spare; SSSI; SSCMI; SI; octet o. SOR-CMCI 12oct; octet (o+1)* octet p*. SOR-CMCI is as below Length of SOR-CMCI contents 2oct; octet o+1 octet o+2. SOR-CMCI rule 1 Length of SOR-CMCI rule contents 2oct; octet q+1 octet q+2. Tsor-cm timer value 1oct 21; octet q+3. Criterion type 1oct 01; octet q+4. Criterion value 6oct; octet (q+5)* octet r*...; octet (o+1)* octet p*. SOR-SNPN-SI; octet (p+1)* octet u. Figure 9.11.3.51.2A: SOR transparentcontainer information element for list type with value "1", SOR datatype with value "0", additional parameters with value "1" So updated variables and v_data variable according to figure Figure 9.11.3.51.2A. |
| Summary of change | - Added variables for Length of PLMN ID and access technology list, Length of SOR-CMCI contents , Length of SOR-CMCI rule contents and modified v_Data |
| TTCN module | SOR_CMCI_NR5GC |
| MCC160 Comment | Accepted in principle, but implemented differently. |

Before Change

function fl_TC_6_3_2_3_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233458 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= 'FF'O;//@sic R5-233458 sic@ var O1_Type v_Length :='03'O; var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233458 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; //@siclog "Step 1" siclog@ //Void//@sic R5-233458 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233458 sic@ //@siclog "Step 22-34b3a1" siclog@ v_Data := v_PlmnID & v_AccessTechId & v_SORInfo& v_Length & v_TsorcmTimer & v_CriterionType; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(crs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, int2oct(lengthof(v_Data) + 19, 2)))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); f_NR5GC_TransportSorCmCheckfunction(nr_Cell12,nr_Cell11,v_PduToSend,-,"Step 26 PASS","Step 28 PASS"); }//End of fl_TC_6_3_2_3_TestBody

After Change

function fl_TC_6_3_2_3_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233458 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= 'FF'O;//@sic R5-233458 sic@ var O1_Type v_Length_SOR_CMCI_rule_contents :=int2oct(2,2); var O1_Type v_Length_SOR_CMCI_contents :=int2oct(4,2); var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233458 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1); //@siclog "Step 1" siclog@ //Void//@sic R5-233458 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233458 sic@ //@siclog "Step 22-34b3a1" siclog@ v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(crs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, int2oct(lengthof(v_Data) + 19, 2)))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); f_NR5GC_TransportSorCmCheckfunction(nr_Cell12,nr_Cell11,v_PduToSend,-,"Step 26 PASS","Step 28 PASS"); }//End of fl_TC_6_3_2_3_TestBody

MCC160 proposed implementation

After Change

function fl_TC_6_3_2_3_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233458 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= 'FF'O;//@sic R5-233458 sic@ var O1_Type v_Length_SOR_CMCI_rule_contents := '01'O; var O1_Type v_Length_SOR_CMCI_contents := '02'O; var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233458 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1); //@siclog "Step 1" siclog@ //Void//@sic R5-233458 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233458 sic@ //@siclog "Step 22-34b3a1" siclog@ v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(crs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, int2oct(lengthof(v_Data) + 19, 2)))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); f_NR5GC_TransportSorCmCheckfunction(nr_Cell12,nr_Cell11,v_PduToSend,-,"Step 26 PASS","Step 28 PASS"); }//End of fl_TC_6_3_2_3_TestBody

- Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

- Execution Log Files

  - Mediatek MT6815

The Mediatek MT6815 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_3_2_3_LOG.html

- PICS/PIXIT parameter file:

TC_6_3_2_3_PIXIT.xml

- References

| [1] | R5s230591: NR5GC FR1: Supporting information for addition of Rel-17 eCPSOR test case 6.3.2.3 |
| --- | --- |