3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230600

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3161 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC test case 6.3.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester Technology , Mediatek inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A conformance UE may fail the test case in Step25E due to a mismatch on 5GMM cause. As per 38.523-1 Table 6.3.1.3.5-6, 5GMM cause shall be checked against value ‘00010110’B(Cause #22 (Congestion)). |  |  |  |  |  |  |  |  |
| Summary of change: |  | At Step 25E, 5GMM cause is set as ‘00010110’B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Pengchong Shen
	shenpengchong@cictmobile.com

- Corrections required

  - Change 1

| Function name | fl_TC_6_3_1_3_TestBody() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | A conformance UE may fail the test case in Step25E due to a mismatch on 5GMM cause. As per 38.523-1 Table 6.3.1.3.5-6, 5GMM cause shall be checked against value ‘00010110’B(Cause #22 (Congestion)). |
| Summary of change | At Step 25E, 5GMM cause is set as ‘00010110’B |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle, but implemented differently. |

Before Change

function fl_TC_6_3_1_3_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202599 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202599 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202599 sic@ var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var NAS_PlmnId v_PlmnID_3 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-204543 sic@ var O2_Type v_AccessTechId :='0800'O;//@sic R5s200680 sic@ var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); timer t_Tmax; ...... ...... //@siclog "Step 24" siclog@ //The SS transmits an RRCSetup message on NR Cell 12.//@sic R5-202599 sic@ //@siclog "Step 25" siclog@ //The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message on NR Cell 12.//@sic R5-202599 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell12); //@siclog "Step 25A-25F" siclog@ // Steps 4-9 of Table 4.9.8.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. // Do Authentication and SMC f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PlmnID_3, v_TypeOfRegistration); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_EmmCause_IllegalMe))))); // @sic R5s190435 sic@ f_NR_RRCRelease (nr_Cell12); //@siclog "Steps 26-28a1 Void" siclog@//@sic R5-225375 sic@ }//End of fl_TC_6_3_1_3_TestBody

After Change

function fl_TC_6_3_1_3_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202599 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202599 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202599 sic@ var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var NAS_PlmnId v_PlmnID_3 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-204543 sic@ var O2_Type v_AccessTechId :='0800'O;//@sic R5s200680 sic@ var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); timer t_Tmax; ...... ...... //@siclog "Step 24" siclog@ //The SS transmits an RRCSetup message on NR Cell 12.//@sic R5-202599 sic@ //@siclog "Step 25" siclog@ //The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message on NR Cell 12.//@sic R5-202599 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell12); //@siclog "Step 25A-25F" siclog@ // Steps 4-9 of Table 4.9.8.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. // Do Authentication and SMC f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PlmnID_3, v_TypeOfRegistration); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion))))); // @sic R5s190435 sic@ f_NR_RRCRelease (nr_Cell12); //@siclog "Steps 26-28a1 Void" siclog@//@sic R5-225375 sic@ }//End of fl_TC_6_3_1_3_TestBody

MCC160 implementation

function fl_TC_6_3_1_3_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202599 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202599 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202599 sic@ var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var NAS_PlmnId v_PlmnID_3 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-204543 sic@ var O2_Type v_AccessTechId :='0800'O;//@sic R5s200680 sic@ var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); timer t_Tmax; ...... ...... //@siclog "Step 24" siclog@ //The SS transmits an RRCSetup message on NR Cell 12.//@sic R5-202599 sic@ //@siclog "Step 25" siclog@ //The UE transmits an RRCSetupComplete message and a REGISTRATION REQUEST message on NR Cell 12.//@sic R5-202599 sic@ v_TypeOfRegistration := f_NR5GC_InitialOrMobilityRegistrationRequest (v_GMM_MobilityInfo, v_ReceivedMsg, nr_Cell12); //@siclog "Step 25A-25F" siclog@ // Steps 4-9 of Table 4.9.8.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. // Do Authentication and SMC f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PlmnID_3, v_TypeOfRegistration); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT((cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion),cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00001'B))))); // @sic R5s190435 sic@ f_NR_RRCRelease (nr_Cell12); //@siclog "Steps 26-28a1 Void" siclog@//@sic R5-225375 sic@ }//End of fl_TC_6_3_1_3_TestBody