3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230601

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3162 | rev | - | Current version: | 17.7.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ## Correction to NR5GC RRC test case 8.1.5.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.523-1-h30 Table 8.1.5.3.1.3.2-1,step8 is missing in test procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add step8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN not implemented accroding to the test procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.3.1.NR5GC, 8.1.5.3.2.NR5GC, 8.1.5.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Correction to NR5GC RRC test case 8.1.5.3.1	1

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Ting Gao
	gaoting @starpointcomm.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_5_3_CommonTestBody |
| --- | --- |
| Reason for change | According to TS38.523-1-h30 Table 8.1.5.3.1.3.2-1,step8 is missing in test procedure. |
| Summary of change | Add step8. |
| TTCN module | RRC_Others_PWS_Notification_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_5_3_CommonTestBody(NR_RRC_STATE_Type p_UE_State) runs on NR5GC_PTC { var template (value) SIB7 v_Sib7_Seg1; var template (value) SIB7 v_Sib7_Seg2; var template (value) SIB7 v_Sib7_Seg3; var template (value) SIB8 v_Sib8_Seg1; var template (value) SIB8 v_Sib8_Seg2; var template (value) SIB8 v_Sib8_Seg3; var template (omit) BCCH_DL_SCH_Message v_BCCH_SI1 := omit; var template (omit) BCCH_DL_SCH_Message v_BCCH_SI2 := omit; var template (omit) BCCH_DL_SCH_Message v_BCCH_SI3 := omit; var template (value) NR_SI_List_Type v_NR_SI_List; var WarningMessageSegment_List v_WarningMsgSegList ; v_WarningMsgSegList := f_SegmentWarningCB_Data(px_PWS_CB_DataPage1, px_PWS_CB_DataPage2); //Table 8.1.5.3.1.3.2-1: Main behaviour //If pc_ETWS_NR is true, steps 1-8 will be excuted if(pc_ETWS_NR) { //@siclog "Steps 1-2" siclog@ //The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. //The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB6 on NR Cell 1 //The SS includes an ETWS message with new messageIdentifier and serialNumber in SIB6 f_NR_SetSysinfoCombinationToNR_10(nr_Cell1, cs_38508_SIB6_Def); f_NR_ModifySysinfo_PWS(nr_Cell1, p_UE_State); //@sic R5s230129 sic@ //@siclog "Step 3" siclog@ //Check: Does the UE indicate the contents of the "warning message" to the user and alert or DCI 1_0activate alerting the user (NOTE 1)? if(f_UT_CheckETWS_CMAS_Alert(UT)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3"); } //@siclog "Step 4" siclog@ //The SS waits for 10s. f_Delay(10.0); //@siclog "Steps 5-6" siclog@ //The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. //The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB6 and SIB7 on NR Cell 1 //The SS includes an ETWS message with new messageIdentifier and serialNumber in SIB7. (NOTE 2). v_Sib7_Seg1 := cs_38508_SIB7_Def(0, v_WarningMsgSegList[0], notLastSegment, tsc_NR_PWS_DataCodingScheme); v_Sib7_Seg2 := cs_38508_SIB7_Def(1, v_WarningMsgSegList[1], notLastSegment); v_Sib7_Seg3 := cs_38508_SIB7_Def(2, v_WarningMsgSegList[2]); v_BCCH_SI1 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB7(v_Sib7_Seg1))); v_BCCH_SI2 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB7(v_Sib7_Seg2))); v_BCCH_SI3 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB7(v_Sib7_Seg3))); v_NR_SI_List := {v_BCCH_SI1, v_BCCH_SI2, v_BCCH_SI3}; f_NR_SetSysinfoCombination_ToNR_9Or_11(nr_Cell1, NR_11, {v_NR_SI_List}); f_NR_ModifySysinfo_PWS(nr_Cell1, p_UE_State); //@sic R5s230129 sic@ //@siclog "Steps 7" siclog@ //Check: Does the UE indicate the completely reassembled contents of the "warning message" to the user, and alert or activate alerting the user (NOTE 2)? if(f_UT_CheckETWS_CMAS_Alert(UT) and (f_UT_CheckETWS_CMAS(UT,px_PWS_CB_DataPage1, px_PWS_CB_DataPage2))) //@sicR5-195676, R5-197860 sic@ { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7"); } } //If pc_CMAS_NR is true, steps 9-11 will be excuted if(pc_CMAS_NR) { //@siclog "Steps 9-10" siclog@ //The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB8 on NR Cell 1. //The SS includes a CMAS message with new messageIdentifier and serialNumber in SIB8. (NOTE 4). v_Sib8_Seg1 := cs_38508_SIB8_Def(0, v_WarningMsgSegList[0], notLastSegment, tsc_NR_PWS_DataCodingScheme); v_Sib8_Seg2 := cs_38508_SIB8_Def(1, v_WarningMsgSegList[1], notLastSegment); v_Sib8_Seg3 := cs_38508_SIB8_Def(2, v_WarningMsgSegList[2]); v_BCCH_SI1 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB8(v_Sib8_Seg1))); v_BCCH_SI2 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB8(v_Sib8_Seg2))); v_BCCH_SI3 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB8(v_Sib8_Seg3))); v_NR_SI_List := {v_BCCH_SI1, v_BCCH_SI2, v_BCCH_SI3}; f_NR_SetSysinfoCombination_ToNR_9Or_11(nr_Cell1, NR_9, {v_NR_SI_List}); f_NR_ModifySysinfo_PWS(nr_Cell1, p_UE_State); //@sic R5s230129 sic@ //@siclog "Steps 11" siclog@ //Check: Does the UE indicate the completely reassembled contents of the "warning message" to the user, and alert or activate alerting the user (NOTE 2)? if(f_UT_CheckETWS_CMAS_Alert(UT) and f_UT_CheckETWS_CMAS(UT,px_PWS_CB_DataPage1, px_PWS_CB_DataPage2, true)) //@sic R5-195676, R5-197860 sic@ { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11"); } } }//End of f_TC_8_1_5_3_CommonTestBody()

After Change:

function f_TC_8_1_5_3_CommonTestBody(NR_RRC_STATE_Type p_UE_State) runs on NR5GC_PTC { var template (value) SIB7 v_Sib7_Seg1; var template (value) SIB7 v_Sib7_Seg2; var template (value) SIB7 v_Sib7_Seg3; var template (value) SIB8 v_Sib8_Seg1; var template (value) SIB8 v_Sib8_Seg2; var template (value) SIB8 v_Sib8_Seg3; var template (omit) BCCH_DL_SCH_Message v_BCCH_SI1 := omit; var template (omit) BCCH_DL_SCH_Message v_BCCH_SI2 := omit; var template (omit) BCCH_DL_SCH_Message v_BCCH_SI3 := omit; var template (value) NR_SI_List_Type v_NR_SI_List; var WarningMessageSegment_List v_WarningMsgSegList ; v_WarningMsgSegList := f_SegmentWarningCB_Data(px_PWS_CB_DataPage1, px_PWS_CB_DataPage2); //Table 8.1.5.3.1.3.2-1: Main behaviour //If pc_ETWS_NR is true, steps 1-8 will be excuted if(pc_ETWS_NR) { //@siclog "Steps 1-2" siclog@ //The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. //The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB6 on NR Cell 1 //The SS includes an ETWS message with new messageIdentifier and serialNumber in SIB6 f_NR_SetSysinfoCombinationToNR_10(nr_Cell1, cs_38508_SIB6_Def); f_NR_ModifySysinfo_PWS(nr_Cell1, p_UE_State); //@sic R5s230129 sic@ //@siclog "Step 3" siclog@ //Check: Does the UE indicate the contents of the "warning message" to the user and alert or DCI 1_0activate alerting the user (NOTE 1)? if(f_UT_CheckETWS_CMAS_Alert(UT)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3"); } //@siclog "Step 4" siclog@ //The SS waits for 10s. f_Delay(10.0); //@siclog "Steps 5-6" siclog@ //The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. //The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB6 and SIB7 on NR Cell 1 //The SS includes an ETWS message with new messageIdentifier and serialNumber in SIB7. (NOTE 2). v_Sib7_Seg1 := cs_38508_SIB7_Def(0, v_WarningMsgSegList[0], notLastSegment, tsc_NR_PWS_DataCodingScheme); v_Sib7_Seg2 := cs_38508_SIB7_Def(1, v_WarningMsgSegList[1], notLastSegment); v_Sib7_Seg3 := cs_38508_SIB7_Def(2, v_WarningMsgSegList[2]); v_BCCH_SI1 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB7(v_Sib7_Seg1))); v_BCCH_SI2 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB7(v_Sib7_Seg2))); v_BCCH_SI3 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB7(v_Sib7_Seg3))); v_NR_SI_List := {v_BCCH_SI1, v_BCCH_SI2, v_BCCH_SI3}; f_NR_SetSysinfoCombination_ToNR_9Or_11(nr_Cell1, NR_11, {v_NR_SI_List}); f_NR_ModifySysinfo_PWS(nr_Cell1, p_UE_State); //@sic R5s230129 sic@ //@siclog "Steps 7" siclog@ //Check: Does the UE indicate the completely reassembled contents of the "warning message" to the user, and alert or activate alerting the user (NOTE 2)? if(f_UT_CheckETWS_CMAS_Alert(UT) and (f_UT_CheckETWS_CMAS(UT,px_PWS_CB_DataPage1, px_PWS_CB_DataPage2))) //@sicR5-195676, R5-197860 sic@ { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7"); } //@siclog "Step 8" siclog@ //The SS waits for 10s. f_Delay(10.0); } //If pc_CMAS_NR is true, steps 9-11 will be excuted if(pc_CMAS_NR) { //@siclog "Steps 9-10" siclog@ //The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB8 on NR Cell 1. //The SS includes a CMAS message with new messageIdentifier and serialNumber in SIB8. (NOTE 4). v_Sib8_Seg1 := cs_38508_SIB8_Def(0, v_WarningMsgSegList[0], notLastSegment, tsc_NR_PWS_DataCodingScheme); v_Sib8_Seg2 := cs_38508_SIB8_Def(1, v_WarningMsgSegList[1], notLastSegment); v_Sib8_Seg3 := cs_38508_SIB8_Def(2, v_WarningMsgSegList[2]); v_BCCH_SI1 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB8(v_Sib8_Seg1))); v_BCCH_SI2 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB8(v_Sib8_Seg2))); v_BCCH_SI3 := cs_NR_BCCH_DL_SCH_SIB(cs_NR_SI(cs_NR_SystemInformation_SIB8(v_Sib8_Seg3))); v_NR_SI_List := {v_BCCH_SI1, v_BCCH_SI2, v_BCCH_SI3}; f_NR_SetSysinfoCombination_ToNR_9Or_11(nr_Cell1, NR_9, {v_NR_SI_List}); f_NR_ModifySysinfo_PWS(nr_Cell1, p_UE_State); //@sic R5s230129 sic@ //@siclog "Steps 11" siclog@ //Check: Does the UE indicate the completely reassembled contents of the "warning message" to the user, and alert or activate alerting the user (NOTE 2)? if(f_UT_CheckETWS_CMAS_Alert(UT) and f_UT_CheckETWS_CMAS(UT,px_PWS_CB_DataPage1, px_PWS_CB_DataPage2, true)) //@sic R5-195676, R5-197860 sic@ { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11"); } } }//End of f_TC_8_1_5_3_CommonTestBody()