3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230650

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4756 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NBIOT RRC testcase 22.4.27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, NB_IOTenh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add testcase 22.4.27 to the ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 22.4.27 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to NBIOT testsuite |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	f_TC_22_4_27_NBIOT	3

3	Branches executed	5

4	Execution Log Files	5

4.1	U-blox SARA-R510M8S	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 22.4.27 which is part of the NBIOT test suite in ATS iwd-TTCN3-B2022-09_D23wk24.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

1.1 Verification Test Summary

Test Case: 			22.4.27
ATS Version:			iwd-TTCN3-B2022-09_D23wk24
System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR
UE used:			U-blox SARA-R510M8S
Verification Status:	 		PASS

# Corrections required

## A2.1 f_TC_22_4_27_NBIOT

| Function name | f_TC_22_4_27_NBIOT |
| --- | --- |
| Reason for change | UE won’t be able to read the paging message at Step 42, as the cell selection criteria wont be satisfied since q-RxLevMin broadcasted in SIB1 is -106dBm and the cell reference power is set to -110dBm as per T3 of Table 22.4.27.3.2-1, at Step 40. #associated proseCR(R5-234094) is agreed in RAN5#100 meeting |
| Summary of change | Set the q-RxLevMin broadcasted in SIB1 to -120dBm on Cell 1 throughout the testcase procedure. |
| TTCN module | NBIOT_RRC_ControlPlane.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_TC_22_4_27_NBIOT() runs on NBIOT_PTC

{ // NB-IoT / RRC connection establishment / Access barring enhancement

var integer v_DelayLoopbackDataTime := 15;

var template (value) SystemInformationBlockType2_NB_r13 v_SIB2;

var template (value) SystemInformationBlockType14_NB_r13 v_SIB14;

var octetstring v_UserData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var float v_ModificationPeriod_s;

var boolean v_Rxd_RRCConnReq := false;

var integer v_WaitTime := 30;

var float v_WaitForConnection := f_NBIOT_SetTimerToleranceMax(rrcTimer, int2float(v_WaitTime));

var template (value) RSRP_ThresholdsNPRACH_InfoList_NB_r13 v_RSRP_ThresholdsNPRACH_InfoList_NB := {60, 40};

var template (value) NPRACH_ParametersList_NB_r13 v_NPRACH_ParametersList_NB := {

cs_NPRACH_Parameters_Default(ms640, ms8,   -, n3, n1),  // CE level 0: nprach-StartTime := 8,   maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 1

cs_NPRACH_Parameters_Default(ms640, ms32,  -, n3, n2),  // CE level 1: nprach-StartTime := 32,  maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 2

cs_NPRACH_Parameters_Default(ms640, ms128, -, n3, n4)   // CE level 2: nprach-StartTime := 128, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 4

};

var template (value) RACH_InfoList_NB_r13 v_RACH_InfoList := { cs_RACH_Info_NB_Default, cs_RACH_Info_NB_Default, cs_RACH_Info_NB_Default };

var template (value) NPUSCH_ACK_NACK_NumRepetitions_Msg4_Type v_NPUSCH_ACK_NACK_NumRepetitions_Msg4 := { r8, r8, r8 };

timer t_Wait;

f_NBIOT_Init(c4);

f_NBIOT_CellInfo_InitMaxReferencePower(nbiot_Cell1, -70);

//Modify system information

v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1);

v_SIB2.radioResourceConfigCommon_r13.nprach_Config_r13 := cs_508_NPRACH_ConfigSIB_NB(v_RSRP_ThresholdsNPRACH_InfoList_NB, v_NPRACH_ParametersList_NB);   /* Table 22.4.27.3.3-2 */

v_SIB2.radioResourceConfigCommon_r13.rach_ConfigCommon_r13 := cs_508_RACH_ConfigCommon_NB_Default(n7, v_RACH_InfoList);                                  /* Table 22.4.27.3.3-5 */

v_SIB2.radioResourceConfigCommon_r13.npusch_ConfigCommon_r13 := cs_508_NPUSCH_ConfigCommon_NB_Default(v_NPUSCH_ACK_NACK_NumRepetitions_Msg4);            /* Table 22.4.27.3.3-6 */

f_NBIOT_CellInfo_SetSIB2(nbiot_Cell1, v_SIB2);

//Set SIB14 acc. to Table 22.4.27.3.3-7

v_SIB14 := f_NBIOT_CellInfo_GetSIB14(nbiot_Cell1);

v_SIB14.ab_Param_r13 := omit;

v_SIB14.ab_PerNRSRP_r15 := thresh1;

f_NBIOT_CellInfo_SetSIB14(nbiot_Cell1, v_SIB14);

//Get ModificationPeriod

v_ModificationPeriod_s := int2float(f_NB_CalculateModificationPeriod (nbiot_Cell1))/100.0;

//Create and configure all cells

f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE);

//Equip UE with access class 0 and does not belong to special access class 11..15

f_UT_USIM_Insert(UT, "USIM must be configured for Access Class 0 and does not belong to special access class 11..15");

//Bring UE to initial state

f_NBIOT_Preamble(nbiot_Cell1, CONTROL_PLANE, STATE2A_NB_TESTLOOP_ModeG);

f_NBIOT_CloseUE_TestLoopModeGH(nbiot_Cell1, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS, 1, v_DelayLoopbackDataTime);

f_NBIOT_TestBody_Set(true);

// Skipped code

}

After change:

function f_TC_22_4_27_NBIOT() runs on NBIOT_PTC

{ // NB-IoT / RRC connection establishment / Access barring enhancement

var integer v_DelayLoopbackDataTime := 15;

var template (value) SystemInformationBlockType1_NB v_SIB1;

var template (value) SystemInformationBlockType2_NB_r13 v_SIB2;

var template (value) SystemInformationBlockType14_NB_r13 v_SIB14;

var octetstring v_UserData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var float v_ModificationPeriod_s;

var boolean v_Rxd_RRCConnReq := false;

var integer v_WaitTime := 30;

var float v_WaitForConnection := f_NBIOT_SetTimerToleranceMax(rrcTimer, int2float(v_WaitTime));

var template (value) RSRP_ThresholdsNPRACH_InfoList_NB_r13 v_RSRP_ThresholdsNPRACH_InfoList_NB := {60, 40};

var template (value) NPRACH_ParametersList_NB_r13 v_NPRACH_ParametersList_NB := {

cs_NPRACH_Parameters_Default(ms640, ms8,   -, n3, n1),  // CE level 0: nprach-StartTime := 8,   maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 1

cs_NPRACH_Parameters_Default(ms640, ms32,  -, n3, n2),  // CE level 1: nprach-StartTime := 32,  maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 2

cs_NPRACH_Parameters_Default(ms640, ms128, -, n3, n4)   // CE level 2: nprach-StartTime := 128, maxNumPreambleAttemptCE := 3, numRepetitionsPerPreambleAttempt := 4

};

var template (value) RACH_InfoList_NB_r13 v_RACH_InfoList := { cs_RACH_Info_NB_Default, cs_RACH_Info_NB_Default, cs_RACH_Info_NB_Default };

var template (value) NPUSCH_ACK_NACK_NumRepetitions_Msg4_Type v_NPUSCH_ACK_NACK_NumRepetitions_Msg4 := { r8, r8, r8 };

timer t_Wait;

f_NBIOT_Init(c4);

f_NBIOT_CellInfo_InitMaxReferencePower(nbiot_Cell1, -70);

//Modify system information

v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1);

v_SIB2.radioResourceConfigCommon_r13.nprach_Config_r13 := cs_508_NPRACH_ConfigSIB_NB(v_RSRP_ThresholdsNPRACH_InfoList_NB, v_NPRACH_ParametersList_NB);   /* Table 22.4.27.3.3-2 */

v_SIB2.radioResourceConfigCommon_r13.rach_ConfigCommon_r13 := cs_508_RACH_ConfigCommon_NB_Default(n7, v_RACH_InfoList);                                  /* Table 22.4.27.3.3-5 */

v_SIB2.radioResourceConfigCommon_r13.npusch_ConfigCommon_r13 := cs_508_NPUSCH_ConfigCommon_NB_Default(v_NPUSCH_ACK_NACK_NumRepetitions_Msg4);            /* Table 22.4.27.3.3-6 */

f_NBIOT_CellInfo_SetSIB2(nbiot_Cell1, v_SIB2);

//Set SIB14 acc. to Table 22.4.27.3.3-7

v_SIB14 := f_NBIOT_CellInfo_GetSIB14(nbiot_Cell1);

v_SIB14.ab_Param_r13 := omit;

v_SIB14.ab_PerNRSRP_r15 := thresh1;

f_NBIOT_CellInfo_SetSIB14(nbiot_Cell1, v_SIB14);

//Get ModificationPeriod

v_ModificationPeriod_s := int2float(f_NB_CalculateModificationPeriod (nbiot_Cell1))/100.0;

// modify system information

v_SIB1 := f_NBIOT_CellInfo_GetSIB1(nbiot_Cell1);

v_SIB1.cellSelectionInfo_r13.q_RxLevMin_r13 := -60;

f_NBIOT_CellInfo_SetSIB1(nbiot_Cell1, v_SIB1);

//Create and configure all cells

f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE);

//Equip UE with access class 0 and does not belong to special access class 11..15

f_UT_USIM_Insert(UT, "USIM must be configured for Access Class 0 and does not belong to special access class 11..15");

//Bring UE to initial state

f_NBIOT_Preamble(nbiot_Cell1, CONTROL_PLANE, STATE2A_NB_TESTLOOP_ModeG);

f_NBIOT_CloseUE_TestLoopModeGH(nbiot_Cell1, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS, 1, v_DelayLoopbackDataTime);

f_NBIOT_TestBody_Set(true);

// Skipped code

}

# Branches executed

The test case implementation was executed with Integrity and ciphering enabled (eea1 and eia1) using FDD band 4.

# Execution Log Files

## U-Blox SARA-R510M8S UE

The U-Blox SARA-R510M8S UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_22_4_27_LOG.xml

- PICS/PIXIT parameter file:

TC_ 22_4_27_PIXIT.txt

# References

[1] R5s230651: Supporting information for addition of NBIOT test case 22.4.27