3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230659

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3186 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 7.1.1.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In step 10 when handling different possibilities in altstep, one of the curly brackets is misplaced for the case when RRC message is not yet received. In curret implementation the v_RRC_MsgRcvd would never get updated to TRUE because its update is enclosed in the condition where decoding of ReconfigurationCOmplete doesn’t match and would fail the test case anyway. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The closing curly bracket for decoding mismatch is moved right after the “FatalError” so it doesn’t include update of the v_RRC_MsgRcvd and the eventual “repeat” on the next line. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For certain UE behaviour in step 10 the test can fail conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.7.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_7_ENDC_NR |
| --- | --- |
| Reason for change | In step 10 when handling different possibilities in altstep, one of the curly brackets is misplaced for the case when RRC message is not yet received. In curret implementation the v_RRC_MsgRcvd would never get updated to TRUE because its update is enclosed in the condition where decoding of ReconfigurationCOmplete doesn’t match and would fail the test case anyway. |
| Summary of change | The closing curly bracket for decoding mismatch is moved right after the “FatalError” so it doesn’t include update of the v_RRC_MsgRcvd and the eventual “repeat” on the next line. |
| TTCN module | ENDC\7_1_1\MAC_ENDC_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_3_7_ENDC_NR() runs on ENDC_NR_PTC { … //@siclog "Steps 10" siclog@ alt { [not v_RRC_MsgRcvd] EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_OctetData := v_ReceivedMsg.OctetData.Data; v_EncodedComplete := oct2bit(v_OctetData); v_Result := decvalue(v_EncodedComplete, v_Complete); if (v_Result != 0 or not match (v_Complete, cr_38508_RRCReconfigurationCompleteIE)) { FatalError(__FILE__, __LINE__, "Decoding error Step 11"); v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } //misplaced bracket } [not v_PHR_Rcvd]SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } [] t_Watchdog.timeout //@sic R5s220484 sic@ { if(not v_PHR_Rcvd) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3: TP1 failed, UE didn't send PHR after phr-PeriodicTimer is configured in UE "); } } }; //@sic R5s210883 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, 200); // @siclog "Step 13" siclog@

After Change:

function f_TC_7_1_1_3_7_ENDC_NR() runs on ENDC_NR_PTC { … //@siclog "Steps 10" siclog@ alt { [not v_RRC_MsgRcvd] EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); v_OctetData := v_ReceivedMsg.OctetData.Data; v_EncodedComplete := oct2bit(v_OctetData); v_Result := decvalue(v_EncodedComplete, v_Complete); if (v_Result != 0 or not match (v_Complete, cr_38508_RRCReconfigurationCompleteIE)) { FatalError(__FILE__, __LINE__, "Decoding error Step 11"); } //correctly closed here v_RRC_MsgRcvd := true; if(not v_PHR_Rcvd){ repeat;} } [not v_PHR_Rcvd]SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(nr_Cell1, cr_NR_MAC_CE_MultiEntryPHR(?)))->value v_NR_SYSTEM_IND { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_TimingInfo1 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame; v_PHR_Rcvd := true; if(not v_RRC_MsgRcvd){ repeat;} } [] t_Watchdog.timeout //@sic R5s220484 sic@ { if(not v_PHR_Rcvd) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3: TP1 failed, UE didn't send PHR after phr-PeriodicTimer is configured in UE "); } } }; //@sic R5s210883 sic@ v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, 200); // @siclog "Step 13" siclog@