3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230706

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4761 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NTN common functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently when RRC Connection release procedure is triggered v_MilliSecondsDelay =300msec is being used to calculate the next search space. When querying for activation time, there is an active DL transmission being triggered and therefore using 300msec delay would result in SS not able to send the RRC connection release message at the correct activation time. It is proposed to use f_NBIOT_NTN_Get_t_NTNdelay to get the next search space cycle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed c8 condition from the if statement and also updated v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension in the else if statement above to update SIB1 accordingly with NTN parameters. It is proposed to use f_NBIOT_NTN_Get_t_NTNdelay to get the next search space cycle. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ALL NB-NTN test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for NTN common functions	2

2.1	Correction to f_NBIOT_InitSystemInformation	2

2.2	Correction to f_NBIOT_RRC_ConnectionRelease_Common	9

2.3	Correction to f_NBIOT_CellArray_Init	10

2.4	Correction to f_NBIOT_CellInfo_Get_Kmac	14

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required for NTN common functions

## Correction to f_NBIOT_InitSystemInformation

| Function name | f_NBIOT_InitSystemInformation |
| --- | --- |
| Reason for change | When test case is executed with c8 combination the v_SI_List is updated with SIB22 configuration. According to 36.508-1 c8 condition includes SIB2 and SIB 31 |
| Summary of change | Removed c8 condition from the if statement and also updated v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension in the else if statement above to update SIB1 accordingly with NTN parameters. |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment | Accepted Additionally, for more clarity combination c9 is removed form conditon check else if (p_Combination == c8 or p_Combination == c9 or p_Combination == c10 or p_Combination == c11 or p_Combination == c12 or p_Combination == c13) { //@sic R5-231904, R5-231933, R5-233367, R5-233365, R5-235331 sic@ as it has its own branch now due to the implementation of R5-235420 |

Before Change:

function f_NBIOT_InitSystemInformation(NBIOT_CellId_Type p_CellId, EUTRA_ASN1_CellIdentity_Type p_CellIdentity, NB_Combination_Type p_Combination, integer p_FrequencyBand, NB_OperationMode_Type p_OperationMode, template (value) PLMN_IdentityList_NB_r13 p_PLMN_Identity, EUTRA_ASN1_TrackingAreaCode_Type p_TrackingAreaCode, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC return template (value) NB_BcchInfo_Type { var template (value) NB_BcchInfo_Type v_CellBcchInfo; var template (value) BCCH_BCH_Message_NB v_MIB; var template (value) BCCH_DL_SCH_Message_NB v_SIB1; var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB2; var template (value) SystemInformation_NB_r13_IEs v_SIB_SI2; var template (value) SystemInformation_NB_r13_IEs v_SIB_SI3; var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset; var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize; var template (value) NB_OperationModeInfo_Type v_OperationMode; var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList; var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI2; var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI3; var NB_CarrierFreq_Type v_CarrierFreq := fl_NBIOT_Get_CarrierFreq(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN); // @sic R5-185113 sic@ select (p_OperationMode) { case (standalone) { v_OperationMode := cs_NB_OperationModeInfo_Standalone; v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } case (inband_samePCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); // @sic R5-181295 sic@ v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := dB0; } case (inband_differentPCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@ v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := omit; } case (guardband) { v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@ v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } } // --- Initialise MIB --- v_MIB := cs_508_MasterInformationBlock_NB_Init(v_OperationMode); // --- Initialise SIB1 --- // -- A default scheduling assigned ---------- v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(p_PLMN_Identity, p_TrackingAreaCode, p_CellIdentity, p_FrequencyBand, v_NB_EutraControlRegionSize, v_NRS_CRS_PowerOffset, cs_SchedulingInfo_NB_1SI); // --- Initialise SIB2 --- v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); // --- Initialise all SYSINFO -- if (p_Combination == c1) { v_SchedulingInfoList := cs_SchedulingInfo_NB_1SI; } else if (p_Combination == c5) { // @sic R5-185113 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB20(cs_508_SystemInformationBlockType20_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType20_NB_r14}); } else if (p_Combination == c6) { // @sic R5-185113 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType22_NB_r14}); } else if (p_Combination == c7) { //@sic R5-2226052 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB23(cs_508_SystemInformationBlockType23_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {sibType22_NB_r14}, -, -, -, {}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.schedulingInfoList_v1530 := cs_SchedulingInfo_NB_c7_r15; } else if (p_Combination == c8 or p_Combination == c9 or p_Combination == c10 or p_Combination == c11 or p_Combination == c12 or p_Combination == c13) { //@sic R5-231904, R5-231933, R5-233367, R5-233365, R5-235331 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(f_NBIOT_InitSIB31()); //@sic R5-235420, R5-235468 sic@ v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({}); v_MIB.message_.partEARFCN_17.earfcn_LSB := fl_NBIOT_Get_EARFCN_2LSB(v_CarrierFreq); v_MIB.message_.schedulingInfoSIB1_r13 := 5; //@sic R5-235420 sic@ v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred; v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17; v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); //@sic R5s230478 sic@ if (p_Combination == c8 or p_Combination == c10 or p_Combination == c11 or p_Combination == c12 or p_Combination == c13){ v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode); if(p_Combination == c10 or p_Combination == c11 or p_Combination == c13){ //c8+SIB3 v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def)); if(p_Combination == c10){ //+ SIB4 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB4(cs_508_SystemInformationBlockType4_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType4_NB_r13}); } else if(p_Combination == c11){ //+ SIB5 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB5(f_NBIOT_InitSystemInformation5(p_CellId, p_FrequencyBand, p_OperationMode))); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType5_NB_r13}); } else{//+ SIB14 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB14(cs_SystemInformationBlockType14_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType14_NB_r13}); } v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c10_c11_c13_r15); } else{//c12 = c8 + SIB22 v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {}, -, -, -, {sibType22_NB_r14}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c12_r15); } } else if (p_Combination == c9) {//@sic R5-235420 sic@ v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17; v_SI_List[0] :=cs_SystemInformation_Common_NB(cs_SI_NB_SIB2_and_SIB31(v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13, f_NBIOT_InitSIB31().sib_TypeAndInfo_r13[0].sib31_v1700)); // @sic R5-235468 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType3_NB_r13}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c9_r15); v_MIB.message_.partEARFCN_17.earfcn_LSB := fl_NBIOT_Get_EARFCN_2LSB(v_CarrierFreq); v_MIB.message_.schedulingInfoSIB1_r13 := 5; }

After Change:

function f_NBIOT_InitSystemInformation(NBIOT_CellId_Type p_CellId, EUTRA_ASN1_CellIdentity_Type p_CellIdentity, NB_Combination_Type p_Combination, integer p_FrequencyBand, NB_OperationMode_Type p_OperationMode, template (value) PLMN_IdentityList_NB_r13 p_PLMN_Identity, EUTRA_ASN1_TrackingAreaCode_Type p_TrackingAreaCode, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC return template (value) NB_BcchInfo_Type { var template (value) NB_BcchInfo_Type v_CellBcchInfo; var template (value) BCCH_BCH_Message_NB v_MIB; var template (value) BCCH_DL_SCH_Message_NB v_SIB1; var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB2; var template (value) SystemInformation_NB_r13_IEs v_SIB_SI2; var template (value) SystemInformation_NB_r13_IEs v_SIB_SI3; var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset; var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize; var template (value) NB_OperationModeInfo_Type v_OperationMode; var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList; var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI2; var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI3; var NB_CarrierFreq_Type v_CarrierFreq := fl_NBIOT_Get_CarrierFreq(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN); // @sic R5-185113 sic@ select (p_OperationMode) { case (standalone) { v_OperationMode := cs_NB_OperationModeInfo_Standalone; v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } case (inband_samePCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); // @sic R5-181295 sic@ v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := dB0; } case (inband_differentPCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@ v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := omit; } case (guardband) { v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@ v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } } // --- Initialise MIB --- v_MIB := cs_508_MasterInformationBlock_NB_Init(v_OperationMode); // --- Initialise SIB1 --- // -- A default scheduling assigned ---------- v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(p_PLMN_Identity, p_TrackingAreaCode, p_CellIdentity, p_FrequencyBand, v_NB_EutraControlRegionSize, v_NRS_CRS_PowerOffset, cs_SchedulingInfo_NB_1SI); // --- Initialise SIB2 --- v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); // --- Initialise all SYSINFO -- if (p_Combination == c1) { v_SchedulingInfoList := cs_SchedulingInfo_NB_1SI; } else if (p_Combination == c5) { // @sic R5-185113 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB20(cs_508_SystemInformationBlockType20_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType20_NB_r14}); } else if (p_Combination == c6) { // @sic R5-185113 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType22_NB_r14}); } else if (p_Combination == c7) { //@sic R5-2226052 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB23(cs_508_SystemInformationBlockType23_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {sibType22_NB_r14}, -, -, -, {}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.schedulingInfoList_v1530 := cs_SchedulingInfo_NB_c7_r15; } else if (p_Combination == c8 or p_Combination == c9 or p_Combination == c10 or p_Combination == c11 or p_Combination == c12 or p_Combination == c13) { //@sic R5-231904, R5-231933, R5-233367, R5-233365, R5-235331 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(f_NBIOT_InitSIB31()); //@sic R5-235420, R5-235468 sic@ v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({}); v_MIB.message_.partEARFCN_17.earfcn_LSB := fl_NBIOT_Get_EARFCN_2LSB(v_CarrierFreq); v_MIB.message_.schedulingInfoSIB1_r13 := 5; //@sic R5-235420 sic@ v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred; v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode); //WA#WI= moved code here to update for c8 v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17; v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); //@sic R5s230478 sic@ if (p_Combination == c10 or p_Combination == c11 or p_Combination == c12 or p_Combination == c13){ // removed c8 combination to avoid else statement updating c8 if(p_Combination == c10 or p_Combination == c11 or p_Combination == c13){ //c8+SIB3 v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def)); if(p_Combination == c10){ //+ SIB4 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB4(cs_508_SystemInformationBlockType4_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType4_NB_r13}); } else if(p_Combination == c11){ //+ SIB5 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB5(f_NBIOT_InitSystemInformation5(p_CellId, p_FrequencyBand, p_OperationMode))); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType5_NB_r13}); } else{//+ SIB14 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB14(cs_SystemInformationBlockType14_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType14_NB_r13}); } v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c10_c11_c13_r15); } else{//c12 = c8 + SIB22 v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {}, -, -, -, {sibType22_NB_r14}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c12_r15); } } else if (p_Combination == c9) {//@sic R5-235420 sic@ v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17; v_SI_List[0] :=cs_SystemInformation_Common_NB(cs_SI_NB_SIB2_and_SIB31(v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13, f_NBIOT_InitSIB31().sib_TypeAndInfo_r13[0].sib31_v1700)); // @sic R5-235468 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType3_NB_r13}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c9_r15); v_MIB.message_.partEARFCN_17.earfcn_LSB := fl_NBIOT_Get_EARFCN_2LSB(v_CarrierFreq); v_MIB.message_.schedulingInfoSIB1_r13 := 5; }

MCC160 Additional change:

function f_NBIOT_InitSystemInformation(NBIOT_CellId_Type p_CellId, EUTRA_ASN1_CellIdentity_Type p_CellIdentity, NB_Combination_Type p_Combination, integer p_FrequencyBand, NB_OperationMode_Type p_OperationMode, template (value) PLMN_IdentityList_NB_r13 p_PLMN_Identity, EUTRA_ASN1_TrackingAreaCode_Type p_TrackingAreaCode, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC return template (value) NB_BcchInfo_Type { var template (value) NB_BcchInfo_Type v_CellBcchInfo; var template (value) BCCH_BCH_Message_NB v_MIB; var template (value) BCCH_DL_SCH_Message_NB v_SIB1; var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB2; var template (value) SystemInformation_NB_r13_IEs v_SIB_SI2; var template (value) SystemInformation_NB_r13_IEs v_SIB_SI3; var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset; var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize; var template (value) NB_OperationModeInfo_Type v_OperationMode; var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList; var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI2; var SIB_MappingInfo_NB_r13 v_SIB_MappingInfo_SI3; var NB_CarrierFreq_Type v_CarrierFreq := fl_NBIOT_Get_CarrierFreq(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN); // @sic R5-185113 sic@ select (p_OperationMode) { case (standalone) { v_OperationMode := cs_NB_OperationModeInfo_Standalone; v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } case (inband_samePCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); // @sic R5-181295 sic@ v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := dB0; } case (inband_differentPCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@ v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := omit; } case (guardband) { v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); // @sic R5-176998 R5s180571 sic@ v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } } // --- Initialise MIB --- v_MIB := cs_508_MasterInformationBlock_NB_Init(v_OperationMode); // --- Initialise SIB1 --- // -- A default scheduling assigned ---------- v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(p_PLMN_Identity, p_TrackingAreaCode, p_CellIdentity, p_FrequencyBand, v_NB_EutraControlRegionSize, v_NRS_CRS_PowerOffset, cs_SchedulingInfo_NB_1SI); // --- Initialise SIB2 --- v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); // --- Initialise all SYSINFO -- if (p_Combination == c1) { v_SchedulingInfoList := cs_SchedulingInfo_NB_1SI; } else if (p_Combination == c5) { // @sic R5-185113 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB20(cs_508_SystemInformationBlockType20_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType20_NB_r14}); } else if (p_Combination == c6) { // @sic R5-185113 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType22_NB_r14}); } else if (p_Combination == c7) { //@sic R5-2226052 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB23(cs_508_SystemInformationBlockType23_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {sibType22_NB_r14}, -, -, -, {}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.schedulingInfoList_v1530 := cs_SchedulingInfo_NB_c7_r15; } else if (p_Combination == c8 or p_Combination == c10 or p_Combination == c11 or p_Combination == c12 or p_Combination == c13) { //@sic R5-231904, R5-231933, R5-233367, R5-233365, R5-235331 sic@ //removed c9 @sic R5s230706 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(f_NBIOT_InitSIB31()); //@sic R5-235420, R5-235468 sic@ v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({}); v_MIB.message_.partEARFCN_17.earfcn_LSB := fl_NBIOT_Get_EARFCN_2LSB(v_CarrierFreq); v_MIB.message_.schedulingInfoSIB1_r13 := 5; //@sic R5-235420 sic@ v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred; v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode); //WA#WI= moved code here to update for c8 v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17; v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); //@sic R5s230478 sic@ if (p_Combination == c10 or p_Combination == c11 or p_Combination == c12 or p_Combination == c13){ // removed c8 combination to avoid else statement updating c8 if(p_Combination == c10 or p_Combination == c11 or p_Combination == c13){ //c8+SIB3 v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def)); if(p_Combination == c10){ //+ SIB4 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB4(cs_508_SystemInformationBlockType4_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType4_NB_r13}); } else if(p_Combination == c11){ //+ SIB5 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB5(f_NBIOT_InitSystemInformation5(p_CellId, p_FrequencyBand, p_OperationMode))); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType5_NB_r13}); } else{//+ SIB14 v_SI_List[3] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB14(cs_SystemInformationBlockType14_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_4SI_NTN({sibType14_NB_r13}); } v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c10_c11_c13_r15); } else{//c12 = c8 + SIB22 v_SI_List[2] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL))); v_SchedulingInfoList := cs_SchedulingInfo_NB_3SI(-, -, -, {}, -, -, -, {sibType22_NB_r14}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c12_r15); } } } else if (p_Combination == c9) {//@sic R5-235420, R5s230706 sic@ v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17; v_SI_List[0] :=cs_SystemInformation_Common_NB(cs_SI_NB_SIB2_and_SIB31(v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13, f_NBIOT_InitSIB31().sib_TypeAndInfo_r13[0].sib31_v1700)); // @sic R5-235468 sic@ v_SI_List[1] := cs_SystemInformation_Common_NB(cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def)); v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({sibType3_NB_r13}); v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(p_TrackingAreaCode, cs_SchedulingInfo_NB_c9_r15); v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred; v_MIB.message_.partEARFCN_17.earfcn_LSB := fl_NBIOT_Get_EARFCN_2LSB(v_CarrierFreq); v_MIB.message_.schedulingInfoSIB1_r13 := 5; //} }

## Correction to f_NBIOT_RRC_ConnectionRelease_Common

| Function name | f_NBIOT_RRC_ConnectionRelease_Common |
| --- | --- |
| Reason for change | Currently when RRC Connection release procedure is triggered v_MilliSecondsDelay =300msec is being used to calculate the next search space. When querying for activation time, there is an active DL transmission being triggered and therefore using 300msec delay would result in SS not able to send the RRC connection release message at the correct activation time. It is proposed to use f_NBIOT_NTN_Get_t_NTNdelay to get the next search space cycle. |
| Summary of change | For ntn scenarios it is proposed to use v_MilliSecondsDelay := f_NBIOT_NTN_Get_t_NTNdelay(p_CellId); |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment | Accepted conditional CR approval for 36.523-3 |

Before Change:

function f_NBIOT_RRC_ConnectionRelease_Common(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, template (value) DL_DCCH_Message_NB p_RRCConnectionRelease, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NB_SRB_Identity_Type p_SrbId := omit) runs on NBIOT_PTC { /* @sic R5s170250: p_IOT_State instead of p_SrbId sic@ */ var NB_SRB_Identity_Type v_SrbId; var SubFrameTiming_Type v_TimingAtT; // time T: sending of RRCConnectionRelease acc. to 36.523-3 clause 7A.7 var SubFrameTiming_Type v_TimingNow := f_NBIOT_GetCurrentTiming(p_CellId); var integer v_MilliSecondsDelay := 300; var integer v_Duration; timer t_Timer; if (isvalue(p_SrbId)) {

After Change:

function f_NBIOT_RRC_ConnectionRelease_Common(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, template (value) DL_DCCH_Message_NB p_RRCConnectionRelease, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NB_SRB_Identity_Type p_SrbId := omit) runs on NBIOT_PTC { /* @sic R5s170250: p_IOT_State instead of p_SrbId sic@ */ var NB_SRB_Identity_Type v_SrbId; var SubFrameTiming_Type v_TimingAtT; // time T: sending of RRCConnectionRelease acc. to 36.523-3 clause 7A.7 var SubFrameTiming_Type v_TimingNow := f_NBIOT_GetCurrentTiming(p_CellId); var integer v_MilliSecondsDelay := 300; var integer v_Duration; timer t_Timer; if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None) {// NTN delay v_MilliSecondsDelay := f_NBIOT_NTN_Get_t_NTNdelay(p_CellId); } if (isvalue(p_SrbId)) {

## Correction to f_NBIOT_CellArray_Init

| Function name | f_NBIOT_CellArray_Init |
| --- | --- |
| Reason for change | When p_Neighbour_NTN_Cells is provided (TC 22.2.13), f_NBIOT_CellInfo_SetSIB31-> f_NBIOT_CellInfo_Get is called to retrieve vc_NBIOT_Global.CellArray, this results in runtime error as the vc_NBIOT_Global.CellArray is not populated yet. When p_Neighbour_NTN_Cells is provided (TC 22.2.13), the NtnCellNumber needs to be updated based on its current value. |
| Summary of change | Removed the call for the function f_NBIOT_CellInfo_SetSIB31 and updated SIB31 based on v_NBIOT_Cells[v_Index]. Added v_NBIOT_Cells[v_Index].NtnInfo.NtnCellNumber := v_NBIOT_Cells[v_Index].NtnInfo.NtnCellNumber+1, so that the 2nd list in cs_NTN_TimingAdvanceConfig_GSO_Array is choosen. |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment | Accepted. To simplify maintenance it is proposed to move existing code from f_NBIOT_CellArray_Init to fl_NBIOT_Common_Init after variable vc_NBIOT_Global.CellArray is initiated |

Before Change:

function f_NBIOT_CellArray_Init(NB_Combination_Type p_SysinfoCombination, boolean p_DoAttachWithoutPDN, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC return NBIOT_CellInfoList_Type { var NBIOT_CellInfoList_Type v_NBIOT_Cells; var integer v_Index; var integer v_PrimaryFrequencyBand := px_ePrimaryFrequencyBand; var integer v_SecondaryFrequencyBand := px_eSecondaryFrequencyBand; var NB_SubCarrierSpacingUL_Type v_SubCarrierSpacingUL := subCarrierSpacing_15kHz; var integer i; v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell1); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell1, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell2); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell2, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell3); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell3, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell4); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell4, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell5); // @sic R5-176139 sic@ v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell5, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell6); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell6, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell10); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell10, v_SecondaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell11); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell11, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell12); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell12, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell13); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell13, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell14); /*v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell14, FFS Waiting for a prose CR to 36.508 to delete this cell v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); */ v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell18); // @sic R5-176139 sic@ v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell18, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell23); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell23, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); if(ispresent(p_Neighbour_NTN_Cells)) {//@sic R5-235468 sic@ for(i:=0; i<lengthof(p_Neighbour_NTN_Cells); i:=i+1) { v_Index := f_NBIOT_CellInfo_GetIndex(valueof(p_Neighbour_NTN_Cells[i])); f_NBIOT_CellInfo_SetSIB31(valueof(p_Neighbour_NTN_Cells[i]), cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]); v_NBIOT_Cells[v_Index].NtnInfo.NtnCellNumber := i+1; } } return v_NBIOT_Cells; }

After Change:

function f_NBIOT_CellArray_Init(NB_Combination_Type p_SysinfoCombination, boolean p_DoAttachWithoutPDN, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC return NBIOT_CellInfoList_Type { var NBIOT_CellInfoList_Type v_NBIOT_Cells; var integer v_Index; var integer v_PrimaryFrequencyBand := px_ePrimaryFrequencyBand; var integer v_SecondaryFrequencyBand := px_eSecondaryFrequencyBand; var NB_SubCarrierSpacingUL_Type v_SubCarrierSpacingUL := subCarrierSpacing_15kHz; var integer i; v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell1); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell1, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell2); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell2, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell3); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell3, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell4); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell4, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell5); // @sic R5-176139 sic@ v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell5, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell6); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell6, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell10); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell10, v_SecondaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell11); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell11, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell12); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell12, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell13); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell13, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell14); /*v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell14, FFS Waiting for a prose CR to 36.508 to delete this cell v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); */ v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell18); // @sic R5-176139 sic@ v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell18, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); v_Index := f_NBIOT_CellInfo_GetIndex(nbiot_Cell23); v_NBIOT_Cells[v_Index] := f_NBIOT_CellInfo_Init(nbiot_Cell23, v_PrimaryFrequencyBand, p_SysinfoCombination, p_DoAttachWithoutPDN, v_SubCarrierSpacingUL); if(ispresent(p_Neighbour_NTN_Cells)) {//@sic R5-235468 sic@ for(i:=0; i<lengthof(p_Neighbour_NTN_Cells); i:=i+1) { v_Index := f_NBIOT_CellInfo_GetIndex(valueof(p_Neighbour_NTN_Cells[i])); // removed f_NBIOT_CellInfo_SetSIB31(v_CellInfo, cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]); global variabl is not updated yet to retreive the data if (valueof(v_NBIOT_Cells[v_Index].Sysinfo.Combination) != c9){ v_NBIOT_Cells[v_Index].Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib31_v1700 := cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]; } else{ v_NBIOT_Cells[v_Index].Sysinfo.BCCH_Info.SIs[0].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[1].sib31_v1700 := cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]; } v_NBIOT_Cells[v_Index].NtnInfo.NtnCellNumber := v_NBIOT_Cells[v_Index].NtnInfo.NtnCellNumber+1; } } return v_NBIOT_Cells; }

MCC160 Alternative implementation:

function fl_NBIOT_Common_Init(NB_InitParams_Local_Type p_Params, boolean p_DoAttachWithoutPDN, template (omit)NBIOT_CellId_List_Type p_Neighbour_NTN_Cells := omit) runs on NBIOT_PTC { vc_NBIOT_Global.Ctrl.Default := null; vc_NBIOT_Global.Ctrl.TestBody := false; vc_NBIOT_Global.Ctrl.VerdictCtrl := normal; vc_NBIOT_Global.Ctrl.HarqErrorHandling := ignore; vc_NBIOT_Global.Ctrl.LoopbackIsActivated := false; // Initialise security parameters vc_NBIOT_Global.Security := f_NBIOT_Security_Init(); // Initialise Cell array vc_NBIOT_Global.CellArray := f_NBIOT_CellArray_Init(p_Params.SysinfoCombination, p_DoAttachWithoutPDN); //@sic R5-235468, R5s230706 removed p_Neighbour_NTN_Cells sic@ vc_NBIOT_Global.CellArray := f_NBIOT_NAS_CellArray_Init(vc_NBIOT_Global.CellArray, p_Params.CellsOnSamePLMN, p_Params.SysinfoCombination, p_DoAttachWithoutPDN, p_Params.CellSpecialFreq); if(ispresent(p_Neighbour_NTN_Cells)) {//@sic R5-235468, R5s230706 sic@ for(i:=0; i<lengthof(p_Neighbour_NTN_Cells); i:=i+1) { v_Index := f_NBIOT_CellInfo_GetIndex(valueof(p_Neighbour_NTN_Cells[i])); f_NBIOT_CellInfo_SetSIB31(valueof(p_Neighbour_NTN_Cells[i]), cs_SystemInformationBlockType31_NB_r17_GSO_Array[i+1]); vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellNumber := vc_NBIOT_Global.CellArray[v_Index].NtnInfo.NtnCellNumber+1; } } //Initialise Mobile Info vc_NBIOT_Global.MobileInfo.Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.P_Tmsi := '00000000'O; vc_NBIOT_Global.MobileInfo.PDNs := omit; f_NBIOT_ActivateDefault(StandardDefault); }

## Correction to f_NBIOT_CellInfo_Get_Kmac

| Function name | f_NBIOT_CellInfo_Get_Kmac |
| --- | --- |
| Reason for change | When f_NBIOT_CellInfo_Get_Kmac is called, the execution results in runtime error as the value of Kmac is set to Omit in v_SIB31 |
| Summary of change | Added a check to see if the IE is present and assigned 0 if it is omitted. |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment | Accepted |

Before Change:

function f_NBIOT_CellInfo_Get_Kmac(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC return integer { var SystemInformationBlockType31_NB_r17 v_SIB31 := f_NBIOT_CellInfo_GetSIB31(p_CellId); return v_SIB31.servingSatelliteInfo_r17.k_Mac_r17 }

After Change:

function f_NBIOT_CellInfo_Get_Kmac(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC return integer { var SystemInformationBlockType31_NB_r17 v_SIB31 := f_NBIOT_CellInfo_GetSIB31(p_CellId); if (ispresent(v_SIB31.servingSatelliteInfo_r17.k_Mac_r17)) { return v_SIB31.servingSatelliteInfo_r17.k_Mac_r17 } else { return 0 }