3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230708

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4763 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NB-IoT NTN test case 22.4.30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add verified NB-IOT NTN testcase 22.4.30 to NBIOT testsuite |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 22.4.30 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to NBIOT testsuite |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of NB-IoT NTN test case 22.4.30 which is part of the NBIOT test suite in iwd-TTCN3-B2022-09_D23wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	22_4_30_NBIOT

ATS Version:	iwd-TTCN3-B2022-09_D23wk37

System Simulator used:	R&S® CMW 500 Protocol Conformance Test platform

UE used:	MediaTek MTK 6825

Verification Status:	PASS

# Corrections required for test case 22.4.30

Changes listed in R5s230706 have also been implemented.

## Correction to f_TC_22_4_30_NBIOT

| Function name | f_TC_22_4_30_NBIOT |
| --- | --- |
| Reason for change | - The user data is incorrect assigned to 01_Type, the length of the data is 10 bytes - If the UE supports re-establishment then the test case expects the UE to sends RRCConnectionReestablishmentRequest-NB at Step 27. The network in SIB2 shall indicate the support of cp_Reestablishment_r14 for the UE to trigger this. - When the function f_NBIOT_InitialRegistration_Step2_4 is called it is called during preamble, therefore the last parameter needs to be updated to PREAMBLE - Closed UE test loop mode function is called with incorrect cell id - After Step 20 and Step 24D v_Delay is used to delay the sending of data, but this is not required according to the prose - ESM data transport message for CP procedure needs to use tsc_PTI_Unassigned - When Starting uplink grant, its not required to use Activation time as this could delay the start of uplink grant. - The Start of T317 timer cannot be exactly synchronised, therefore it is proposed to start T317_min and T317_max, therefore it is proposed to start after the reception of data at Step 24B. T317_min once expired is used to start T318, and at expiry of T317_max the uplink grant is assigned. - If the UE supports pc_RRC_re_establishment_CP_CIoT, then the expectation of the current prose is to do a complete re-establishment procedure. According to 36.331 “RRCConnectionReestablishment-NB message is used to re-establish SRB1”. The current test case execution is for control plane and SRB1 is not configured. It is proposed to modify the test sequence to receive RRCConnectionReestablishmentRequest-NB and wait for T301 to expire and the UE goes into idle. The change requires an update to prose. - The current TTCN implementation was incorrectly using r13 template to handle RRCConnectionReestablishmentRequest-NB - The data transmitted at Step 24D, may be in the UE buffer and the UE will trigger Control plane service request procedure to loop back the data and this needs to be handled. The change requires an update to prose. |
| Summary of change | - Change the variable type to O10_Type - Added SIB2 timers and set cp_Reestablishment_r14 to true - Updated the last parameter in f_NBIOT_InitialRegistration_Step2_4 to PREAMBLE - Updated first parameter in f_NBIOT_CloseUE_TestLoopModeGH to nbiot_Cell1 - Removed the code to delay the test by v_Delay at Step Step 20 and 24D - Updated the test case to use tsc_PTI_Unassigned cs_ESM_DATA_TRANSPORT transmission / reception - Updated Step 24 to start continuous uplink grant immediately. - Moved T317 start timer after data and used T317_min and T317_max to control the start of T318 and sending the grant (Steps 24E and 24F). - Update the test case to handle RRCConnectionReestablishmentRequest-NB and wait for T301 to expire - Updated the TTCN code to use existing function f_NBIOT_RRC_RRCConnectionReestablishmentRequest_r14 - Added else statement to handle Control plane service request procedure and then perform a detach at the end of the test case |
| TTCN module | NBIOT_RRC_NTN |
| MCC160 Comment | - Accepted - Accepted conditional prose CR approval - Accepted - Accepted - Accepted conditional prose CR approval - Accepted - Accepted conditional prose CR approval - Accepted conditional prose CR approval - Accepted - Accepted - Accepted |

Before Change:

function f_TC_22_4_30_NBIOT() runs on NBIOT_PTC {// NB-IoT / NTN / Ephemeris information update / T317 Expiry / T318 Expiry @sic R5-235429 sic@ var NAS_MSG_Indication_Type v_NAS_Ind; var EUTRA_ASN1_RRC_TransactionIdentifier_Type v_RRC_TI := tsc_RRC_TI_Def; var NBIOT_SecurityParams_Type v_SecurityParams; var EUTRA_ASN1_ShortMAC_I_Type v_ShortMAC_I; var EUTRA_ASN1_PhysCellId_Type v_PhysCellId; var template (present) EUTRA_ASN1_ReestabUE_Identity_Type v_UE_Identity; var O1_Type v_UserData; var integer v_PDNIndex; var float v_Delay; var SubFrameTiming_Type v_TimingStart; var template (value) TimingInfo_Type v_TimingInfo; var template (value) NB_DciUlInfo_Type v_DciUlInfo; timer t_T317 := 60.0; //Corresponds to the value (60sec) of ul_SyncValidityDuration_r17 in SIB31-NB acc. Table 22.4.30.3.3-2 timer t_T318 := 8.0 ; //Corresponds to the value (8000ms) of t318-r17 acc 36.508 Table 4.6.3-14 //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8, CONTROL_PLANE); //Table 22.4.30.3.3-2, Value of T317=60s f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(nbiot_Cell1,s60); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT); //Preamble: The UE is in State 1-NB switched off according to TS 36.508 [18]. f_UT_PowerOffUE (UT); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //UE acquires SystemInformationBlockType31-NB and start T317 = 60 seconds. //@siclog "Steps 3-18b1" siclog@ //Steps 1 to 16b1 of the Attach, connected mode and test mode activated procedure described in TS 36.508 [18] subclause 8.1.5.2A.3 with TEST LOOP MODE A are performed on Ncell 1. // Acc to 36.508 Table 8.1.5.2A.3-1: NB-IoT UE Attach, connected mode and test mode activated procedure (state 2-NB to state 2A-NB) //Note: After receiving the message in step 4, SS start a timer T1 with value = T317. v_NAS_Ind := f_NBIOT_InitialRegistration_Step2_4(nbiot_Cell1, CONTROL_PLANE, NORMAL); t_T317.start; //T1 started f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2A_NB_TESTLOOP_ModeG, v_NAS_Ind); //@siclog "Steps 18A-18B" siclog@ //Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 8.1.5.2B.3 are performed on Ncell 1. //FFS: Prose CR needed to actiate a corresponding loopback A->G f_NBIOT_CloseUE_TestLoopModeGH (nbiot_Cell50, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS); //@siclog "Step 19" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1, true, false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 19A" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 20" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB. (Note 1) //Note 1: This could be done near the expiry of T1 in step 23 and T3 in step 24E v_Delay := t_T317.read - 1.0; if(v_Delay > 1.0){ f_Delay(v_Delay); //T317 is 6 sec, within last 1 sec the DL should be send out } v_UserData := f_GetN_OctetsFromPRBS(0, 10); v_PDNIndex := f_NBIOT_MobileInfo_GetNoOfPDNsConfigured(); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, f_NBIOT_MobileInfo_GetPTI(v_PDNIndex), cs_UserDataContainer(v_UserData))))); //@siclog "Steps 21-22" siclog@ //Void //@siclog "Step 23" siclog@ //Wait until expiry of T1. //Note: UE starts timer T318. SS start a timer T2 with value = T318. t_T317.timeout; //T1 expires t_T318.start; //T2 started //@siclog "Step 24" siclog@ //The SS starts the UL periodic grant transmission, allowing the UE to return the IP packet as received in step 20 v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 1.0)); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant(v_DciUlInfo, 1)), v_TimingInfo); //@siclog "Step 24A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //Note: Timer T318 shall be stopped and T317 shall be started in UE side. SS starts a timer T3 with value = T317. t_T318.stop; //T2 stopped t_T317.start; //T3 started //@siclog "Step 24B" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NBcorresponding to grant in step 24? SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 24B"); //@siclog "Step 24C" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1,true,false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 24CA" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 24D" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB (Note 1). //Note 1: This could be done near the expiry of T1 in step 23 and T3 in step 24E v_Delay := t_T317.read - 1.0; if(v_Delay > 1.0){ f_Delay(v_Delay); //T317 is 6 sec, within last 1 sec the DL should be send out } v_UserData := f_GetN_OctetsFromPRBS(10, 10); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, f_NBIOT_MobileInfo_GetPTI(v_PDNIndex), cs_UserDataContainer(v_UserData))))); //@siclog "Step 24E" siclog@ //Wait until expiry of T3. //Note: UE starts timer T318. SS starts a timer T4 with value = T318 t_T317.timeout; //T3 expired t_T318.start; //T4 started //@siclog "Step 24F" siclog@ //SS starts periodic UL grant transmission, allowing the UE to return the IP packet as received in step 24D. v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 1.0)); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant(v_DciUlInfo, 1)), v_TimingInfo); //@siclog "Step 25" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NB corresponding to grant in step 24F? alt{ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); } [] t_T318.timeout //T4 expires { //@siclog "Step 26" siclog@ //Wait until expiry of T4. } } //@siclog "Step 26A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //EXCEPTION: Steps 27-30 describe behaviour that depends on whether the UE supports RRC connection re-establishment. if(pc_RRC_re_establishment_CP_CIoT) { //@siclog "Step 27" siclog@ //Check: Does the UE transmit an RRCConnectionReestablishmentRequest-NB message on SRB0? v_PhysCellId := f_NBIOT_CellInfo_GetPhyCellId(nbiot_Cell1); v_ShortMAC_I := f_NBIOT_Calculate_ShortMAC(nbiot_Cell1); v_UE_Identity := cr_ReestabUE_Identity_NB(tsc_C_RNTI_Def, v_PhysCellId, v_ShortMAC_I); SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionReestablishmentRequest_NB(v_UE_Identity, otherFailure))); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, " Step 27"); //@siclog "Step 28" siclog@ //The SS transmits a RRCConnectionReestablishment-NB message on SRB0 v_SecurityParams := f_NBIOT_Security_Get(); v_SecurityParams := f_NBIOT_SS_AS_ReestablishResumeSecurity(nbiot_Cell1, v_SecurityParams, rrcConnectionReestablishment); f_NBIOT_Security_Set(v_SecurityParams); //@siclog "Step 29" siclog@ //The UE transmits RRCConnectionReestablishmentComplete-NB message on SRB1. SRB.receive(car_NB_SRB_RrcPdu_IND(nbiot_Cell1, tsc_SRB1, cr_RRCConnectionReestablishmentComplete_NB(v_RRC_TI))); //@siclog "Step 30" siclog@ //The SS transmits an RRCConnectionRelease-NB message to move to RC_IDLE state f_NBIOT_RRC_ConnectionRelease (nbiot_Cell1); } f_NBIOT_TestBody_Set(false); //Switch/power off UE f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N3_TESTMODE); }

After Change:

module NBIOT_RRC_NTN { … … … import from NBIOT_RRC_ControlPlane all; function f_TC_22_4_30_NBIOT() runs on NBIOT_PTC {// NB-IoT / NTN / Ephemeris information update / T317 Expiry / T318 Expiry @sic R5-235429 sic@ var NAS_MSG_Indication_Type v_NAS_Ind; var EUTRA_ASN1_RRC_TransactionIdentifier_Type v_RRC_TI := tsc_RRC_TI_Def; var NBIOT_SecurityParams_Type v_SecurityParams; var EUTRA_ASN1_ShortMAC_I_Type v_ShortMAC_I; var EUTRA_ASN1_PhysCellId_Type v_PhysCellId; var template (present) EUTRA_ASN1_ReestabUE_Identity_Type v_UE_Identity; var O10_Type v_UserData; // length 10 var integer v_PDNIndex; var float v_Delay; var SubFrameTiming_Type v_TimingStart; var template (value) TimingInfo_Type v_TimingInfo; var template (value) NB_DciUlInfo_Type v_DciUlInfo; var DRB_IdentityList_Type v_DrbIdList; // var integer v_SelectedPlmnIndex; // var SystemInformationBlockType2_NB_r13 v_SIB2; var float v_T301 := f_NBIOT_SetTimerToleranceMin(rrcTimer, 2.5); timer t_T317 := 60.0; //Corresponds to the value (60sec) of ul_SyncValidityDuration_r17 in SIB31-NB acc. Table 22.4.30.3.3-2 timer t_T318 := 8.0 ; //Corresponds to the value (8000ms) of t318-r17 acc 36.508 Table 4.6.3-14 timer t_T317_min := f_NBIOT_SetTimerToleranceMin(rrcTimer,60.0); timer t_T317_max := f_NBIOT_SetTimerToleranceMax(rrcTimer,60.0); timer t_T318_max := f_NBIOT_SetTimerToleranceMax(rrcTimer,8.0); timer t_T318_min := f_NBIOT_SetTimerToleranceMin(rrcTimer,8.0); timer t_wait := 30.0; // wait timer at the end of the test case timer t_wait_res; //System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used. f_NBIOT_Init(c8, CONTROL_PLANE); //Table 22.4.30.3.3-2, Value of T317=60s f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(nbiot_Cell1,s60); // update SIB2 to indicate support for cp_reestablishment v_SIB2 := f_NBIOT_CellInfo_GetSIB2(nbiot_Cell1); //v_SIB2.ue_TimersAndConstants_r13.t301_r13 := ms2500; v_SIB2.ue_TimersAndConstants_r13.t311_r13 := ms10000; v_SIB2.ue_TimersAndConstants_r13.t301_r13 := ms2500; v_SIB2.cp_Reestablishment_r14 := true_; f_NBIOT_CellInfo_SetSIB2 (nbiot_Cell1, v_SIB2); //Create and configure all cells f_NBIOT_CellConfig_Def(nbiot_Cell1, CONTROL_PLANE); f_UT_SwitchOnUE(UT, false); //The UE is in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT); //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_UT_ConfigureNTNPosition(UT); //Preamble: The UE is in State 1-NB switched off according to TS 36.508 [18]. f_UT_PowerOffUE (UT); f_NBIOT_SetCellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE); f_NBIOT_TestBody_Set(true); //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Step 2" siclog@ //UE acquires SystemInformationBlockType31-NB and start T317 = 60 seconds. //@siclog "Steps 3-18b1" siclog@ //Steps 1 to 16b1 of the Attach, connected mode and test mode activated procedure described in TS 36.508 [18] subclause 8.1.5.2A.3 with TEST LOOP MODE A are performed on Ncell 1. // Acc to 36.508 Table 8.1.5.2A.3-1: NB-IoT UE Attach, connected mode and test mode activated procedure (state 2-NB to state 2A-NB) //Note: After receiving the message in step 4, SS start a timer T1 with value = T317. v_NAS_Ind := f_NBIOT_InitialRegistration_Step2_4(nbiot_Cell1, CONTROL_PLANE, PREAMBLE); // changed from normal to preamble t_T317.start; //T1 started f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2A_NB_TESTLOOP_ModeG, v_NAS_Ind); //@siclog "Steps 18A-18B" siclog@ //Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 8.1.5.2B.3 are performed on Ncell 1. //FFS: Prose CR needed to actiate a corresponding loopback A->G f_NBIOT_CloseUE_TestLoopModeGH (nbiot_Cell1, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS); // 1181614 incorrect cell reference //@siclog "Step 19" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1, true, false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 19A" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 20" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB. (Note 1) //Note 1: This could be done near the expiry of T1 in step 23 and T3 in step 24E /*v_Delay := t_T317.read - 1.0; if(v_Delay > 1.0){ f_Delay(v_Delay); //T317 is 6 sec, within last 1 sec the DL should be send out }*/ v_UserData := f_GetN_OctetsFromPRBS(0, 10); v_PDNIndex := f_NBIOT_MobileInfo_GetNoOfPDNsConfigured(); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); // changed PTI to un assigned f_NBIOT_MobileInfo_GetPTI(v_PDNIndex) //@siclog "Steps 21-22" siclog@ //Void //@siclog "Step 23" siclog@ //Wait until expiry of T1. //Note: UE starts timer T318. SS start a timer T2 with value = T318. t_T317.timeout; //T1 expires t_T318.start; //T2 started //@siclog "Step 24" siclog@ //The SS starts the UL periodic grant transmission, allowing the UE to return the IP packet as received in step 20 //v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); //v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 1.0)); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); //@siclog "Step 24A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //Note: Timer T318 shall be stopped and T317 shall be started in UE side. SS starts a timer T3 with value = T317. t_T318.stop; //T2 stopped //@siclog "Step 24B" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NBcorresponding to grant in step 24? SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 24B"); t_T317_max.start; t_T317_min.start; //T3 started //@siclog "Step 24C" siclog@ //SS stops broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationTo_C1(nbiot_Cell1,true,false); //now only SIB2 will be broadcasted with cellBarred and DottachWithoutPDN = false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //@siclog "Step 24CA" siclog@ //SS stops periodic grant allocation f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Stop); //@siclog "Step 24D" siclog@ //The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB (Note 1). //Note 1: This could be done near the expiry of T1 in step 23 and T3 in step 24E /*v_Delay := t_T317.read - 1.0; if(v_Delay > 1.0){ f_Delay(v_Delay); //T317 is 6 sec, within last 1 sec the DL should be send out }*/ v_UserData := f_GetN_OctetsFromPRBS(10, 10); SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData))))); // changed PTI to un assigned f_NBIOT_MobileInfo_GetPTI(v_PDNIndex) //@siclog "Step 24E" siclog@ //Wait until expiry of T3. //Note: UE starts timer T318. SS starts a timer T4 with value = T318 t_T317_min.timeout; //T3 expired t_T318_min.start; //T4 started t_T317_max.timeout; //@siclog "Step 24F" siclog@ //SS starts periodic UL grant transmission, allowing the UE to return the IP packet as received in step 24D. // v_TimingStart := f_NBIOT_GetNextSearchSpace(nbiot_Cell1); // v_TimingInfo := cs_TimingInfo_SubFrameFull(f_NBIOT_IncrementSearchSpace(v_TimingStart, 1.0)); v_DciUlInfo := cs_NB_DciInfo_CcchDcchDtchUL (1,10); f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); //@siclog "Step 25" siclog@ //Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NB corresponding to grant in step 24F? alt{ [] SRB.receive(car_NB_SRB_NasPdu_IND(nbiot_Cell1, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer)))) { f_NBIOT_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 25"); } [] t_T318_min.timeout //T4 expires { //@siclog "Step 26" siclog@ //Wait until expiry of T4. } } //@siclog "Step 26A" siclog@ //SS resumes broadcasting SystemInformationBlockType31-NB. f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(nbiot_Cell1, s60, false); //now SIB31-NB will be broadcasted again with T317=60s and DoAttachWithoutPDN=false for CONTROL_PLANE f_NBIOT_SS_ConfigureSysinfo(nbiot_Cell1, false); //EXCEPTION: Steps 27-30 describe behaviour that depends on whether the UE supports RRC connection re-establishment. if(pc_RRC_re_establishment_CP_CIoT) { // Locally release v_DrbIdList := f_NBIOT_SS_ActiveDRBs(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SS_SRBs_DRBs_Release(nbiot_Cell1, CONTROL_PLANE, -, v_DrbIdList); f_NBIOT_SS_SRBs_DRBs_Config(nbiot_Cell1, CONTROL_PLANE, -, omit); f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell1); //@siclog "Step 27" siclog@ //Check: Does the UE transmit an RRCConnectionReestablishmentRequest-NB message on SRB0? v_PhysCellId := f_NBIOT_CellInfo_GetPhyCellId(nbiot_Cell1); v_ShortMAC_I := f_NBIOT_Calculate_ShortMAC(nbiot_Cell1); v_UE_Identity := cr_ReestabUE_Identity_NB(tsc_C_RNTI_Def, v_PhysCellId, v_ShortMAC_I); //SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionReestablishmentRequest_NB(v_UE_Identity, otherFailure))); f_NBIOT_RRC_RRCConnectionReestablishmentRequest_r14(nbiot_Cell1); f_NBIOT_PreliminaryPass(__FILE__, __LINE__, " Step 27"); //The SS does not respond for 2.5s (T301) //@siclog "Step 28" siclog@ t_wait_res.start(v_T301); // @sic R5s180539, R5-186796 sic@ alt { [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell2, cr_RRCConnectionReestablishmentRequest_NB_r14(?, otherFailure))) { repeat; } [] t_wait_res.timeout {} } //The SS transmits a RRCConnectionReestablishment-NB message on SRB0 /*v_SecurityParams := f_NBIOT_Security_Get(); v_SecurityParams := f_NBIOT_SS_AS_ReestablishResumeSecurity(nbiot_Cell1, v_SecurityParams, rrcConnectionReestablishment); f_NBIOT_Security_Set(v_SecurityParams); */ //The UE transmits RRCConnectionReestablishmentComplete-NB message on SRB1. //SRB.receive(car_NB_SRB_RrcPdu_IND(nbiot_Cell1, tsc_SRB1, cr_RRCConnectionReestablishmentComplete_NB(v_RRC_TI))); //The SS transmits an RRCConnectionRelease-NB message to move to RC_IDLE state //f_NBIOT_RRC_ConnectionRelease (nbiot_Cell1); f_NBIOT_SS_ConfigRachProcedureMsg4(nbiot_Cell1, f_NBIOT_508_RRCConnectionSetup()); } else { // Locally release the connection as UE will go to idle mode v_DrbIdList := f_NBIOT_SS_ActiveDRBs(nbiot_Cell1, CONTROL_PLANE); f_NBIOT_SS_SRBs_DRBs_Release(nbiot_Cell1, CONTROL_PLANE, -, v_DrbIdList); f_NBIOT_SS_SRBs_DRBs_Config(nbiot_Cell1, CONTROL_PLANE, -, omit); } //@siclog "Step 31" siclog@ t_wait.start; alt { //@siclog "Step 32a1" siclog@ [] SRB.receive(car_NB_SRB0_RrcPdu_IND(nbiot_Cell1, cr_RRCConnectionRequest_NB(?, ?))) { //@siclog "Step 32a2" siclog@ t_wait.stop; /* NOTE: acc. to ES 201 873-1 cl. 23.3 the timer can be stopped even if it has not been started before */ // switch on UL grant assignments @sic R5s170506: UL grants started only if there has been an RRCConnectionRequest sic@ */ f_NBIOT_ULGrantTransmission(nbiot_Cell1, cs_NB_UL_GrantScheduling_Start(cs_NB_ContinuousGrant)); f_NBIOT_RRC_ConnectionSetup_Def(); // UE shall establish SRB1 acc. to 36.331 cl. 5.3.3.4 and 5.3.9 // UE enters RRC_CONNECTED state //@siclog "Step 32a3" siclog@ // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_SelectedPlmnIndex := f_NBIOT_CellInfo_GetSelectedPlmnIndex(nbiot_Cell1); f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def, v_SelectedPlmnIndex), cr_NAS_IndicationWithPiggybacking (tsc_SHT_IntegrityProtected_PartiallyCiphered, cr_CONTROL_PLANE_SERVICE_REQUEST(?), cr_ESM_DATA_TRANSPORT(?, cr_UserDataContainer))); //@siclog "Step 17b6" siclog@ //@sic R5-195313 sic@ SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, // @sic R5s190574 sic@ tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); f_NBIOT_UE_Detach_SwitchOffUe(nbiot_Cell1, CONTROL_PLANE, RRC_CONNECTED); // @sic R5s190574 sic@ } [] t_wait.timeout { //@siclog "Step 32b1" siclog@ f_NBIOT_Postamble(nbiot_Cell1, CONTROL_PLANE, N1_IDLE); } }

## Correction to f_NBIOT_ModifySysinfoCombinationFromC1_To_C8

| Function name | f_NBIOT_ModifySysinfoCombinationFromC1_To_C8 |
| --- | --- |
| Reason for change | In condition c8 system information type 31 uses the first array in cs_SystemInformationBlockType31_NB_r17_GSO_Array, but v_CellInfo.NtnInfo.NtnCellNumber is assigned as 1 (NtnCellNumber is ranged from (1..3))) and this results in the second array being assigned in cs_SystemInformationBlockType31_NB_r17_GSO_Array |
| Summary of change | Removed the 3rd parameter so that the default value for NtnCellNumber is being used for SIB31. |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment | Accepted in principle. Alternative implementation is proposed to avoid resetting the actual NtnCellNumber to a default one Additional change to MIB contents as per R5-235420: v_MIB.message_.schedulingInfoSIB1_r13 := 5; |

Before Change:

function f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(NBIOT_CellId_Type p_CellId, template (omit) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17 := omit, boolean p_DoAttachWithoutPDN := true, //AllowAttachWithoutPDN = CONTROL_PLANE integer p_FrequencyBand := px_ePrimaryFrequencyBand, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC { var template (value) NB_BcchInfo_Type v_CellBcchInfo; var template (value) BCCH_BCH_Message_NB v_MIB; var template (value) BCCH_DL_SCH_Message_NB v_SIB1; var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB2; var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset; var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize; var template (value) NB_OperationModeInfo_Type v_OperationMode; var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList; var NB_OperationMode_Type v_TypeOperationMode := f_NBIOT_OperationMode(); var EUTRA_ASN1_TrackingAreaCode_Type v_TAC_CellId := f_NBIOT_CellInfo_GetTAC (p_CellId); var EUTRA_ASN1_CellIdentity_Type v_CellIdentity := f_NBIOT_CellInfo_GetCellIdentity(p_CellId); var GutiParameters_Type v_Guti_Params := f_NBIOT_CellInfo_GetGuti(p_CellId); var template (value) PLMN_IdentityList_NB_r13 v_PLMN_Identity := { f_NBIOT_PLMN_Identity(v_Guti_Params.PLMN_Identity, p_DoAttachWithoutPDN) }; var NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); select (v_TypeOperationMode) { case (standalone) { v_OperationMode := cs_NB_OperationModeInfo_Standalone; v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } case (inband_samePCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := dB0; } case (inband_differentPCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := omit; } case (guardband) { v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } } // --- Initialise MIB --- v_MIB := cs_508_MasterInformationBlock_NB_Init(v_OperationMode); // --- Initialise SIB1 --- v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(v_PLMN_Identity, v_TAC_CellId, v_CellIdentity, p_FrequencyBand, v_NB_EutraControlRegionSize, v_NRS_CRS_PowerOffset, cs_SchedulingInfo_NB_1SI); v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred; // --- Keep current SIB2 --- v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_CellInfo_GetSIB2(p_CellId)); v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); // --- Initialise SIB31 --- v_SI_List[1] := cs_SystemInformation_Common_NB(f_NBIOT_InitSIB31(p_UL_SyncValidityDuration_r17, v_CellInfo.NtnInfo.NtnCellNumber)); //@sic R5-235420, R5-235468 sic@ v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({}); //Update scheduling info list v_SIB1.message_.c1.systemInformationBlockType1_r13.schedulingInfoList_r13 := v_SchedulingInfoList; v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(v_TAC_CellId); v_CellBcchInfo := cs_BCCHInfo_NB_C1(v_MIB, v_SIB1, v_SI_List); f_NBIOT_CellInfo_SetBCCH_Info(p_CellId, v_CellBcchInfo); f_NBIOT_CellInfo_SetCombination(p_CellId, c1); }

After Change:

function f_NBIOT_ModifySysinfoCombinationFromC1_To_C8(NBIOT_CellId_Type p_CellId, template (omit) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17 := omit, boolean p_DoAttachWithoutPDN := true, //AllowAttachWithoutPDN = CONTROL_PLANE integer p_FrequencyBand := px_ePrimaryFrequencyBand, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC { var template (value) NB_BcchInfo_Type v_CellBcchInfo; var template (value) BCCH_BCH_Message_NB v_MIB; var template (value) BCCH_DL_SCH_Message_NB v_SIB1; var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB2; var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset; var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize; var template (value) NB_OperationModeInfo_Type v_OperationMode; var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList; var NB_OperationMode_Type v_TypeOperationMode := f_NBIOT_OperationMode(); var EUTRA_ASN1_TrackingAreaCode_Type v_TAC_CellId := f_NBIOT_CellInfo_GetTAC (p_CellId); var EUTRA_ASN1_CellIdentity_Type v_CellIdentity := f_NBIOT_CellInfo_GetCellIdentity(p_CellId); var GutiParameters_Type v_Guti_Params := f_NBIOT_CellInfo_GetGuti(p_CellId); var template (value) PLMN_IdentityList_NB_r13 v_PLMN_Identity := { f_NBIOT_PLMN_Identity(v_Guti_Params.PLMN_Identity, p_DoAttachWithoutPDN) }; var NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); select (v_TypeOperationMode) { case (standalone) { v_OperationMode := cs_NB_OperationModeInfo_Standalone; v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } case (inband_samePCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := dB0; } case (inband_differentPCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := omit; } case (guardband) { v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } } // --- Initialise MIB --- v_MIB := cs_508_MasterInformationBlock_NB_Init(v_OperationMode); // --- Initialise SIB1 --- v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(v_PLMN_Identity, v_TAC_CellId, v_CellIdentity, p_FrequencyBand, v_NB_EutraControlRegionSize, v_NRS_CRS_PowerOffset, cs_SchedulingInfo_NB_1SI); v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred; // --- Keep current SIB2 --- v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_CellInfo_GetSIB2(p_CellId)); v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.ue_TimersAndConstants_r13.t311_r13 := ms10000; v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.ue_TimersAndConstants_r13.t301_r13 := ms2500; v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.cp_Reestablishment_r14 := true_; v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); // --- Initialise SIB31 --- v_SI_List[1] := cs_SystemInformation_Common_NB(f_NBIOT_InitSIB31(p_UL_SyncValidityDuration_r17)); //@sic R5-235420, R5-235468 sic@ //removed v_CellInfo.NtnInfo.NtnCellNumber v_SchedulingInfoList := cs_SchedulingInfo_NB_2SI({}); //Update scheduling info list v_SIB1.message_.c1.systemInformationBlockType1_r13.schedulingInfoList_r13 := v_SchedulingInfoList; v_SIB1.message_.c1.systemInformationBlockType1_r13.nonCriticalExtension := cs_SystemInformationBlockType1_NB_v1350_NTN_r17(v_TAC_CellId); v_CellBcchInfo := cs_BCCHInfo_NB_C1(v_MIB, v_SIB1, v_SI_List); f_NBIOT_CellInfo_SetBCCH_Info(p_CellId, v_CellBcchInfo); f_NBIOT_CellInfo_SetCombination(p_CellId, c1); }

MCC160 Alternative Implementation:

type integer NtnCellNumber_Type (1..3); /* * @desc To initialise SIB 31 with the cell-specific requirements of test case 22.5.13 * @param p_UL_SyncValidityDuration_r17 (default value: omit) * @param p_NTN_CellNumber (default value: 1) * @return template (value) SystemInformation_NB_r13_IEs * @status APPROVED (NBIOT) */ function f_NBIOT_InitSIB31(template (omit) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17 := omit, NtnCellNumber_Type p_NTN_CellNumber := 1) runs on NBIOT_PTC return template (value) SystemInformation_NB_r13_IEs { var template (value) SystemInformationBlockType31_NB_r17 v_SIB31; if(f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO) {//@sic R5s230478, R5-235468, R5s230708 sic@ v_SIB31 :=f_NBIOT_GetSIB31_ForNTNCell(p_NTN_CellNumber); //init as cell number acc to 36.508 Table 6.3.5.1-1/2-1 if(isvalue(p_UL_SyncValidityDuration_r17)) { v_SIB31.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 := p_UL_SyncValidityDuration_r17 //compiler warning may be ignored due to isvalue check } } else { FatalError(__FILE__, __LINE__, "NGSO configuration is not yet supported"); } return cs_SI_NB_SIB31(v_SIB31); } /* * @desc To retrieve SIB 31 configured for a given cell according to 36.508 Table 6.3.5.1-1/6.3.5.2-1 depending on satellite scenario * @param p_NTN_CellNumber (default value: 1) * @return template (value) SystemInformationBlockType31_NB_r17 * @status */ function f_NBIOT_GetSIB31_ForNTNCell(NtnCellNumber_Type p_NTN_CellNumber := 1) runs on NBIOT_PTC return template (value) SystemInformationBlockType31_NB_r17 { if (f_NBIOT_MobileInfo_GetNTNSatelliteType() == ntn_GSO) { return cs_SystemInformationBlockType31_NB_r17_GSO_Array[p_NTN_CellNumber - 1] } else { FatalError(__FILE__, __LINE__, "NGSO configuration is not yet supported"); return cs_SystemInformationBlockType31_NB_r17_GSO_Array[p_NTN_CellNumber - 1] // temporary, to avoid compiler error } }

## Correction to f_NBIOT_ModifySysinfoCombinationTo_C1

| Function name | f_NBIOT_ModifySysinfoCombinationTo_C1 |
| --- | --- |
| Reason for change | In f_NBIOT_ModifySysinfoCombination SIB2 is getting initialised, the test case requires an update to SIB2 to cp_Reestablishment_r14 to true to indicate the support for Reestablishment and also the timer values defined at the start of the test case |
| Summary of change | Updated the variable and timer values Prose change is required to 38.523-1 |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment | Accepted, conditional prose CR approval |

Before Change:

function f_NBIOT_ModifySysinfoCombinationTo_C1(NBIOT_CellId_Type p_CellId, boolean p_CellBarred := false, boolean p_DoAttachWithoutPDN := true, //AllowAttachWithoutPDN = CONTROL_PLANE integer p_FrequencyBand := px_ePrimaryFrequencyBand, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC { var template (value) NB_BcchInfo_Type v_CellBcchInfo; var template (value) BCCH_BCH_Message_NB v_MIB; var template (value) BCCH_DL_SCH_Message_NB v_SIB1; var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB2; var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset; var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize; var template (value) NB_OperationModeInfo_Type v_OperationMode; var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList; var NB_OperationMode_Type v_TypeOperationMode := f_NBIOT_OperationMode(); var EUTRA_ASN1_TrackingAreaCode_Type v_TAC_CellId := f_NBIOT_CellInfo_GetTAC (p_CellId); var EUTRA_ASN1_CellIdentity_Type v_CellIdentity := f_NBIOT_CellInfo_GetCellIdentity(p_CellId); var GutiParameters_Type v_Guti_Params := f_NBIOT_CellInfo_GetGuti(p_CellId); var template (value) PLMN_IdentityList_NB_r13 v_PLMN_Identity := { f_NBIOT_PLMN_Identity(v_Guti_Params.PLMN_Identity, p_DoAttachWithoutPDN) }; select (v_TypeOperationMode) { case (standalone) { v_OperationMode := cs_NB_OperationModeInfo_Standalone; v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } case (inband_samePCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := dB0; } case (inband_differentPCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := omit; } case (guardband) { v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } } // --- Initialise MIB --- v_MIB := cs_508_MasterInformationBlock_NB_Init(v_OperationMode); // --- Initialise SIB1 --- // -- A default scheduling assigned ---------- v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(v_PLMN_Identity, v_TAC_CellId, v_CellIdentity, p_FrequencyBand, v_NB_EutraControlRegionSize, v_NRS_CRS_PowerOffset, cs_SchedulingInfo_NB_1SI); if(p_CellBarred){ v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred; } // --- Initialise SIB2 --- v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); v_SchedulingInfoList := cs_SchedulingInfo_NB_1SI; v_SIB1.message_.c1.systemInformationBlockType1_r13.schedulingInfoList_r13 := v_SchedulingInfoList; v_CellBcchInfo := cs_BCCHInfo_NB_C1(v_MIB, v_SIB1, v_SI_List); f_NBIOT_CellInfo_SetBCCH_Info(p_CellId, v_CellBcchInfo); f_NBIOT_CellInfo_SetCombination(p_CellId, c1); }

After Change:

function f_NBIOT_ModifySysinfoCombinationTo_C1(NBIOT_CellId_Type p_CellId, boolean p_CellBarred := false, boolean p_DoAttachWithoutPDN := true, //AllowAttachWithoutPDN = CONTROL_PLANE integer p_FrequencyBand := px_ePrimaryFrequencyBand, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN := LTE_AllCellsOnSamePLMN) runs on NBIOT_PTC { var template (value) NB_BcchInfo_Type v_CellBcchInfo; var template (value) BCCH_BCH_Message_NB v_MIB; var template (value) BCCH_DL_SCH_Message_NB v_SIB1; var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB2; var template (omit) NRS_CRS_PowerOffset_Type v_NRS_CRS_PowerOffset; var template (omit) NB_EutraControlRegionSize_Type v_NB_EutraControlRegionSize; var template (value) NB_OperationModeInfo_Type v_OperationMode; var template (value) SchedulingInfoList_NB_r13 v_SchedulingInfoList; var NB_OperationMode_Type v_TypeOperationMode := f_NBIOT_OperationMode(); var EUTRA_ASN1_TrackingAreaCode_Type v_TAC_CellId := f_NBIOT_CellInfo_GetTAC (p_CellId); var EUTRA_ASN1_CellIdentity_Type v_CellIdentity := f_NBIOT_CellInfo_GetCellIdentity(p_CellId); var GutiParameters_Type v_Guti_Params := f_NBIOT_CellInfo_GetGuti(p_CellId); var template (value) PLMN_IdentityList_NB_r13 v_PLMN_Identity := { f_NBIOT_PLMN_Identity(v_Guti_Params.PLMN_Identity, p_DoAttachWithoutPDN) }; select (v_TypeOperationMode) { case (standalone) { v_OperationMode := cs_NB_OperationModeInfo_Standalone; v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } case (inband_samePCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandSamePCI (fl_NBIOT_MIB_GetEUTRA_CRS_SequenceInfo(p_FrequencyBand)); v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := dB0; } case (inband_differentPCI) { v_OperationMode := cs_NB_OperationModeInfo_InBandDifferentPCI (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_NB_EutraControlRegionSize := n2; v_NRS_CRS_PowerOffset := omit; } case (guardband) { v_OperationMode := cs_NB_OperationModeInfo_GuardBand (fl_NBIOT_MIB_GetRasterOffset (p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_NB_EutraControlRegionSize := omit; v_NRS_CRS_PowerOffset := omit; } } // --- Initialise MIB --- v_MIB := cs_508_MasterInformationBlock_NB_Init(v_OperationMode); // --- Initialise SIB1 --- // -- A default scheduling assigned ---------- v_SIB1 := cs_508_SystemInformationBlockType1_NB_Def(v_PLMN_Identity, v_TAC_CellId, v_CellIdentity, p_FrequencyBand, v_NB_EutraControlRegionSize, v_NRS_CRS_PowerOffset, cs_SchedulingInfo_NB_1SI); if(p_CellBarred){ v_SIB1.message_.c1.systemInformationBlockType1_r13.cellAccessRelatedInfo_r13.cellBarred_r13 := barred; } // --- Initialise SIB2 --- v_SIB2 := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, v_TypeOperationMode, p_CellsOnPLMN)); v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.ue_TimersAndConstants_r13.t311_r13 := ms10000; v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.ue_TimersAndConstants_r13.t301_r13 := ms2500; v_SIB2.sib_TypeAndInfo_r13[0].sib2_r13.cp_Reestablishment_r14 := true_; v_SI_List[0] := cs_SystemInformation_Common_NB(v_SIB2); v_SchedulingInfoList := cs_SchedulingInfo_NB_1SI; v_SIB1.message_.c1.systemInformationBlockType1_r13.schedulingInfoList_r13 := v_SchedulingInfoList; v_CellBcchInfo := cs_BCCHInfo_NB_C1(v_MIB, v_SIB1, v_SI_List); f_NBIOT_CellInfo_SetBCCH_Info(p_CellId, v_CellBcchInfo); f_NBIOT_CellInfo_SetCombination(p_CellId, c1); }

# Branches executed in test case 22.4.30

The test case implementation was executed with Integrity and ciphering enabled using NTN bands 255 and 256.

# Execution Log Files

## MediaTek 6825

The MediaTek 6825 device passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

Test Case Execution log file: 
tc_22_4_30_MTK.log 
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

n the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230709 This archive comprises Execution log files, PICS/PIXIT files |
| --- | --- |