3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230713

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3205 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the test cases 8.1.5.8.1 and 8.1.5.8.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Change 1: - In the current TTCN implementation of the function f_TC_8_1_5_8_1_NR5GC_TestBody at Step 14a5, Resume procedure delay is calculated using the function f_NR5GC_RRC_ProcedureDelay. - As per the RAN5#100 document R5-234265, for the resume procedure it is agreed to have the function f_NR5GC_RRC_ProcedureDelayResume which provides flexible implementation to receive the resume complete either using UL DCI Grant or RAR grant.(this implementation is already present for 8.1.5.8.2.x) - It is proposed to have the same implementation in place for f_TC_8_1_5_8_1_NR5GC_TestBody,at step 14a5 when calculating the procedure delay for resume procedure Change 2: - In the current TTCN implementation of the function f_NR5GC_RRC_ProcedureDelayResume , when checking the procedure delay based on UL DCI , current condition is as follows if (not v_RACH_ProcedureStarted or (f_SubFrameTiming_Duration(v_T3, v_T4)>0)) V_T3( Ul grant time ) can be lesser than V_T4 ( UL message received time ) in case of Rach Procedure triggered resume procedure as well . So it is proposed to detect rach based and UL DCI based flows only based on the variable v_RACH_ProcedureStarted |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the function f_TC_8_1_5_8_1_NR5GC_TestBody,at step 14a5 when calculating the procedure delay for resume procedure, function f_NR5GC_RRC_ProcedureDelayResume is used In the function f_NR5GC_RRC_ProcedureDelayResume , UL DCI based / Rach based flow is detected only based on the variable v_RACH_ProcedureStarted |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Change (1) is for 8.1.5.8.1.NR5GC Change (2) is common for 8.1.5.8.1 and 8.1.5.8.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Corrections required	4

1.1	Correction to the Function f_TC_8_1_5_8_1_NR5GC_TestBody	4

1.2	Correction to the Function f_NR5GC_RRC_ProcedureDelayResume	6

# Corrections required

## Correction to the Function f_TC_8_1_5_8_1_NR5GC_TestBody

| Function name | f_TC_8_1_5_8_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | - In the current TTCN implementation of the function f_TC_8_1_5_8_1_NR5GC_TestBody at Step 14a5, Resume procedure delay is calculated using the function f_NR5GC_RRC_ProcedureDelay. - As per the RAN5#100 document R5-234265, for the resume procedure it is agreed to have the function f_NR5GC_RRC_ProcedureDelayResume which provides flexible implementation to receive the resume complete either using UL DCI Grant or RAR grant.(this implementation is already present for 8.1.5.8.2.x) - It is proposed to have the same implementation in place for f_TC_8_1_5_8_1_NR5GC_TestBody,at step 14a5 when calculating the procedure delay for resume procedure |
| Summary of change | - In the function f_TC_8_1_5_8_1_NR5GC_TestBody,at step 14a5 when calculating the procedure delay for resume procedure, function f_NR5GC_RRC_ProcedureDelayResume is used |
| TTCN module | ttcn/develop/NR5GC/8_1_5/RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted, implemented as per R5-237357 |

Before Change:

function f_TC_8_1_5_8_1_NR5GC() runs on NR5GC_PTC { << SKIPPED CODE >> //@siclog "Step 14a5Aa1 - 14a5Aa2" siclog@ //The SS starts transmitting a periodic DCI from (10* 2 2^numerology +1+TDL) slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (6*2^numerology+1+TDL+K2) slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelay( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 6, "Step 14a5Aa2" ); } else { //@siclog "Step 14a5Ab1 - 14a5Ab2" siclog@ //The SS starts transmitting a periodic DCI from n slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (10* 2 2^numerology +1+TDL+K2)slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelay( nr_Cell1, cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 10, "Step 14a5Ab2" ); << SKIPPED CODE >> }

After Change:

function f_TC_8_1_5_8_1_NR5GC() runs on NR5GC_PTC { //Processing delay / RRC_Idle to RRC_Connected / RRC_Inactive to RRC_Connected / Success / Latency check << SKIPPED CODE >> //@siclog "Step 14a5Aa1 - 14a5Aa2" siclog@ //The SS starts transmitting a periodic DCI from (10* 2 2^numerology +1+TDL) slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (6*2^numerology+1+TDL+K2) slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayResume( nr_Cell1, //WA#WI=1195922 cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 6, "Step 14a5Aa2" ); } else { //@siclog "Step 14a5Ab1 - 14a5Ab2" siclog@ //The SS starts transmitting a periodic DCI from n slots after step 14a5 to schedule PUSCH. //Check: Does the UE transmit an RRCResumeComplete message within (10* 2 2^numerology +1+TDL+K2)slots after successful completion of step 14a5? f_NR5GC_RRC_ProcedureDelayResume( nr_Cell1, //WA#WI=1195922 cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCResume(tsc_NR_RRC_TI_Def, v_EncodedSCG_Config, v_RadioBearerConfigResume)),//@sic R5s220683 sic@ car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeComplete), 10, "Step 14a5Ab2" ); << SKIPPED CODE >> }

## Correction to the Function f_NR5GC_RRC_ProcedureDelayResume

| Function name | f_NR5GC_RRC_ProcedureDelayResume |
| --- | --- |
| Reason for change | - In the current TTCN implementation of the function f_NR5GC_RRC_ProcedureDelayResume , when checking the procedure delay based on UL DCI , current condition is as follows if (not v_RACH_ProcedureStarted or (f_SubFrameTiming_Duration(v_T3, v_T4)>0)) V_T3( Ul grant time ) can be lesser than V_T4 ( UL message received time ) in case of Rach Procedure triggered resume procedure as well . So it is proposed to detect rach based and UL DCI based flows only based on the variable v_RACH_ProcedureStarted |
| Summary of change | - In the function f_NR5GC_RRC_ProcedureDelayResume , UL DCI based / Rach based flow is detected only based on the variable v_RACH_ProcedureStarted |
| TTCN module | ttcn/develop/Common/NR5GC/NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted in principle, implemented as per R5-237429 |

Before Change:

f_NR5GC_RRC_ProcedureDelayResume(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { << SKIPPED CODE >> if (not v_RACH_ProcedureStarted or (f_SubFrameTiming_Duration(v_T3, v_T4)>0)) { // No RACH procedure was initiated by UE or RRCResumeComplete sent after UL grant hence not with RLC ACK in RACH procedure // Timing calculation as per 38.523-3 clause 7.3.7.2 << SKIPPED CODE >> }

After Change:

f_NR5GC_RRC_ProcedureDelayResume(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { << SKIPPED CODE >> if (not v_RACH_ProcedureStarted /* or (f_SubFrameTiming_Duration(v_T3, v_T4)>0)*/) { // No RACH procedure was initiated by UE or RRCResumeComplete sent after UL grant hence not with RLC ACK in RACH procedure // Timing calculation as per 38.523-3 clause 7.3.7.2 << SKIPPED CODE >> }