3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230733

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3217 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 38.213 sec 12: For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP. Hence, before Step 9, the local end configuration cannot be switched only for DL BWP The same reason as highlighted above holds true for the local end configuration before Step 11A where only the UL is switched. The same reason as highlighted above holds true for the lcoal end configuration after Step 22A |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Both the DL and UL BWP configuration is switched before Step 9. - The configuration to switch only the UL before Step 11A is removed as having a configuration to switch both DL and UL would cause the PDCCH Order not to be transmitted to the UE at Step 12. - Both the DL and UL BWP configuration is switched after Step 22A before the start of the Postamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.NR5GC, 7.1.1.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_8_1_NR_CommonTestBody |
| --- | --- |
| Reason for change | According to TS 38.213 sec 12: For unpaired spectrum operation, a DL BWP from the set of configured DL BWPs with index provided by BWP-Id is linked with an UL BWP from the set of configured UL BWPs with index provided by BWP-Id when the DL BWP index and the UL BWP index are same. For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP. Hence, before Step 9, the local end configuration cannot be switched only for DL BWP The same reason as highlighted above holds true for the local end configuration before Step 11A where only the UL is switched. The same reason as highlighted above holds true for the lcoal end configuration after Step 22A |
| Summary of change | - Both the DL and UL BWP configuration is switched before Step 9. - The configuration to switch only the UL before Step 11A is removed as having a configuration to switch both DL and UL would cause the PDCCH Order not to be transmitted to the UE at Step 12. - Both the DL and UL BWP configuration is switched after Step 22A before the start of the Postamble. |
| TTCN module | MAC_BWP_Common_NR.ttcn |
| MCC160 Comment | 1. Not needed, UL BWP is already configured on tsc_NR_BWP_Id_2/ tsc_NR_BWP_Id_Initial. In the test model omit means ‘keep as it is’. 2. The proposed change does not reconfigure UL BWPs to receive data in step 13D. See proposed implementation below. 3. Accepted, but implemented differently. The reconfiguration of DL/UL BWP is done before step 20, there is no need to do it again: delete le DL BWP reconfiguration. |

Before Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ <<SKIPPED CODE>> //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } f_NR_SS_ConfigureBWPs_UL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); // Configure UL BWP to receive data at step 13D <<SKIPPED CODE>> // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2); } else { f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); } }

After Change:

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId, DRB_Identity p_DrbId, template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC { //@sic R5-213459 R5-227417 sic@ <<SKIPPED CODE>> //@sic R5s230220 change 9.2 sic@ if (v_BWP_SameNumerology_upto4) { //@sic R5-2134599 sic@ f_NR_SS_ConfigureBWPs(p_NR_CellId, -,tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); //WA# } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); //WA# } // @siclog "Step 9" siclog@ v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300); v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN); <<SKIPPED CODE>> //@sic R5-2134599 R5s230634 sic@ if (v_BWP_SameNumerology_upto4) { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3; } else { v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1; } // REMOVED f_NR_SS_ConfigureBWPs_UL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial); // Configure UL BWP to receive data at step 13D WA# <<SKIPPED CODE>> // @siclog "Steps 22-22A" siclog@ fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22"); if (v_BWP_SameNumerology_upto4 ) { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2); WA# } else { f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial); WA# } }

MCC160 proposed implementation for change 2

function f_TC_7_1_1_8_1_NR_CommonTestBody(NR_CellId_Type p_NR_CellId,

DRB_Identity p_DrbId,

template (value) NR_ResourceAllocation_Type p_ResourceAllocationUL) runs on NR_BASE_PTC

{ //@sic R5-213459 R5-227417 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters;

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure;

var template (value) NR_PDCCH_Order_Type v_PDCCH_Order;

var BWP_Id v_BWP_Id_PDCCH_Order;

var SubFrameTiming_Type v_TimingInfoDL;

var SubFrameTiming_Type v_TimingInfoUL;

var SubFrameTiming_Type v_TimingInfoConfig;

var octetstring v_RLC_SDU1;

var octetstring v_RLC_SDU2;

var integer v_RA_PreambleIndex;

var integer v_SN := 0;

var boolean v_BWP_SameNumerology_upto4 := fl_NR_ReturnBWP_SameNumerologyUpto4_Capability(p_NR_CellId); //@sic R5-231578 sic@

var SubFrameTiming_Type v_TimingInfoUL_SS;

// @siclog "Step 1" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// @siclog "Step 2" siclog@

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100);            // 100ms after step 1

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 3-3A" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3");

v_SN := v_SN + 1;

if (v_BWP_SameNumerology_upto4) { //@sic  R5s230634 sic@

f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2);

} else {

f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_FirstActive, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial);

}

// @siclog "Step 5" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// @siclog "Step 6" siclog@

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100);    // 100ms after step 5

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 7-7A" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

v_SN := v_SN + 1;

//@sic R5s230220 change 9.2 sic@

if (v_BWP_SameNumerology_upto4) {  //@sic R5-2134599 sic@

f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_2);

} else {

f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, -, tsc_NR_BWP_Id_Initial);

}

// @siclog "Step 9" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId, 300);

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

//@sic R5s230220 change 9.2 sic@

v_TimingInfoUL_SS := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 90);              // 90 ms after step 9 - reconfigure UL/DL BWP in SS

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100);                // 100ms after step 9

if (v_BWP_SameNumerology_upto4) {  //@sic R5-2134599  R5s230634 sic@

f_NR_SS_ConfigureBWPs( p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_3, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_3);

} else {

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL_SS), tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1);

}

// @siclog "Step 10" siclog@

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 11-11AA" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

// configure CBRA CRNTI Based RACH procedure

//@sic R5s230220 change 10 sic@

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters, p_ResourceAllocationUL, -, tsc_NR_ENDC_Msg3Grant_Def, cs_NR_AssignedBWPs_InitialBWP);

f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedure);

v_SN := v_SN + 1;

//@sic R5-2134599  R5s230634 sic@

if (v_BWP_SameNumerology_upto4) {

v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_3;

} else {

v_BWP_Id_PDCCH_Order := tsc_NR_BWP_Id_1;

}

// @siclog "Step 11A" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId);

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// Enable PRACH reception indication

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable));

// @siclog "Step 12" siclog@

v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100);                            // 100ms after step 9

v_RA_PreambleIndex := 0;                                                                            // => contention-based random access

v_PDCCH_Order := cs_NR_PDCCH_Order(f_NR_CellInfo_GetSSB_Index(p_NR_CellId), v_RA_PreambleIndex);

f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_DciTriggerPDCCHOrder_REQ(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoDL), v_PDCCH_Order, cs_NR_AssignedBWPs_DedicatedBWP(v_BWP_Id_PDCCH_Order)));

// @sic R5s230733 sic@

v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 10);

f_NR_SS_ConfigureBWPs(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial);

// @siclog "Step 13" siclog@

SYSIND.receive(car_NR_PRACH_Preamble_IND(p_NR_CellId));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13");

// @siclog "Step 13A - 13C" siclog@

// steps are done automatically according to configuration of the RACH procedure

// @siclog "Steps 13D-13E" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13D");

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@sic R5s230634 sic@

v_SN := v_SN + 1;

//@sic  R5s230634 sic@

f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1, tsc_NR_BWP_Id_1);

// @siclog "Step 16" siclog@

v_TimingInfoDL := f_TC_7_1_1_8_1_NR_GetNextSendOccasion_DL(p_NR_CellId);   //@sic R5s230220 change 10 sic@

v_TimingInfoConfig := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 100);                        // 100ms after step 16

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

v_SN := v_SN + 1;  //@sic R5s230220 change 10 sic@

f_NR_SS_ConfigureBWPs_DL(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoConfig), tsc_NR_BWP_Id_1);

// @siclog "Step 17" siclog@

v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 200);                            // 200ms after step 16 @sic R5s230220 R5-231578 sic@

v_RLC_SDU2 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// @siclog "Step 18" siclog@

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 200);                // 200ms after step 17 @sic R5s230220 R5-231578 sic@

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 19-19A" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1, v_RLC_SDU2}, true, v_SN); //@sic R5-231578 R5s230220 sic@

v_SN := v_SN + 1;

//@sic R5-216169 R5s230220 R5s230634 sic@

if (v_BWP_SameNumerology_upto4 ) {

f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2, tsc_NR_BWP_Id_2);

} else {

f_NR_SS_ConfigureBWPs(p_NR_CellId, -, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial, tsc_NR_BWP_Id_Initial);

}

// @siclog "Step 20" siclog@

v_TimingInfoDL := f_SubFrameTiming_AddMilliSeconds(v_TimingInfoDL, 1400);                           // 1400ms after step 17 => 1000ms after step 18

v_RLC_SDU1 := fl_TC_7_1_1_8_1_SendData(p_NR_CellId, p_DrbId, cs_TimingInfo_NR(v_TimingInfoDL), v_SN);

// @siclog "Step 21" siclog@

v_TimingInfoUL := f_NR_GetNextSendOccasion_ULafterDL(p_NR_CellId, v_TimingInfoDL, 100);                // 100ms after step 20

f_NR_OneULGrantTransmission(p_NR_CellId, cs_TimingInfo_NR(v_TimingInfoUL), p_ResourceAllocationUL);

// @siclog "Steps 22-22A" siclog@

fl_TC_7_1_1_8_1_ReceiveData(p_NR_CellId, p_DrbId, {v_RLC_SDU1}, true, v_SN); //@sic R5-231578 R5s230220 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 22");

//DL and UL BWP already configured on tsc_NR_BWP_Id_2/tsc_NR_BWP_Id_Initial @sic R5s230733 sic@

}

## Additional change received from R&S

| Function name | f_NR_SS_ConfigureBWPs |
| --- | --- |
| Reason for change | The parameter for IE ActiveBWP in the Downlink is being incorrectly set to p_BwpId_DCI_DL rather than p_BwpId_Data_DL. This is should match the IE ActiveBWP in the Uplink which is being assigned with the variable v_ActiveUplinkBWP_Id which in turn is set to p_BwpId_Data_UL |
| Summary of change | The parameter p_BwpId_Data_DL is used to assign to the IE ActiveBWP in the Downlink |
| TTCN module | MAC_BWP_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_SS_ConfigureBWPs(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, BWP_Id p_BwpId_DCI_DL, BWP_Id p_BwpId_Data_DL, BWP_Id p_BwpId_DCI_UL, BWP_Id p_BwpId_Data_UL, integer p_NoPfBWPs := 4, template (omit) NR_FreqDomainResourceAssignmentDL_Type p_FreqDomainSchedul := omit, template (omit) NR_DownlinkBWP_List_Type p_DownlinkBWP_List := omit, template (omit) NR_UplinkBWP_List_Type p_UplinkBWP_List := omit ) runs on NR_BASE_PTC { //@sic R5s230634 change 2.1 sic@ var template (value) NR_AssignedBWPs_Type v_DCI_AssignedBWPs_DL; var template (value) NR_DciCommon_BWPIndicator_Type v_BWPIndicator; var template (value) NR_DciFormat_1_X_ResourceAssignment_Type v_ResourceAssignment_DL; var template (value) NR_DciFormat_1_X_SpecificInfo_Type v_DlDciFormat := cs_NR_DciFormat_1_1_AllOmit; var template (value) NR_DciDlInfo_Type v_DciDlInfo; var template (value) NR_ActiveUplinkBWP_Id_Type v_ActiveUplinkBWP_Id; var template (value) NR_AssignedBWPs_Type v_DCI_AssignedBWPs_UL; var template (value) NR_DciUlInfo_Type v_DciUlInfo; var template (value) NR_DciFormat_0_X_SpecificInfo_Type v_DciFormat_UL := cs_NR_DciFormat_0_1_AllOmit; //DL v_DCI_AssignedBWPs_DL := cs_NR_AssignedBWPs_DedicatedBWP(p_BwpId_DCI_DL); // BWP assigned to the searchspace (i.e. in which DCI is sent) v_BWPIndicator := f_NR_DciCommon_BWPIndicator(p_NoPfBWPs, p_BwpId_Data_DL); // BWP indicator in DCI format 1_1 v_DlDciFormat.Format_1_1.BWPIndicator := v_BWPIndicator; // Only the BWP indicator needs to be modified //PUCCH configured with ony one resource in Table 7.1.1.8.1.3.3-2G //@sic R5s230220 change 6 sic@ v_DlDciFormat.Format_1_1.PucchResourceIndicator.Value := '000'B; v_ResourceAssignment_DL := cs_NR_DciFormat_1_X_ResourceAssignment(-, p_FreqDomainSchedul, cs_NR_DciCommon_TimeDomainResourceAssignment('1'B)); //@sic R5s230220 change 6 sic@ v_DciDlInfo := cs_NR_DciDlInfo(v_ResourceAssignment_DL, -, v_DlDciFormat); //UL v_ActiveUplinkBWP_Id := cs_ActiveUplinkBWP_Id_Explicit(p_BwpId_Data_UL); // Active UL BWP: same as in DCI format 0_1 v_DCI_AssignedBWPs_UL := cs_NR_AssignedBWPs_DedicatedBWP(p_BwpId_DCI_UL); // BWP in which DCI is sent v_BWPIndicator := f_NR_DciCommon_BWPIndicator(p_NoPfBWPs, p_BwpId_Data_UL); // BWP indicator in DCI format 0_1 v_DciFormat_UL.Format_0_1.BWPIndicator := v_BWPIndicator; // only the BWP indicator needs to be modified v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; v_DciUlInfo.Format := v_DciFormat_UL; f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_BWP_CONFIG_REQ(p_NR_CellId, p_TimingInfo, p_BwpId_DCI_DL, v_DCI_AssignedBWPs_DL, v_DciDlInfo, v_ActiveUplinkBWP_Id, v_DCI_AssignedBWPs_UL, v_DciUlInfo, p_DownlinkBWP_List, p_UplinkBWP_List )); }

After Change:

function f_NR_SS_ConfigureBWPs(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, BWP_Id p_BwpId_DCI_DL, BWP_Id p_BwpId_Data_DL, BWP_Id p_BwpId_DCI_UL, BWP_Id p_BwpId_Data_UL, integer p_NoPfBWPs := 4, template (omit) NR_FreqDomainResourceAssignmentDL_Type p_FreqDomainSchedul := omit, template (omit) NR_DownlinkBWP_List_Type p_DownlinkBWP_List := omit, template (omit) NR_UplinkBWP_List_Type p_UplinkBWP_List := omit ) runs on NR_BASE_PTC { //@sic R5s230634 change 2.1 sic@ var template (value) NR_AssignedBWPs_Type v_DCI_AssignedBWPs_DL; var template (value) NR_DciCommon_BWPIndicator_Type v_BWPIndicator; var template (value) NR_DciFormat_1_X_ResourceAssignment_Type v_ResourceAssignment_DL; var template (value) NR_DciFormat_1_X_SpecificInfo_Type v_DlDciFormat := cs_NR_DciFormat_1_1_AllOmit; var template (value) NR_DciDlInfo_Type v_DciDlInfo; var template (value) NR_ActiveUplinkBWP_Id_Type v_ActiveUplinkBWP_Id; var template (value) NR_AssignedBWPs_Type v_DCI_AssignedBWPs_UL; var template (value) NR_DciUlInfo_Type v_DciUlInfo; var template (value) NR_DciFormat_0_X_SpecificInfo_Type v_DciFormat_UL := cs_NR_DciFormat_0_1_AllOmit; //DL v_DCI_AssignedBWPs_DL := cs_NR_AssignedBWPs_DedicatedBWP(p_BwpId_DCI_DL); // BWP assigned to the searchspace (i.e. in which DCI is sent) v_BWPIndicator := f_NR_DciCommon_BWPIndicator(p_NoPfBWPs, p_BwpId_Data_DL); // BWP indicator in DCI format 1_1 v_DlDciFormat.Format_1_1.BWPIndicator := v_BWPIndicator; // Only the BWP indicator needs to be modified //PUCCH configured with ony one resource in Table 7.1.1.8.1.3.3-2G //@sic R5s230220 change 6 sic@ v_DlDciFormat.Format_1_1.PucchResourceIndicator.Value := '000'B; v_ResourceAssignment_DL := cs_NR_DciFormat_1_X_ResourceAssignment(-, p_FreqDomainSchedul, cs_NR_DciCommon_TimeDomainResourceAssignment('1'B)); //@sic R5s230220 change 6 sic@ v_DciDlInfo := cs_NR_DciDlInfo(v_ResourceAssignment_DL, -, v_DlDciFormat); //UL v_ActiveUplinkBWP_Id := cs_ActiveUplinkBWP_Id_Explicit(p_BwpId_Data_UL); // Active UL BWP: same as in DCI format 0_1 v_DCI_AssignedBWPs_UL := cs_NR_AssignedBWPs_DedicatedBWP(p_BwpId_DCI_UL); // BWP in which DCI is sent v_BWPIndicator := f_NR_DciCommon_BWPIndicator(p_NoPfBWPs, p_BwpId_Data_UL); // BWP indicator in DCI format 0_1 v_DciFormat_UL.Format_0_1.BWPIndicator := v_BWPIndicator; // only the BWP indicator needs to be modified v_DciUlInfo := cs_NR_DciUlInfo_AllOmit; v_DciUlInfo.Format := v_DciFormat_UL; f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_BWP_CONFIG_REQ(p_NR_CellId, p_TimingInfo, p_BwpId_Data_DL, //WA# v_DCI_AssignedBWPs_DL, v_DciDlInfo, v_ActiveUplinkBWP_Id, v_DCI_AssignedBWPs_UL, v_DciUlInfo, p_DownlinkBWP_List, p_UplinkBWP_List )); }