3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230749

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3225 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for Rel-17 eCPSOR test case 6.3.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SOR transparent container IE’s IEI and IE length part (first 3 octets) should not be included in the Payload container IE, according to 24.501 9.11.3.39. Current implementation is encoding the IE length (2 octets) inside cs_NG_SORTransparentContainerValue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set IE length to omit in calling the template cs_NG_SORTransparentContainerValue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_6_3_2_1_TestBody () |
| --- | --- |
| Reason for change | SOR transparent container IE’s IEI and IE length part (first 3 octets) should not be included in the Payload container IE, according to 24.501 9.11.3.39. Current implementation is encoding the IE length (2 octets) inside cs_NG_SORTransparentContainerValue. |
| Summary of change | Set IE length to omit in calling the template cs_NG_SORTransparentContainerValue |
| TTCN module | SOR_CMCI_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_6_3_2_1_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233456 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= '01'O; var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet); var O2_Type v_Length_SOR_CMCI_rule_contents :=int2oct(2+lengthof(v_CriterionValue),2);//@sic R5s230554 sic@ var O2_Type v_Length_SOR_CMCI_contents :=int2oct(4+lengthof(v_CriterionValue),2);//@sic R5s230554 sic@ var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233456 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1);//@sic R5s230554 sic@ //@siclog "Step 1" siclog@ //Void//@sic R5-233456 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233456 sic@ //@siclog "Step 22-34b3a1" siclog@//@sic R5s230554 sic@ v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType & v_CriterionValue; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, int2oct(lengthof(v_Data) + 19, 2)))); v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); f_NR5GC_TransportSorCmCheckfunction(nr_Cell12,nr_Cell11,v_PduToSend,-,"Step 26 PASS","Step 28 PASS"); }//End of fl_TC_6_3_2_1_TestBody

After Change:

function fl_TC_6_3_2_1_TestBody() runs on NR5GC_PTC { var octetstring v_Data; var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var O2_Type v_AccessTechId :='0800'O; var O1_Type v_SORInfo :='01'O;//SI/SSCMI/SSSI//@sic R5-233456 sic@ var octetstring v_NAS_TxMsgContainer_SOR_CMCI; var O16_Type v_Mac; var B128_Type v_Mac_A17; var NG_NAS_SecurityParams_Type v_NAS_SecurityParams; var octetstring v_CriterionType:= '01'O; var octetstring v_CriterionValue:= f_DomainName_Encode(pc_APN_ID_Internet); var O2_Type v_Length_SOR_CMCI_rule_contents :=int2oct(2+lengthof(v_CriterionValue),2);//@sic R5s230554 sic@ var O2_Type v_Length_SOR_CMCI_contents :=int2oct(4+lengthof(v_CriterionValue),2);//@sic R5s230554 sic@ var octetstring v_TsorcmTimer:= '21'O; var O2_Type v_CounterSOR :='0001'O; var template (value) SOR_Header v_Header := cs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B,'1'B);//@sic R5-233456 sic@ var template (value) NG_NAS_DL_Pdu_Type v_PduToSend; var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1);//@sic R5s230554 sic@ //@siclog "Step 1" siclog@ //Void//@sic R5-233456 sic@ //@siclog "Steps 2-21a1" siclog@ //Void//@sic R5-233456 sic@ //@siclog "Step 22-34b3a1" siclog@//@sic R5s230554 sic@ v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_SORInfo & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType & v_CriterionValue; v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_CounterSOR, v_Data, v_NAS_SecurityParams.KDF); v_Mac := bit2oct(v_Mac_A17); v_NAS_TxMsgContainer_SOR_CMCI := bit2oct(encvalue(cs_NG_SORTransparentContainerValue(v_Header, v_Mac, v_CounterSOR, v_Data, omit, omit))); //CR v_PduToSend := f_Get_NG_DLNASTransport (cs_PayloadContainerType(tsc_PayloadContainerSORMsg), omit, cs_PayloadContainer(omit, v_NAS_TxMsgContainer_SOR_CMCI)); f_NR5GC_TransportSorCmCheckfunction(nr_Cell12,nr_Cell11,v_PduToSend,-,"Step 26 PASS","Step 28 PASS"); }//End of fl_TC_6_3_2_1_TestBody