3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230753

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3228 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Test case 9.1.5.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the 9.1.5.1.2 the UE is switched off and on several times during the test and each time it it is supposed to find service on the correct equivalent PLMN among three different interF cells with different PLMN each. The PLMN search procedure may take relatively longer amount of time and while this test relies on UE to perform several such searches, we propose to increase the guard timer (GT) so UE has enough time to go through the test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - GT increased from 15 minutes to 25 minutes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case due to GT being too short. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| TC name | TC_9_1_5_1_2_NR5GC |
| --- | --- |
| Reason for change | - The 9.1.5.1.2 involves changing of the USIM as part of the test body. This procedure may take relatively significant amount of time which given the current guard timer (GT) of only 5 minutes may cause the test case fail unfairly due to GT. Therefore we propose to increase GT to 10 minutes so there is enough time for the test procedure including the USIM change. |
| Summary of change | - GT increased from 5 minutes to 10 minutes. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_9_1_5_1_2_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Initial registration / 5GS services / Equivalent PLMN list handling var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(900); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_9_1_5_1_2_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_9_1_5_1_2_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Initial registration / 5GS services / Equivalent PLMN list handling var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1500); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_9_1_5_1_2_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }