3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230759

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4766 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE test case 8.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TTCN-3 Core Language specification "15.6.2 Referencing record and set fields": c) Ifpresent attribute: referencing a subfield within a structured field to which the ifpresent attribute is attached, shall cause an error (irrespective of the value or the matching mechanism to which ifpresent is appended). - rel9 v_EutraCapability.nonCriticalExtension.nonCriticalExtension / v_EutraCapability.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension / v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension already in template cr_UeCapabilitiesRAT_Container_r9 are *, it’s can match omit and ?, So can removed the TTCN. rel10 v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension already in template cr_UeCapabilitiesRAT_Container_r10 are *, it’s can match omit and ?, So can removed the TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the included {} ifpresent format and it’s already match in template UeCapabilitiesRAT_Container |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | fl_EUTRA_FullCapabilityCheck |
| --- | --- |
| Reason for change | According to TTCN-3 Core Language specification "15.6.2 Referencing record and set fields": c) Ifpresent attribute: referencing a subfield within a structured field to which the ifpresent attribute is attached, shall cause an error (irrespective of the value or the matching mechanism to which ifpresent is appended). - rel9 v_EutraCapability.nonCriticalExtension.nonCriticalExtension / v_EutraCapability.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension / v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension already in template cr_UeCapabilitiesRAT_Container_r9 are *, it’s can match omit and ?, So can removed the TTCN. rel10 v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension already in template cr_UeCapabilitiesRAT_Container_r10 are *, it’s can match omit and ?, So can removed the TTCN. |
| Summary of change | Removed the included {} ifpresent format and it’s already match in template UeCapabilitiesRAT_Container |
| TTCN module | LTE\ 8_5\ RRC_Others.ttcn |
| MCC160 comment | Accepted |

Before Change:

function fl_EUTRA_FullCapabilityCheck(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC { var UE_EUTRA_Capability v_ReceivedCapabilityMsg; var template UE_EUTRA_Capability v_EutraCapability; var template UE_EUTRA_Capability_v9a0_IEs v_Expected_UE_EUTRA_Capability_v9a0_IEs := omit; // Transmit UE Capability Enquiry message to ask for EUTRA Capability information SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI,-,-,-,-,enabled))); // @sic R5-217903 sic@ // Receive UE Capability Information message for EUTRA capabilities and decode // Either a container is received or segments from which the container is to be established (only Rel-16 onwards) v_ReceivedCapabilityMsg := f_EUTRA_GetCaps(p_CellId, p_RRC_TI); // @sic R5-217903 sic@ select (v_ReceivedCapabilityMsg.accessStratumRelease) { // @sic R5s120250 sic@ case (rel8) { v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r8); // Check Rel 8 vs. IEs provided if (isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension)) { // @sic R5-123750 sic@ // if mixed releases then Rel 9 IEs may be povided v_EutraCapability.nonCriticalExtension := ?; } else { v_EutraCapability.nonCriticalExtension := omit; } // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r8(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@ } case (rel9) { v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r9); if (not isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension)) { // no Rel 9 (UE-EUTRA-Capability-v940-IEs) IEs and thus no additional Rel 9 FGIs provided v_EutraCapability.nonCriticalExtension.nonCriticalExtension := omit; // @sic R5s120623 sic@ } else { // Rel 9 (UE-EUTRA-Capability-v940-IEs) IEs provided !! if (isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension)) { // Rel 9 (UE-EUTRA-Capability-v9a0-IEs) IEs provided !! v_EutraCapability.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension := ?; } else { v_EutraCapability.nonCriticalExtension.nonCriticalExtension.lateNonCriticalExtension := omit; } if (isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { // Rel 10 (UE-EUTRA-Capability-v1020-IEs) IEs may be provided by Rel 9 UE, but not checked v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := ?; // @sic R5-150656 sic@ } else { v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := omit; } } // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r9(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@ } case (rel10) { v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r10); if (not isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { // @sic R5s130088 sic@ // no Rel 10 IEs provided although Rel 10 FGIs must be provided (36.331 Annex C.1) f_SetVerdict(fail, __FILE__, __LINE__, "UE-EUTRA-Capability-v1020-IEs must be provided by Rel 10 UE"); // @sic R5-133612 sic@ } else { v_EutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := ?; } if (not isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.featureGroupIndRel10_r10)) { // @sic R5s130269 sic@ @sic R5-133612 sic@ f_SetVerdict(fail, __FILE__, __LINE__, "UE-EUTRA-Capability-v1020-IEs FGIs must be provided by Rel 10 UE"); } // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r10(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@ } case (rel11) { // @sic R5s130195 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r11); // @sic R5s160666, R5-166324 sic@ v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r11(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@ } case (rel12) { // @sic R5s150329 - Rel-12 Baseline move sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r12); // @sic R5s160666, R5-166324 sic@ v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r12(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@ } case (rel13) { // @sic R5s160711 BASELINE MOVING 2016 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r13); // @sic R5s160666, R5-166324 sic@ v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r13(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@ } case (rel14) { // @sic R5s170597 BASELINE MOVING 2017 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r14); v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r14(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@ } case (rel15) { // @sic R5s180552 BASELINE MOVING 2018 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r15); v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r15(v_ReceivedCapabilityMsg); // @sic R5s180552 BASELINE MOVING 2018 sic@ } case (rel16) { // @sic R5s201386 BASELINE MOVING 2020 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r16); v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r16(v_ReceivedCapabilityMsg); // @sic R5s201386 BASELINE MOVING 2020 sic@ } case (rel17) { // @sic R5s221178 BASELINE MOVING 2022 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r17); v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r17(v_ReceivedCapabilityMsg); // @sic R5s221178 BASELINE MOVING 2022 sic@ } case else { f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not testable"); // a UE implementing an unsupported release must not be failed ! } } // Match the received UECapabilityInformation with the expected UE_EUTRA_Capability template and, if applicable, the UE_EUTRA_Capability_v9a0_IEs template. f_EUTRA_UE_Capability_MatchTemplates(v_ReceivedCapabilityMsg, v_EutraCapability, v_Expected_UE_EUTRA_Capability_v9a0_IEs); } // fl_EUTRA_FullCapabilityCheck

After Change:

function fl_EUTRA_FullCapabilityCheck(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI) runs on EUTRA_PTC { var UE_EUTRA_Capability v_ReceivedCapabilityMsg; var template UE_EUTRA_Capability v_EutraCapability; var template UE_EUTRA_Capability_v9a0_IEs v_Expected_UE_EUTRA_Capability_v9a0_IEs := omit; // Transmit UE Capability Enquiry message to ask for EUTRA Capability information SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry(p_RRC_TI,-,-,-,-,enabled))); // @sic R5-217903 sic@ // Receive UE Capability Information message for EUTRA capabilities and decode // Either a container is received or segments from which the container is to be established (only Rel-16 onwards) v_ReceivedCapabilityMsg := f_EUTRA_GetCaps(p_CellId, p_RRC_TI); // @sic R5-217903 sic@ select (v_ReceivedCapabilityMsg.accessStratumRelease) { // @sic R5s120250 sic@ case (rel8) { v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r8); // Check Rel 8 vs. IEs provided if (isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension)) { // @sic R5-123750 sic@ // if mixed releases then Rel 9 IEs may be povided v_EutraCapability.nonCriticalExtension := ?; } else { v_EutraCapability.nonCriticalExtension := omit; } // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r8(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@ } case (rel9) { v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r9); // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r9(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@ } case (rel10) { v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r10); if (not isvalue (v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { // @sic R5s130088 sic@ // no Rel 10 IEs provided although Rel 10 FGIs must be provided (36.331 Annex C.1) f_SetVerdict(fail, __FILE__, __LINE__, "UE-EUTRA-Capability-v1020-IEs must be provided by Rel 10 UE"); // @sic R5-133612 sic@ } if (not isvalue(v_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.featureGroupIndRel10_r10)) { // @sic R5s130269 sic@ @sic R5-133612 sic@ f_SetVerdict(fail, __FILE__, __LINE__, "UE-EUTRA-Capability-v1020-IEs FGIs must be provided by Rel 10 UE"); } // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r10(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@ } case (rel11) { // @sic R5s130195 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r11); // @sic R5s160666, R5-166324 sic@ v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r11(v_ReceivedCapabilityMsg); // @sic R5-130706, R5s170597 sic@ } case (rel12) { // @sic R5s150329 - Rel-12 Baseline move sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r12); // @sic R5s160666, R5-166324 sic@ v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r12(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@ } case (rel13) { // @sic R5s160711 BASELINE MOVING 2016 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r13); // @sic R5s160666, R5-166324 sic@ v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r13(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@ } case (rel14) { // @sic R5s170597 BASELINE MOVING 2017 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r14); v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r14(v_ReceivedCapabilityMsg); // @sic R5s170597 sic@ } case (rel15) { // @sic R5s180552 BASELINE MOVING 2018 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r15); v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r15(v_ReceivedCapabilityMsg); // @sic R5s180552 BASELINE MOVING 2018 sic@ } case (rel16) { // @sic R5s201386 BASELINE MOVING 2020 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r16); v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r16(v_ReceivedCapabilityMsg); // @sic R5s201386 BASELINE MOVING 2020 sic@ } case (rel17) { // @sic R5s221178 BASELINE MOVING 2022 sic@ v_EutraCapability := f_EUTRA_SetExpectedUeEutraCap(cr_UeCapabilitiesRAT_Container_r17); v_Expected_UE_EUTRA_Capability_v9a0_IEs := cr_UE_EUTRA_Capability_v9a0_IEs_r11; // Match the ueCapabilitiesRAT-Container f_EUTRA_MatchEutraCapMsgs_r17(v_ReceivedCapabilityMsg); // @sic R5s221178 BASELINE MOVING 2022 sic@ } case else { f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not testable"); // a UE implementing an unsupported release must not be failed ! } } // Match the received UECapabilityInformation with the expected UE_EUTRA_Capability template and, if applicable, the UE_EUTRA_Capability_v9a0_IEs template. f_EUTRA_UE_Capability_MatchTemplates(v_ReceivedCapabilityMsg, v_EutraCapability, v_Expected_UE_EUTRA_Capability_v9a0_IEs); } // fl_EUTRA_FullCapabilityCheck