3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230772

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4768 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE test case 8.2.4.18.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2023-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the testcase is run with band bandCA_66C, it is observed that the testcase fail at ARFCN_ValueEUTRA OutOfValue. The ARFCN is 66984 which is larger than 65535, which results in this error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add a new template cs_Measurement_Config_A3_InterFreq_CA_r9 where the parameters p_EUTRA_DL_CarrierFreq1 and p_EUTRA_DL_CarrierFreq2 are set to be of type ARFCN_ValueEUTRA_r9 . Also used this template in main testcase body in function f_TC_8_2_4_18_Common when band > 64. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.18.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Similar changes have been done for R5-171441 for other templates. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 	Change 1	2

2.2 	Change 2	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 	Change 1

| Function name | cs_Measurement_Config_A3_InterFreq_CA_r9 |
| --- | --- |
| Reason for change | When the testcase is run with band bandCA_66C, it is observed that the testcase fail at ARFCN_ValueEUTRA OutOfValue. The ARFCN is 66984 which is larger than 65535, which results in this error. |
| Summary of change | Added a new template cs_Measurement_Config_A3_InterFreq_CA_r9 where the parameters p_EUTRA_DL_CarrierFreq1 and p_EUTRA_DL_CarrierFreq2 are set to be of type ARFCN_ValueEUTRA_r9 . |
| TTCN module | LTE_A_R10_R11\COMMON\EUTRA\ EUTRA_SecurityFunctions.ttcn |
| MCC160 Comments | All reason for changes refer to cs_Measurement_Config_A3_InterFreq_CA_r9, but the actual changes proposed are for cs_Measurement_Config_A3_IntraFreq_CA_r9. The changes accepted for cs_Measurement_Config_A3_IntraFreq_CA_r9. |

After Change:

template (value) MeasConfig cs_Measurement_Config_A3_IntraFreq_CA_r9(ARFCN_ValueEUTRA_r9 p_EUTRA_DL_CarrierFreq1, AllowedMeasBandwidth p_MeasurementBandwidth1, ARFCN_ValueEUTRA_r9 p_EUTRA_DL_CarrierFreq2, AllowedMeasBandwidth p_MeasurementBandwidth2, MeasObjectId p_MeasObjectId_2, //@sic R5s130422 sic@ template (omit) MeasGapConfig p_MeasGapConfig := omit, MeasObjectId p_MeasObjectId_1 := tsc_IdMeasObject_f1, //@sic R5s144680 sic@ template (value) ReportConfigToAddMod_ReportConfig_Type p_ReportConfig := cs_508_ReportConfigEUTRA_A3) := /* @status APPROVED (LTE_A_R10_R11) */ //@sic R5s180224 sic@ cs_MeasConfig(omit, cs_MeasObjectToAddModList_2Entries(p_MeasObjectId_1, cs_MeasObject_measObjectEUTRA_Common(f_EUTRA_ARFCN_Value (p_EUTRA_DL_CarrierFreq1), p_MeasurementBandwidth1), p_MeasObjectId_2, cs_MeasObject_measObjectEUTRA_Common(f_EUTRA_ARFCN_Value (p_EUTRA_DL_CarrierFreq2), p_MeasurementBandwidth2,-,-,-,-,sf512)), //@sic R5s160833 sic@ omit, cs_ReportConfigToAddModList_1Entry(tsc_IdReportConfig_A3, p_ReportConfig), omit, cs_MeasIdToAddModList_1Entry( 1, p_MeasObjectId_1, tsc_IdReportConfig_A3 ), //@sic R5-130845 R5-131022 sic@ -, p_MeasGapConfig, -,-,-, f_ReturnMeasObjectToAddModList_2Entries_v9e0(p_EUTRA_DL_CarrierFreq1, p_EUTRA_DL_CarrierFreq2 ) );

## 2.2 	Change 2

| Function name | f_TC_8_2_4_18_Common |
| --- | --- |
| Reason for change | When the testcase is run with band bandCA_66C, it is observed that the testcase fail at ARFCN_ValueEUTRA OutOfValue. The ARFCN is 66984 which is larger than 65535, which results in this error. |
| Summary of change | Please see change 1 above. Used this template in main testcase body in function f_TC_8_2_4_18_Common when band > 64. |
| TTCN module | LTE_A_R10_R11\ 8.2\ RRC_Handover_CA.ttcn |
| MCC160 Comments | Accepted |

function f_TC_8_2_4_18_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_NeighCell, EUTRA_CellId_Type p_SCellId, CA_Tested_Type p_CA_Type) runs on EUTRA_PTC { var PhysCellId v_PhysicalCellIdentity_NeighCell; var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_PCell; var Frequency_IE_Type v_Frequency_IE_Scell; var ChannelBandwidthDependency_Type v_ChBandDependency_PCell; var ChannelBandwidthDependency_Type v_ChBandDependency_Scell; var template (value) CellPowerList_Type v_CellPowerList_AtT0; var template (value) CellPowerList_Type v_CellPowerList_AtT1; var boolean v_UL_CA; //@sic R5s170752 sic@ //var template (value) CarrierFreqEUTRA v_CarrierFreq_Scell; var MeasObjectId v_MeasObjectId ; var MeasObjectId v_MeasObjectId_PCell :=tsc_IdMeasObject_f1 ; //@sic R5s144680 sic@ var default v_DefaultRef; //@sic R5s151045 sic@ v_CellPowerList_AtT0 := { cs_CellPower(p_NeighCell, -91), cs_CellPower(p_SCellId, -97) }; v_CellPowerList_AtT1 := { cs_CellPower(p_NeighCell, -79) }; if (p_CA_Type == CA_IntraBand or p_CA_Type == CA_IntraBandNonContiguous ) { /* @sic R5-141113 R5s150037 sic@ */ v_MeasObjectId := tsc_IdMeasObject_f2; /* if (p_PCellId == eutra_Cell6) //@sic R5s144680 sic@ { v_MeasObjectId_PCell := tsc_IdMeasObject_f3; } */ } else { v_MeasObjectId := tsc_IdMeasObject_f5; } f_EUTRA_Init(c3); f_EUTRA_CA_InitCells({p_PCellId, p_SCellId}, p_CA_Type, c3); v_UL_CA := f_EUTRA_MobileInfo_GetUL_CA(); //@sic R5s170752 sic@ //@sic R5-144655, R5-144681 sic@ // Only needed for the case of non contigious as cell 13 PLMN as per table 4.4.2-2 is PLMN 3 f_EUTRA_CellInfo_SetPLMN_1Entry(p_NeighCell, cs_HPLMN_Def);//Home PLMN f_EUTRA_CellInfo_SetTAC(p_NeighCell, oct2bit(tsc_TAC1)); //Set maximum cell power level for intra freq cell to be used in creation f_EUTRA_CellInfo_InitMaxReferencePower(p_NeighCell, -79); //Get cell dependant parameters v_Frequency_IE_Scell := f_EUTRA_CellInfo_GetFrequencyIEs(p_SCellId); v_ChBandDependency_Scell := f_EUTRA_BandDependentParam(v_Frequency_IE_Scell.DL_ChBandwidth, v_Frequency_IE_Scell.UL_ChBandwidth); v_Frequency_IE_PCell := f_EUTRA_CellInfo_GetFrequencyIEs(p_PCellId); v_ChBandDependency_PCell := f_EUTRA_BandDependentParam(v_Frequency_IE_PCell.DL_ChBandwidth, v_Frequency_IE_PCell.UL_ChBandwidth); //@sic R5-165892 sic@ //v_CarrierFreq_Scell := cs_CarrierFreqEUTRA(v_Frequency_IE_Scell.UL_DL_Earfcn.dl_CarrierFreq, v_Frequency_IE_Scell.UL_DL_Earfcn.ul_CarrierFreq); v_PhysicalCellIdentity_NeighCell := f_EUTRA_CellInfo_GetPhyCellId(p_NeighCell); //Set C-RNTI to be used in s-cell creation f_EUTRA_CellInfo_SetC_RNTI(p_SCellId, tsc_C_RNTI_Def); //Create and configure all cells f_EUTRA_CellConfig_Pcell_Def(p_PCellId); f_EUTRA_CellConfig_Scell(p_SCellId); //No DRB's created f_EUTRA_CellConfig_Def(p_NeighCell); //Normal cell configured // ring UE to initial state f_EUTRA_Preamble(p_PCellId, STATE2_IDLEUPDATE); f_EUTRA_RbEst_Def(p_PCellId); //Set cell power level according to T0 f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0); f_EUTRA_TestBody_Set(true); //@siclog "Step 1 - 2" siclog@ //Transmit RRCConnectionReconfiguration to add Scell //Receive RRCConnectionReconfigurationComplete f_EUTRA_508RRC_AddMod_1Scell(p_PCellId, p_SCellId, 1, //SCellIndex_r10 // p_CA_Type v_UL_CA); //@siclog "Step 3-4" siclog@ if (f_EUTRA_CellInfo_IsLAA (p_SCellId)) { //@sic R5s180031 sic@ f_EUTRA_SS_CommonCellConfig(p_SCellId, cas_PhysicalLayerConfigDL_REQ(p_SCellId, cs_TimingInfo_Now, cs_PhysicalLayerConfigDS_RS(cs_Dmtc_PeriodOffsetms160_r12(10), cs_Ds_OccasionDuration_r12_FDD(1)))); f_EUTRA_RRCConnectionReconfiguration_MeasConfig(p_PCellId, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_Measurement_Config_A3_IntraFreq_CA_LAA(v_Frequency_IE_PCell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_PCell.AllowedMeasBandwidth, v_Frequency_IE_Scell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_Scell.AllowedMeasBandwidth, v_MeasObjectId, omit,//@sic R5s144680 sic@ v_MeasObjectId_PCell, cs_ReportConfig_reportConfigEUTRA_eventA3(0, true, 0, ms2560, rsrp, both, 1, ms1024, r1)))); //@sic R5-140756 sic@ }else { f_EUTRA_RRCConnectionReconfiguration_MeasConfig(p_PCellId, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_Measurement_Config_A3_IntraFreq_CA(v_Frequency_IE_PCell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_PCell.AllowedMeasBandwidth, v_Frequency_IE_Scell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_Scell.AllowedMeasBandwidth, v_MeasObjectId, omit,//@sic R5s144680 sic@ v_MeasObjectId_PCell, cs_ReportConfig_reportConfigEUTRA_eventA3(0, true, 0, ms2560, rsrp, both, 1, ms1024, r1)))); //@sic R5-140756 sic@ } //@siclog "Step 5" siclog@ //Set cell power level according to T1 f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 6" siclog@ //Receive MeasurementReport SRB.receive(car_SRB1_RrcPdu_IND(p_PCellId, cr_MeasurementReport(1, //Meas id ?, //RSRP result ?, //RSRQ result cr_MeasResultNeighCells_measResultListEUTRA_1Entry(v_PhysicalCellIdentity_NeighCell, omit, //CGI info ?, //RSRP result ?),//RSRQ result cr_MeasResultServFreqList(1, ?, ?)))); //Activate the default to receive further measurement reports for cell 2 on cell 1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(p_PCellId, cr_MeasurementReport(1, ?, ?, cr_MeasResultNeighCells_measResultListEUTRA_1Entry(v_PhysicalCellIdentity_NeighCell, omit, ?, ?), cr_MeasResultServFreqList(1, ?, ?)))); //* @verdict Measurement Report received on Cell 3 f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.4.18 or 8.2.4.21 Step 6"); //@siclog "Step 7-8" siclog@ f_EUTRA_508RRC_IntraLTE_CA_HO_ToNormalInterCell(p_PCellId, p_SCellId, 1, p_NeighCell); deactivate(v_DefaultRef); //@siclog "Step 9" siclog@ //Check if UE is in state RRC_Connected f_EUTRA_508Check_ConnectedState(p_NeighCell); f_EUTRA_TestBody_Set(false); //Switch/power off UE f_EUTRA_Postamble(p_NeighCell, E2_CONNECTED); } }

After Change:

function f_TC_8_2_4_18_Common(EUTRA_CellId_Type p_PCellId, EUTRA_CellId_Type p_NeighCell, EUTRA_CellId_Type p_SCellId, CA_Tested_Type p_CA_Type) runs on EUTRA_PTC { var PhysCellId v_PhysicalCellIdentity_NeighCell; var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var Frequency_IE_Type v_Frequency_IE_PCell; var Frequency_IE_Type v_Frequency_IE_Scell; var ChannelBandwidthDependency_Type v_ChBandDependency_PCell; var ChannelBandwidthDependency_Type v_ChBandDependency_Scell; var template (value) CellPowerList_Type v_CellPowerList_AtT0; var template (value) CellPowerList_Type v_CellPowerList_AtT1; var boolean v_UL_CA; //@sic R5s170752 sic@ //var template (value) CarrierFreqEUTRA v_CarrierFreq_Scell; var MeasObjectId v_MeasObjectId ; var MeasObjectId v_MeasObjectId_PCell :=tsc_IdMeasObject_f1 ; //@sic R5s144680 sic@ var default v_DefaultRef; //@sic R5s151045 sic@ v_CellPowerList_AtT0 := { cs_CellPower(p_NeighCell, -91), cs_CellPower(p_SCellId, -97) }; v_CellPowerList_AtT1 := { cs_CellPower(p_NeighCell, -79) }; if (p_CA_Type == CA_IntraBand or p_CA_Type == CA_IntraBandNonContiguous ) { /* @sic R5-141113 R5s150037 sic@ */ v_MeasObjectId := tsc_IdMeasObject_f2; /* if (p_PCellId == eutra_Cell6) //@sic R5s144680 sic@ { v_MeasObjectId_PCell := tsc_IdMeasObject_f3; } */ } else { v_MeasObjectId := tsc_IdMeasObject_f5; } f_EUTRA_Init(c3); f_EUTRA_CA_InitCells({p_PCellId, p_SCellId}, p_CA_Type, c3); v_UL_CA := f_EUTRA_MobileInfo_GetUL_CA(); //@sic R5s170752 sic@ //@sic R5-144655, R5-144681 sic@ // Only needed for the case of non contigious as cell 13 PLMN as per table 4.4.2-2 is PLMN 3 f_EUTRA_CellInfo_SetPLMN_1Entry(p_NeighCell, cs_HPLMN_Def);//Home PLMN f_EUTRA_CellInfo_SetTAC(p_NeighCell, oct2bit(tsc_TAC1)); //Set maximum cell power level for intra freq cell to be used in creation f_EUTRA_CellInfo_InitMaxReferencePower(p_NeighCell, -79); //Get cell dependant parameters v_Frequency_IE_Scell := f_EUTRA_CellInfo_GetFrequencyIEs(p_SCellId); v_ChBandDependency_Scell := f_EUTRA_BandDependentParam(v_Frequency_IE_Scell.DL_ChBandwidth, v_Frequency_IE_Scell.UL_ChBandwidth); v_Frequency_IE_PCell := f_EUTRA_CellInfo_GetFrequencyIEs(p_PCellId); v_ChBandDependency_PCell := f_EUTRA_BandDependentParam(v_Frequency_IE_PCell.DL_ChBandwidth, v_Frequency_IE_PCell.UL_ChBandwidth); //@sic R5-165892 sic@ //v_CarrierFreq_Scell := cs_CarrierFreqEUTRA(v_Frequency_IE_Scell.UL_DL_Earfcn.dl_CarrierFreq, v_Frequency_IE_Scell.UL_DL_Earfcn.ul_CarrierFreq); v_PhysicalCellIdentity_NeighCell := f_EUTRA_CellInfo_GetPhyCellId(p_NeighCell); //Set C-RNTI to be used in s-cell creation f_EUTRA_CellInfo_SetC_RNTI(p_SCellId, tsc_C_RNTI_Def); //Create and configure all cells f_EUTRA_CellConfig_Pcell_Def(p_PCellId); f_EUTRA_CellConfig_Scell(p_SCellId); //No DRB's created f_EUTRA_CellConfig_Def(p_NeighCell); //Normal cell configured // ring UE to initial state f_EUTRA_Preamble(p_PCellId, STATE2_IDLEUPDATE); f_EUTRA_RbEst_Def(p_PCellId); //Set cell power level according to T0 f_EUTRA_SetCellPowerList(v_CellPowerList_AtT0); f_EUTRA_TestBody_Set(true); //@siclog "Step 1 - 2" siclog@ //Transmit RRCConnectionReconfiguration to add Scell //Receive RRCConnectionReconfigurationComplete f_EUTRA_508RRC_AddMod_1Scell(p_PCellId, p_SCellId, 1, //SCellIndex_r10 // p_CA_Type v_UL_CA); //@siclog "Step 3-4" siclog@ if (f_EUTRA_CellInfo_IsLAA (p_SCellId)) { //@sic R5s180031 sic@ f_EUTRA_SS_CommonCellConfig(p_SCellId, cas_PhysicalLayerConfigDL_REQ(p_SCellId, cs_TimingInfo_Now, cs_PhysicalLayerConfigDS_RS(cs_Dmtc_PeriodOffsetms160_r12(10), cs_Ds_OccasionDuration_r12_FDD(1)))); f_EUTRA_RRCConnectionReconfiguration_MeasConfig(p_PCellId, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_Measurement_Config_A3_IntraFreq_CA_LAA(v_Frequency_IE_PCell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_PCell.AllowedMeasBandwidth, v_Frequency_IE_Scell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_Scell.AllowedMeasBandwidth, v_MeasObjectId, omit,//@sic R5s144680 sic@ v_MeasObjectId_PCell, cs_ReportConfig_reportConfigEUTRA_eventA3(0, true, 0, ms2560, rsrp, both, 1, ms1024, r1)))); //@sic R5-140756 sic@ } else if(v_Frequency_IE_PCell.UL_DL_Earfcn.dl_CarrierFreq_r9 > maxEARFCN) {//band >64 f_EUTRA_RRCConnectionReconfiguration_MeasConfig(p_PCellId, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_Measurement_Config_A3_IntraFreq_CA_r9(v_Frequency_IE_PCell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_PCell.AllowedMeasBandwidth, v_Frequency_IE_Scell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_Scell.AllowedMeasBandwidth, v_MeasObjectId, omit,//@sic R5s144680 sic@ v_MeasObjectId_PCell, cs_ReportConfig_reportConfigEUTRA_eventA3(0, true, 0, ms2560, rsrp, both, 1, ms1024, r1)))); //@sic R5-140756 sic@ }else { //Band <64} f_EUTRA_RRCConnectionReconfiguration_MeasConfig(p_PCellId, cs_508_RRCConnectionReconfiguration_ConditionMEAS(v_RRC_TI, cs_Measurement_Config_A3_IntraFreq_CA(v_Frequency_IE_PCell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_PCell.AllowedMeasBandwidth, v_Frequency_IE_Scell.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5-171441 sic@ v_ChBandDependency_Scell.AllowedMeasBandwidth, v_MeasObjectId, omit,//@sic R5s144680 sic@ v_MeasObjectId_PCell, cs_ReportConfig_reportConfigEUTRA_eventA3(0, true, 0, ms2560, rsrp, both, 1, ms1024, r1)))); //@sic R5-140756 sic@ } //@siclog "Step 5" siclog@ //Set cell power level according to T1 f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 6" siclog@ //Receive MeasurementReport SRB.receive(car_SRB1_RrcPdu_IND(p_PCellId, cr_MeasurementReport(1, //Meas id ?, //RSRP result ?, //RSRQ result cr_MeasResultNeighCells_measResultListEUTRA_1Entry(v_PhysicalCellIdentity_NeighCell, omit, //CGI info ?, //RSRP result ?),//RSRQ result cr_MeasResultServFreqList(1, ?, ?)))); //Activate the default to receive further measurement reports for cell 2 on cell 1 v_DefaultRef := activate(a_EUTRA_AddDefault_PeriodicalMeasurmentReport(p_PCellId, cr_MeasurementReport(1, ?, ?, cr_MeasResultNeighCells_measResultListEUTRA_1Entry(v_PhysicalCellIdentity_NeighCell, omit, ?, ?), cr_MeasResultServFreqList(1, ?, ?)))); //* @verdict Measurement Report received on Cell 3 f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.2.4.18 or 8.2.4.21 Step 6"); //@siclog "Step 7-8" siclog@ f_EUTRA_508RRC_IntraLTE_CA_HO_ToNormalInterCell(p_PCellId, p_SCellId, 1, p_NeighCell); deactivate(v_DefaultRef); //@siclog "Step 9" siclog@ //Check if UE is in state RRC_Connected f_EUTRA_508Check_ConnectedState(p_NeighCell); f_EUTRA_TestBody_Set(false); //Switch/power off UE f_EUTRA_Postamble(p_NeighCell, E2_CONNECTED); }