3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230813

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4772 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_GERAN_SysInfo_GetSI2q_2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When the testcase is run in Eutra band 38, the current TTCN implementation configures two ARFCNs, 37850 and 38150 respectively. In the function f_GERAN_SystemInformation_InterRAT, for the case No UTRAN, if lengthof(v_EUTRA_SysInfo) is 3 or 4, then the IE v_CellArray[v_Index].sysInfo.si2quater_2 is initialized, otherwise not. With this setting, the function call f_TC_6_2_2_2_GERAN v_SI2quater2 := f_GERAN_SysInfo_GetSI2q_2 (geran_Cell24); is called, it results in a runtime error, because the IE si2quater_2 is not initialized. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to update the function f_GERAN_SysInfo_GetSI2q_2 to be able to return template (omit) SYSTEMINFORMATIONTYPE2quater |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | Function fl_TC_8_2_3_17_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | When the testcase is run in Eutra band 38, the current TTCN implementation configures two ARFCNs, 37850 and 38150 respectively. In the function f_GERAN_SystemInformation_InterRAT, for the case No UTRAN, if lengthof(v_EUTRA_SysInfo) is 3 or 4, then the IE v_CellArray[v_Index].sysInfo.si2quater_2 is initialized, otherwise not. With this setting, the function call f_TC_6_2_2_2_GERAN v_SI2quater2 := f_GERAN_SysInfo_GetSI2q_2 (geran_Cell24); is called, it results in a runtime error, because the IE si2quater_2 is not initialized. This needs to be addressed. |
| Summary of change | It is proposed to update the function f_GERAN_SysInfo_GetSI2q_2 to be able to return template (omit) SYSTEMINFORMATIONTYPE2quater |
| TTCN module | GERAN_Component.ttcn |
| MCC160 Comment | Accepted. Also corrected typo in f_GERAN_SystemInformation_InterRATas shown below |

function f_GERAN_SysInfo_GetSI2q_2(GERAN_CellId_Type p_CellId) runs on GERAN_PTC return SYSTEMINFORMATIONTYPE2quater { var GERAN_SystemInformation_Type v_SysInfo := f_GERAN_SysInfo_Get (p_CellId); return v_SysInfo.si2quater_2; }

After Change:

function f_GERAN_SysInfo_GetSI2q_2(GERAN_CellId_Type p_CellId) runs on GERAN_PTC return template (omit)SYSTEMINFORMATIONTYPE2quater //WA#6.2.2.2 E38-GSM { var GERAN_SystemInformation_Type v_SysInfo := f_GERAN_SysInfo_Get (p_CellId); return v_SysInfo.si2quater_2; }

function f_GERAN_SystemInformation_InterRAT ( GERAN_CellArray_Type p_CellArray,

IRAT_CoOrd_SysInfo_List_Type p_SysInfoList) return GERAN_CellArray_Type

{

var CoOrd_EUTRASysInfo_List_Type v_EUTRA_SysInfo;

var CoOrd_UTRANSysInfo_List_Type v_UTRAN_SysInfo;

var template (value) GERAN_CellArray_Type v_CellArray := p_CellArray;

var template (value) EUTRANMeasParams_SI2 v_EUTRA_SI2_1;

var template (value) EUTRANMeasParams_SI2 v_EUTRA_SI2_2;

var template (value) EUTRANMeasParams_SI2 v_EUTRA_SI2_3;

var template (value) EUTRANMeasParams_SI2 v_EUTRA_SI2_4;

var integer v_Index := 0;

var integer v_NoOfPTCs := lengthof (p_SysInfoList);

var template (omit) RptdEARFCN v_XtdEARFCN1 := omit;

var template (omit) RptdEARFCN v_XtdEARFCN2 := omit;

var template (omit) RptdEARFCN v_XtdEARFCN3 := omit;

var template (omit) RptdEARFCN v_XtdEARFCN4 := omit;

var B4_Type v_NoOfInstances := '0101'B;

v_CellArray[v_Index].sysInfo.si2quater_2 := omit;

v_CellArray[v_Index].sysInfo.si2quater_3 := omit;  // so everything can fit in one instance @sic R5s120021 sic@

v_CellArray[v_Index].sysInfo.si2quater_4 := omit;  // so everything can fit in one instance @sic R5s120129 sic@

v_CellArray[v_Index].sysInfo.si2quater_5 := omit;

v_CellArray[v_Index].sysInfo.si2quater_6 := omit;  // so everything can fit in one instance @sic R5s120021 sic@

...

} else {

if (isvalue(v_XtdEARFCN1) or isvalue(v_XtdEARFCN2) ) { // @sic R5-173669 sic@

if ( isvalue(v_XtdEARFCN2)) { // @sic R5-177078 sic@

v_NoOfInstances := '0001'B;

} else {

v_NoOfInstances := '0000'B;

}

v_CellArray[v_Index].sysInfo.si2quater_1 := cs_SysInfoType2quater(cds_SI2quaterRO_Rel11('0000'B, v_NoOfInstances, '0'B, omit, // @sic GP-120371 sic@

omit,

omit,

v_EUTRA_SI2_1, '0'B,

v_XtdEARFCN1));

if (v_NoOfInstances == '0001'B) { // @sic R5-177078 sic@

v_CellArray[v_Index].sysInfo.si2quater_2 := cs_SysInfoType2quater(cds_SI2quaterRO_Rel11('0001'B, v_NoOfInstances, '0'B, omit, // @sic GP-120371, R5s180468 sic@

omit,

omit,

v_EUTRA_SI2_2, '0'B,

v_XtdEARFCN2));

}

} else {

v_CellArray[v_Index].sysInfo.si2quater_1 := cs_SysInfoType2quater(cs_SI2quaterRO('0000'B, '0000'B, '0'B, omit, // @sic GP-120371, R5s120129, R5s180468 sic@

omit,

omit,

v_EUTRA_SI2_1));

}

}

}

}

return valueof (v_CellArray);

}