3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230864

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3270 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC redcap test case 6.1.2.27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. As per Table 6.1.2.27.3.3-4: SIB4 for NR Cell 1, redCapAccessAllowed-r17 is not present within interfreqcarrierfreqlist-v1700 IE but current TTCN implementation has omitted both IEs which is incorrect because as per TS 38.331, clause 5.2.2.4.5, UE would meet a condition(3) if interfreqcarrierfreqlist-v1700 is also absent, and thus UE would select the first frequency band in the frequencyBandList defeating the test purpose here. i.e., [TS 38.331, clause 5.2.2.4.5] Upon receiving SIB4 the UE shall: 1> if in RRC_IDLE, or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running: 2> for each entry in the interFreqCarrierFreqList: 3> if the UE is not a RedCap UE; or 3> if the UE is a RedCap UE and the interFreqCarrierFreqList-v1700 is absent; or 3> if the UE is a RedCap UE and redCapAccessAllowed is present in interFreqCarrierFreqList-v1700: 4> select the first frequency band in the frequencyBandList, and frequencyBandListSUL, if present, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in NR-NS-PmaxList, if present: 2. SIB4 template interFreqCarrierFreqList has two elements but second element is not being updated accordingly. 3. Tempate cs_InterFreqCarrierFreqInfo_v1700_Def input parameter needs to modify to control the presence/absence of redCapAccessAllowed_r17 IE within InterFreqCarrierFreqInfo_v1700 IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Corrected the TTCN implementation in else case just to omit redCapAccessAllowed-r17 but enable the root element interfreqcarrierfreqlist-v1700 presence by using the updated template implementation of cs_InterFreqCarrierFreqInfo_v1700_Def. 2. Added second element entry as well for both if and else cases. 3. Updated p_RedCapAccessAllowed with default value as omit inorder to use it to control the presence/absence of redCapAccessAllowed_r17 IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.27.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	5

2.1	Function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed()	5

2.2	Tempate cs_InterFreqCarrierFreqInfo_v1700_Def	7

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed()

| Function name | function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed |
| --- | --- |
| Reason for change | 1. As per Table 6.1.2.27.3.3-4: SIB4 for NR Cell 1, redCapAccessAllowed-r17 is not present within interfreqcarrierfreqlist-v1700 IE but current TTCN implementation has omitted both IEs which is incorrect because as per TS 38.331, clause 5.2.2.4.5, UE would meet a condition(3) if interfreqcarrierfreqlist-v1700 is also absent, and thus UE would select the first frequency band in the frequencyBandList defeating the test purpose here. i.e., [TS 38.331, clause 5.2.2.4.5] Upon receiving SIB4 the UE shall: 1> if in RRC_IDLE, or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running: 2> for each entry in the interFreqCarrierFreqList: 3> if the UE is not a RedCap UE; or 3> if the UE is a RedCap UE and the interFreqCarrierFreqList-v1700 is absent; or 3> if the UE is a RedCap UE and redCapAccessAllowed is present in interFreqCarrierFreqList-v1700: 4> select the first frequency band in the frequencyBandList, and frequencyBandListSUL, if present, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in NR-NS-PmaxList, if present: 2. SIB4 template interFreqCarrierFreqList has two elements but second element is not being updated accordingly. |
| Summary of change | 1. Corrected the TTCN implementation in else case just to omit redCapAccessAllowed-r17 but enable the root element interfreqcarrierfreqlist-v1700 presence by using the updated template implementation of cs_InterFreqCarrierFreqInfo_v1700_Def. 2. Added second element entry as well for both if and else cases. |
| TTCN module | 6_1_2\Idle_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed(NR_CellId_Type p_NR_CellId, boolean p_RedCapAccessAllowed := true) runs on NR_BASE_PTC { // @sic R5s230293 sic@ var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var integer v_SIB4_SI_Index := f_NR_GetSIB4_SI_Index(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination); if (p_RedCapAccessAllowed) {//@sic R5-235781 sic@ v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[0] := valueof(cs_InterFreqCarrierFreqInfo_v1700_Def(true_)); } else {//@sic R5-235781 sic@ v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700 := omit; } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change:

function f_NR5GC_CellInfo_SetSIB4_RedCap_AccessAllowed(NR_CellId_Type p_NR_CellId, boolean p_RedCapAccessAllowed := true) runs on NR_BASE_PTC { // @sic R5s230293 sic@ var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var integer v_SIB4_SI_Index := f_NR_GetSIB4_SI_Index(v_NR_CellInfo.Sysinfo.NR_SysinfoCombination); if (p_RedCapAccessAllowed) {//@sic R5-235781 sic@ v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[0] := valueof(cs_InterFreqCarrierFreqInfo_v1700_Def(true_)); v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[1] := valueof(cs_InterFreqCarrierFreqInfo_v1700_Def(true_)); //WA=WA#1208869 } else {//@sic R5-235781 sic@ v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[0] := valueof(cs_InterFreqCarrierFreqInfo_v1700_Def); //WA=WA#1208869 v_NR_CellInfo.Sysinfo.BcchInfo.SIs[v_SIB4_SI_Index].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib4.interFreqCarrierFreqList_v1700[1] := valueof(cs_InterFreqCarrierFreqInfo_v1700_Def); //WA=WA#1208869 } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

## Tempate cs_InterFreqCarrierFreqInfo_v1700_Def

| Function name | Tempate cs_InterFreqCarrierFreqInfo_v1700_Def |
| --- | --- |
| Reason for change | 1. Tempate cs_InterFreqCarrierFreqInfo_v1700_Def input parameter needs to modify to control the presence/absence of redCapAccessAllowed_r17 IE within InterFreqCarrierFreqInfo_v1700 IE. |
| Summary of change | 1. Updated p_RedCapAccessAllowed with default value as omit inorder to use it to control the presence/absence of redCapAccessAllowed_r17 IE. |
| TTCN module | ..\Common\NR\NR_SysInfo_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change:

template(value) InterFreqCarrierFreqInfo_v1700 cs_InterFreqCarrierFreqInfo_v1700_Def(InterFreqCarrierFreqInfo_v1700.redCapAccessAllowed_r17 p_RedCapAccessAllowed):= { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ interFreqNeighHSDN_CellList_r17 := omit, highSpeedMeasInterFreq_r17 := omit, redCapAccessAllowed_r17 := p_RedCapAccessAllowed, ssb_PositionQCL_Common_r17 := omit, interFreqNeighCellList_v1710 := omit }

After Change:

template(value) InterFreqCarrierFreqInfo_v1700 cs_InterFreqCarrierFreqInfo_v1700_Def( template(omit) InterFreqCarrierFreqInfo_v1700.redCapAccessAllowed_r17 p_RedCapAccessAllowed := omit):= //WA=WA#1208869 { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ interFreqNeighHSDN_CellList_r17 := omit, highSpeedMeasInterFreq_r17 := omit, redCapAccessAllowed_r17 := p_RedCapAccessAllowed, ssb_PositionQCL_Common_r17 := omit, interFreqNeighCellList_v1710 := omit }