3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230865

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3271 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, the AT command to trigger PDU Session X is AT+CGDCONT=1,"IPV4V6","ABCC",,,,,,,,,,,,,,0,"",0, where cid = 1. After this at Step 0A, there is another trigger to establish PDU Session ID Y, which uses AT command AT+CGDCONT=2,"IPV4V6","ABCC",,,,,,,,,,,,,,0,"",0, where cid =2. After this at Step 1 the PDU Session X is released. The TTCN implementation at Step 16 then uses f_NR5GC_MobileInfo_GetNoOfSessions()+1 to deduce the cid. Since there is only 1 PDU Session (Y) is active, this is calculated as 2. This sets the cid = 2 for Step 16. This is where, some UEs fail the testcase. Issue here is that some UE implementations associate the cid with a PDU Session ID, and some don’t. A device that associates the cid with PDU session ID does not trigger a PDU session establishment at Step 16 because for this device cid =2 is already used and is associated with PDU Session Y which is still active. This needs to be addressed. The solution here is to not re-use cid. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Get PDU session number before f_NR5GC_PDUSessionRelease in step1. This will result in cid = 3 at Step 16 which is not used before, thus all UE implementations should be fine. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_10_1_4_1_Body() |
| --- | --- |
| Reason for change | In the current TTCN implementation, the AT command to trigger PDU Session X is AT+CGDCONT=1,"IPV4V6","ABCC",,,,,,,,,,,,,,0,"",0, where cid = 1. After this at Step 0A, there is another trigger to establish PDU Session ID Y, which uses AT command AT+CGDCONT=2,"IPV4V6","ABCC",,,,,,,,,,,,,,0,"",0, where cid =2. After this at Step 1 the PDU Session X is released. The TTCN implementation at Step 16 then uses f_NR5GC_MobileInfo_GetNoOfSessions()+1 to deduce the cid. Since there is only 1 PDU Session (Y) is active, this is calculated as 2. This sets the cid = 2 for Step 16. This is where, some UEs fail the testcase. Issue here is that some UE implementations associate the cid with a PDU Session ID, and some don’t. A device that associates the cid with PDU session ID does not trigger a PDU session establishment at Step 16 because for this device cid =2 is already used and is associated with PDU Session Y which is still active. This needs to be addressed. The solution here is to not re-use cid. |
| Summary of change | Get PDU session number before f_NR5GC_PDUSessionRelease in step1. This will result in cid = 3 at Step 16 which is not used before, thus all UE implementations should be fine. |
| TTCN module | NR5GC\10_1\GSM_NR5GC.ttcn |
| MCC160 Comment | Accepted. See MCC160 implementation. |

Before Change:

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToRelease; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (omit) DNN v_RequestedDNN; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_Count := 0; <Code Skipped> // @sic R5-206357 sic@ //@siclog "Steps 1" siclog@ // The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed withThe SS transmits an PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release PDU session X. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s230497 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause "Reactivation requested" //@siclog "Steps 2 - 15 void" siclog@ //@siclog "Steps 16" siclog@ //@sic R5-202664 sic@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2). t_Wait.start (1.0); alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; } []t_Wait.timeout { // @sic R5-213490 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, v_DNN, -, -, v_NSSAI); //AT command to trigger a PDU session @sic R5s201230 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } }

After Change:

| function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToRelease; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (omit) DNN v_RequestedDNN; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_Count := 0; var integer v_PduSessionNumber := 0; <Code Skipped> // @sic R5-206357 sic@ //@siclog "Steps 1" siclog@ // The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed withThe SS transmits an PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release PDU session X. v_PduSessionNumber := f_NR5GC_MobileInfo_GetNoOfSessions(); f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s230497 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause "Reactivation requested" //@siclog "Steps 2 - 15 void" siclog@ //@siclog "Steps 16" siclog@ //@sic R5-202664 sic@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2). t_Wait.start (1.0); alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; } []t_Wait.timeout { // @sic R5-213490 sic@ f_UT_RequestPDUSession(UT, v_PduSessionNumber+1, v_DNN, -, -, v_NSSAI); //AT command to trigger a PDU session @sic R5s201230 sic@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } } |  |
| --- | --- |

## MCC160 Implementation

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ?

var O1_Type v_PDUSessionIdToRelease;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

var template (omit) DNN v_RequestedDNN;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_Count := 0;

var float v_T3580Val := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 16.0);

var charstring v_NSSAI := ""; // @sic R5-213490 sic@

var charstring v_DNN := ""; // @sic R5-213490 sic@

var template (omit)S_NSSAI_Type v_RequestedNSSAI; // @sic R5s210148 sic@

var integer v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5s230865 sic@

timer t_Wait;

f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_NoOfSessions +1, -, tsc_RandomAPN);// @sic R5-217818  R5s230865 sic@

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); // @sic R5-217818 sic@

v_PDUSessionIdToRelease := v_PDUSessionInfoList[v_NoOfSessions -1].SessionId; // @sic R5-217818 R5s230865 sic@

//v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease);

v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease);

// @sic R5-213490 sic@

if (isvalue(v_RequestedNSSAI)){ // compiler warning may be ignored due to check

// @sic R5s201028 sic@

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI.lvPart); // @sic R5s221179 Baseline Moving sic@

}

if (isvalue(v_RequestedDNN)){

// @sic R5s210437 sic@

v_DNN := oct2char(valueof(substr((v_RequestedDNN.dnnValue),1,lengthof(v_RequestedDNN.dnnValue)-1)));

}

// @sic R5-206357 sic@

//@siclog "Steps 1" siclog@

// The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed withThe SS transmits an PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release PDU session X.

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA,  // @sic R5s230497 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause "Reactivation requested"

//@siclog "Steps 2 - 15 void" siclog@

//@siclog "Steps 16" siclog@

//@sic R5-202664 sic@

//The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2).

t_Wait.start (1.0);

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

t_Wait.stop;

}

[]t_Wait.timeout

{

// @sic R5-213490 sic@

f_UT_RequestPDUSession(UT, v_NoOfSessions +2, v_DNN, -, -, v_NSSAI); // @sic R5s230865 sic@ //AT command to trigger a PDU session @sic R5s201230 sic@

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

}

}

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

….

}