3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230867

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0939 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS test case 7.24a and 7.24b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A conformance UE may fail the test case in Step-23A due to a mismatch in the Contact header of the SIP Update Request. According to TS 34.229-1, clause A.2.5 the Contact header should be the same as 183 Session Progress for the same dialog sent by SS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the parameter in the function f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7() to pass px_IMS_CalleeContactUri2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.24a.NR5GC, 7.24b.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	moiht.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | function f_TC_7_24a_NR5GC_IMS1() |
| --- | --- |
| Reason for change | A conformance UE may fail the test case in Step-23A due to a mismatch in the Contact header of the SIP Update Request. According to TS 34.229-1, clause A.2.5 the Contact header should be the same as 183 Session Progress for the same dialog sent by SS. |
| Summary of change | Modified the parameter in the function f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7() to pass px_IMS_CalleeContactUri2. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted in principle: The implementation of f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7 is somehow inconsistent as on one hand in f_IMS_MOCallSetup_ReceivePRACK_Send200OK the contact URI is taken from p_MessageHeader183 on the other hand in f_IMS_MOCallSetup_ReceiveUpdateSend200OK the handed over p_CalleeContactUri is used as contact URI of the SS. contact URI shall always be taken from p_MessageHeader183 and there is no need for p_CalleeContactUri in f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7. (see MCC160 implementation below) NOTE 1: There is a similar issue with f_TC_7_26_NR5GC_IMS1 which should also be fixed by changing f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7 as described above NOTE 2: A similar implementation is done for 4G already (f_IMS_MOCallSetup_AnnexC21C25_Steps5_8) |

Before Change:

function f_TC_7_24a_NR5GC_IMS1() runs on IMS_PTC { <<< Skipped >>> // @siclog "Step 21" siclog@ 183 Session progress (dialog 2) v_MessageHeader := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri2); v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2, 65); /* @sic R5-226704: EVS_A2 sic@ */ IMS_Server.send(cas_IMS_DATA_RSP(v_InviteRequestWithSdp.RoutingInfo, cs_Response(c_statusLine183, v_MessageHeader, cs_MessageBody_SDP(v_SdpAnswer)))); // @siclog "Step 22 - 23B" siclog@ v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2); /* Updated offer for 2nd remote UE @sic R5-226704: EVS_A2 sic@ */ f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7(IPCAN_MO_SpeechCall, v_InviteRequestWithSdp, v_MessageHeader, -, v_SdpOffer, -, v_RadioBearersEstablished); /* @sic R5s220385 Change 1.1 + R5-222101 sic@ @sic R5-231508/RP-230602: optional UPDATE / 200 OK sic@ */ // @siclog "Step 24 - 26" siclog@ f_IMS_MOCallSetup_Send180_Reliable(v_InviteRequestWithSdp, px_IMS_CalleeContactUri2); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 25"); <<< Skipped >>> }

After Change:

function f_TC_7_24a_NR5GC_IMS1() runs on IMS_PTC { <<< Skipped >>> // @siclog "Step 21" siclog@ 183 Session progress (dialog 2) v_MessageHeader := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri2); v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2, 65); /* @sic R5-226704: EVS_A2 sic@ */ IMS_Server.send(cas_IMS_DATA_RSP(v_InviteRequestWithSdp.RoutingInfo, cs_Response(c_statusLine183, v_MessageHeader, cs_MessageBody_SDP(v_SdpAnswer)))); // @siclog "Step 22 - 23B" siclog@ v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2); /* Updated offer for 2nd remote UE @sic R5-226704: EVS_A2 sic@ */ f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7(IPCAN_MO_SpeechCall, v_InviteRequestWithSdp, v_MessageHeader, px_IMS_CalleeContactUri2, v_SdpOffer, -, v_RadioBearersEstablished); /* @sic R5s220385 Change 1.1 + R5-222101 sic@ @sic R5-231508/RP-230602: optional UPDATE / 200 OK sic@ */ // @siclog "Step 24 - 26" siclog@ f_IMS_MOCallSetup_Send180_Reliable(v_InviteRequestWithSdp, px_IMS_CalleeContactUri2); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 25"); <<< Skipped >>> }

## Change 2

| Function name | function f_TC_7_24b_NR5GC_IMS1() |
| --- | --- |
| Reason for change | A conformance UE may fail the test case in Step-23A due to a mismatch in the Contact header of the SIP Update Request. According to TS 34.229-1, clause A.2.5 the Contact header should be the same as 183 Session Progress for the same dialog sent by SS. |
| Summary of change | Modified the parameter in the function f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7() to pass px_IMS_CalleeContactUri2. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | (see change 1) |

Before Change:

function f_TC_7_24b_NR5GC_IMS1() runs on IMS_PTC { <<< Skipped >>> // @siclog "Step 21" siclog@ 183 Session progress (dialog 2) v_MessageHeader := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri2); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_MessageHeader); v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2, 65); // @sic R5-225929: EVS_A2 sic@ IMS_Server.send(cas_IMS_DATA_RSP(v_InviteRequestWithSdp.RoutingInfo, cs_Response(c_statusLine183, v_MessageHeader, cs_MessageBody_SDP(v_SdpAnswer)))); // @siclog "Step 22 - 23B siclog@ v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2); // @sic R5-225929: EVS_A2 sic@ f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7(IPCAN_MO_SpeechCall, v_InviteRequestWithSdp, v_MessageHeader, -, v_SdpOffer, -, v_RadioBearersEstablished); /* @sic R5s220385 Change 1.1 + R5-222101 sic@ @sic R5-231508/RP-230602: optional UPDATE / 200 OK sic@ */ // @siclog "Step 24 - 26" siclog@ f_IMS_MOCallSetup_Send180_Reliable(v_InviteRequestWithSdp, px_IMS_CalleeContactUri2); <<< Skipped >>> }

After Change:

function f_TC_7_24b_NR5GC_IMS1() runs on IMS_PTC { <<< Skipped >>> // @siclog "Step 21" siclog@ 183 Session progress (dialog 2) v_MessageHeader := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri2); v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_MessageHeader); v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2, 65); // @sic R5-225929: EVS_A2 sic@ IMS_Server.send(cas_IMS_DATA_RSP(v_InviteRequestWithSdp.RoutingInfo, cs_Response(c_statusLine183, v_MessageHeader, cs_MessageBody_SDP(v_SdpAnswer)))); // @siclog "Step 22 - 23B siclog@ v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, EVS_A2); // @sic R5-225929: EVS_A2 sic@ f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7(IPCAN_MO_SpeechCall, v_InviteRequestWithSdp, v_MessageHeader, px_IMS_CalleeContactUri2, v_SdpOffer, -, v_RadioBearersEstablished); /* @sic R5s220385 Change 1.1 + R5-222101 sic@ @sic R5-231508/RP-230602: optional UPDATE / 200 OK sic@ */ // @siclog "Step 24 - 26" siclog@ f_IMS_MOCallSetup_Send180_Reliable(v_InviteRequestWithSdp, px_IMS_CalleeContactUri2); <<< Skipped >>> }

MCC160 Implementation

function f_IMS_MOCallSetup_NR5GC_WithPreconditions_Step4_7(IMS_TestProcedure_Type p_TestProcedure, IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, template (value) MessageHeader p_MessageHeader183, template (present) SDP_Message p_UpdatedSdpOffer, IMS_CallType_Type p_CallType := NORMAL_CALL, boolean p_RadioBearersEstablished := false) runs on IMS_PTC return SDP_Message { var template SDP_Message v_SdpOfferPRACK := omit; var template ContentType v_ContentTypePRACK := omit; var template (omit) SDP_Message v_LatestSdpOffer := omit; var SipUrl v_ContactUrl; var charstring v_ContactUrlStr; // step 4 - 5 PRACK, 200 OK if (not p_RadioBearersEstablished) { f_IMS_MOCallSetup_ReceivePRACK_Send200OK(p_InviteRequestWithSdp, p_MessageHeader183, -, -, p_CallType); f_IMS_CC_TriggerDedicatedBearerActivation(p_TestProcedure); // to trigger step 13 of 38.508-1 Table 4.9.15.2.2-1 } else { v_ContentTypePRACK := cr_ContentTypeSDP ifpresent; v_SdpOfferPRACK := p_UpdatedSdpOffer ifpresent; v_LatestSdpOffer := f_IMS_MOCallSetup_ReceivePRACK_Send200OK(p_InviteRequestWithSdp, p_MessageHeader183, v_ContentTypePRACK, v_SdpOfferPRACK, p_CallType); } if (not isvalue(v_LatestSdpOffer)) { // step 6 - 7 UPDATE, 200 OK v_ContactUrl := f_MessageHeader_GetContactSipUrl(p_MessageHeader183); v_ContactUrlStr := f_SIP_SipUrl2Str(v_ContactUrl); v_LatestSdpOffer := f_IMS_MOCallSetup_ReceiveUpdateSend200OK(p_InviteRequestWithSdp.Invite, v_ContactUrlStr, p_UpdatedSdpOffer, -, p_CallType); } return valueof(v_LatestSdpOffer); }