3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230871

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3273 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_GetHighestBandwidthFrequency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Recap n79 the current TTCN implementation is using the incorrect values for the frequency definition (I.e it is using values from 4.3.1.1.1.79-1, n79 SCS 15KHz, where as it should refer to 4.3.1.1.1.79-2 n79 SCS 30KHz. See Table (Table 6.2.3.1-5A: Test frequencies for NR TDD FR1 bands using 20 MHz channel bandwidth for RedCap UE) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction highest frequency for N79 of RedCap parameters according Table 4.3.1.1.1.79-2 n79 SCS 30KHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.1.1.ENDC, 7.1.1.4.1.NR5GC, 7.1.1.4.2.1.ENDC, 7.1.1.4.2.1.NR5GC, 7.1.1.4.2.2.ENDC, 7.1.1.4.2.2.NR5GC,7.1.1.4.2.3.ENDC, 7.1.1.4.2.3.NR5GC,7.1.1.4.2.4.ENDC, 7.1.1.4.2.4.NR5GC,7.1.1.4.2.5.ENDC, 7.1.1.4.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | function f_NR_GetHighestBandwidthFrequency() |
| --- | --- |
| Reason for change | For Recap n79 the current TTCN implementation is using the incorrect values for the frequency definition (I.e it is using values from 4.3.1.1.1.79-1, n79 SCS 15KHz, where as it should refer to 4.3.1.1.1.79-2 n79 SCS 30KHz. See Table (Table 6.2.3.1-5A: Test frequencies for NR TDD FR1 bands using 20 MHz channel bandwidth for RedCap UE) |
| Summary of change | Correction highest frequency for N79 of RedCap parameters according Table 4.3.1.1.1.79-2 n79 SCS 30KHz |
| TTCN module | \Common\NR_CommonFrequency\NR_FrequencyHighestBandwidth.ttcn |
| MCC160 Comment | Accepted Additional changes are needed for bands n51, n94 and n101 – see below |

Before Change:

function f_NR_GetHighestBandwidthFrequency ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) { var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(); select ( p_FreqBandIndicatorNR ) { case( 79 ) { //@sic R5-227163, R5-227166, R5-227168, R5-227169 sic@ if ( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 693792 , 79 , 693364 , 0 , kHz30 , 106 ), 20 , kHz30 , 8 , 0 , 15 , 11 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 106 , v_P_Max), 20 );//WA#WI=1211199(7114xx RedCap) } else { //@sic R5-201024 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 694176 , 79 , 693392 , 0 , kHz30 , 273 ), 100 , kHz30 , 16 , 1 , 44 , 18 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 , v_P_Max), 100 ); } } ……. };

After Change:

| function f_NR_GetHighestBandwidthFrequency ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) { var template(value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(); select ( p_FreqBandIndicatorNR ) { case( 79 ) { //@sic R5-227163, R5-227166, R5-227168, R5-227169 sic@ if ( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 693696 , 79 , 693388 , 0 , kHz30 , 51 ), 20 , kHz30 , 20 , 0 , 4 , 2 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 51 , v_P_Max), 20 ); } else { //@sic R5-201024 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 694176 , 79 , 693392 , 0 , kHz30 , 273 ), 100 , kHz30 , 16 , 1 , 44 , 18 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 , v_P_Max), 100 ); } } ……. }; |  |
| --- | --- |

## MCC160 additional changes

function f_NR_GetHighestBandwidthFrequency ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return template (value) NR_Frequencyf1tof4_Type

{

var template (value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy;

var P_Max v_P_Max := f_NR_InitP_Max();

select ( p_FreqBandIndicatorNR ) {

case( 1 ) {

//@sic R5-227163 R5-227166 R5-227168 R5-227169 sic@

if ( pc_supportOfRedCap_r17 ){

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 422650 , 1 , 422092 , 0 , kHz15 , 106 ), 20 , kHz15 , 6 , 2 , 5 , 1 , 4 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 1 }, 384092 , 0 , kHz15 , 106 , v_P_Max), 20 );

} else {

//@sic R5-197968 R5-200499 R5-201961 R5-203998 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 422650 , 1 , 422140 , 0 , kHz15 , 270 ), 50 , kHz15 , 2 , 2 , 4 , 0 , 4 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 1 }, 384140 , 0 , kHz15 , 270 , v_P_Max), 50 );

}

}

…

case( 51 ) {

//@sic R5-190642 R5-197984 R5-200808 R5-201977 R5s230871 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 285870 , 51 , 285450 , 0 , kHz15 , 25 ), 5 , kHz15 , 8 , 0 , 1 , 1 , 0 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz15 , 25 , v_P_Max), 5 );

}

…

case( 94 ) {

//@sic R5-225953  R5s230871 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 287290 , 94 , 286564 , 0 , kHz30 , 51 ), 20 , kHz15 , 2 , 0 , 10 , 0 , 5 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 94 }, 176164 , 0 , kHz30 , 51 , v_P_Max), 20 );

}

case( 100 ) {

//@sic R5-230066 R5-230067 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 184330 , 100 , 183930 , 0 , kHz15 , 25 ), 5 , kHz15 , 0 , 0 , 0 , 0 , 0 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL({ 100 }, 174930 , 0 , kHz15 , 25 , v_P_Max), 5 );

}

case( 101 ) {

//@sic R5-230066 R5-230067 R5s230871 sic@

v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 380430 , 101 , 380064 , 0 , kHz15 , 52 ), 10 , kHz15 , 2 , 2 , 5 , 1 , 4 );

v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz15 , 52 , v_P_Max), 10 );

}

…

case else { FatalError (__FILE__, __LINE__, "Invalid NR-SA band combination") }

}

return v_NR_Frequencyf1tof4;

}