3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230881

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3280 | rev | - | Current version: | 17.8.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to template cr_TAU_Request_N1ToS1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint,Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS24.301 9.9.3.53 UE additional security capability: The UE additional security capability information element is used by the UE to indicate which additional security algorithms are supported by the UE for S1 mode in dual connectivity with NR or for N1 mode or both. In TC11.4.12, N1 mode is disabled, if redcap UE do not support ENDC, UE may not report additionalSecurityCap in TAU request message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add if present for ueAddSecurityCap |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

- Corrections required

  - Change 1

| Function name | cr_TAU_Request_N1ToS1 |
| --- | --- |
| Reason for change | According to TS24.301 9.9.3.53 UE additional security capability: The UE additional security capability information element is used by the UE to indicate which additional security algorithms are supported by the UE for S1 mode in dual connectivity with NR or for N1 mode or both. In TC11.4.12, N1 mode is disabled, if redcap UE do not support ENDC, UE may not report additionalSecurityCap in TAU request message. |
| Summary of change | Add if present for ueAddSecurityCap |
| TTCN module | EUTRA38_NR5GCProcedures.ttcn |
| MCC160 Comment | Accepted in principle and implemented as per R5-241528 agreement at RAN5#102, please see sub-clauses 2.2-2.4 |

Before Change:

template (present) NAS_UL_Message_Type cr_TAU_Request_N1ToS1(template (present) EPS_UpdateTypeValue p_EpsUpdate_TypeValue, template (present) NAS_KsiValue p_KsiValue, template DRXparameter p_DRXparameter, template AdditionalUpdateType p_AdditionalUpdateType, template (present) MobileIdentity p_MobileId := cr_MobileIdAny, template TrackingAreaId p_TAI := cr_TAI_Any ifpresent, template (present) B1_Type p_ActiveFlag := ?, // @sic R5-202667, R5-211371 sic@ B1_Type p_TSC := tsc_NasKsi_NativeSecurityContext, template NAS_KeySetIdentifier p_NonCurrentKSI := omit, template MobileIdentity p_AdditionalGUTI := cr_MobileIdAny ('50'O) ifpresent, // @sic R5-202563 sic@ template UERadioCapInfoUpdateNeeded p_UeRadioCap := cr_UeRadioCap, template EPS_BearerContextStatus p_EPS_BearerContextStatus := cr_EPS_BearerContextStatus_Any, template (present) UE_NetworkCap p_UENWCap := cdr_NR5GC_UENetworkCap('58'O)) := { /* @status APPROVED (NR5GC_IRAT) */ tRACKING_AREA_UPDATE_REQUEST := { securityHeaderType := tsc_SHT_NoSecurityProtection, protocolDiscriminator := tsc_PD_EMM, messageType := tsc_MT_TauRequest, asmeNasKeySetId := cr_NAS_KeySetIdentifier(p_KsiValue, p_TSC), // @sic R5s200509 sic@ epsUpdateType := cr_EPS_UpdateType(p_ActiveFlag, p_EpsUpdate_TypeValue), // @sic R5-202667 sic@ oldGuti := p_MobileId, nonCurrentNativeNasKeySetId := p_NonCurrentKSI, /* @sic R5s100135, R5s200509 sic@ */ gprsCipheringKeySeqNum := cr_CiphKeySeqNum('8'H, ?) ifpresent, /* @sic R5s100135, R5-135073 sic@ */ oldPtmsiSignature := omit, additionalGuti := p_AdditionalGUTI, // @sic R5-201215 sic@ nonce := omit, ueNetworkCapability := p_UENWCap, // @sic R5s211353 sic@ lastVisitedRegisteredTai := p_TAI, drxParameter := p_DRXparameter, // @sic R5s100660, R5s120210 sic@ ueRadioCapNeeded := p_UeRadioCap, // @sic R5s200583 sic@ epsBearerContextStatus := p_EPS_BearerContextStatus, // @sic R5s200760 sic@ msNetworkCapability := cr_MS_NetworkCap_Any ifpresent, oldLai := cr_LAI_Any ifpresent, tmsiStatus := cr_TMSI_Status ifpresent, msClassmark2 := cr_MS_Clsmk2_Any_tlv ifpresent, // @sic R5s100662 sic@ msClassmark3 := cr_MSCLSMK3_Any ifpresent, supportedCodecList := cr_CodecListAny ifpresent, addUpdateType := p_AdditionalUpdateType, /* @sic R5-103681 sic@ */ voiceDomainPref := cr_VoiceDomainPref_Any ifpresent, /* @sic R5s110176 Baseline Moving sic@ */ oldGUTI_Type := cr_GUTI_Type_Native, deviceProperties := cr_DeviceProperties('D'H) ifpresent, /* cl. 9.9.2.0A O TV 1 IEI=0xD- @sic R5s120178 Baseline Moving sic@*/ msNetworkFeatureSupport := cr_MS_NetworkFeatureSupport('C'H) ifpresent, /* cl. 9.9.2.0A O TV 1 IEI=0xC- @sic R5s120178 Baseline Moving sic@*/ tmsiBasedNRIContainer := cr_NwkResourceIdAny ifpresent, // @sic R5s130195 Baseline Moving sic@ t3324Value := cr_GPRS_Timer2_3('6A'O) ifpresent, // @sic R5s150329 Baseline Moving sic@ t3412ExtdValue := cr_GPRS_Timer2_3 ('5E'O) ifpresent, // @sic R5s150329 Baseline Moving sic@ extendedDRXParams := cr_ExtdDRXParamsAny ifpresent, // @sic R5s160711 Baseline Moving sic@ ueAddSecurityCap := cr_UE_AddSecurityCap_Any , ueStatus := cr_UEStatus_N1, addInfoReq := cr_AdditionalInfoReqAny ifpresent, /* cl. 9.9.3.55 O TLV 2 IEI=0x17 @sic R5s180552 Baseline Moving 2018 Phase 2 sic@ */ n1UENetworkCap := cr_N1_UENetworkCapAny ifpresent, /* cl. 9.9.3.57 O TLV 3-15 IEI=0x32 @sic R5s201386 Baseline Moving sic@ */ ueRadioCapIdAvailability := cr_UERadioCapIdAvailabilityAny ifpresent, /* cl. 9.9.3.58 O TV 3 IEI=0x34 @sic R5s201386 Baseline Moving sic@ */ requestedWUSAssistInfo := cr_WUSAssistInfoAny ifpresent, /* cl. 9.9.3.62 O TLV 3-n IEI=0x35 @sic R5s201386 Baseline Moving sic@ */ drxInNB_S1 := cr_DRXParamCommonAny ('36'O) ifpresent, /* cl. 9.9.3.63 O TLV 3 IEI=0x36 @sic R5s201386 Baseline Moving sic@ */ requestedIMSI_Offset := cr_IMSI_Offset ifpresent, /* cl. 9.9.3.64 O TLV 4 IEI=0x38 Sep22 @sic R5s221178 Baseline Moving sic@ */ ueRequestType := cr_UE_RequestType ifpresent, /* cl. 9.9.3.65 O TLV 3 IEI=0x29 Sep22 @sic R5s221178 Baseline Moving sic@ */ pagingRestriction := cr_PagingRestriction ifpresent /* cl. 9.9.3.66 O TLV 3-5 IEI=0x28 Sep22 @sic R5s221178 Baseline Moving sic@ */ } };

After Change:

template (present) NAS_UL_Message_Type cr_TAU_Request_N1ToS1(template (present) EPS_UpdateTypeValue p_EpsUpdate_TypeValue, template (present) NAS_KsiValue p_KsiValue, template DRXparameter p_DRXparameter, template AdditionalUpdateType p_AdditionalUpdateType, template (present) MobileIdentity p_MobileId := cr_MobileIdAny, template TrackingAreaId p_TAI := cr_TAI_Any ifpresent, template (present) B1_Type p_ActiveFlag := ?, // @sic R5-202667, R5-211371 sic@ B1_Type p_TSC := tsc_NasKsi_NativeSecurityContext, template NAS_KeySetIdentifier p_NonCurrentKSI := omit, template MobileIdentity p_AdditionalGUTI := cr_MobileIdAny ('50'O) ifpresent, // @sic R5-202563 sic@ template UERadioCapInfoUpdateNeeded p_UeRadioCap := cr_UeRadioCap, template EPS_BearerContextStatus p_EPS_BearerContextStatus := cr_EPS_BearerContextStatus_Any, template (present) UE_NetworkCap p_UENWCap := cdr_NR5GC_UENetworkCap('58'O)) := { /* @status APPROVED (NR5GC_IRAT) */ tRACKING_AREA_UPDATE_REQUEST := { securityHeaderType := tsc_SHT_NoSecurityProtection, protocolDiscriminator := tsc_PD_EMM, messageType := tsc_MT_TauRequest, asmeNasKeySetId := cr_NAS_KeySetIdentifier(p_KsiValue, p_TSC), // @sic R5s200509 sic@ epsUpdateType := cr_EPS_UpdateType(p_ActiveFlag, p_EpsUpdate_TypeValue), // @sic R5-202667 sic@ oldGuti := p_MobileId, nonCurrentNativeNasKeySetId := p_NonCurrentKSI, /* @sic R5s100135, R5s200509 sic@ */ gprsCipheringKeySeqNum := cr_CiphKeySeqNum('8'H, ?) ifpresent, /* @sic R5s100135, R5-135073 sic@ */ oldPtmsiSignature := omit, additionalGuti := p_AdditionalGUTI, // @sic R5-201215 sic@ nonce := omit, ueNetworkCapability := p_UENWCap, // @sic R5s211353 sic@ lastVisitedRegisteredTai := p_TAI, drxParameter := p_DRXparameter, // @sic R5s100660, R5s120210 sic@ ueRadioCapNeeded := p_UeRadioCap, // @sic R5s200583 sic@ epsBearerContextStatus := p_EPS_BearerContextStatus, // @sic R5s200760 sic@ msNetworkCapability := cr_MS_NetworkCap_Any ifpresent, oldLai := cr_LAI_Any ifpresent, tmsiStatus := cr_TMSI_Status ifpresent, msClassmark2 := cr_MS_Clsmk2_Any_tlv ifpresent, // @sic R5s100662 sic@ msClassmark3 := cr_MSCLSMK3_Any ifpresent, supportedCodecList := cr_CodecListAny ifpresent, addUpdateType := p_AdditionalUpdateType, /* @sic R5-103681 sic@ */ voiceDomainPref := cr_VoiceDomainPref_Any ifpresent, /* @sic R5s110176 Baseline Moving sic@ */ oldGUTI_Type := cr_GUTI_Type_Native, deviceProperties := cr_DeviceProperties('D'H) ifpresent, /* cl. 9.9.2.0A O TV 1 IEI=0xD- @sic R5s120178 Baseline Moving sic@*/ msNetworkFeatureSupport := cr_MS_NetworkFeatureSupport('C'H) ifpresent, /* cl. 9.9.2.0A O TV 1 IEI=0xC- @sic R5s120178 Baseline Moving sic@*/ tmsiBasedNRIContainer := cr_NwkResourceIdAny ifpresent, // @sic R5s130195 Baseline Moving sic@ t3324Value := cr_GPRS_Timer2_3('6A'O) ifpresent, // @sic R5s150329 Baseline Moving sic@ t3412ExtdValue := cr_GPRS_Timer2_3 ('5E'O) ifpresent, // @sic R5s150329 Baseline Moving sic@ extendedDRXParams := cr_ExtdDRXParamsAny ifpresent, // @sic R5s160711 Baseline Moving sic@ ueAddSecurityCap := cr_UE_AddSecurityCap_Any ifpresent, ueStatus := cr_UEStatus_N1, addInfoReq := cr_AdditionalInfoReqAny ifpresent, /* cl. 9.9.3.55 O TLV 2 IEI=0x17 @sic R5s180552 Baseline Moving 2018 Phase 2 sic@ */ n1UENetworkCap := cr_N1_UENetworkCapAny ifpresent, /* cl. 9.9.3.57 O TLV 3-15 IEI=0x32 @sic R5s201386 Baseline Moving sic@ */ ueRadioCapIdAvailability := cr_UERadioCapIdAvailabilityAny ifpresent, /* cl. 9.9.3.58 O TV 3 IEI=0x34 @sic R5s201386 Baseline Moving sic@ */ requestedWUSAssistInfo := cr_WUSAssistInfoAny ifpresent, /* cl. 9.9.3.62 O TLV 3-n IEI=0x35 @sic R5s201386 Baseline Moving sic@ */ drxInNB_S1 := cr_DRXParamCommonAny ('36'O) ifpresent, /* cl. 9.9.3.63 O TLV 3 IEI=0x36 @sic R5s201386 Baseline Moving sic@ */ requestedIMSI_Offset := cr_IMSI_Offset ifpresent, /* cl. 9.9.3.64 O TLV 4 IEI=0x38 Sep22 @sic R5s221178 Baseline Moving sic@ */ ueRequestType := cr_UE_RequestType ifpresent, /* cl. 9.9.3.65 O TLV 3 IEI=0x29 Sep22 @sic R5s221178 Baseline Moving sic@ */ pagingRestriction := cr_PagingRestriction ifpresent /* cl. 9.9.3.66 O TLV 3-5 IEI=0x28 Sep22 @sic R5s221178 Baseline Moving sic@ */ } };

  - Additional MCC160 implementation 1

| Function name | f_TC_11_4_12_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | - If some UE do not support ENDC, UE may not report additionalSecurityCap in TAU request message at step 4. |
| Summary of change | - UE additional security capability exsists depending on pc_EN_DC. |
| TTCN module | Emergency_IRAT_NR5GC_EUTRA.ttcn |

function fl_TC_11_4_12_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC

{

var NAS_MSG_Indication_Type v_NasInd;

var NasCount_Type v_NasCount;

var template UE_AdditionalSecurityCapability v_UEAddSecurityCap; // @sic R5s230881 R5-241528 sic@

//@siclog "Step 0" siclog@ //@sic R5-223438 sic@

//Set the power levels according to T0 as per Table 11.4.12.3.1-1/2

f_EUTRA38_SetCellPower(eutra_Cell1, -91, -91);

//@siclog "Step 1" siclog@

//Make the UE initiate an Emergency call.

f_UT_RequestIMSEmergencyCall(UT);

//@siclog "Step 2 - 4" siclog@

//Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'?

//SS transmits an RRCConnectionSetup message.

//Check: Does the UE transmit an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area.

//For some consequences related to the content of the TRACKING AREA UPDATE REQUEST message see the Notes in TS.36.508 [7], Table 4.5A.2.1-1.

if (pc_EN_DC == true ) { // @sic R5s230881 R5-241528 sic@

v_UEAddSecurityCap := cr_UE_AddSecurityCap_Any;

}

Else {

v_UEAddSecurityCap := omit;

}

v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_Cell1,

tsc_RRC_TI_Def,

-,

MAPPED_CXT,

-,

emergency,

-,

v_UEAddSecurityCap // @sic R5s230881 R5-241528 sic@

);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap_AnyproSeCap('58'O))){ // @sic R5s221058 sic@

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability indication is not correct at step 4");

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

};

v_NasCount := v_NasInd.SecurityProtection.NasCount;

//@siclog "Steps 5-8" siclog@

//Steps 5-8 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed.

f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1,

tsc_RRC_TI_Def,

v_NasCount);

f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1,

tsc_RRC_TI_Def);

//@siclog "Step 9" siclog@

//SS responds with TRACKING AREA UPDATE ACCEPT message.

//@siclog "Step 10" siclog@

//The UE transmits a TRACKING AREA UPDATE COMPLETE message.

f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_Cell1,

v_NasInd,

MAPPED_CXT,

noRrcConnectionRelease,

-, true,

-, true);

//@siclog "Steps 11-17" siclog@

//Steps 9-15 from the Generic Test Procedure for IMS Emergency call establishment in EUTRA: Normal Service as specified in TS 36.508 [7], Table 4.5A.4.3-1 are performed.

f_EUTRA38_MultiPDN_EstablishAdditionalPDNs(eutra_Cell1, 1, -, -, -, '100'B, omit, true); // @sic R5s211357 R5-231416 sic@

// @siclog "Step 19-20" siclog@

f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1, Emergency_PDN);

// Tell IMS PTC to release the call

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s221058 sic@

// Emergency call release in IMS

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "Call Released");

// The SS deactivates the EPS bearer. [This is the rest of C.32]

f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1,

int2hex(valueof(f_Get_EPSBearerIdForAPNType(Emergency_PDN)), 1)); // @sic R5s211357 R5s221058 sic@

}

  - Additional MCC160 implementation 2

| Function name | template cr_TAU_Request_N1ToS1 |
| --- | --- |
| Reason for change | - In TC 11.4.12,if some UE do not support ENDC, UE may not report additionalSecurityCap in TAU request message at step 4. |
| Summary of change | - Add UE additional security capability |
| TTCN module | EUTRA38_NR5GCProcedures.ttcn |

template (present) NAS_UL_Message_Type cr_TAU_Request_N1ToS1(template (present) EPS_UpdateTypeValue p_EpsUpdate_TypeValue,

template (present) NAS_KsiValue p_KsiValue,

template DRXparameter p_DRXparameter,

template AdditionalUpdateType p_AdditionalUpdateType,

template (present) MobileIdentity p_MobileId := cr_MobileIdAny,

template TrackingAreaId p_TAI := cr_TAI_Any ifpresent,

template (present) B1_Type p_ActiveFlag := ?,            // @sic R5-202667, R5-211371 sic@

B1_Type  p_TSC := tsc_NasKsi_NativeSecurityContext,

template NAS_KeySetIdentifier p_NonCurrentKSI := omit,

template MobileIdentity p_AdditionalGUTI := cr_MobileIdAny ('50'O) ifpresent, // @sic R5-202563 sic@

template UERadioCapInfoUpdateNeeded p_UeRadioCap := cr_UeRadioCap,

template EPS_BearerContextStatus p_EPS_BearerContextStatus := cr_EPS_BearerContextStatus_Any,

template (present) UE_NetworkCap p_UENWCap := cdr_NR5GC_UENetworkCap('58'O),

template UE_AdditionalSecurityCapability p_UE_AddSecurityCap := cr_UE_AddSecurityCap_Any // @sic R5s230881 R5-241528 sic@

) :=

{ /* @status    APPROVED (NR5GC_IRAT) */

tRACKING_AREA_UPDATE_REQUEST := {

securityHeaderType         := tsc_SHT_NoSecurityProtection,

protocolDiscriminator      := tsc_PD_EMM,

messageType                := tsc_MT_TauRequest,

asmeNasKeySetId            := cr_NAS_KeySetIdentifier(p_KsiValue, p_TSC),  // @sic R5s200509 sic@

epsUpdateType              := cr_EPS_UpdateType(p_ActiveFlag, p_EpsUpdate_TypeValue), // @sic R5-202667 sic@

oldGuti                    := p_MobileId,

nonCurrentNativeNasKeySetId  := p_NonCurrentKSI,   /* @sic R5s100135, R5s200509 sic@ */

gprsCipheringKeySeqNum     := cr_CiphKeySeqNum('8'H, ?) ifpresent,        /* @sic R5s100135, R5-135073 sic@ */

oldPtmsiSignature          := omit,

additionalGuti             := p_AdditionalGUTI,  // @sic R5-201215 sic@

nonce                      := omit,

ueNetworkCapability        := p_UENWCap,  // @sic R5s211353 sic@

lastVisitedRegisteredTai   := p_TAI,

drxParameter               := p_DRXparameter, // @sic R5s100660, R5s120210 sic@

ueRadioCapNeeded           := p_UeRadioCap, // @sic R5s200583 sic@

epsBearerContextStatus     := p_EPS_BearerContextStatus,  // @sic R5s200760 sic@

msNetworkCapability        := cr_MS_NetworkCap_Any ifpresent,

oldLai                     := cr_LAI_Any ifpresent,

tmsiStatus                 := cr_TMSI_Status ifpresent,

msClassmark2               := cr_MS_Clsmk2_Any_tlv ifpresent,  // @sic R5s100662 sic@

msClassmark3               := cr_MSCLSMK3_Any ifpresent,

supportedCodecList         := cr_CodecListAny ifpresent,

addUpdateType              := p_AdditionalUpdateType,         /* @sic R5-103681 sic@ */

voiceDomainPref            := cr_VoiceDomainPref_Any ifpresent, /* @sic R5s110176 Baseline Moving sic@ */

oldGUTI_Type               := cr_GUTI_Type_Native,

deviceProperties           := cr_DeviceProperties('D'H) ifpresent,  /* cl. 9.9.2.0A O TV 1 IEI=0xD- @sic R5s120178 Baseline Moving sic@*/

msNetworkFeatureSupport    := cr_MS_NetworkFeatureSupport('C'H) ifpresent,   /* cl. 9.9.2.0A O TV 1 IEI=0xC- @sic R5s120178 Baseline Moving sic@*/

tmsiBasedNRIContainer      := cr_NwkResourceIdAny ifpresent, // @sic R5s130195 Baseline Moving sic@

t3324Value                 := cr_GPRS_Timer2_3('6A'O) ifpresent, // @sic R5s150329 Baseline Moving sic@

t3412ExtdValue             := cr_GPRS_Timer2_3 ('5E'O) ifpresent, // @sic R5s150329 Baseline Moving sic@

extendedDRXParams          := cr_ExtdDRXParamsAny ifpresent,  // @sic R5s160711 Baseline Moving sic@

ueAddSecurityCap           := p_UE_AddSecurityCap,  // @sic R5s230881 R5-241528 sic@

ueStatus                   := cr_UEStatus_N1,

addInfoReq                 := cr_AdditionalInfoReqAny ifpresent,     /* cl. 9.9.3.55 O TLV 2    IEI=0x17 @sic R5s180552 Baseline Moving 2018 Phase 2 sic@ */

n1UENetworkCap             := cr_N1_UENetworkCapAny ifpresent,  /* cl. 9.9.3.57 O TLV 3-15 IEI=0x32 @sic R5s201386 Baseline Moving sic@ */

ueRadioCapIdAvailability   := cr_UERadioCapIdAvailabilityAny ifpresent,  /* cl. 9.9.3.58 O TV 3 IEI=0x34 @sic R5s201386 Baseline Moving sic@ */

requestedWUSAssistInfo     := cr_WUSAssistInfoAny ifpresent,  /* cl. 9.9.3.62 O TLV 3-n IEI=0x35 @sic R5s201386 Baseline Moving sic@ */

drxInNB_S1                 := cr_DRXParamCommonAny ('36'O) ifpresent,   /* cl. 9.9.3.63 O TLV 3 IEI=0x36 @sic R5s201386 Baseline Moving sic@ */

requestedIMSI_Offset       := cr_IMSI_Offset ifpresent,  /* cl. 9.9.3.64 O TLV 4 IEI=0x38 Sep22 @sic R5s221178 Baseline Moving sic@ */

ueRequestType              := cr_UE_RequestType ifpresent,     /* cl. 9.9.3.65 O TLV 3 IEI=0x29 Sep22 @sic R5s221178 Baseline Moving sic@ */

pagingRestriction          := cr_PagingRestriction ifpresent   /* cl. 9.9.3.66 O TLV 3-5 IEI=0x28 Sep22 @sic R5s221178 Baseline Moving sic@ */

}

};

  - Additional MCC160 implementation 3

| Function name | f_EUTRA38_TAUReqFromN1 |
| --- | --- |
| Reason for change | - In TC 11.4.12,if some UE do not support ENDC, UE may not report additionalSecurityCap in TAU request message at step 4. |
| Summary of change | - Add UE additional security capability |
| TTCN module | EUTRA38_NR5GCProcedures.ttcn |

function f_EUTRA38_TAUReqFromN1(EUTRA_CellId_Type p_CellId,

RRC_TransactionIdentifier p_RRC_TI,

template MobileIdentity p_MobileId := omit,

N1ToS1_SecurityContext_Type p_CxtType := MAPPED_CXT,

template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific,

template (present) EstablishmentCause p_EstablishmentCause := mo_Signalling,

boolean p_FirstN1_S1Transition := true,

template UE_AdditionalSecurityCapability p_UE_AddSecurityCap := cr_UE_AddSecurityCap_Any // @sic R5s230881 R5-241528 sic@

) runs on EUTRA_Multi5GS_PTC return NAS_MSG_Indication_Type

{

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL);

var NAS_MSG_Indication_Type v_NasInd;

var EUTRA_SecurityParams_Type v_SecurityParams;

var EUTRA_NR_CoOrd_Security_Type v_SecurityRx;

var template TrackingAreaId v_TAI:= *; // @sic R5s220225 sic@

var EUTRA_CellId_Type v_OldCellId;

var GutiParameters_Type v_GutiParams;

var NAS_PlmnId v_PLMN;

var TrackingAreaCode v_Tac;

var template UERadioCapInfoUpdateNeeded v_UeRadioCap:= cr_UeRadioCap;

var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList := f_EUTRA38_WaitForPDUSessionInfoList(p_CellId); // @sic R5s210149 sic@

var integer v_NoOfDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs();

var EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); // @sic R5s201363 sic@

var template (value) EUTRA38_DRBInfoList_Type v_NewDRBInfoList := {};

var integer v_NewDRBId;

var integer i;

var integer j := 0;

var template (present) MobileIdentity v_MobileId := cr_MobileIdAny;

var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR);

var template (value) MobileIdentity v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit);

var EPS_BearerContextStatus v_EPS_BearerContextStatus;

var IntegerList_Type v_TransferredBearers;

var template (present) UE_NetworkCap v_UENWCap := cdr_NR5GC_UENetworkCap('58'O);

if (isvalue(p_MobileId)) { // @sic R5s200564 sic@

v_MobileId := p_MobileId;

}

if (p_CxtType == MAPPED_CXT) { // @sic R5s200564 sic@

v_MobileId := v_MappedGUTI;

}

if (isvalue(p_CellIdPreviouslyCampedOn)) {

v_OldCellId := valueof(p_CellIdPreviouslyCampedOn);

if (v_OldCellId == eutra_Cell_NonSpecific) {

v_OldCellId := p_CellId;

}

v_GutiParams:= f_EUTRA_CellInfo_GetGuti(v_OldCellId);

v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity);

v_Tac := f_EUTRA_CellInfo_GetTAC(v_OldCellId);

v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac));

}

// If we need a new algorithm, have to store the new values before calling this

// tell NAS EMU to check next message with 5G integrity

f_EUTRA_NB_SS_NAS_SecurityConfig(NASCTRL, cas_NasSecurity_5GIntegrity_REQ(true));

if (not p_FirstN1_S1Transition) { // @sic R5s200583 sic@

v_UeRadioCap := cr_UeRadioCap ifpresent;

}

if (valueof(p_EstablishmentCause) == emergency) { // @sic R5-217714 sic@

v_UENWCap.cIoT_proSeCap := ?; // @sic R5-220644 sic@

v_UENWCap.cIoT_V2X := ?

}

/* Steps 1-3 */

//Receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH):

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(p_CellId,

p_RRC_TI,

p_EstablishmentCause,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_TAU_Request_N1ToS1(('000'B, '001'B, '010'B), // @sic R5-201171, R5-202563, R5-211371 sic@

?,

cr_DRXparameter_Any ('5C'O) ifpresent,

v_AdditionalUpdateType,

v_MobileId,  // @sic R5-204399 sic@

v_TAI,

-, -, -,  // @sic R5s200509 sic@

cr_MobileIdAny ('50'O) ifpresent, // @sic R5202563 sic@

v_UeRadioCap,

-,

v_UENWCap,

p_UE_AddSecurityCap  // @sic R5s230881 R5-241528 sic@

)));

// return NAS EMU to normal 4G integrity

f_EUTRA_NB_SS_NAS_SecurityConfig(NASCTRL, cas_NasSecurity_5GIntegrity_REQ(false));

// now tell NR it's ok to send the security parameters

f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_Trigger);

v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (NR);

// @sic R5-204399, R5s201390, R5-206289 sic@

v_EPS_BearerContextStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

v_TransferredBearers := f_EUTRA38_NR_ConvertEPS_BearerContextStatus(v_EPS_BearerContextStatus);

// Store EPS bearers transferred, if not stored already

for (i:=0;  i < lengthof(v_PduSessionInfoList); i :=i+1) {

if (f_IntegerList_Search(v_TransferredBearers, v_PduSessionInfoList[i].EPS_Bearer) != -1) {

// don't store if not in transferred list @sic R5-206289 sic@

v_NewDRBId := (v_PduSessionInfoList[i].EPS_Bearer - 4);

if (f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, v_NewDRBId) == -1) {

// add new DRB to list

v_NewDRBInfoList[j] := cs_DRB_Info (v_NewDRBId, (v_NewDRBId+4), false, cs_OneDRB_ConfigAM(v_NewDRBId), cs_508_DRB_ToAddMod_DEFAULT_AM(v_NewDRBId), f_GetMultiPdnTypeFromIndex(v_PduSessionInfoList[i].PdnIndex));  // @sic R5s210305 sic@

j := j+1;

f_EUTRA_MobileInfo_SetEPS_BearerContext(int2hex(v_PduSessionInfoList[i].EPS_Bearer, 1), v_PduSessionInfoList[i]); // @sic R5s210414 sic@

}

}

}

// now store the full list, but don't configure any

if (v_NoOfDRBs > 0) {

v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList();

v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (v_ExistingDRBs, valueof(v_NewDRBInfoList));

} else {

v_ExistingDRBs := valueof(v_NewDRBInfoList);

}

f_EUTRA38_MobileInfo_SetDRBInfoList (valueof(v_ExistingDRBs));

// Now that we have the security parameters we can check the KSI received

if (not match(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.asmeNasKeySetId.nasKeySetId, v_SecurityRx.KSI)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU Req KSI failed");

}

v_SecurityParams := f_EUTRA38_MappedContextFromN1_InitNAS(p_CellId, v_SecurityRx, false);

f_EUTRA_Security_Set(v_SecurityParams);

return v_NasInd;

}