3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230896

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3287 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 11.7.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 11.7.2 to the IWD_23wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.7.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 11.7.2	4

2.1	f_TC_11_7_2_NR5GC_TestBody	4

2.2	f_GetTestcaseAttrib_EarlyContentionResolution	10

2.3	f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle	11

2.4	f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive	13

2.5	cs_NG_NAS_EDRXParams	14

3	Branches executed	15

4	Execution Log Files	15

4.1	Hisilicon Balong 5000	15

5	References	16

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.7.2 which is part of the NR5GC test suite in iWD_TTCN3-B2022-09_D23wk37.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	11.7.2.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000

Verification Status:	PASS

# Corrections required to test case 11.7.2

## f_TC_11_7_2_NR5GC_TestBody

| Function name | f_TC_11_7_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation: - For Steps 19Aa1-20 the function f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); is called. This does not handle the case when pc_noOf_PDUsNewConnection >0. This needs to be handled. - At Step 19, the TTCN needs to use the latest PDCP counts and refresh the AS security with those sequence numbers otherwise the rrcResume message may not be received by the UE. This needs to be addressed. Same change is also needed after Steps 27 and 34. - In Step 27, the eDRX cycle length is set to 2.56s, whereas a\ccording to the Prose it should be 5.12s. This needs to be corrected. |
| Summary of change | - For Steps 19Aa1-20, instead of calling f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); called function f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); This takes care of both situations. - Called the function f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); after Steps 19, 27 and 34. - Updated the TTCN at Step 27 to use correct eDRX cycle length. |
| TTCN module | EDRX_NR5GC.ttcn |
| MCC160 Comment | 1,2 Accepted 3- Not needed as fourth parameter in f_NR5GC_UE_Page_eDRX will be used to calculate ‘T’, which is min [eDRX(=5,12), ran-ExtendedPagingCycle-r17(=2,56)]. See MCC160’s implementation to improve this part along with the updates in common functions from R5s240061 needed to calculate ‘T’ and v_N. |

Before Change:

function f_TC_11_7_2_NR5GC_TestBody() runs on NR5GC_PTC { var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var template (value) ExtdDRXParams v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0001'B, '0001'B, '00000001'B);// Step 14 - Table 11.7.2.3.3-2: REGISTRATION ACCEPT // @sic R5-233412 sic@ var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var template (value) SuspendConfig v_SuspendConfig; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-15" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 [4] Table 4.5.3.2-1 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE? //Check: Does the UE transmit a REGISTRATION COMPLETE message? f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 16-18" siclog@ //Void //@siclog "Steps 19Aa1-20" siclog@ //Steps 19Aa1-20 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf512); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 21" siclog@ //The SS transmits a Paging message within a paging occasion f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 1); //@siclog "Step 22" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 22"); //@siclog "Step 23" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 24" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 25" siclog@ //Void //@siclog "Step 26" siclog@ //The SS transmits an RRCRelease message with suspend. v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf256); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 27" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by the value of ran-ExtendedPagingCycle IE. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 0); //@siclog "Step 28" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 28"); //@siclog "Step 29" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 30" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 31" siclog@ //The SS transmits an RRCRelease message with suspend. f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 32" siclog@ //The SIB1 is updated according to Table 11.7.2.3.3-1 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_CellInfo_SetSIB1_hyperSFN_r17(nr_Cell1, omit); // return value by default f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(nr_Cell1); f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(nr_Cell1); f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //Modify Sysinfo in SS //@siclog "Step 33" siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 34" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by normal DRX and not belonging to extended DRX. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 35" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 35"); //@siclog "Step 36" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 37" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); }//end of f_TC_11_7_2_NR5GC_TestBody

After Change:

function f_TC_11_7_2_NR5GC_TestBody() runs on NR5GC_PTC { var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var template (value) ExtdDRXParams v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0001'B, '0001'B, '00000001'B);// Step 14 - Table 11.7.2.3.3-2: REGISTRATION ACCEPT // @sic R5-233412 sic@ var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var template (value) SuspendConfig v_SuspendConfig; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-15" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 [4] Table 4.5.3.2-1 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE? //Check: Does the UE transmit a REGISTRATION COMPLETE message? f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 16-18" siclog@ //Void //@siclog "Steps 19Aa1-20" siclog@ //Steps 19Aa1-20 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. //WA#11_7_2 COMMENTED OUT f_NR5GC_PDUSessionEstablishment (nr_Cell1, -, v_SRB); f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); //WA#11_7_2 v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf512); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 21" siclog@ //The SS transmits a Paging message within a paging occasion f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //WA#11_7_2 //@siclog "Step 22" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 22"); //@siclog "Step 23" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 24" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 25" siclog@ //Void //@siclog "Step 26" siclog@ //The SS transmits an RRCRelease message with suspend. v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf256); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 27" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by the value of ran-ExtendedPagingCycle IE. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 1 /*WA#11_7_2 -->1*/); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //WA#11_7_2 //@siclog "Step 28" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 28"); //@siclog "Step 29" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 30" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 31" siclog@ //The SS transmits an RRCRelease message with suspend. f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 32" siclog@ //The SIB1 is updated according to Table 11.7.2.3.3-1 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_CellInfo_SetSIB1_hyperSFN_r17(nr_Cell1, omit); // return value by default f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(nr_Cell1); f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(nr_Cell1); f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //Modify Sysinfo in SS //@siclog "Step 33" siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 34" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by normal DRX and not belonging to extended DRX. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //WA#11_7_2 //@siclog "Step 35" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 35"); //@siclog "Step 36" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 37" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); }//end of f_TC_11_7_2_NR5GC_TestBody

MCC160 Implementation

function f_TC_11_7_2_NR5GC_TestBody() runs on NR5GC_PTC {// @sic R5s230896 sic@ var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var template (value) ExtdDRXParams v_NegotiatedExtdDRXParams := cs_NG_NAS_EDRXParams('0001'B, '0001'B, '00000001'B);// Step 14 - Table 11.7.2.3.3-2: REGISTRATION ACCEPT // @sic R5-233412 sic@ var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var template (value) SuspendConfig v_SuspendConfig; //@siclog "Step 1" siclog@ //The UE is switched on f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2-15" siclog@ //Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 [4] Table 4.5.3.2-1 are performed. //The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE //Steps 5-13 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. //The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE? //Check: Does the UE transmit a REGISTRATION COMPLETE message? f_NR5GC_RRC_Idle_Steps2_15_eDRX(nr_Cell1, v_NegotiatedExtdDRXParams); //@siclog "Steps 16-18" siclog@ //Void //@siclog "Steps 19Aa1-20" siclog@ //Steps 19Aa1-20 of Table 4.5.3.2-1 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); //WA#11_7_2 v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf512); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 21" siclog@ //The SS transmits a Paging message within a paging occasion f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 1, 1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //WA#11_7_2 //@siclog "Step 22" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 22"); //@siclog "Step 23" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 24" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 25" siclog@ //Void //@siclog "Step 26" siclog@ //The SS transmits an RRCRelease message with suspend. v_SuspendConfig := cs_SuspendConfigDef(cs_RAN_NotificationAreaInfo_1Cell(v_CellIdentity, omit), -, -, rf256); f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 27" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by the value of ran-ExtendedPagingCycle IE. f_NR5GC_UE_Page_eDRX(nr_Cell1, cs_PagingRecordList_OneRecord(cs_PagingUE_Identity_FullI_RNTI(tsc_NR_I_RNTI_Value1)), 1, 1, 0); /* RRC_INACTIVE state, if the UE operates in eDRX and eDRX is configured by RRC, i.e., TeDRX, RAN , and/or upper layers, i.e., TeDRX, CN, as defined in clause 7.4: If both TeDRX, CN and used TeDRX, RAN are no longer than 1024 radio frames, T = min{TeDRX, RAN, TeDRX, CN} = min (5.12, 2.56)*/ f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //@siclog "Step 28" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 28"); //@siclog "Step 29" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 30" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 31" siclog@ //The SS transmits an RRCRelease message with suspend. f_NR_RRCRelease(nr_Cell1, v_SuspendConfig); //@siclog "Step 32" siclog@ //The SIB1 is updated according to Table 11.7.2.3.3-1 and transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_CellInfo_SetSIB1_hyperSFN_r17(nr_Cell1, omit); // return value by default f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(nr_Cell1); f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(nr_Cell1); f_NR_ModifySysinfo(nr_Cell1, true, RRC_INACTIVE); //Modify Sysinfo in SS //@siclog "Step 33" siclog@ //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 34" siclog@ //The SS transmits a Paging message to the UE in a valid PO which is derived by normal DRX and not belonging to extended DRX. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1,tsc_NR_38508_NextHopChainingCount); //WA#11_7_2 //@siclog "Step 35" siclog@ //Check: Does the UE transmit an RRCResumeRequest message? f_NR_RRC_ResumeRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 11.7.2 Step 35"); //@siclog "Step 36" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 37" siclog@ //The UE transmits an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell1); }//end of f_TC_11_7_2_NR5GC_TestBody

Module NR5GC_SysinfoPaging

function f_NR_Calculate_PF_PO (SubFrameTiming_Type p_CurrentTiming, PagingCycle p_T, PCCH_Config p_PCCH_Config, B32_Type p_NG_TMSI, integer p_SlotsNumberPerSubframe, SubcarrierSpacing p_SCS, integer p_SlotOffset, integer p_UE_ID_Mod_Value := 1024, template (omit) float p_eDRX_Value := omit, template (omit) float p_Ran_ExtendedPagingCycle := omit) return SubFrameTiming_Type { //@sic R5s190291 change 1: change first parameter p_Sfn -> p_CurrentTiming (type SubFrameTiming_Type) R5s240061 sic@ var integer v_UE_ID := bit2int(p_NG_TMSI) mod p_UE_ID_Mod_Value; // 5G-S-TMSI mod 1024 or 4096 var integer v_PF_Offset := fl_NR_ConvertPF_Offset(p_PCCH_Config.nAndPagingFrameOffset); // var integer v_Ns := fl_NR_ConvertNs(p_PCCH_Config.ns); //number of paging occasions for a PF var integer v_NextSfn := p_CurrentTiming.SFN.Number + tsc_NR_SfnDelay; //Absolute value - no mod 1024 var integer v_SfnPlusOffsetModT; var integer v_FirstPagingFrame; var integer v_T := fl_NR_ConvertPagingCycle(p_T); var integer v_N; // N: number of total paging frames in T @sic R5s190150 change 4 sic@ var SubFrameTiming_Type v_PagingOccasion; var template (omit) float v_Tmin := omit; if (ispresent(p_eDRX_Value)) { v_Tmin := p_eDRX_Value; if (ispresent(p_Ran_ExtendedPagingCycle) and (valueof(p_Ran_ExtendedPagingCycle) < valueof(p_eDRX_Value))){ v_Tmin := p_Ran_ExtendedPagingCycle; } v_T := float2int(valueof(v_Tmin) * 100.00); // check with ispresent allows this line } v_N := fl_NR_GetN(p_PCCH_Config.nAndPagingFrameOffset, v_T); //Calculate the first paging Frame //SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) //Absolute value - no mod 1024 @sic R5-211417 sic@ v_SfnPlusOffsetModT := (v_T / v_N) * (v_UE_ID mod v_N); if ((v_NextSfn mod v_T) < v_SfnPlusOffsetModT) { v_FirstPagingFrame := ((v_NextSfn - v_NextSfn mod v_T) + v_SfnPlusOffsetModT) - v_PF_Offset; } else { v_FirstPagingFrame := (v_NextSfn - v_NextSfn mod v_T) + v_SfnPlusOffsetModT + v_T - v_PF_Offset; } //* Calculate i_s = floor (UE_ID/N) mod Ns // v_I_s := (v_UE_ID mod v_N) mod v_Ns; Note: unused so far //PDCCH monitoring occasions according to paging-SearchSpace and firstPDCCH-MonitoringOccasionOfPO //PDCCH monitoring occasions according to the default association: PDCCH monitoring occasions for paging are same as for RMSI as defined in Section 13 in TS 38.213 //For Ns = 1, there is only one PO which starts in the PF. //For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF. v_PagingOccasion.HSFN.Number := (p_CurrentTiming.HSFN.Number + (v_FirstPagingFrame)/1024 ) mod 1024; //@sic R5-211417 sic@ v_PagingOccasion.SFN.Number := v_FirstPagingFrame mod 1024; //@sic R5-211417 sic@ v_PagingOccasion.Subframe.Number := p_SlotOffset / p_SlotsNumberPerSubframe; v_PagingOccasion.Symbol := valueof(cs_SymbolTimingInfo_FirstSlot); //@sic R5s221246 sic@ v_PagingOccasion := f_NR_SubFrameTiming_SetSlot(v_PagingOccasion, p_SlotOffset mod p_SlotsNumberPerSubframe, p_SCS); return v_PagingOccasion; } .. group Paging_eDRX { type float T_RedCap_NR5GC_CycleParameters[13]; type float T_NR_ExtendedPagingCycle_r17_Parameters[3]; // Table 10.5.5.32/3GPP TS 24.008: Extended DRX parameters information element const T_RedCap_NR5GC_CycleParameters tsc_NR5GC_eDRX_Cycle_Length := { 2.56, 5.12, 10.24, 20.48, 40.96, 81.92, 163.84, 327.68, 655.36, 1310.72, 2621.44, 5242.88, 10485.76 }; const T_NR_ExtendedPagingCycle_r17_Parameters tsc_NR5GC_Ran_ExtendedPagingCycle := { 2.56, 5.12, 10.24 }; .. function f_NR5GC_PO_List_eDRX(NR_CellId_Type p_NR_CellId, integer p_PTWLength_Index, integer p_EDRXValue_Index, template (omit) integer p_Ran_ExtendedPagingCycle_Index := omit) runs on NR5GC_PTC return SubFrameTimingList_Type { // @sic R5s240061 sic@ var SubFrameTiming_Type v_CurrentTiming; var PCCH_Config v_PcchConfig := f_NR_GetSIB1_PcchConfig(p_NR_CellId); var B32_Type v_NG_TMSI := f_NR5GC_CellInfo_GetNG_TMSI(p_NR_CellId); var SubFrameTimingList_Type v_PoList := {}; var SubFrameTiming_Type v_PagingOccasion; var float v_EDRXValue; var template (omit) float v_Ran_ExtendedPagingCycle := omit; var float v_PTWLengthValue; var PagingCycle v_PagingCycle := f_NR_GetSIB1_PagingCycle(p_NR_CellId); var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_SlotOffsetPaging := f_NR_GetSlotOffsetForPaging(p_NR_CellId); var integer v_UE_ID_Mod_Value := 4096; v_PTWLengthValue := tsc_PTWLengthParameters[p_PTWLength_Index]; v_EDRXValue := tsc_NR5GC_eDRX_Cycle_Length[p_EDRXValue_Index]; if (isvalue(p_Ran_ExtendedPagingCycle_Index)){ v_Ran_ExtendedPagingCycle := tsc_NR5GC_Ran_ExtendedPagingCycle[valueof(p_Ran_ExtendedPagingCycle_Index)]; // ispresent allows assignement with valueof } //Get current SFN and subframe v_CurrentTiming := f_NR_GetCurrentTiming(p_NR_CellId); // Calculate next Paging Occasion // TS 38.304 clause 7.4: If the UE is configured with an extended DRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE configured with eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE. if (p_EDRXValue_Index < 3){ // TS 38.304 cl. 7.1: calculate the first Paging occasion v_PagingOccasion := f_NR_Calculate_PF_PO(v_CurrentTiming, v_PagingCycle, v_PcchConfig, v_NG_TMSI, v_SlotsNumberPerSubframe, v_SCS, v_SlotOffsetPaging, v_UE_ID_Mod_Value, v_EDRXValue, v_Ran_ExtendedPagingCycle); v_PoList[0] := v_PagingOccasion; } else { v_PoList := f_NR5GC_Calculate_PTW_PH(p_NR_CellId, v_CurrentTiming.SFN.Number, v_CurrentTiming.HSFN.Number, v_PTWLengthValue, v_EDRXValue); } return v_PoList; } function f_NR5GC_UE_Page_eDRX(NR_CellId_Type p_NR_CellId, template (value) PagingRecordList p_PagingRecordList, integer p_PTWLength_Index, integer p_EDRXValue_Index, template (omit) integer p_Ran_ExtendedPagingCycle_Index := omit) runs on NR5GC_PTC { var SubFrameTimingList_Type v_PoList; f_Delay(tsc_NR_DelayForCellSelection); v_PoList := f_NR5GC_PO_List_eDRX (p_NR_CellId, p_PTWLength_Index, p_EDRXValue_Index, p_Ran_ExtendedPagingCycle_Index); SYS.send(cas_NR_CellConfig_Paging_REQ(p_NR_CellId, v_PoList[0], p_PagingRecordList)); }

## f_GetTestcaseAttrib_EarlyContentionResolution

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | In the function f_TC_11_7_2_NR5GC_TestBody at Step 22 upon reception of the rrcResumeREQ, a preconfigured rrcSetup will be transmitted. This is incorrect, and needs to be addressed.. Hence it is proposed that the testcase 11.7.2 be incuded in this function, after which the preconfigured rrcSetup will not be transmitted. |
| Summary of change | Included testcase 11.7.2 in function f_GetTestcaseAttrib_EarlyContentionResolution |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } <<SKIPPED CODE>>

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_11_7_2_NR5GC") { return true; } //WA#11_7_2 case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } <<SKIPPED CODE>>

## f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle

| Function name | f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle |
| --- | --- |
| Reason for change | The current TTCN implementation only allows to set the IE eDRX_AllowedIdle_r17 to TRUE, but it does not allow it to set to OMIT. This needs to be corrected. |
| Summary of change | Updated the function so that depending on the parameter p_EDRX_AllowedIdle_r17 the IE eDRX_AllowedIdle_r17 is set to TRUE or OMIT. Please see below. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(NR_CellId_Type p_NR_CellId, template (omit) SIB1_v1700_IEs.eDRX_AllowedIdle_r17 p_EDRX_AllowedIdle_r17 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (isvalue(p_EDRX_AllowedIdle_r17)) { if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedIdle_r17 := valueof(p_EDRX_AllowedIdle_r17); } } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change:

function f_NR_CellInfo_Set_SIB1_eDRX_AllowedIdle(NR_CellId_Type p_NR_CellId, template (omit) SIB1_v1700_IEs.eDRX_AllowedIdle_r17 p_EDRX_AllowedIdle_r17 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (isvalue(p_EDRX_AllowedIdle_r17)) { if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedIdle_r17 := valueof(p_EDRX_AllowedIdle_r17); } } else { //WA#11_7_2 v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedIdle_r17 := omit; } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

## f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive

| Function name | f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive |
| --- | --- |
| Reason for change | The current TTCN implementation only allows to set the IE eDRX_AllowedInactive_r17 to TRUE, but it does not allow it to set to OMIT. This needs to be corrected. |
| Summary of change | Updated the function so that depending on the parameter p_EDRX_AllowedIdle_r17 the IE eDRX_AllowedInactive_r17 is set to TRUE or OMIT. Please see below. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(NR_CellId_Type p_NR_CellId, template (omit) SIB1_v1700_IEs.eDRX_AllowedInactive_r17 p_EDRX_AllowedIdleInactive_r17 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (isvalue(p_EDRX_AllowedIdleInactive_r17)) { if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedInactive_r17 := valueof(p_EDRX_AllowedIdleInactive_r17); } } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change:

function f_NR_CellInfo_Set_SIB1_eDRX_AllowedInactive(NR_CellId_Type p_NR_CellId, template (omit) SIB1_v1700_IEs.eDRX_AllowedInactive_r17 p_EDRX_AllowedIdleInactive_r17 := omit) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ if (isvalue(p_EDRX_AllowedIdleInactive_r17)) { if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedInactive_r17 := valueof(p_EDRX_AllowedIdleInactive_r17); } } else { //WA#11_7_2 v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.eDRX_AllowedInactive_r17 := omit; } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

## cs_NG_NAS_EDRXParams

| Function name | cs_NG_NAS_EDRXParams |
| --- | --- |
| Reason for change | In the current TTCN implementation, the iel is set incorrectly, this needs to be corrected. |
| Summary of change | Changed the iel to ‘02’O. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change:

template (value) ExtdDRXParams cs_NG_NAS_EDRXParams (B4_Type p_PTW, B4_Type p_EDRXVal, template (omit) B8_Type p_ExtdPTW := omit) := { /* @status */ iei := '6E'O, iel := '01'O, pagingTimeWindow := p_PTW, eDRXValue := p_EDRXVal, extdPTW := p_ExtdPTW };

After Change:

template (value) ExtdDRXParams cs_NG_NAS_EDRXParams (B4_Type p_PTW, B4_Type p_EDRXVal, template (omit) B8_Type p_ExtdPTW := omit) := { /* @status */ iei := '6E'O, iel := '02'O, // length //WA#11_7_1 01 --> 02 (Also needed for 11_7_2, 11_7_3 and 8_1_5_2_3) pagingTimeWindow := p_PTW, eDRXValue := p_EDRXVal, extdPTW := p_ExtdPTW };

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA1 ciphering algorithms.

# Execution Log Files

## Hisilicon Balong 5000

The Hisilicon Balong 5000 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:

tc_11_7_2_Hisilicon.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s230897: Supporting information for addition of NR5GC testcase 11.7.2 in FR1 |
| --- | --- |