3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230902

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3290 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_SorRegistration_Step5_20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HUAWEI TECHNOLOGIES Co. Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is restriction for the parameter p_Header_UE. Header_UE is a mandatory parameter in record SORTransparentContainer, so this template can’t be used as omit, it should be a “present” template. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the restriction for the p_Header_UE, and set the default value as “?”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	liuyang179@hisilicon.com

# Corrections required

## Change 1

| Function name | f_NR5GC_SorRegistration_Step5_20 |
| --- | --- |
| Reason for change | There is restriction for the parameter p_Header_UE. Header_UE is a mandatory parameter in record SORTransparentContainer, so this template can’t be used as omit, it should be a “present” template. |
| Summary of change | Correct the restriction for the p_Header_UE, and set the default value as “?”. |
| TTCN module | SOR_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function f_NR5GC_SorRegistration_Step5_20(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, template (omit) SOR_Header p_Header_UE:=omit, boolean p_MacError:= false, TypeOfRegistration_Type p_RegType := Initial_NoSecurity, GMM_MobilityInfo_Type p_GMM_MobilityInfo, NG_NAS_MSG_Indication_Type p_ReceivedMsg, boolean p_AckSor:= true) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var O16_Type v_Mac_UE; v_Mac_UE:=f_NR5GC_SorRegistration_Step5_13 (p_NR_CellId, p_GMM_MobilityInfo, p_ReceivedMsg, p_Data, p_Header, p_RegType, p_MacError); //@sic R5s210236 sic@ if(p_AckSor) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(p_Header_UE,v_Mac_UE)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } f_NR5GC_RRC_Idle_Steps16_20(p_NR_CellId,-, TESTMode_OFF,rrcConnectionRelease); } else{ f_NR5GC_RRC_Idle_Steps15_20(p_NR_CellId, TESTMode_OFF, rrcConnectionRelease); } }

After Change

function f_NR5GC_SorRegistration_Step5_20(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, template (present) SOR_Header p_Header_UE:=?, boolean p_MacError:= false, TypeOfRegistration_Type p_RegType := Initial_NoSecurity, GMM_MobilityInfo_Type p_GMM_MobilityInfo, NG_NAS_MSG_Indication_Type p_ReceivedMsg, boolean p_AckSor:= true) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var O16_Type v_Mac_UE; v_Mac_UE:=f_NR5GC_SorRegistration_Step5_13 (p_NR_CellId, p_GMM_MobilityInfo, p_ReceivedMsg, p_Data, p_Header, p_RegType, p_MacError); //@sic R5s210236 sic@ if(p_AckSor) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(p_Header_UE,v_Mac_UE)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } f_NR5GC_RRC_Idle_Steps16_20(p_NR_CellId,-, TESTMode_OFF,rrcConnectionRelease); } else{ f_NR5GC_RRC_Idle_Steps15_20(p_NR_CellId, TESTMode_OFF, rrcConnectionRelease); } }