3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230904

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3292 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A conformant UE would request for an UL grant immediately to transmit the optional MAC PDU at Step 4A as soon as it has received the DL MAC PDU at Step 1. However, due to configurations involving RRC Reconfiguration procedure at Step 3 – 4 and the SS local end configuration involving the RACH procedure config after the RRC Reconfiguration procedure, the UE tries to send the optional MAC PDU at Step 4A using the OnSR UL grant procedure started in function f_NR_RLC_ReconfigurationHO_Steps1_8 However, there is no handling for the MAC PDU in function fl_TC_7_1_1_9_1_NR_TestBody as this MAC PDU is expected to be handled in function f_TC_7_1_1_9_1_Step5to9A and this leads to the MAC PDU being handled by the TTCN default handler and causes TC failure |  |  |  |  |  |  |  |  |
| Summary of change: |  | To cater for all UEs, it is recommended to move the handling of the optional MAC PDU at Step 4A to the function fl_TC_7_1_1_9_1_NR_TestBody and fl_TC_7_1_1_9_1_ENDC_TestBody from f_TC_7_1_1_9_1_Step5to9A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC, 7.1.1.9.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	9

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_7_1_1_9_1_NR_TestBody |
| --- | --- |
| Reason for change | A conformant UE would request for an UL grant to transmit the optional MAC PDU at Step 4A as soon as it has received the DL MAC PDU at Step 1. However, due to configurations involving RRC Reconfiguration procedure at Step 3 – 4 and the SS local end configuration involving the RACH procedure config after the RRC Reconfiguration procedure, the UE tries to send the optional MAC PDU at Step 4A using the UL grant procedure started in function f_NR_RLC_ReconfigurationHO_Steps1_8 However, there is no handling for the MAC PDU in function fl_TC_7_1_1_9_1_NR_TestBody as this MAC PDU is expected to be handled in function f_TC_7_1_1_9_1_Step5to9A and this leads to the MAC PDU being handled by the TTCN default handler and causes TC failure |
| Summary of change | To cater for all UEs, it is recommended to move the handling of the optional MAC PDU at Step 4A to the function fl_TC_7_1_1_9_1_NR_TestBody from f_TC_7_1_1_9_1_Step5to9A |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle. To be inline with the test sequence in TS 38.523-1, it is proposed not to start the grant inside f_NR_RLC_ReconfigurationHO_Steps1_8. See proposed implementation below. |

Before Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; //@sic R5-232641 R5s230370 sic@ v_SS_Drb_ConfigList[0]:= cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_PollRetransmit, -, -, -, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_Reassembly, -, true_); v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5s201457 sic@ v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0]; //@sic R5s201457 sic@ <<SKIPPED CODE>> // @siclog "Step 4" siclog@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@ // @siclog "Step 4a - Void" // Steps left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 // Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); // @siclog "Steps 5-9A" siclog@ v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList); <<SKIPPED CODE>>

After Change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC { //@sic R5-206320 R5-213132 sic@ <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; //@sic R5-232641 R5s230370 sic@ //WA# var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; v_SS_Drb_ConfigList[0]:= cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode); v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_PollRetransmit, -, -, -, f_NR_38508_SetSNFieldLengthAM(), //@sic R5-227518 sic@ v_T_Reassembly, -, true_); v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5s201457 sic@ v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0]; //@sic R5s201457 sic@ <<SKIPPED CODE>> // @siclog "Step 4" siclog@ SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@ // @siclog "Step 4a - Void" WA# //Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } } // Steps left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10 // Step 9 (after step 8) Re-configure RACH procedure as for initial access, enable TA transmissions. f_NR_SS_RachProcedureConfig_Def(nr_Cell1); // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); // @siclog "Steps 5-9A" siclog@ v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList, -, v_IsDataRecvd); //WA#

MCC160 proposed implementation

function f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step1_8(NR_CellId_Type                         p_NR_CellId,

template (value) DL_DCCH_Message       p_RRCReconfiguration,

boolean                                p_RBConfig_KeyChange := true,

template (value) NR_RadioBearerList_Type   p_NR_RadioBearerList,

template (value) NR_RachProcedureConfig_Type   p_RachProcedureConfig,

RNTI_Value                             p_RNTI_Value :=  tsc_C_RNTI_Value3,

template (omit) MeasConfig             p_MeasConfig := omit,

NextHopChainingCount                   p_NCC := tsc_NR_38508_NextHopChainingCount,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit,

boolean p_SetGetOfPDCPCount := true,

template (omit) UL_GrantConfig_Type    p_UL_GrantConfig := cs_UL_GrantConfig_OnSR

) runs on NR_BASE_PTC return SubFrameTiming_Type

{ //Added p_SetGetOfPDCPCount to be set to false in PDCP tc with TM mode

// @sic R5s220825: deleted p_TimingInfo sic@

// @sic R5s230904: added p_UL_GrantConfig sic@

var SubFrameTiming_Type v_Timing;

var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_CellId);  // List of DRBs to release in the cell

var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList;

var NR_SecurityParams_Type v_RRC_SecurityParams;

// Step 1. Before T: Get PDCP count for AM DRBs and SRBs (if applied).

// Not needed in PDCP tc with PDCP configured in TM mode

v_NR_PdcpCountInfoList := f_NR_GetPdcpCountInfoListIntraNR_HO(p_NR_CellId, p_NR_RadioBearerList, p_RBConfig_KeyChange); //@sic R5s220123 sic@

// @sic R5s191200 R5s201150 sic@

/* For intra cell HO the HO procedure and the SS configuration must be executed with timing info.

* This is to make sure SS reconfiguration is done properly and in time before UE access the new cell. */

v_Timing := f_NR_GetNextSendOccasion_DL(p_NR_CellId, tsc_NR_DelayBeforeIntraCellHO);     // @sic R5s220825 sic@

//Step 2 At T: Send RRCReconfiguration.

SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, cs_TimingInfo_NR(v_Timing), p_RRCReconfiguration));

/* Note: current implementation releases SRBs and reconfigures C-RNTI 5ms after sending the RRCConnectionReconfiguration

=> RLC ACK is not taken into consideration */

//Step 3 At T + 5ms: Release SRBs and DRBs.

v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5);   // add 5ms

f_NR_SS_SRBs_DRBs_Release(p_NR_CellId, cs_TimingInfo_NR(v_Timing), v_DrbIdList, tsc_NoCnfReq, tsc_FollowOnFlag);   //@sic R5-206369 sic@

// Step 4 At T + 5ms: Configure RACH procedure either dedicated or C-RNTI based.

// NOTE: Since the RACH procedure may require a new C-RNTI to be used it cannot be configured before sending out the RRCReconfiguration.

// NOTE 2: The FollowOnFlag is set to true in the ASPs reconfiguring the RadioBearerList (step 3) and C-RNTI (step 4).

f_NR_SS_C_RNTI_Config(p_NR_CellId, p_RNTI_Value, cs_TimingInfo_NR(v_Timing), tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5s190747 change 5.3 R5-206369 sic@

f_NR_SS_RachProcedureConfigHO(p_NR_CellId, p_RachProcedureConfig, cs_TimingInfo_NR(v_Timing), p_RACH_ConfigDedicated);

f_NR_CellInfo_SetRNTI(p_NR_CellId, p_RNTI_Value);

// Step 5 At T + 5ms: (Re-)configure measurement gap if provided in the RRCReconfiguration message.

// @sic R5-197222 sic@

if (isvalue(p_MeasConfig.measGapConfig)) {

f_NR_SS_MeasGapCtrlConfig(p_NR_CellId, cs_NR_MeasGapCtrl_Config(p_MeasConfig.measGapConfig), cs_TimingInfo_NR(v_Timing));

}

// Step 6 At T + 10ms: (Re-) configure SRBs and DRBs.

v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5);   // add 5ms

f_NR_SS_SRBs_DRBs_Config(p_NR_CellId, cs_TimingInfo_NR(v_Timing), p_NR_RadioBearerList);

// Step 7 At T + 10ms:  Restore the PDCP count for AM DRBs and SRBs (if applied).

//NOTE 2:   For AM DRBs the PDCP count is maintained while for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the  RRCReconfiguration message content

// Not needed in PDCP tc with PDCP configured in TM mode

if (p_SetGetOfPDCPCount){

f_NR_SS_PdcpCount_Set(p_NR_CellId, v_NR_PdcpCountInfoList, cs_TimingInfo_NR(v_Timing)); // @sic R5s191200_r2 sic@

}

// Step 8 At T + 10ms: Re-establish security, disable TA transmission.

// NOTE 2:   For AM DRBs the PDCP count is maintained while for SRBs and UM DRBs the PDCP count is reset to 0.

if ((p_NCC > tsc_NR_38508_NextHopChainingCount) or p_RBConfig_KeyChange) { // @sic R5s191200 R5s200309 sic@

f_NR_SS_AS_ActivateSecurity_Current(p_NR_CellId, p_NCC, cs_TimingInfo_NR(v_Timing),-, v_NR_PdcpCountInfoList); //@sic R5s200754 R5s220123 sic@

} else {

v_RRC_SecurityParams := f_NR_Security_Get();

v_RRC_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, false, -, v_NR_PdcpCountInfoList); //@sic R5s220123 sic@

f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId,

v_RRC_SecurityParams.AS_Integrity,

v_RRC_SecurityParams.AS_Ciphering,

cs_TimingInfo_NR(v_Timing)); // @sic R5s191200_r2 sic@

}

// Disable TA transmission

f_NR_ULGrantConfiguration_Common(p_NR_CellId, cs_TimingInfo_NR(v_Timing), -, cs_NR_UplinkTimeAlignment_Stop, p_UL_GrantConfig);

return v_Timing;  // @sic R5s220825 sic@

}

//=========================================================================================================================================

function f_NR_RLC_ReconfigurationHO_Steps1_8(NR_CellId_Type p_NR_CellId,

DRB_Identity p_DRBId,

template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList,

template (value) PDCP_Config p_PDCP_Config,

template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList:= omit,

boolean p_OmitRecoverPDCP := false,

template (omit) CellGroupConfig p_CellGroupConfig := omit,

boolean p_ReestablishPDCP := false,

integer p_TBS := tsc_NR_Msg4GrantHO_Def,

template (omit) UL_GrantConfig_Type p_UL_GrantConfig := cs_UL_GrantConfig_OnSR

) runs on NR5GC_PTC return SubFrameTiming_Type

{ //@sic R5-194296 R5-204512 R5s221105: p_SS_Drb_Config updated to a list p_SS_Drb_ConfigList sic@

//@sic R5-231905: added p_ReestablishPDCP sic@

//@sic R5-232972: added p_TBS sic@

//@sic R%s230904: added p_UL_GrantConfig sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s221105 sic@

var template (value) DL_DCCH_Message v_RRCReconfiguration;

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

var SubFrameTiming_Type v_Timing;

if (ispresent(p_CellGroupConfig)) { // @sic R5s191200 sic@

v_CellGroupConfig := p_CellGroupConfig; // compiler warning may be ignored due to check

} else {

v_CellGroupConfig := f_NR_GetCellGroupConfigHO (p_NR_CellId, p_RLC_BearerConfigList); //tsc_C_RNTI_Value3

}

v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(false, p_PDCP_Config); // RBConfig_NoKeyChange

if (p_OmitRecoverPDCP) { // @sic R5s200180 sic@

v_RadioBearerConfig.drb_ToAddModList[0].recoverPDCP := omit;

}

if (p_ReestablishPDCP) { // @sic R5-231905 sic@

v_RadioBearerConfig.drb_ToAddModList[0].reestablishPDCP := true_;

}

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)));

v_RRCReconfiguration := cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, -, v_V1530Ext);

if (isvalue(p_SS_Drb_ConfigList)) { // @sic R5s190905_r1 R5s221105 sic@

v_SS_Drb_ConfigList := p_SS_Drb_ConfigList; // compiler warning may be ignored due to check

} else {

v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_AM_Test(p_DRBId, f_NR_38508_SetSNFieldLengthAM())}; // size12 or size18 for RedCap  @sic R5-227518 sic@

}

v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, tsc_C_RNTI_Value3, p_TBS);

v_Timing := f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step1_8(p_NR_CellId, v_RRCReconfiguration, false, v_SS_Drb_ConfigList, v_RachProcedureConfig, tsc_C_RNTI_Value3, -, -, -, -, p_UL_GrantConfig);  //@sic R5s221105 sic@

return v_Timing;

}

//=========================================================================================================================================

/*

* @desc      Test Body 7.1.1.9.1 Step 4Aa to Step 9A

* @param     p_NR_DRB_Id

* @param     p_LCID

* @param     p_NR_RadioBearerList

* @param     p_StopGrant         (default value: true)

* @return    boolean

* @status    APPROVED (ENDC, NR5GC)

*/

function f_TC_7_1_1_9_1_Step4Aato9A ( DRB_Identity p_NR_DRB_Id,

B6_Type p_LCID,

template (value) NR_RadioBearerList_Type p_NR_RadioBearerList,

boolean p_StopGrant:= true

) runs on NR_BASE_PTC return boolean

{ //@sic R5-206320 sic@

//@sic R5s230904: renamed function f_TC_7_1_1_9_1_Step5to9A -> f_TC_7_1_1_9_1_Step4Aato9A sic@

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var default v_IgnoreNRSR:= null; //@sic R5-204418 sic@

var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s211710 sic@

var boolean v_IsDataRecvd := false;

var NR_RLC_SS_State_Type v_RLC_Rec;

timer t_Watchdog;

// Initialization of RLC variables

v_RLC_Rec := f_NR_InitRLC_Record();

v_RLC_Rec.AM_RX_Next := 1;

// @siclog "Step 4Aa" siclog@

//@sic R5-232641 R5s230370 sic@

t_Watchdog.start(1.0);

alt {

[] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id,  cr_TimingInfo_Any, ?))

{

f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id);

// @siclog "UE has looped back step1 data" siclog@

v_IsDataRecvd := true;

t_Watchdog.stop;

}

[] t_Watchdog.timeout

{ }

}

// @siclog "Step 5" siclog@

// void

// @sic R5-213132 sic@

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, p_NR_RadioBearerList);

//The SS gets prepared for SR indication and stops default UL grant allocation.

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable));

…

}

//=========================================================================================================================================

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id,

template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC

{ //@sic R5-206320 R5-213132 sic@

var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6);

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList;

var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList;       //@sic R5s201457 sic@

var RLC_Config.am.ul_AM_RLC.t_PollRetransmit  v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1));

var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1));

var template (value) CellGroupConfig v_CellGroupConfig;  //@sic R5S210138 sic@

var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_UL := { SpecificSubset := {0,2,3,4,6,7 }};  //@sic R5s220669  sic@

var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};  //@sic R5s220669 sic@

var template (value) NR_DciUlInfo_Type v_DciUlInfo := cs_NR_DciUlInfo_AllOmit;

var SubFrameTiming_Type v_Timing;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); //@sic R5s221105 R5-227418 sic@

var boolean v_IsDataRecvd := false;     //@sic R5-232641 R5s230370 sic@

v_SS_Drb_ConfigList[0]:=  cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode);

v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id,

f_NR_38508_SetSNFieldLengthAM(),  //@sic R5-227518 sic@

v_T_PollRetransmit,

-,

-,

-,

f_NR_38508_SetSNFieldLengthAM(),  //@sic R5-227518 sic@

v_T_Reassembly,

-,

true_);

v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_);         //@sic R5s201457 sic@

v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0];                          //@sic R5s201457 sic@

//Initialise CellGroupConfig before sending data to UE to avoid additional delay

v_CellGroupConfig := f_NR_GetCellGroupConfigHO(nr_Cell1, v_NR_RLC_BearerToAddModList);      //@sic R5s210138 sic@

f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID);

v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1);

f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing),

cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL)

, -, -))); //@sic R5s220669 sic@

//@siclog "Steps 3-4a" siclog@

//@sic R5s210950 R5s220795 R5-231905 R5s230322 R5-232641 R5s230904 sic@

f_NR_RLC_ReconfigurationHO_Steps1_8(nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit, omit), v_SS_Drb_ConfigList, true, v_CellGroupConfig, true, -, omit);//@sic R5-213132 sic@

// @siclog "Step 4" siclog@

SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));// @sic R5-222044 sic@

// Step 10 left from HO: f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step9_10

// HO Step 10 Re-configure RACH procedure as for initial access, enable TA transmissions.

f_NR_SS_RachProcedureConfig_Def(nr_Cell1);

// enable TA transmissions

f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR);

// @siclog "Steps 4a-9A" siclog@

v_IsDataRecvd := f_TC_7_1_1_9_1_Step4Aato9A(p_NR_DRB_Id,v_LCID, p_NR_RadioBearerList);

//@siclog "Steps 16A-21" siclog@

//SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell

f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id, v_LCID, v_IsDataRecvd);

//@siclog "Steps 22" siclog@

//SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell

//@sic R5s210950 sic@

v_DciUlInfo.ResoureAssignment := cs_NR_DciFormat_0_X_ResourceAssignment(-, -, -, -,v_NR_HarqProcessConfig_UL);//@sic R5s220669  sic@

f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, v_DciUlInfo, -, -);

//@sic R5s230322 R5-232641 sic@

f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(-, omit, omit), v_SS_Drb_ConfigList,true, v_CellGroupConfig, true); //@sic R5-204418 R5s210138 R5-227518  R5-23231905 sic@

f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList);

//Reconfiguring MAC to normal mode //@sic R5s220687 sic@

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_Drb_ConfigList[0]});

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, -, -, -, -, cs_NR_HarqProcessConfig_AnyProcess);  //@sic R5s230680 sic@

}

//=========================================================================================================================================

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC

{

var B6_Type v_LCID := tsc_LCID_SCG_Bearer;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList;

var RLC_Config.am.ul_AM_RLC.t_PollRetransmit  v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1));

var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1));

var template (value)  NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode);

var template (value)  NR_RadioBearerList_Type v_SS_DrbConfigList_TestMode  := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value;

var template (value) NR_HarqProcessConfig_Type v_NR_HarqProcessConfig_DL := { SpecificSubset := {0,2,3,4,6,7 }};//@sic R5s220669 sic@

var SubFrameTiming_Type v_Timing;

var boolean v_IsDataRecvd := false;  //@sic R5-232641 R5s230370 sic@

f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID);

//Restrict the SS to use of the HARQ proccess id used by the test cases

v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); //@sic R5s220805 sic@

f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_DciDlInfo_REQ(nr_Cell1,cs_TimingInfo_NR(v_Timing),

cs_NR_DciDlInfo(cs_NR_DciFormat_1_X_ResourceAssignment(-,-,-,-,v_NR_HarqProcessConfig_DL),

-, -)));//@sic R5s220669 sic@

//@siclog "Step 3" siclog@

//SS transmits RRCConnectionReconfiguration message to E-UTRA PTC containing NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell

//Send NR RRCreconfiguration message to E-UTRA PTC

//Note: it is assumed that the timing can be achieved without timed sending

//Set PDCP parameter discardTimer to 60ms

v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id,

-,

v_T_PollRetransmit,

-,

-,

-,

-,

v_T_Reassembly,

-,

true_); // @sic R5-197023 sic@

f_NR_SS_CommonRadioBearerConfig(nr_Cell1,{v_SS_DRB_Config});//@sic R5s200855 sic@

f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config, true, -, false); //@sic R5s200855 R5-213132 R5s210950 R5s220805 R5-227518 sic@

//f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config(-, omit), true, -, false);

//EXCEPTION: Void

//@sic R5-213132 R5-222044 sic@

EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value;

f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value));

f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR);   //@sic R5s230860 sic@

v_IsDataRecvd := f_TC_7_1_1_9_1_Step4Aato9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode); //@sic R5s200855 R5s220838 sic@

f_TC_7_1_1_9_1_Step16Ato21(p_NR_DRB_Id, v_LCID, v_IsDataRecvd);//@sic R5s200855 sic@

//@siclog "Step 22-23" siclog@

//The SS transmits NR RRCReconfiguration message to perform SCG change with reconfigurationWithSync with the same PSCell.

//@sic R5s210950 sic@

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config});

f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config,true); //@sic R5-197023, R5s200855 R5-227518 sic@

//f_NR_RLC_ReconfigurationHO_ENDC (nr_Cell1, v_RLC_BearerConfigList, cs_38508_PDCP_Config( -, omit),true);

f_TC_7_1_1_9_1_Step24to29(p_NR_DRB_Id, v_LCID,v_SS_DrbConfigList_TestMode);   //@sic R5s200855 sic@

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {v_SS_DRB_Config});                 //@sic R5s220805 sic@

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, -, -, -, -, cs_NR_HarqProcessConfig_AnyProcess);  //@sic R5s230680 sic@

};

## Change 2

| Function name | fl_TC_7_1_1_9_1_ENDC_TestBody |
| --- | --- |
| Reason for change | The change mentioned in Change 1 necessitates the handling of the optional MAC PDU at Step 4A within function fl_TC_7_1_1_9_1_ENDC_TestBody as f_TC_7_1_1_9_1_Step5to9A is common between NR5GC and ENDC components |
| Summary of change | Moved the handling of the optional MAC PDU at Step 4A to the function fl_TC_7_1_1_9_1_ENDC_TestBody from f_TC_7_1_1_9_1_Step5to9A |
| TTCN module | MAC_ENDC_NR.ttcn |
| MCC160 Comment | See change 1 |

Before Change:

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC { <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; //@sic R5-232641 R5s230370 sic@ f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); <<SKIPPED CODE>> //@sic R5-213132 R5-222044 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); //@sic R5s230860 sic@ v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode); //@sic R5s200855 R5s220838 sic@ <<SKIPPED CODE>>

After Change:

function fl_TC_7_1_1_9_1_ENDC_TestBody(DRB_Identity p_NR_DRB_Id) runs on ENDC_NR_PTC { <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; //@sic R5-232641 R5s230370 sic@ //WA var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; f_TC_7_1_1_9_1_Step0to2(p_NR_DRB_Id,v_LCID); <<SKIPPED CODE>> //@sic R5-213132 R5-222044 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR); //@sic R5s230860 sic@ // Initialization of RLC variables WA# v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } } v_IsDataRecvd := f_TC_7_1_1_9_1_Step5to9A(p_NR_DRB_Id,v_LCID, v_SS_DrbConfigList_TestMode, -, v_IsDataRecvd); //@sic R5s200855 R5s220838 sic@ WA# <<SKIPPED CODE>>

## Change 3

| Function name | f_TC_7_1_1_9_1_Step5to9A |
| --- | --- |
| Reason for change | A conformant UE would request for an UL grant to transmit the optional MAC PDU at Step 4A as soon as it has received the DL MAC PDU at Step 1. However, due to configurations involving RRC Reconfiguration procedure at Step 3 – 4 and the SS local end configuration involving the RACH procedure config after the RRC Reconfiguration procedure, the UE tries to send the optional MAC PDU at Step 4A using the UL grant procedure started in function f_NR_RLC_ReconfigurationHO_Steps1_8 However, there is no handling for the MAC PDU in function fl_TC_7_1_1_9_1_NR_TestBody as this MAC PDU is expected to be handled in function f_TC_7_1_1_9_1_Step5to9A and this leads to the MAC PDU being handled by the TTCN default handler and causes TC failure |
| Summary of change | To cater for all UEs, it is recommended to move the handling of the optional MAC PDU at Step 4A to the function fl_TC_7_1_1_9_1_NR_TestBody and fl_TC_7_1_1_9_1_ENDC_TestBody from f_TC_7_1_1_9_1_Step5to9A |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | See change 1 |

Before Change:

function f_TC_7_1_1_9_1_Step5to9A ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true ) runs on NR_BASE_PTC return boolean { //@sic R5-206320 sic@ <<SKIPPED CODE>> var boolean v_IsDataRecvd := false; var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } } <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_9_1_Step5to9A ( DRB_Identity p_NR_DRB_Id, B6_Type p_LCID, template (value) NR_RadioBearerList_Type p_NR_RadioBearerList, boolean p_StopGrant:= true, boolean p_IsDataRecvd //WA# ) runs on NR_BASE_PTC return boolean { //@sic R5-206320 sic@ <<SKIPPED CODE>> var boolean v_IsDataRecvd := p_IsDataRecvd; //WA# /*REMOVED var NR_RLC_SS_State_Type v_RLC_Rec; timer t_Watchdog; // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); v_RLC_Rec.AM_RX_Next := 1; // @siclog "Step 4Aa" siclog@ //@sic R5-232641 R5s230370 sic@ t_Watchdog.start(1.0); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, cr_TimingInfo_Any, ?)) { f_NR_TxSTATUS_PDU(v_RLC_Rec, p_NR_DRB_Id); // @siclog "UE has looped back step1 data" siclog@ v_IsDataRecvd := true; t_Watchdog.stop; } [] t_Watchdog.timeout { } }*/ <<SKIPPED CODE>>