3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240007

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3297 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Implementation of the Prose CR R5-236453 has been done only for the peer signalling RRC Reconfiguration (NR5GC) message and RRC Connection Reconfiguration (ENDC) message. The local end SS configuration is not updated with the PUSCH Time Domain Allocation list for the initial UL BWP. This should also be updated to inform SS stack of the updated K2 values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated function f_NR_SS_BWP_List_UL to initialise the PUSCH Time Domain Allocation List for the initial UL BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1.NR5GC, 7.1.1.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_NR_SS_BWP_List_UL |
| --- | --- |
| Reason for change | Implementation of the Prose CR R5-236453 has been done only for the peer signalling RRC Reconfiguration (NR5GC) message and RRC Connection Reconfiguration (ENDC) message. The local end SS configuration is not updated with the PUSCH Time Domain Allocation list for the initial UL BWP. This should also be updated to inform SS stack of the updated K2 values. |
| Summary of change | Updated function f_NR_SS_BWP_List_UL to initialise the PUSCH Time Domain Allocation List for the initial UL BWP |
| TTCN module | MAC_BWP_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_SS_BWP_List_UL(NR_CellId_Type p_NR_CellId, template (value) UplinkConfig.uplinkBWP_ToAddModList p_DedicatedBwpListUL) runs on NR_BASE_PTC return template (value) NR_UplinkBWP_List_Type { var BWP_Uplink v_BWP_Uplink; var template (value) NR_UplinkBWP_List_Type v_UplinkBWP_List := { f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, tsc_NR_BWP_Id_Initial) }; // initial BWP as stored for the cell var integer k := lengthof(v_UplinkBWP_List); var integer i; for (i := 0; i < lengthof(p_DedicatedBwpListUL); i := i + 1) { v_BWP_Uplink := valueof(p_DedicatedBwpListUL[i]); // valueof to get around useless compiler warnings v_UplinkBWP_List[i+k] := cs_NR_UplinkBWP(v_BWP_Uplink.bwp_Id, v_BWP_Uplink.bwp_Common, v_BWP_Uplink.bwp_Dedicated); } return v_UplinkBWP_List; }

After Change:

function f_NR_SS_BWP_List_UL(NR_CellId_Type p_NR_CellId, template (value) UplinkConfig.uplinkBWP_ToAddModList p_DedicatedBwpListUL) runs on NR_BASE_PTC return template (value) NR_UplinkBWP_List_Type { var BWP_Uplink v_BWP_Uplink; var template (value) NR_UplinkBWP_List_Type v_UplinkBWP_List := { f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, tsc_NR_BWP_Id_Initial) }; // initial BWP as stored for the cell var integer k := lengthof(v_UplinkBWP_List); var integer i; //WA# v_BWP_Uplink := valueof(p_DedicatedBwpListUL[0]); v_UplinkBWP_List[0].Dedicated := { R15 := v_BWP_Uplink.bwp_Dedicated } for (i := 0; i < lengthof(p_DedicatedBwpListUL); i := i + 1) { v_BWP_Uplink := valueof(p_DedicatedBwpListUL[i]); // valueof to get around useless compiler warnings v_UplinkBWP_List[i+k] := cs_NR_UplinkBWP(v_BWP_Uplink.bwp_Id, v_BWP_Uplink.bwp_Common, v_BWP_Uplink.bwp_Dedicated); } return v_UplinkBWP_List; }