3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240029

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3309 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.6.1.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per prose change done in R5-223995, to enquire the connEstFailReportReq and rlf-Report at the end of the TC, additional steps 18 and 19 have been added and listed as implemented in 22wk37 TTCN drop but missed out to be part of any TTCN drop |  |  |  |  |  |  |  |  |
| Summary of change: |  | Steps 18 and 19 have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Jerry Wang
	jerry.wang@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_6_1_4_4_TestBody |
| --- | --- |
| Reason for change | As per prose change done in R5-223995, to enquire the connEstFailReportReq and rlf-Report at the end of the TC, additional steps 18 and 19 have been added and listed as implemented in 22wk37 TTCN drop but missed out to be part of any TTCN drop. |
| Summary of change | Steps 18 and 19 have been added as per R5-223995 requirement. |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_6_1_4_4_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), // @sic R5-220613 sic@ cs_NR_CellPower(nr_Cell2, -80, -82) // @sic R5-220613 R5-234797 sic@ }; //@siclog "Step 1" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 3" siclog@ //The SS waits for 2sec to ensure that the UE detects T300 expiry. f_Delay(2.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 7" siclog@ //The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(-, -, -, true_)))); // @sic R5-225389 sic@ //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 12" siclog@ //The SS changes Cell 1 and Cell 2 parameters according to the row "T1"in table 8.1.6.1.4.4.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 13" siclog@ //The UE transmits an RRCReestablishmentRequest message on Cell 2. f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell2, otherFailure); //@sic R5-221467 sic@ //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReestablishment message on Cell 2. //Check: Does the UE send an RRCReestablishmentComplete message with connEstFailInfoAvailable on Cell 2? v_RRCReestablishmentComplete_v1610_IEs := { ue_MeasurementsAvailable_r16 := cr_UE_MeasurementsAvailable_r16(-, -, -, true_, true_), // @sic R5-215681 sic@ nonCriticalExtension := * }; f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell2, -, -, v_RRCReestablishmentComplete_v1610_IEs); // @sic R5s211281 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Steps 16-17" siclog@ //@sic R5-215681, R5s210873 sic@ //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationtComplete message. f_NR5GC_RRCReestablishment_Steps8To13(nr_Cell1, nr_Cell2); }

After Change:

function fl_TC_8_1_6_1_4_4_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), // @sic R5-220613 sic@ cs_NR_CellPower(nr_Cell2, -80, -82) // @sic R5-220613 R5-234797 sic@ }; //@siclog "Step 1" siclog@ //The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Step 2" siclog@ //The UE transmits an RRCSetupRequest message. f_NR_RRC_SetupRequest_Def(nr_Cell1); //@siclog "Step 3" siclog@ //The SS waits for 2sec to ensure that the UE detects T300 expiry. f_Delay(2.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Step 7" siclog@ //The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(-, -, -, true_)))); // @sic R5-225389 sic@ //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 12" siclog@ //The SS changes Cell 1 and Cell 2 parameters according to the row "T1"in table 8.1.6.1.4.4.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 13" siclog@ //The UE transmits an RRCReestablishmentRequest message on Cell 2. f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell2, otherFailure); //@sic R5-221467 sic@ //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReestablishment message on Cell 2. //Check: Does the UE send an RRCReestablishmentComplete message with connEstFailInfoAvailable on Cell 2? v_RRCReestablishmentComplete_v1610_IEs := { ue_MeasurementsAvailable_r16 := cr_UE_MeasurementsAvailable_r16(-, -, -, true_, true_), // @sic R5-215681 sic@ nonCriticalExtension := * }; f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell2, -, -, v_RRCReestablishmentComplete_v1610_IEs); // @sic R5s211281 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); //@siclog "Steps 16-17" siclog@ //@sic R5-215681, R5s210873 sic@ //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationtComplete message. f_NR5GC_RRCReestablishment_Steps8To13(nr_Cell1, nr_Cell2); //@siclog "Step 18" siclog@ //The SS sends a UEInformationRequest message to get connEstFailReportReq and rlf-Report. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, true_, -, true_))); //@siclog "Step 19" siclog@ //Check: Does the UE send a UEInformationResponse message with connEstFailReport and rlf-Report? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def, -, -, -, ?, -, ?)));