3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240042

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3317 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_RRC_ProcedureDelayWithRACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of function : f_NR5GC_RRC_ProcedureDelayWithRACH, - p_Nms = 16 case is not considered for the calculation of v_DeltaTDLmax ( this is necessary for the test case 8.1.5.8.2.x ) - v_T1 is always calculated in this function , but where as in the case of 8.1.5.8.2.x , v_T1 is already given as part of the p_SRB_COMMON_REQ message . so v_T1 should be calculated only when p_SRB_COMMON_REQ message is having immediate activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - p_Nms = 16 case is considered for the calculation of v_DeltaTDLmax - v_T1 is calculated only when incoming p_SRB_COMMON_REQ message is having immediate activation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.2.1.NR5GC, 8.1.5.8.2.2.NR5GC, 8.1.5.8.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the Function f_NR5GC_RRC_ProcedureDelayWithRACH	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the Function f_NR5GC_RRC_ProcedureDelayWithRACH

| Function name | f_NR5GC_RRC_ProcedureDelayWithRACH |
| --- | --- |
| Reason for change | In the current TTCN implementation of function : f_NR5GC_RRC_ProcedureDelayWithRACH, - p_Nms = 16 case is not considered for the calculation of v_DeltaTDLmax ( this is necessary for the test case 8.1.5.8.2.x ) - v_T1 always calculated in this function , but where as in the case of 8.1.5.8.2.x , v_T1 is already given as part of the p_SRB_COMMON_REQ message . so v_T1 should be calculated only when p_SRB_COMMON_REQ message is having immediate activation. |
| Summary of change | - p_Nms = 16 case is considered for the calculation of v_DeltaTDLmax - v_T1 is calculated only when incoming p_SRB_COMMON_REQ message is having immediate activation |
| TTCN module | Common\NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_ProcedureDelayWithRACH(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { //@sic R5-237419 renamed function to f_NR5GC_RRC_ProcedureDelayWithRACH sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var boolean v_RACH_ProcedureStarted := false; var integer v_DeltaTDLmax := 0; var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; timer t_Watchdog; // The SS transmits an RRCReestablishment/RRCResume message on slot #i in the radio frame,where: // i = 1 for micro=0,1 // i = 21 for micro=3 v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024; select (v_SCS) { case (kHz15) { v_T1.Subframe.Number := 1; v_T1.Slot.SlotOffset.Numerology0 := true; if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)) { if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 1; } else { v_T1.Subframe.Number := 5; } } } case (kHz30) { v_T1.Subframe.Number := 0; v_T1.Slot.SlotOffset.Numerology1 := 1; } case (kHz120) { v_T1.Subframe.Number := 2; v_T1.Slot.SlotOffset.Numerology3 := 5; if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 5; v_T1.Subframe.Number := 2; } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS");} } v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; v_DeltaTDLmax := 7; if (f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId)) { select (p_Nms) { //@sic R5-232191 R5-237429 sic@ case (10) {v_DeltaTDLmax := 2;} case (6) {v_DeltaTDLmax := 6;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; select (p_Nms) { //@sic R5-237429 sic@ case (10) {v_DeltaTDLmax := 7;} case (6) {v_DeltaTDLmax := 11;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; select (p_Nms) { //@sic R5-237429 sic@ case (10) {v_DeltaTDLmax := 17;} case (6) {v_DeltaTDLmax := 25;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; select (p_Nms) { //@sic R5-237429 sic@ case (10) {v_DeltaTDLmax := 32;} case (6) {v_DeltaTDLmax := 24;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } //Calculate T3 - UL grant when CSS is used v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //Enable RACH preamble reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, -, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } //@sic R5s221179 sic@ /* UE initiates RACH procedure */ [] SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)) { f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable), -, tsc_NoCnfReq); // @sic R5s220683 sic@ v_RACH_ProcedureStarted := true; repeat; //@sic R5s220813 sic@ } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; if (not v_RACH_ProcedureStarted) { // No RACH procedure was initiated by UE // Timing calculation as per 38.523-3 clause 7.3.7.2 //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } } else { // RACH procedure was initiated by UE // Timing calculation as per 38.523-3 clause 7.3.7.3 v_T1Slots := (((v_T1.SFN.Number * 10) + v_T1.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T1) + v_MaxNumberSlots; //@sic R5s220683 sic@ if ( (v_T4Slots - v_T1Slots) <= ( v_Nslots + v_DeltaTDLmax) ) { //@sic R5s220683 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } } return v_ReceivedNR_SRB_COMMON_IND; }

After Change:

function f_NR5GC_RRC_ProcedureDelayWithRACH(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { //@sic R5-237419 renamed function to f_NR5GC_RRC_ProcedureDelayWithRACH sic@ var integer v_T1Slots; var integer v_T2Slots; var integer v_T3Slots; var integer v_T4Slots; var SubFrameTiming_Type v_T1; // Send DL Msg var SubFrameTiming_Type v_T2; // HARQ ACK receive var SubFrameTiming_Type v_T3; // Start UL Grant start var SubFrameTiming_Type v_T4; // UL RRC PDU received var SubFrameTiming_Type v_Tnow; var integer v_SlotsNumberPerSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_Nslots := p_Nms*v_SlotsNumberPerSubframe + 1; //@sic R5s210842 change 2.1 sic@ var integer v_Delta1; var integer v_Delta2; var integer v_GrantPeriodicity; var integer v_DL_RetransmissionCnt := 0; var NR_SYSTEM_IND v_SYS_IND; var boolean v_T2Flag := false; var boolean v_T4Flag := false; var integer v_K1; var integer v_K2 := 4; var NR_SRB_COMMON_IND v_ReceivedNR_SRB_COMMON_IND; var integer v_MaxNumberSlots := 1024 * 10 * v_SlotsNumberPerSubframe; // Maximum number per SFN complete cycle var boolean v_RACH_ProcedureStarted := false; var integer v_DeltaTDLmax := 0; var template (value) NR_SRB_COMMON_REQ v_SRB_COMMON_REQ := p_SRB_COMMON_REQ; timer t_Watchdog; // The SS transmits an RRCReestablishment/RRCResume message on slot #i in the radio frame,where: // i = 1 for micro=0,1 // i = 21 for micro=3 if (f_TimingInfo_IsNow(v_SRB_COMMON_REQ.Common.TimingInfo)) { //WA#WI=1229735 v_T1 := f_NR_GetNextSendOccasion_DL(p_NR_CellId); v_T1.SFN.Number := (v_T1.SFN.Number + 1) mod 1024; select (v_SCS) { case (kHz15) { v_T1.Subframe.Number := 1; v_T1.Slot.SlotOffset.Numerology0 := true; if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)) { if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 1; } else { v_T1.Subframe.Number := 5; } } } case (kHz30) { v_T1.Subframe.Number := 0; v_T1.Slot.SlotOffset.Numerology1 := 1; } case (kHz120) { v_T1.Subframe.Number := 2; v_T1.Slot.SlotOffset.Numerology3 := 5; if (p_Nms == 6) { //pc_reducedCP_Latency @sic R5-237357 sic@ v_T1.Subframe.Number := 5; //v_T1.Subframe.Number := 2; //WA#WI=1229735 } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS");} } v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame := v_T1; } else{ //WA#WI=1229735 v_T1 := valueof(v_SRB_COMMON_REQ.Common.TimingInfo.SubFrame); } v_T1Slots := f_NR_ConvertTiming_To_TimingInSlots(v_T1, v_SlotsNumberPerSubframe); // Initialise GrantPeriodicity, Delta1, K1 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_GrantPeriodicity := 1; // Grant repeated in every slot v_Delta1 := 0; v_K1 := 7; v_DeltaTDLmax := 7; if (f_NR_CellInfo_GetHalfDuplexFDD(p_NR_CellId)) { select (p_Nms) { //@sic R5-232191 R5-237429 sic@ case (10,16) {v_DeltaTDLmax := 2;} //WA#WI=1229735 case (6) {v_DeltaTDLmax := 6;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } } else { // TDD select(v_SCS) { case(kHz15) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5 - (v_T1Slots + v_Nslots)) mod 5; v_K1 := 7; select (p_Nms) { //@sic R5-237429 sic@ case (10,16) {v_DeltaTDLmax := 7;} //WA#WI=1229735 case (6) {v_DeltaTDLmax := 11;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case(kHz30) { v_GrantPeriodicity := 10; //Grant repeated every 10 slots v_Delta1 := (4 - (v_T1Slots + v_Nslots) mod 10) mod 10; v_K1 := 5; select (p_Nms) { //@sic R5-237429 sic@ case (10,16) {v_DeltaTDLmax := 17;} //WA#WI=1229735 case (6) {v_DeltaTDLmax := 25;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case(kHz120) { v_GrantPeriodicity := 5; //Grant repeated every 5 slots v_Delta1 := (5- (v_T1Slots + v_Nslots) mod 5) mod 5; v_K1 := 5; select (p_Nms) { //@sic R5-237429 sic@ case (10,16) {v_DeltaTDLmax := 32;} //WA#WI=1229735 case (6) {v_DeltaTDLmax := 24;} case else { FatalError(__FILE__, __LINE__, "Invalid Nsf"); } } } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } //Calculate T3 - UL grant when CSS is used v_T3Slots := v_T1Slots + v_Nslots + v_Delta1; v_T3 := f_NR_ConvertTimingInSlots_To_Timing(v_T3Slots, v_SCS, v_SlotsNumberPerSubframe); //T1: send DL Msg SRB.send(v_SRB_COMMON_REQ); //Enable HARQ ack/nack reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(enable), cs_TimingInfo_NR(v_T1)); //@sic R5s210842 change 2.4 sic@ //Enable RACH preamble reporting f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(enable)); //T3 configure SS with periodic UL Grant at T3 f_NR_ULGrantConfiguration_Start(p_NR_CellId, cs_TimingInfo_NR(v_T3), -, -, -, cs_UL_GrantConfig_ContPeriodic(v_GrantPeriodicity)); // Start WatchDog timer v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); t_Watchdog.start (int2float(f_SubFrameTiming_Duration(v_Tnow, v_T1) + p_Nms + 50) / 1000.0); /* 50ms added to be at the safe side */ alt { /* HARQ NACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) { v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1; if (v_DL_RetransmissionCnt > 4) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4"); } else { repeat; } } /* HARQ ACK */ []SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND { v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame; f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq); /* disable indication of HARQ ack/nack; @sic R5s220683 sic@ */ v_T2Flag := true; repeat; } /* HARQ error indication */ [] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) { f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test"); t_Watchdog.stop; } //@sic R5s221179 sic@ /* UE initiates RACH procedure */ [] SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)) { f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable), -, tsc_NoCnfReq); // @sic R5s220683 sic@ v_RACH_ProcedureStarted := true; repeat; //@sic R5s220813 sic@ } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } } //Stop UL Grant f_NR_ULGrantConfiguration_Stop(p_NR_CellId); if (not v_T2Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL ACK not received"); } if (not v_T4Flag) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "UL Msg not received"); } v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; if (not v_RACH_ProcedureStarted) { // No RACH procedure was initiated by UE // Timing calculation as per 38.523-3 clause 7.3.7.2 //Check the Delay v_T2Slots := (((v_T2.SFN.Number * 10) + v_T2.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T2) + v_MaxNumberSlots; v_T4Slots := (((v_T4.SFN.Number * 10) + v_T4.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T4) + v_MaxNumberSlots; //Initialise Delta2 if (f_NR_CellInfo_GetIsFDD(p_NR_CellId)){ v_Delta2 := 0; } else { //TDD select(v_SCS) { case(kHz15) { v_Delta2 := (5 - (v_T2Slots - v_K1 + v_Nslots)) mod 5; } case(kHz30) { v_Delta2 := (4 - (v_T2Slots - v_K1 + v_Nslots) mod 10) mod 10; } case(kHz120) { v_Delta2 := (5- (v_T2Slots - v_K1 + v_Nslots) mod 5) mod 5; } case else { FatalError(__FILE__, __LINE__, "Invalid SCS"); } } } if ( ((v_T4Slots - v_K2) - (v_T2Slots - v_K1))mod v_MaxNumberSlots <= (v_Nslots + v_Delta2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } } else { // RACH procedure was initiated by UE // Timing calculation as per 38.523-3 clause 7.3.7.3 v_T1Slots := (((v_T1.SFN.Number * 10) + v_T1.Subframe.Number) * v_SlotsNumberPerSubframe) + f_NR_SubFrameTiming_GetSlotNumber(v_T1) + v_MaxNumberSlots; //@sic R5s220683 sic@ if ( (v_T4Slots - v_T1Slots) <= ( v_Nslots + v_DeltaTDLmax) ) { //@sic R5s220683 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestcaseStep); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep); } } return v_ReceivedNR_SRB_COMMON_IND; }