3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240059

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3329 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of the function fl_TC_9_1_7_1_Body at Step 9 – 10 , as part of function f_NR_RRCRelease_Local, at T+60 f_NR_SS_SRBs_DRBs_Config is performed. right after this at the same time f_NR_ULGrantConfiguration_Start configuration is also done . It is proposed to configure f_NR_ULGrantConfiguration_Start at T+10 from f_NR_SS_SRBs_DRBs_Config, so that SS can process the RB configuration in time and continue with the UL grant configuration configuration afterwards |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to configure f_NR_ULGrantConfiguration_Start at T+10 from f_NR_SS_SRBs_DRBs_Config, so that SS can process the RB configuration in time and continue with the UL grant configuration configuration afterwards |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase stability issues will be seen during UL grant configuration . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the Function fl_TC_9_1_7_1_Body	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the Function fl_TC_9_1_7_1_Body

| Function name | fl_TC_9_1_7_1_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function fl_TC_9_1_7_1_Body at Step 9 – 10 , as part of function f_NR_RRCRelease_Local, at T+60 f_NR_SS_SRBs_DRBs_Config is performed. right after this at the same time f_NR_ULGrantConfiguration_Start configuration is also done . It is proposed to configure f_NR_ULGrantConfiguration_Start at T+10 from f_NR_SS_SRBs_DRBs_Config, so that SS can process the RB configuration in time and continue with the UL grant configuration configuration afterwards |
| Summary of change | It is proposed to configure f_NR_ULGrantConfiguration_Start at T+10 from f_NR_SS_SRBs_DRBs_Config, so that SS can process the RB configuration in time and continue with the UL grant configuration configuration afterwards |
| TTCN module | NR5GC\9_1_7\Service_Request_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_7_1_Body() runs on NR5GC_PTC { << SKIPPED CODE >> []t_T3517.timeout { //@sic R5s210839 sic@ v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA, tsc_NR_DelayBeforeIntraCellHO)); //@sic R5s200087, R5s200662 sic@, R5s220551 sic@ f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //UL grant already configured with activation time v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, -, -, -, -, -, false); //@sic R5s200087 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cr_NAS_KeySetIdentifier(v_KeySetId, tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else //@sic R5s200087 sic@ { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } } << SKIPPED CODE >>

After Change:

import from Common4G5G_Timing all; //WA#WI=1227913 function fl_TC_8_2_3_8_1x_TestBody(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { << SKIPPED CODE >> []t_T3517.timeout { //@sic R5s210839 sic@ v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA, tsc_NR_DelayBeforeIntraCellHO)); //@sic R5s200087, R5s200662 sic@, R5s220551 sic@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 10); //WA#WI=1227913 f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //UL grant already configured with activation time v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, -, -, -, -, -, false); //@sic R5s200087 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cr_NAS_KeySetIdentifier(v_KeySetId, tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else //@sic R5s200087 sic@ { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } } << SKIPPED CODE >>