3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240064

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3331 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT testcase 9.1.9.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per prose change in the attached Prose draft, TAU shall be performed: without including the UE radio capability information update needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Steps 4 check on TAU has been removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | As per prose change in the attached Prose draft, TAU shall be performed: without including the UE radio capability information update needed |
| Summary of change | Steps 4 check on TAU has been removed |
| TTCN module | RACS_IRAT_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; // @siclog "Step 1" siclog@ //The SS configures:- NGC Cell A as "Serving cell"- E-UTRA Cell A as "Non-suitable "off" cell". f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); f_EUTRA38_SetCellPower(eutra_CellA, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 2" siclog@ //Check: Does the UE perform test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1 on NGC Cell A? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_CellA); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 3" siclog@ //The SS configures:- E-UTRA Cell A as "Serving cell"- NGC Cell A as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 4" siclog@ //Check: Does the UE perform Test procedure for UE Tracking area updating for Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1, 'connected without release' on E-UTRA Cell A? v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_CellA,tsc_RRC_TI_Def); if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapNeeded, cr_UeRadioCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU req failed"); } f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_CellA, v_NasInd, -, noRrcConnectionRelease); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4"); }

After Change:

function fl_TC_9_1_9_7_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NAS_MSG_Indication_Type v_NasInd; // @siclog "Step 1" siclog@ //The SS configures:- NGC Cell A as "Serving cell"- E-UTRA Cell A as "Non-suitable "off" cell". f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); f_EUTRA38_SetCellPower(eutra_CellA, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 2" siclog@ //Check: Does the UE perform test procedure for UE Tracking area updating for Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.9.2.2-1 on NGC Cell A? f_EUTRA38_NR5GC_S1ToN1_IdleMode(eutra_CellA); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 3" siclog@ //The SS configures:- E-UTRA Cell A as "Serving cell"- NGC Cell A as "Non-suitable "off" cell". f_EUTRA38_SetCellPower(eutra_CellA, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_NR_WaitForCoOrd_Trigger(NR);// @sic R5s221235 sic@ // @siclog "Step 4" siclog@ //Check: Does the UE perform Test procedure for UE Tracking area updating for Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode as described in TS 38.508-1 [4], Table 4.9.7.2.2-1, 'connected without release' on E-UTRA Cell A? v_NasInd := f_EUTRA38_TAUReqFromN1(eutra_CellA,tsc_RRC_TI_Def); // Removed /* if (not match (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapNeeded, cr_UeRadioCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU req failed"); }*/ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(eutra_CellA, v_NasInd, -, noRrcConnectionRelease); f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4"); }