3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240066

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3332 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 11.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, function f_TC_11_4_3_NR5GC f_UT_USIM_Remove(UT) is called. This sets the parameter p_Description to OMIT. This causes a runtime error when function f_UT_USIM_Remove fl_UT_ApplyCommand is called. What happens is that during the call TC_11_4_3_NR5GC f_MTC_MainLoop_NR5GC a_MTC_ReceiveFromPTC_SendToUT fl_UT_HandleCmd for the case REMOVE_USIM the variable v_ParamList remains uninitialized because of the setting above. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling f_UT_USIM_Remove(UT), called f_UT_USIM_Remove(UT,-, "Remove Primary USIM card"); |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

  - Correction to function f_TC_11_4_3_NR5GC

| Function name | f_TC_11_4_3_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation, function f_TC_11_4_3_NR5GC f_UT_USIM_Remove(UT) is called. This sets the parameter p_Description to OMIT. This causes a runtime error when function f_UT_USIM_Remove fl_UT_ApplyCommand is called. What happens is that during the call TC_11_4_3_NR5GC f_MTC_MainLoop_NR5GC a_MTC_ReceiveFromPTC_SendToUT fl_UT_HandleCmd for the case REMOVE_USIM the variable v_ParamList remains uninitialized because of the setting above. This needs to be addressed. |
| Summary of change | Instead of calling f_UT_USIM_Remove(UT), called f_UT_USIM_Remove(UT,-, "Remove Primary USIM card"); |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_TC_11_4_3_NR5GC() runs on NR5GC_PTC

{

……

f_UT_USIM_Remove(UT);

……

}<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_11_4_3_NR5GC() runs on NR5GC_PTC

{

……

f_UT_USIM_Remove(UT,-, "Remove Primary USIM card");

……

}<<SKIPPED CODE>>