3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240073

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4773 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE test case 13.1.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-11-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, a failure is observed at f_UTRAN_CS_Fallback_WithHandover f_UTRAN_UE_CapabilityInformation cs_108_UE_CapabilityEnq (v_SecurityInfo.dl_IntegrityCheckInfo). This is because in the testflow priotr to above call, v_SecurityInfo.dl_IntegrityCheckInfo is not initialized at all. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Before calling f_UTRAN_CS_Fallback_WithHandover, called the following: v_SecurityInfo.dl_IntegrityCheckInfo := valueof(cs_IntegrityCheckInfo0);//WA#13.1.16 f_UTRAN_Security_Set(v_SecurityInfo);//WA#13.1.16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.1.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_13_1_16_UTRAN |
| --- | --- |
| Reason for change | In the current TTCN implementation, a failure is observed at f_UTRAN_CS_Fallback_WithHandover f_UTRAN_UE_CapabilityInformation cs_108_UE_CapabilityEnq (v_SecurityInfo.dl_IntegrityCheckInfo). This is because in the testflow priotr to above call, v_SecurityInfo.dl_IntegrityCheckInfo is not initialized at all. This needs to be addressed. |
| Summary of change | Before calling f_UTRAN_CS_Fallback_WithHandover, called the following: v_SecurityInfo.dl_IntegrityCheckInfo := valueof(cs_IntegrityCheckInfo0);//WA#13.1.16 f_UTRAN_Security_Set(v_SecurityInfo);//WA#13.1.16 |
| TTCN module | LTE_IRAT\ COMMON\ UTRAN\ UTRAN_IRAT_CommonProcedures.ttcn |

Before Change:

| function f_TC_13_1_16_UTRAN() runs on UTRAN_IRAT_PTC { var integer v_T0_CPICH_Cell5 := -70; var integer v_T0_PCCPCH_Cell5 := -72; //Initialise all cells f_UTRAN_Init (EUTRA_UTRAN); //Create and configure Cell 5 //Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH (utran_Cell5); //Start sending System Information f_UTRAN_SendDefSysInfo (utran_Cell5); //Send relevant UTRAN cell info to EUTRA PTC f_UTRAN_Send_IR_SysInfoToEUTRA (utran_Cell5); // f_UTRAN_Pre_Preamble (utran_Cell5, NotSpecified); // @sic R5s110776 R5-131302 sic@ //Configure a dedicated channel in the UTRAN FDD with configuration according to 34.108 cl. 6.10.2.4.1.26 f_SS_Config_64kPS_DCH4_RAB (utran_Cell5); //@sic R5s120111 sic@ //Wait for preamble and then set appropiate cell power level f_IRAT_WaitForCoOrd_Trigger (EUTRA); f_UTRAN_SetCellPower(utran_Cell5, v_T0_CPICH_Cell5, v_T0_PCCPCH_Cell5); f_UTRAN_TestBody_Set(true); //Receive authentication parameters and continue with the test case f_UTRAN_InterRAT_InitialiseAuthParams(); //Configure ciphering in SS before HO f_UTRAN_ConfigureCipheringBeforeHO (utran_Cell5); //Notify E-UTRA ready to send Mobility from EUTRA f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); //@siclog "Step 6-13" siclog@ //Perfrom 36.508 cl. 6.4.3.7.7 - CS fallback to UTRAN with Handover / emergency call f_UTRAN_CS_Fallback_WithHandover(utran_Cell5, false, true); //Notify E-UTRA end of test body f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); f_UTRAN_TestBody_Set(false); //Switch/Power off UE and release cell f_UTRAN_Postamble (utran_Cell5, U5_CSCALL); } |  |
| --- | --- |

After Change:

function f_TC_13_1_16_UTRAN() runs on UTRAN_IRAT_PTC { var integer v_T0_CPICH_Cell5 := -70; var integer v_T0_PCCPCH_Cell5 := -72; //Initialise all cells f_UTRAN_Init (EUTRA_UTRAN); //Create and configure Cell 5 //Cell started switched on with default power level, -60 and max attenuation f_UTRAN_SS_CreateCellDCH (utran_Cell5); //Start sending System Information f_UTRAN_SendDefSysInfo (utran_Cell5); //Send relevant UTRAN cell info to EUTRA PTC f_UTRAN_Send_IR_SysInfoToEUTRA (utran_Cell5); // f_UTRAN_Pre_Preamble (utran_Cell5, NotSpecified); // @sic R5s110776 R5-131302 sic@ //Configure a dedicated channel in the UTRAN FDD with configuration according to 34.108 cl. 6.10.2.4.1.26 f_SS_Config_64kPS_DCH4_RAB (utran_Cell5); //@sic R5s120111 sic@ //Wait for preamble and then set appropiate cell power level f_IRAT_WaitForCoOrd_Trigger (EUTRA); f_UTRAN_SetCellPower(utran_Cell5, v_T0_CPICH_Cell5, v_T0_PCCPCH_Cell5); f_UTRAN_TestBody_Set(true); //Receive authentication parameters and continue with the test case f_UTRAN_InterRAT_InitialiseAuthParams(); //Configure ciphering in SS before HO f_UTRAN_ConfigureCipheringBeforeHO (utran_Cell5); //Notify E-UTRA ready to send Mobility from EUTRA f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); v_SecurityInfo.dl_IntegrityCheckInfo := valueof(cs_IntegrityCheckInfo0);//WA#13.1.16 f_UTRAN_Security_Set(v_SecurityInfo);//WA#13.1.16 //@siclog "Step 6-13" siclog@ //Perfrom 36.508 cl. 6.4.3.7.7 - CS fallback to UTRAN with Handover / emergency call f_UTRAN_CS_Fallback_WithHandover(utran_Cell5, false, true); //Notify E-UTRA end of test body f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); f_UTRAN_TestBody_Set(false); //Switch/Power off UE and release cell f_UTRAN_Postamble (utran_Cell5, U5_CSCALL); }