3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240086

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3279 | rev | 1 | Current version: | 17.9.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_GetFrequencySA_f1tof4 for band n79 for redcap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.508-1 Table 6.2.3.1-5A for TDD band n79 for redcap UE, the Low&High frequency should be the same values as for Low and High range in clause 4.3.1.1.1.79 for bandwidth=20 MHz and SCS=30 kHz. Table 4.3.1.1.1.79-2 changed in TS38.508-1 i00. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the value according to TS 38.508-1 i00 Table 4.3.1.1.1.79-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All RedCap testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk37related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | f_NR_GetFrequencySA_f1tof4 |
| --- | --- |
| Reason for change | According to TS38.508-1 Table 6.2.3.1-5A for TDD band n79 for redcap UE, the Low&High frequency should be the same values as for Low and High range in clause 4.3.1.1.1.79 for bandwidth=20 MHz and SCS=30 kHz. Table 4.3.1.1.1.79-2 changed in TS38.508-1 i00. |
| Summary of change | Update the value according to TS 38.508-1 i00 Table 4.3.1.1.1.79-2. |
| TTCN module | NR_FrequencyInit_SA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_GetFrequencySA_f1tof4 ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return template (value) NR_Frequencyf1tof4_Type { var template (value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(); … case( 79 ) { //@sic R5-226863, R5-227702, R5s230175 sic@ if ( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 693792 , 79 , 693388 , 0 , kHz30 , 51 ), 20 , kHz30 , 20 , 1 , 12 , 2 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 51 , v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 706560 , 79 , 705990 , 12 , kHz30 , 51 ), 20 , kHz30 , 18 , 0 , 26 , 1 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 12 , kHz30 , 51 , v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 719424 , 79 , 718590 , 24 , kHz30 , 51 ), 20 , kHz30 , 18 , 0 , 48 , 0 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 24 , kHz30 , 51 , v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f4 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 732384 , 79 , 719960 , 504 , kHz30 , 51 ), 20 , kHz30 , 16 , 0 , 1014 , 3 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f4 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 51 , v_P_Max), 20 ); } else { //@sic R5-186848 R5-190650 R5-201020 R5-201024, R5-201984 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 694176 , 79 , 693392 , 0 , kHz30 , 273 ), 100 , kHz30 , 16 , 1 , 44 , 18 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 704928 , 79 , 704214 , 12 , kHz30 , 273 ), 100 , kHz30 , 18 , 1 , 38 , 6 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 12 , kHz30 , 273 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 717216 , 79 , 715038 , 24 , kHz30 , 273 ), 100 , kHz30 , 18 , 1 , 160 , 54 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 24 , kHz30 , 273 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f4 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 727968 , 79 , 714628 , 504 , kHz30 , 273 ), 100 , kHz30 , 20 , 1 , 1090 , 37 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f4 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 273 , v_P_Max), 100 ); } }… }

After Change:

function f_NR_GetFrequencySA_f1tof4 ( FreqBandIndicatorNR p_FreqBandIndicatorNR ) return template (value) NR_Frequencyf1tof4_Type { var template (value) NR_Frequencyf1tof4_Type v_NR_Frequencyf1tof4 := cs_NR_Frequencyf1tof4_Dummy; var P_Max v_P_Max := f_NR_InitP_Max(); … case( 79 ) { //@sic R5-226863, R5-227702, R5s230175 sic@ if ( pc_supportOfRedCap_r17 ){ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 693696 , 79 , 693388, 0 , kHz30 , 51 ), 20 , kHz30 , 20, 0, 4 , 2, 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 51 , v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 706560 , 79 , 705990 , 12 , kHz30 , 51 ), 20 , kHz30 , 18 , 0 , 26 , 1 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 12 , kHz30 , 51 , v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 719424 , 79 , 718590 , 24 , kHz30 , 51 ), 20 , kHz30 , 18 , 0 , 48 , 0 , 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 24 , kHz30 , 51 , v_P_Max), 20 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f4 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 732384 , 79 , 719958, 504 , kHz30 , 51 ), 20 , kHz30 , 18, 0 , 1014, 3, 0 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f4 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 51 , v_P_Max), 20 ); } else { //@sic R5-186848 R5-190650 R5-201020 R5-201024, R5-201984 sic@ v_NR_Frequencyf1tof4.FrequencyInfoDL_f1 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 694176 , 79 , 693392 , 0 , kHz30 , 273 ), 100 , kHz30 , 16 , 1 , 44 , 18 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f1 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 0 , kHz30 , 273 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f2 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 704928 , 79 , 704214 , 12 , kHz30 , 273 ), 100 , kHz30 , 18 , 1 , 38 , 6 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f2 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 12 , kHz30 , 273 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f3 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 717216 , 79 , 715038 , 24 , kHz30 , 273 ), 100 , kHz30 , 18 , 1 , 160 , 54 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f3 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 24 , kHz30 , 273 , v_P_Max), 100 ); v_NR_Frequencyf1tof4.FrequencyInfoDL_f4 := cs_SS_FrequencyInfoDL(cs_FrequencyInfoDL( 727968 , 79 , 714628 , 504 , kHz30 , 273 ), 100 , kHz30 , 20 , 1 , 1090 , 37 , 4 ); v_NR_Frequencyf1tof4.FrequencyInfoUL_f4 := cs_SS_FrequencyInfoUL(cs_FrequencyInfoUL( -, -, 504 , kHz30 , 273 , v_P_Max), 100 ); } }… }